# 3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315246

# Xiamen, China, October 09 – October 13, 2023

Agenda item:			5.26.1.3

Source:	MediaTek Inc.

Title:	Discussion on LS on PUSCH DMRS bundling for NR NTN coverage enhancement

Document for:	Discussion

# Introduction

One LS is received from RAN1 regarding PUSCH DMRS bundling for NR NTN coverage [1]. And the working assumption was reached in RAN1 as blow.

Working assumption

For NTN-specific PUSCH DMRS bundling, to satisfy the phase difference limit without causing phase discontinuity, it is assumed that pre-compensation to keep phase rotation due to timing drift within the phase difference limit can be performed at UE side.

- UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit.

  - FFS: how to determine the actual TDW

- FFS: specification impact

Send an LS to RAN4

In RAN4#108, the following agreement is reached.

- Agreement:

- RAN4 investigate the feasibility of an NTN UE to meet the DMRS requirement in the new test condition where DL time would be changing for non-GEO satellite.

  - Taking existing requirements specified for the maximum allowable phase difference for DMRS bundling captured in Table 6.4.2.5-1 of TS38.101-1 as starting point

    - FFS any update on the side conditions needed or not which also need to be compliant with RAN1 design

- From RRM requirements perspective to support NTN-specific PUSCH DMRS bundling:

    - Option 1: update the applicability of the timing requirements such that the requirements apply only for the first transmission in the TDW.

    - Other options not precluded.

In this contribution, the corresponding RAN4 requirement regarding the TA pre-compensation is discussed.

# Discussion

In RAN1 working assumption for NTN-specific PUSCH DMRS bundling, it requires that UE shall not perform TA pre-compensation during TDW, in order to keep phase difference within a limit, as in Table 6.4.2.5-1 of TS38.101-1 as starting point.

In R17 NR NTN, the TA pre-compensation was extensively discussed in the UE transmit timing requirement, as specified in 7.1C in 38.133, Thus, in our view, RAN4 can update the applicability of the timing requirements, for NTN-specific PUSCH DMRS bundling. Option 1 seems a reasonable approach.

Proposal 1: On RRM requirement, update the applicability of the timing requirements such that the requirements apply only for the first transmission in the TDW. (Option 1)

# Summary

The observations and proposals in this contribution are summarized:

Proposal 1: On RRM requirement, update the applicability of the timing requirements such that the requirements apply only for the first transmission in the TDW. (Option 1)

# Reference

- R1-2304094, LS on PUSCH DMRS bundling for NR NTN coverage enhancement, RAN1#112bis-e Apr. 2023.

- R4-2313864, WF for NTN general part, RAN4#108, Aug. 2023.