3GPP TSG-RAN WG4 Meeting # 108bis		R4-2315258

Xiamen, China, 9 – 13 October 2023

Agenda Item:	4.16.2

Source: 	Nokia, Bell Mobility

Title: 	TP to TR 37.829: System level simulation methodology and assumptions for coexistence study on 31dBm UE Power Class for NR Band n25

Document for:	Approval

# 1.	Introduction

The revised WID on High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands was approved at TSG RAN #101 [1]. One of the objectives of this work item is to define power class 1 (31 dBm) requirements for NR Band n25. Hence system level simulations need to be performed for the coexistence study similar to those performed on 31 dBm UE Power Class for other NR bands and recorded in the annexes of TR 37.829 [2].

This contribution proposes the system level simulation methodology and assumptions for coexistence study on 31 dBm UE Power Class for NR Band n25 and provides a text proposal to record the simulation methodology and assumptions into a new annex in TR 37.829.

# 2.	Discussion

The simulation methodology and assumptions are based on Annex A in TR 37.829 on 31 dBm UE Power Class for LTE Band 41 and NR Band n41, with modifications on propagation model, inter-site distance and uplink power control parameter CLx-ile to better represent operator deployment scenario for NR Band n25 operation.

#### 2.1	Macro cell Propagation model - Urban and Suburban Areas

Macro cell propagation model for urban area is applicable for scenarios in urban and suburban areas outside the high rise core where the buildings are of nearly uniform height [3]:

L=40⋅(1-4⋅10-3⋅Dhb)⋅log10(R)-18⋅log10(Dhb)+21⋅log10(f)+80dB	(1)

where:

R is the base station-UE separation in kilometres

f is the carrier frequency in MHz

Dhb is the base station antenna height in metres, measured from the average rooftop level

Considering a carrier frequency of 1.9 GHz and a base station antenna height of 15 m above average rooftop level, the propagation model is given by the following equation:

L=127.7+37.6log10(R)	(2)

where:

R is the base station-UE separation in kilometres

#### 2.2	Macro cell Propagation model - Rural Area

For rural area, the Hata model in [3] is not applicable for a carrier frequency above 1.5 GHz, while the modified Hata model can be used [4]:

Case 1:		d  0.04 km

L = 32.4 + 20 log(f) + 10 log(d2 + (Hb - Hm)2/106)	(3)

where:

d is the base station-UE separation in kilometres

f is the carrier frequency in MHz

Hm is the mobile antenna height in metres

Hb is the base station antenna height in metres

Case 2:		d  0.1 km

L = L(urban) - 4.78 logmin{max{150, f}, 2000}2 + 18.33 logmin{max{150, f}, 2000} - 40.94	(4)

where:

L (urban)= 46.3 + 33.9 log(2000) + 10 log(f/2000) - 13.82 log(max{30,Hb}) +44.9 - 6.55 log(max{30,Hb}) log(d)α - a(Hm) - b(Hb)
a(Hm) = (1.1 log(f) - 0.7) min{10, Hm} - (1.56 log(f) - 0.8) + max{0, 20 log(Hm/10)}

b(Hb) = min{0, 20 log(Hb/30)}

Note that for short range devices in the case of low base station antenna height, Hb,

b(Hb) = min{0, 20 log(Hb/30)} is replaced by:

b(Hb) = (1.1 log(f) - 0.7) min{10, Hb} - (1.56 log(f) - 0.8) + max{0, 20 log(Hb/10)}

α  1                                                                                                              for d≤20 km1 + 0.14 + 1.87 × 10-4 f + 1.07 × 10-3 Hb log d200.8    for 20 km<d≤100 km

Case 3:		0.04 km  d  0.1 km

L = L(0.04) + log(d) - log(0.04)log(0.1) - log(0.04)  L(0.1) - L(0.04)	(5)

When L is below the free space attenuation for the same distance, the free space attenuation should be used instead.

Considering a carrier frequency of 1.9 GHz, a base station antenna height of 45 meters above ground, a mobile antenna height of 1.5 meters above ground, and a base station-UE separation smaller than 20 km, equations (3) and (4) can be simplified, respectively, as equations (6) and (7) below:

Case 1:		d  0.04 km

L = 98.0 + 10 log(d2 + 0.00189225)	(6)

Case 2:		d  0.1 km

L = 102.9 + 34.1 log(d)	(7)

Note that the propagation loss using equation (7) is less than that using equation (6) for a base station-UE separation smaller than 0.45 km and the propagation loss in equation (6) should be chosen. Hence equation (5) need not be used and the propagation model can be given by the following equations:

Case 1:		d  0.45 km

L = 98.0 + 10 log(d2 + 0.00189225)	(8)

Case 2:		d  0.45 km

L = 102.9 + 34.1 log(d)	(9)

where:

d is the base station-UE separation in kilometres

#### 2.3	Inter-site distance

The inter-site distances considered in the present document are provided in Table 2.3-1 below.

Table 2.3-1: Inter-site distances and Propagation model

| Environment | ISD (km) | ISD (miles) |
| --- | --- | --- |
| Urban | 2 | 1.24 |
| Suburban | 5.1 | 3.17 |
| Rural | 9.3 | 5.78 |

#### 2.4	Power control modelling

In TR 36.942, the following power control equation is used for the uplink coexistence simulations:

(10)

where Pmax is the maximum transmit power, Rmin is the minimum power reduction ratio to prevent UEs with good channels to transmit at very low power level, CL is the coupling loss defined as max{path loss-G_Tx-G_Rx, MCL}, where path loss is propagation loss plus shadowfading, G_TX is the transmitter antenna gain in the direction of the receiver, G_RX is the receiver antenna gain in the direction of the transmitter and CLx-ile is the x-percentile CL value. With this power control equation, the x percent of UEs that have the highest coupling loss will transmit at Pmax. Finally, 0<<=1 is the balancing factor for UEs with bad channel and UEs with good channel.

The parameter sets for power control are specified in TR 36.942 (copied in Table 2.4-1 below).

Table 2.4-1: Power control algorithm parameter

| Parameter set | Gamma | CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 109 | 110 | 112 | 115 |
| Set 2 | 0,8 | TBD | TBD | 129 | 133 |

However, the power control parameters were specified for carrier frequency of 2 GHz and 500 m cell range. The path loss model is based on the equation below [3]:

(11)

where:

R is the base station-UE separation in kilometres

Based on equations (2), (9) and (11), the power control parameters can be modified to account for different propagation model, carrier frequency, BS antenna height and cell range. For example, in Table 2.4-1, CLx-ile = 109 dB for Set 1 and 20 MHz bandwidth, which corresponds to R = 0.31 km according to equation (11) assuming total antenna gain of 0 dBi. Similarly, CLx-ile = 112 dB for Set 1 and 10 MHz bandwidth corresponds to R = 0.37 km and CLx-ile = 129 dB for Set 2 and 10 MHz bandwidth corresponds to R = 1.06 km. For 2 km inter-site distance and 1.9 GHz carrier frequency, using equation (2), CLx-ile = 127.7 + 37.6*log10(2*2/3/0.5*0.31) = 125 dB for Set 1 and 20 MHz bandwidth. For 9.3 km inter-site distance and 1.9 GHz carrier frequency, using equation (9), CLx-ile = 102.9 + 34.1*log10(9.3*2/3/0.5*1.06) = 141 dB for Set 2 and 10 MHz bandwidth. Similarly, the modified power control parameters for other cases for 23 dBm UE can be calculated as shown in Table 2.4-2 below.

Table 2.4-2 CLx-ile parameters for +23 dBm UE

(a) CLx-ile parameters for +23 dBm UE using 2 km inter-site distance and 1.9 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 125 | 128 |
| Set 2 | 0,8 | 141 | 145 |

(b) CLx-ile parameters for +23 dBm UE using 5.1 km inter-site distance and 1.9 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 140 | 143 |
| Set 2 | 0,8 | 156 | 160 |

(c) CLx-ile parameters for +23 dBm UE using 9.3 km inter-site distance and 1.9 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 123 | 126 |
| Set 2 | 0,8 | 137 | 141 |

Following the approach adopted for the coexistence study to develop the ACLR requirements for 26 dBm HPUE in Band 41, as described in clause 5.8.1.2 of TR 36.886 [5], the uplink power control parameter CLx-ile for 31 dBm HPUE is set to 8 and 10 (8/0.8) dB higher than those for 23 dBm UE for the uplink power control parameter Gamma of 1 and 0.8, respectively, considering the 8 dB higher maximum power of the 31 dBm HPUE. This represents the case where the 31 dBm UE is used to enhance the UL coverage rather than capacity. Consequently, the modified power control parameters for other cases for 23 dBm UE can be calculated as shown in Table 2.4-3 below.

Table 2.4-3 CLx-ile parameters for +31 dBm UE

(a) CLx-ile parameters for +31 dBm UE using 2 km inter-site distance and 1.9 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 133 | 136 |
| Set 2 | 0,8 | 151 | 155 |

(b) CLx-ile parameters for +31 dBm UE using 5.1 km inter-site distance and 1.9 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 148 | 151 |
| Set 2 | 0,8 | 166 | 170 |

(c) CLx-ile parameters for +31 dBm UE using 9.3 km inter-site distance and 1.9 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 131 | 134 |
| Set 2 | 0,8 | 147 | 151 |

# 3.	Conclusion

This contribution proposes the system level simulation methodology and assumptions for coexistence study on 31 dBm UE Power Class for NR Band n25. It is proposed to approve the text proposal below into a new annex in TR 37.829.

# 4.	Text proposal

<New annex>

# Annex	C:

# Coexistence studies for 31 dBm UE Power Class for NR Band n25

# C.1	Simulation assumptions

#### C.1.1	Macro cell Propagation model

##### C.1.1.1	Macro cell Propagation model - Urban and Suburban Areas

The propagation model is a derived from TR 36.942 [2].

Considering a carrier frequency of 1.9 GHz and a base station antenna height of 15 m above average rooftop level, the propagation model is given by the following equation:

L=127.7+37.6log10(R)

where:

R is the base station-UE separation in kilometres

##### C.1.1.2 	Macro cell Propagation model - Rural Area

The propagation model is a derived from TR 36.942.

For rural area, the Hata model is not applicable for a carrier frequency above 1.5 GHz, while the modified Hata model can be used:

Case 1:		d  0.45 km

L = 98.0 + 10 log(d2 + 0.00189225)

Case 2:		d  0.45 km

L = 102.9 + 34.1 log(d)

where: d is the base station-UE separation in kilometres

#### C.1.2	Power Control Simulation Parameters

Table C.1.2-1 CLx-ile parameters for +23 dBm UE

(a) CLx-ile parameters for +23 dBm UE using 2 km inter-site distance and 1.9 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 125 | 128 |
| Set 2 | 0,8 | 141 | 145 |

(b) CLx-ile parameters for +23 dBm UE using 5.1 km inter-site distance and 1.9 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 140 | 143 |
| Set 2 | 0,8 | 156 | 160 |

(c) CLx-ile parameters for +23 dBm UE using 9.3 km inter-site distance and 1.9 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 123 | 126 |
| Set 2 | 0,8 | 137 | 141 |

Table C.1.2-2 CLx-ile power control algorithm parameters for +31 dBm UE

(a) CLx-ile parameters for +31 dBm UE using 2 km inter-site distance and 1.9 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 133 | 136 |
| Set 2 | 0,8 | 151 | 155 |

(b) CLx-ile parameters for +31 dBm UE using 5.1 km inter-site distance and 1.9 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 148 | 151 |
| Set 2 | 0,8 | 166 | 170 |

(c) CLx-ile parameters for +31 dBm UE using 9.3 km inter-site distance and 1.9 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 131 | 134 |
| Set 2 | 0,8 | 147 | 151 |

#### C.1.3	Cell Layout

Base stations with 3 sectors per site are placed on a hexagonal grid with distance of 3*R, where R is the cell radius (see Figure C.1.3-1), with wrap around. The number of sites shall be equal to or higher than 19. Uncoordinated macro cellular deployment is assumed, where interfering UE may be at cell edge of the serving base station but close to the victim base station (hence transmitting with highest power and causing highest interference).

Figure C.1.3-1: Uncoordinated macro cellular deployment

The inter-site distances considered in the present document are provided in Table C.1.3-1 below.

Table C.1.3-1: Inter-site distances and Propagation model

| Environment | ISD (km) | ISD (miles) |
| --- | --- | --- |
| Urban | 2 | 1.24 |
| Suburban | 5.1 | 3.17 |
| Rural | 9.3 | 5.78 |

#### C.1.4	Other Simulation Assumptions

Other simulation assumptions are summarized in Table C.1.4-1 below:

Table C.1.4-1: Simulation parameters for Band 25 system

(a) With 23 dBm UE

|  | Base Station | UE |
| --- | --- | --- |
| Carrier frequency | 1900 MHz |  |
| Channel bandwidth | 20 MHz, 10 MHz |  |
| Inter-site distance | Use Table C.1.3-1 |  |
| Cell layout | Wrap-around 19 tri-sector cells, uncoordinated |  |
| Frequency reuse | 1x3x1 |  |
| Lognormal fading | 10 dB |  |
| Shadowing correlation | Between cells: 0.5, between sites: 1.0 |  |
| MCL (including antenna gain) | 70 dB (urban and suburban areas) 80 dB (rural area) |  |
| Antenna gain and horizontal antenna pattern | 17 dBi, θ3dB = 65 degrees, Am = 20 dB | Omni-directional antenna with -3.5 dBi. |
| Noise figure | 5 dB | 9 dB |
| Transmit power | 46 dBm | 23 dBm |
| Antenna height | 45 m | 1.5 m |
| ACLR | 45 dB | Use Table 5.2 in TR 36.942 ACLR1: 30+X, ACLR2: 43+X Where X is 1 dB |
| ACS | 45 dB | 27 dB (20 MHz), 33 dB (10 MHz) |

(b) With 31 dBm UE

|  | Base Station | HPUE |
| --- | --- | --- |
| Carrier frequency | 1900 MHz |  |
| Channel bandwidth | 20 MHz, 10 MHz |  |
| Inter-site distance | Use Table C.1.3-1 |  |
| Cell layout | Wrap-around 19 tri-sector cells, uncoordinated |  |
| Frequency reuse | 1x3x1 |  |
| Lognormal fading | 10 dB |  |
| Shadowing correlation | Between cells: 0.5, between sites: 1.0 |  |
| MCL (including antenna gain) | 70 dB (urban and suburban areas) 80 dB (rural area) |  |
| Antenna gain and horizontal antenna pattern | 17 dBi, θ3dB = 65 degrees, Am = 20 dB | Omni-directional antenna with -3.5 dBi. |
| Noise figure | 5 dB | 9 dB |
| Transmit power | 46 dBm | 31 dBm |
| Antenna height | 45 m | 1.5 m |
| ACLR | 45 dB | Use Table 5.2 in TR 36.942 ACLR1: 30+X, ACLR2: 43+X Where X is 1 dB |
| ACS | 45 dB | 27 dB (20 MHz), 33 dB (10 MHz) |

Simulations should assume the worst case of 100 % HPUEs in the scenarios with HPUEs.

#### C.1.5	Simulation Procedure

For the co-existence study, the following procedure shall be performed:

1)	Run the Band 25 UL to UL coexistence study, assuming parameters of both systems are according to Table C.1.4-1 (a). Power control parameters in Table C.1.2-1 are used. This corresponds to the coexistence of two commercial networks operating in adjacent channel and with similar deployment parameters. This is used as the reference. Band 25 victim system performance degradation results in this scenario are used as the baseline. Provide a CDF plot of UE transmit power.

2)	Run the Band 25 UL to UL coexistence study, assuming +31 dBm power class UE is deployed in Band 25 interfering system only and obtain the victim system performance degradation results. The simulation parameters in Tables C.1.4-1 (a) and C.1.4-1 (b) are used for the victim and interfering system, respectively. And the power control parameters in Tables C.1.2-1 and C.1.2-2 are used for the victim and interfering system, respectively. Provide a CDF plot of UE transmit power.

3)	Compare the Band 25 victim system performance degradation obtaining in steps 1) and 2), choose the 31 dBm UE ACLR value so that the victim system performance degradation due to 31 dBm UE in 2) is the same as 1).

<End of change>

# References

[1]	RP-231615, “Revised WID: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands”, Nokia.

[2]	R4-2307872, “TR 37.829 v0.6.0”, Nokia.

[3]	3GPP TR 36.942 v13.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”.

[4]	Report ITU-R SM.2028-1, “Monte Carlo simulation methodology for the use in sharing and compatibility studies between different radio services or systems”.

[5]	3GPP TR 36.886 v14.1.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Band 41 High Power UE (HPUE)”.