3GPP TSG-RAN WG4 Meeting #108bis	R4-2315299
Xiamen, China, 9 – 13 Oct, 2023

Title: 	MSD analysis of PC2 and PC1.5 band combinations supporting 3Tx

Source: 	Huawei, HiSilicon

Agenda Item:	5.5.3.2

Document for:	Approval

# 1	Introduction

A series of PC2 and PC1.5 band combinations are approved within the latest WID[1]. In the last WF [2] it was requested to check the status of these band combinations. In this tdoc we are going to check their status and propose the MSD value for the bands that might need them. It should be mentioned that based on a previous agreement if the MSD value for a PC2 (or PC1.5) band combination with 2Tx exists, the same MSD would be valid for the PC2 (or PC1.5) band combination with 3Tx.

# 2	PC2 3Tx MSD analysis

Table 2.1 summarizes the MSD status of these PC2 band combinations. For the band combinations with MSD status highlighted in green, either the CA or DC 2Tx PC2 counter parts already exist in the spec. for other cases the MSD is calculated below.  For the FDD (1Tx)+TDD (2Tx) band combinations, the architecture considered for the MSD analysis is depicted in Figure 2.1.

Table 2.1: Band combinations for 3Tx with CA/EN-DC power class 2

| Band combination | UL configuration | Max Power capabilities | MSD Status |
| --- | --- | --- | --- |
| CA_n28A-n41A | CA_n28A-n41A | PC3@n28 1Tx; PC2@n41 2Tx | No MSD is needed |
| CA_n28A-n78A | CA_n28A-n78A | PC3@n28 1Tx; PC3/2@n78 2Tx | IM5 on n8, H4 on n78, adapt from PC2 2Tx DC_28A_n78A |
| CA_n8A-n78A | CA_n8A-n78A | PC3@n8 1Tx; PC2@n78 2Tx | IM4 on n8, H4 on n78, adapt from PC2 2Tx CA_n8A-n78A |
| CA_n41A-n71A | CA_n41A-n71A | PC3@n71 1Tx; PC2@n41 2Tx | IM4 on n71, PC2 3Tx needs to be provided; H4 on n41 Not needed as it is an even order harmonic |
| CA_n41A-n77A | CA_n41A-n77A | PC2@n41 1Tx; PC2@n77 2Tx; PC2@n41 2Tx; PC2@n77 1Tx; | IM9 on n77, cross band isolation adapt from PC2 2Tx CA_n41A-n77A |
| CA_n26A-n78A | CA_n26A-n78A | PC3@n26 1Tx; PC2@n78 2Tx | IM4 on n26, PC2 3Tx needs to be provided |
| DC_3A_n78A | DC_3A_n78A | PC3@n3 1Tx; PC2@n78 2Tx | IM2, IM4 on n3, adapt from PC2 2Tx CA_n3-n78 |
| DC_40A_n78A | DC_40A_n78A | PC3@n40 1Tx; PC3/2@n78 2Tx | Cross band leakage PC2 3Tx needs to be provided; 3H of n40 could mix with 2H of n78, adapt from PC2 2Tx CA_n3-n77 |

Observation 1: CA_n41A-n71A PC2 is missing in the last WF.

Figure 2.1 FDD-TDD band combination architecture

## CA_n41A-n71A

Based on the MSD analysis, |3fn71-fn41| could fall on n71 Rx, so some MSD would be needed for n71 Rx. Table 2.2 present the parameters of the analysis

Table 2.2 CA_n41A-n71A parameters

| Antenna ISO | 10 | dB |
| --- | --- | --- |
| PCB isolation Paout-Pain | 60 | dB |
| PCB isolation Paout Div-Pain main | 65 | dB |
| Diplexer ISO | 15 | dB |
| n71 TX SAW/duplexer rejection at n41 Tx | 45 | dB |
| n71 RX duplexer rejection at n41 Tx | 40 | dB |
| n71 SAW/Duplexer TX rejection at n71 RX band | 55 | dB |
| filter n41 TX rejection at n71 RX band | 50 | dB |
| n41 Tx attenuation at n71 RX | 34 | dB |
| n41 Tx attenuation at n71 TX | 34 | dB |
| Diplexer IL | 0.5 | dB |
| Antenna switch IL | 1 | dB |
| TX/RX switch IL | 0.5 | dB |
| n71 Dup IL | 4 | dB |
| n41 RX SAW filter IL | 3 | dB |
| Pout n71 | 23 | dBm |
| Pout n41 | 20 | dBm |
| Pout n41 Div | 20 | dBm |
| REFSENS n71 | -97.2 | dBm |

Table 2.3: IMD4 level to n71 Rx main path (PC2)

| Main Path | received interference, IMD4 (dBm) |
| --- | --- |
| Antenna switch | -74.73 |
| Diplexer (H-H or L-L) | -68.79 |
| n71 SAW/Duplexer | -97.47 |
| n71 PA Forward mixing | -68.48 |
| n41 PA Forward mixing | -155.53 |
| n71 PA Reverse mixing | -102.18 |
| n41 PA Reverse mixing | -215.53 |
| Total | -65.12 |
| B71 LNA via filters | -95.35 |
| B71 LNA via PCB coupling | -117.48 |
| Total main IMD | -65.11 |

Table 2.4: IMD4 level to n71 Rx diversity path

| Diversity Path | received interference, IMD4 (dBm) |
| --- | --- |
| From main path | -75.11 |
| Diplexer | -105.59 |
| Antenna switch | -107.59 |
| n71 SAW Filter | -103.29 |
| n41 PA div Forward mixing | -172.03 |
| n71 LNA via antenna ISO | -153.35 |
| n71 LNA via PCB coupling | -125.48 |
| Total div IMD | -75.10 |

Table 2.5: CA_n41A-n71A MSD for n71 Rx due to IMD4 (PC2)

| MSD calculation parameters | received interference, IMD4 (dB) |
| --- | --- |
| MSD | 25.98 |
| SNR | -21.70 |
| MSD at SNR=-1 | 20.70 |

Proposal 1: Consider an MSD= 20.70 dB for CA_n41A-n71A with PC2 3Tx when IM4 falls on n71 Rx.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n71 | n41 | 2614 | 5 | 25 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 20.70 | FDD | IMD4 |

## CA_n26A-n78A

Based on the MSD analysis, |3fn26-fn78| could fall on n26 Rx, so some MSD would be needed for n26 Rx. Table 2.6 present the parameters of the analysis

Table 2.6 CA_n26A-n78A parameters

| Antenna ISO | 10 | dB |
| --- | --- | --- |
| PCB isolation Paout-Pain | 60 | dB |
| PCB isolation Paout Div-Pain main | 65 | dB |
| Diplexer ISO | 15 | dB |
| n26 TX SAW/duplexer rejection at n78 Tx | 35 | dB |
| n26 RX duplexer rejection at n78 Tx | 58 | dB |
| n26 SAW/Duplexer TX rejection at n26 RX band | 60 | dB |
| filter n78 TX rejection at n26 RX band | 47 | dB |
| n78 Tx attenuation at n26 RX | 47 | dB |
| n78 Tx attenuation at n26 TX | 47 | dB |
| Diplexer IL | 0.5 | dB |
| Antenna switch IL | 1 | dB |
| TX/RX switch IL | 0.5 | dB |
| n26 Dup IL | 4 | dB |
| n78 RX SAW filter IL | 3 | dB |
| Pout n26 | 23 | dBm |
| Pout n78 | 20 | dBm |
| Pout n78 Div | 20 | dBm |
| REFSENS n26 | -97.5 | dBm |

Table 2.7: IMD4 level to n26 Rx main path (PC2)

| Main Path | received interference, IMD4 (dBm) |
| --- | --- |
| Antenna switch | -74.73 |
| Diplexer (H-H or L-L) | -68.79 |
| n26 SAW/Duplexer | -87.47 |
| n26 PA Forward mixing | -74.42 |
| n78 PA Forward mixing | -171.34 |
| n26 PA Reverse mixing | -121.11 |
| n78 PA Reverse mixing | -270.34 |
| Total | -66.91 |
| n26 LNA via filters | -129.29 |
| n26 LNA via PCB coupling | -118.42 |
| Total main IMD | -66.91 |

Table 2.8: IMD4 level to n26 Rx diversity path

| Diversity Path | received interference, IMD4 (dBm) |
| --- | --- |
| From main path | -76.91 |
| Diplexer | -105.59 |
| Antenna switch | -107.59 |
| n26 Filter | -103.29 |
| n78 PA div Forward mixing | -187.84 |
| n26 LNA via antenna ISO | -159.29 |
| n26 LNA via PCB coupling | -126.42 |
| Total div IMD | -76.89 |

Table 2.9: CA_ n26A-n78A MSD for n26 Rx due to IMD4 (PC2)

| MSD calculation parameters | received interference, IMD4 (dB) |
| --- | --- |
| MSD | 24.20 |
| SNR | -20.22 |
| MSD at SNR=-1 | 19.22 |

Proposal 2: Consider an MSD= 19.2 dB for CA_ n26A-n78A with PC2 3Tx when IM4 falls on n26 Rx.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n26-n78 | n26 | 836.5 | 5 | 25 | 881.5 | 19.22 | FDD | IMD4 |
|  | n78 | 3391 | 10 | 50 | 3391 | N/A | TDD | N/A |

## DC_40A_n78A

Since the 3rd harmonic of n40 could overlap with 2nd harmonic of n78 and induce degradation of the receiver of the two bands, one can reuse the MSD values proposed for the case of n40 and n77 that is available in 38.101-1 Table 7.3A.4-4.

Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1 UL band; DL band; UL BW; SCS of UL band; UL RB Allocation; DL BW; MSD; UL/DL fc condition; UL/DL harmonic order. (MHz); (kHz); LCRB; (MHz); (dB). n77; n40; 20; 30; 50 (RBstart=0); 10; 13.2; NOTE 3; UL2/DL3. n77; n40; 20; 30; 50 (RBstart=0); 100; 4.4; NOTE 3; UL2/DL3.

\

Proposal 3:

Table 2.11 Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 3Tx EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n78 | 40 | 16.2 | 13.2 | 11.4 | 10.2 |  |  |  |  |  |  |  |

Based on the MSD analysis, the cross band leakage can happen in such a TDD-TDD band combination. For the sake of comparison the same architecture as R4-2312775 is used. It should be mentioned that since in this architecture, every RF chain has an antenna (with 10 dB antenna isolation), its interference is higher than the case where a diplexer (with 15 dB isolation), is used (right before the common antenna on each main and diversity path). Therefore the MSD should be worse than a common antenna architecture. The details of the MSD analysis due to cross band leakage is given in Table 2.11. Here the calculations for the case of 5MHz @B40 is presented. The summary of all CBWs is given in the proposal 4.

Figure 2.2 TDD-TDD architecture similar to R4-2312775

Table 2.11 DC_40A_n78A MSD due to cross band leakage (PC2)

| Item | Unit | parameter | Main | Diversity |
| --- | --- | --- | --- | --- |
| n78 PA output noise power @ B40 freq | dBm/Hz | （100MHz UL） | -122.00 | -122.00 |
| Antenna isolation Primary <-> Diversity | dB | 10.00 |  |  |
| B40 Rx CBW | MHz | 5.00 |  |  |
| n78 PA output noise power on B40 Rx CBW | dBm |  | -55.01 | -55.01 |
| n78 Tx filter Att @ B40 Rx | dB | 25.00 |  |  |
| PCB isolation | dB | 60.00 |  |  |
| n78 PA noise power at B40 LNA | dBm |  | -90.01 | -90.01 |
| n78 PA noise power at B40 LNA via PCB | dBm |  | -115.01 | -115.01 |
| front end loss | dB | 4.00 |  |  |
| n78 PA noise power at Antenna port | dBm |  | -86.00 | -86.00 |
| B40 REFSEN @ 5MHz | dBm | -100.00 |  |  |
| noise figure dB | dB | 9.00 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | dBm/Hz | -165.00 |  |  |
| Thermal noise, dBm | dBm |  | -98.01 | -98.01 |
| Total noise level at ANT port, dBm | dBm |  | -85.73 | -85.73 |
| SNR requirement for QPSK | dB | -1.00 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) | dBm |  | -86.73 | -86.73 |
| combined REFSENS(5MHz BW), dBm | dBm |  | -89.74 |  |
| Margin | dB | 4.00 |  |  |
| combined REFSENS(5MHz BW)after considering Margin, dBm | dBm |  | -85.74 |  |
| MSD | dB |  | 14.26 |  |

Proposal 4: Consider an MSD= 14.26 dB for DC_40A_n78A MSD due to cross band leakage

Table 2.12 Reference sensitivity exceptions (MSD) due to cross band isolation for PC2 3TX EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n78 | 401 | 14.26 | 14.27 | 14.23 | 14.28 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation 1: The MSD due to cross band isolations is stronger than the MSD due to harmonic mixing which is due to the fact that a different UE architecture is used.

# 3	PC1.5 3Tx MSD analysis

Table 3.1 summarizes the MSD status of these PC1.5 band combinations. For the band combinations with MSD status highlighted in green, the CA or DC 2Tx PC3 counter parts already exist in the spec. for other cases, the MSD is calculated below.  For the FDD+TDD band combinations, the architecture considered for the MSD analysis is depicted in Figure 2.1.

Table 3.1: Band combinations for 3Tx with CA/EN-DC power class 1.5

| Band combination | UL configuration | Max Power capabilities | MSD Status |
| --- | --- | --- | --- |
| CA_n41A-n71A | CA_n41A-n71A | PC3@n71 1Tx; PC1.5@n41 2Tx | IM4 on n71, PC1.5 3Tx needs to be provided; H4 on n41 Not needed as it is an even order harmonic |
| CA_n5A-n77A | CA_n5A-n77A | PC3@n5 1Tx; PC1.5@n77 2Tx | IM2 and IM4 on n5, PC1.5 3Tx needs to be provided; |
| CA_n25A-n77A | CA_n25A-n77A | PC3@n25 1Tx; PC1.5@n77 2Tx | IM2 and IM4 on n25, PC1.5 3Tx needs to be provided; n77 UL could overlap with H2 of n25. adapt from PC2 2Tx CA_n25A-n77A |

Observation 2: for CA_n25A-n77A, IM2 on n25 was missing from the WF

## CA_n41A-n71A

Based on the MSD analysis, |3fn71-fn41| could fall on n71 Rx, so some MSD would be needed for n71 Rx. Table 3.2 presents the parameters of the analysis

Table 3.2 CA_n41A-n71A parameters

| Antenna ISO | 10 | dB |
| --- | --- | --- |
| PCB isolation Paout-Pain | 60 | dB |
| PCB isolation Paout Div-Pain main | 65 | dB |
| Diplexer ISO | 15 | dB |
| n71 TX SAW/duplexer rejection at n41 Tx | 45 | dB |
| n71 RX duplexer rejection at n41 Tx | 40 | dB |
| n71 SAW/Duplexer TX rejection at n71 RX band | 55 | dB |
| filter n41 TX rejection at n71 RX band | 50 | dB |
| n41 Tx attenuation at n71 RX | 34 | dB |
| n41 Tx attenuation at n71 TX | 34 | dB |
| Diplexer IL | 0.5 | dB |
| Antenna switch IL | 1 | dB |
| TX/RX switch IL | 0.5 | dB |
| n71 Dup IL | 4 | dB |
| n41 RX SAW filter IL | 3 | dB |
| Pout n71 | 23 | dBm |
| Pout n41 | 24.8 | dBm |
| Pout n41 Div | 24.8 | dBm |
| REFSENS n71 | -97.2 | dBm |

Table 3.3: IMD4 level to n71 Rx main path (PC1.5)

| Main Path | received interference, IMD4 (dBm) |
| --- | --- |
| Antenna switch | -69.93 |
| Diplexer (H-H or L-L) | -63.99 |
| n71 SAW/Duplexer | -92.67 |
| n71 PA Forward mixing | -63.68 |
| n41 PA Forward mixing | -150.73 |
| n71 PA Reverse mixing | -97.38 |
| n41 PA Reverse mixing | -210.73 |
| Total | -60.32 |
| B71 LNA via filters | -90.55 |
| B71 LNA via PCB coupling | -112.68 |
| Total main IMD | -60.31 |

Table 3.4: IMD4 level to n71 Rx diversity path (PC1.5)

| Diversity Path | received interference, IMD4 (dBm) |
| --- | --- |
| From main path | -70.31 |
| Diplexer | -100.79 |
| Antenna switch | -102.79 |
| n71 SAW Filter | -98.49 |
| n41 PA div Forward mixing | -167.23 |
| n71 LNA via antenna ISO | -148.55 |
| n71 LNA via PCB coupling | -120.68 |
| Total div IMD | -70.30 |

Table 3.5: CA_n41A-n71A MSD for n71 Rx due to IMD4 (PC1.5)

| MSD calculation parameters | received interference, IMD4 (dB) |
| --- | --- |
| MSD | 30.77 |
| SNR | -26.49 |
| MSD at SNR=-1 | 25.49 |

Proposal 5: Consider an MSD= 25.5 dB for CA_n41A-n71A with PC1.5 3Tx when IM4 falls on n71 Rx.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n71 | n41 | 2614 | 5 | 25 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 25.5 | FDD | IMD4 |

## CA_n5A-n77A

Based on the MSD analysis, |3fn5-fn77| could fall on n71 Rx, so some MSD would be needed for n5 Rx. Table 3.6 presents the parameters of the analysis

Table 3.6 CA_n5A-n77A parameters

| Antenna ISO | 10 | dB |
| --- | --- | --- |
| PCB isolation Paout-Pain | 60 | dB |
| PCB isolation Paout Div-Pain main | 65 | dB |
| Diplexer ISO | 15 | dB |
| n5 TX SAW/duplexer rejection at n77 Tx | 18 | dB |
| n5 RX duplexer rejection at n77 Tx | 50 | dB |
| n5 SAW/Duplexer TX rejection at n5 RX band | 64 | dB |
| filter n77 TX rejection at n5 RX band | 47 | dB |
| n77 Tx attenuation at n5 RX | 47 | dB |
| n77 Tx attenuation at n5 TX | 47 | dB |
| Diplexer IL | 0.5 | dB |
| Antenna switch IL | 1 | dB |
| TX/RX switch IL | 0.5 | dB |
| n5 Dup IL | 4 | dB |
| n77 RX SAW filter IL | 3 | dB |
| Pout n5 | 23 | dBm |
| Pout n77 | 24.8 | dBm |
| Pout n77 Div | 24.8 | dBm |
| REFSENS n5 | -98 | dBm |

Table 3.7: IMD4 level to n5 Rx main path (PC1.5)

| Main Path | received interference, IMD2 (dBm) | received interference, IMD4 (dBm) |
| --- | --- | --- |
| Antenna switch | -59.93 | -69.93 |
| Diplexer (H-H or L-L) | -59.99 | -63.99 |
| n5 SAW/Duplexer | -51.67 | -65.67 |
| n5 PA Forward mixing | -75.04 | -74.04 |
| n77 PA Forward mixing | -72.73 | -167.81 |
| n5 PA Reverse mixing | -115.74 | -120.74 |
| n77 PA Reverse mixing | -110.74 | -266.81 |
| Total | -50.50 | -60.91 |
| B71 LNA via filters | -80.91 | -128.91 |
| B71 LNA via PCB coupling | -74.04 | -114.04 |
| Total main IMD | -50.47 | -60.91 |

Table 3.8: IMD4 level to n5 Rx diversity path (PC1.5)

| Diversity Path | received interference, IMD2 (dBm) | received interference, IMD4 (dBm) |
| --- | --- | --- |
| From main path | -60.47 | -70.91 |
| Diplexer | -73.79 | -100.79 |
| Antenna switch | -70.79 | -102.79 |
| n5 SAW Filter | -50.49 | -98.49 |
| n77 PA div Forward mixing | -79.24 | -184.31 |
| n5 LNA via antenna ISO | -67.91 | -149.91 |
| n5 LNA via PCB coupling | -78.04 | -122.04 |
| Total div IMD | -49.94 | -70.89 |

Table 3.9: CA_n5A-n77A MSD for n5 Rx due to IMD4 (PC1.5)

| MSD calculation parameters | received interference, IMD2 (dBm) | received interference, IMD4 (dB) |
| --- | --- | --- |
| MSD | 48.25 | 30.18 |
| SNR | -44.78 | -26.70 |
| MSD at SNR=-1 | 43.78 | 25.70 |

Proposal 6: Consider an MSD= 25.7 dB for CA_n5A-n77A with PC1.5 3Tx when IM4 falls on n5 Rx. For IM2 it is better to keep it as N/A (same as PC3 MSD) as the value is too high.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n7713 | n5 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | 25.70 | FDD | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| NOTE 7: In current release the maximum separation bandwidth class is 600MHz, therefore, no IMD2 MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of TS 38.101-1 Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

## CA_n25A-n77A

Based on the MSD analysis {|fn25-fn77| (IM2), |3fn25-fn77| (IM4)} could fall on n25 Rx so some MSD would be needed for n25 Rx. Table 3.10 present the parameters of the analysis..

Table 3.10 CA_n25A-n77A parameters

| Antenna ISO | 10 | dB |
| --- | --- | --- |
| PCB isolation Paout-Pain | 60 | dB |
| PCB isolation Paout Div-Pain main | 65 | dB |
| Diplexer ISO | 15 | dB |
| n25 TX SAW/duplexer rejection at n77 Tx | 40 | dB |
| n25 RX duplexer rejection at n77 Tx | 50 | dB |
| n25 SAW/Duplexer TX rejection at n25 RX band | 53 | dB |
| filter n77 TX rejection at n25 RX band | 45 | dB |
| n77 Tx attenuation at n25 RX | 45 | dB |
| n77 Tx attenuation at n25 TX | 45 | dB |
| Diplexer IL | 0.5 | dB |
| Antenna switch IL | 1 | dB |
| TX/RX switch IL | 0.5 | dB |
| n25 Dup IL | 4 | dB |
| n77 RX SAW filter IL | 3 | dB |
| Pout n25 | 23 | dBm |
| Pout n77 | 24.8 | dBm |
| Pout n77 Div | 24.8 | dBm |
| REFSENS n25 | -96.6 | dBm |

Table 3.11: IMD level to n25 Rx main path (PC1.5)

| Main Path | received interference, IMD2 (dBm) | received interference, IMD4 (dBm) |
| --- | --- | --- |
| Antenna switch | -59.93 | -69.93 |
| Diplexer (H-H or L-L) | -59.99 | -63.99 |
| n25 SAW/Duplexer | -73.67 | -87.67 |
| n25 PA Forward mixing | -57.00 | -56.00 |
| n77 PA Forward mixing | -63.69 | -144.69 |
| n25 PA Reverse mixing | -95.70 | -100.70 |
| n77 PA Reverse mixing | -99.70 | -237.69 |
| Total | -53.48 | -55.21 |
| B71 LNA via filters | -62.87 | -88.87 |
| B71 LNA via PCB coupling | -67.00 | -107.00 |
| Total main IMD | -52.84 | -55.21 |

Table 3.12: IMD level to n25 Rx diversity path (PC1.5)

| Diversity Path | received interference, IMD2 (dBm) | received interference, IMD4 (dBm) |
| --- | --- | --- |
| From main path | -62.84 | -65.21 |
| Diplexer | -73.79 | -100.79 |
| Antenna switch | -70.79 | -102.79 |
| n25 SAW Filter | -50.49 | -98.49 |
| n77 PA div Forward mixing | -70.20 | -161.19 |
| n25 LNA via antenna ISO | -71.87 | -131.87 |
| n25 LNA via PCB coupling | -71.00 | -115.00 |
| Total div IMD | -50.08 | -65.21 |

Table 3.13: CA_n25A-n77A MSD for n25 Rx due to IMDs (PC1.5)

| MSD calculation parameters | received interference, IMD2 (dB) | received interference, IMD2 (dB) |
| --- | --- | --- |
| MSD | 46.79 | 35.86 |
| SNR | -41.81 | -30.88 |
| MSD at SNR=-1 | 40.81 | 29.88 |

Proposal 7: Consider an MSD= 40.8 dB for CA_n25A-n77A with PC1.5 3Tx when IM2 falls on n25 Rx

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 40.81 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | 29.88 | FDD | IMD4 |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |

# 3	Conclusion

Observation 1: CA_n41A-n71A PC2 is missing in the last WF.

| Band combination | UL configuration | Max Power capabilities | MSD Status |
| --- | --- | --- | --- |
| CA_n28A-n41A | CA_n28A-n41A | PC3@n28 1Tx; PC2@n41 2Tx | No MSD is needed |
| CA_n28A-n78A | CA_n28A-n78A | PC3@n28 1Tx; PC3/2@n78 2Tx | IM5 on n8, H4 on n78, adapt from PC2 2Tx DC_28A_n78A |
| CA_n8A-n78A | CA_n8A-n78A | PC3@n8 1Tx; PC2@n78 2Tx | IM4 on n8, H4 on n78, adapt from PC2 2Tx CA_n8A-n78A |
| CA_n41A-n71A | CA_n41A-n71A | PC3@n71 1Tx; PC2@n41 2Tx | IM4 on n71, PC2 3Tx needs to be provided; H4 on n41 Not needed as it is an even order harmonic |
| CA_n41A-n77A | CA_n41A-n77A | PC2@n41 1Tx; PC2@n77 2Tx; PC2@n41 2Tx; PC2@n77 1Tx; | IM9 on n77, cross band isolation adapt from PC2 2Tx CA_n41A-n77A |
| CA_n26A-n78A | CA_n26A-n78A | PC3@n26 1Tx; PC2@n78 2Tx | IM4 on n26, PC2 3Tx needs to be provided |
| DC_3A_n78A | DC_3A_n78A | PC3@n3 1Tx; PC2@n78 2Tx | IM2, IM4 on n3, adapt from PC2 2Tx CA_n3-n78 |
| DC_40A_n78A | DC_40A_n78A | PC3@n40 1Tx; PC3/2@n78 2Tx | Cross band leakage PC2 3Tx needs to be provided; 3H of n40 could mix with 2H of n78, adapt from PC2 2Tx CA_n3-n77 |

Proposal 1: Consider an MSD= 20.70 dB for CA_n41A-n71A with PC2 3Tx when IM4 falls on n71 Rx.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n71 | n41 | 2614 | 5 | 25 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 20.70 | FDD | IMD4 |

Proposal 2: Consider an MSD= 19.2 dB for CA_ n26A-n78A with PC2 3Tx when IM4 falls on n26 Rx.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n26-n78 | n26 | 836.5 | 5 | 25 | 881.5 | 19.22 | FDD | IMD4 |
|  | n78 | 3391 | 10 | 50 | 3391 | N/A | TDD | N/A |

Proposal 3:

Table 2.11 Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 3Tx EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n78 | 40 | 16.2 | 13.2 | 11.4 | 10.2 |  |  |  |  |  |  |  |

Proposal 4: Consider an MSD= 14.26 dB for DC_40A_n78A MSD due to cross band leakage

Table 2.12 Reference sensitivity exceptions (MSD) due to cross band isolation for PC2 3TX EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n78 | 401 | 14.26 | 14.27 | 14.23 | 14.28 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation 2: for CA_n25A-n77A, IM2 on n25 was missing from the WF

Table 3.1: Band combinations for 3Tx with CA/EN-DC power class 1.5

| Band combination | UL configuration | Max Power capabilities | MSD Status |
| --- | --- | --- | --- |
| CA_n41A-n71A | CA_n41A-n71A | PC3@n71 1Tx; PC1.5@n41 2Tx | IM4 on n71, PC1.5 3Tx needs to be provided; H4 on n41 Not needed as it is an even order harmonic |
| CA_n5A-n77A | CA_n5A-n77A | PC3@n5 1Tx; // PC1.5@n77 2Tx | IM2 and IM4 on n5, PC1.5 3Tx needs to be provided; |
| CA_n25A-n77A | CA_n25A-n77A | PC3@n25 1Tx; PC1.5@n77 2Tx | IM2 and IM4 on n25, PC1.5 3Tx needs to be provided; n77 UL could overlap with H2 of n25. adapt from PC2 2Tx CA_n25A-n77A |

Proposal 5: Consider an MSD= 25.5 dB for CA_n41A-n71A with PC1.5 3Tx when IM4 falls on n71 Rx.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n71 | n41 | 2614 | 5 | 25 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 25.5 | FDD | IMD4 |

Proposal 6: Proposal 6: Consider an MSD= 25.7 dB for CA_n5A-n77A with PC1.5 3Tx when IM4 falls on n5 Rx. For IM2 it is better to keep it as N/A (same as PC3 MSD) as the value is too high.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n7713 | n5 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | 25.70 | FDD | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| NOTE 7: In current release the maximum separation bandwidth class is 600MHz, therefore, no IMD2 MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of TS 38.101-1 Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

Proposal 7: Consider an MSD= 40.8 dB for CA_n25A-n77A with PC1.5 3Tx when IM2 falls on n25 Rx

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 40.81 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | 29.88 | FDD | IMD4 |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |

# 4	Reference

[1]	“RP-231719, WID revision for Core part 4Rx handheld UE for low NR bands (1GHz) andor 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC, Oppo.”

[2]	“R4-2314705, WF on 3Tx and 4Rx requirements, Oppo.”