# 3GPP TSG-RAN WG4 Meeting #108bis	R4-2315825

# Xiamen, China, October 09 – October 13, 2023

Source: 	vivo

Title: 	TP for TR 38.786: Addition of definitions and symbols to Chapter 3

Agenda Item:	5.30.1

Document for:	Approval

Introduction

In this text proposal, definitions and symbols for SL evolution were added on the latest version of TR 38.786[1].

Reference

[1] TR 38.786 V1.0.0

# Annex: TP for TR 38.786: Addition of definitions and symbols to Chapter 3

============================ Start of TP ============================

3	Definitions of terms, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Con-current operation: The simultaneous transmission and reception of sidelink and Uu interfaces, irrespective of TDM mode and FDM mode, while operation is agnostic of the service used on each interface.

Inter-band con-current operation: Operation of NR Uu carrier and NR Sidelink carrier in different operating bands.

NR SL CA operation: Aggregation of two or more NR Sidelink component carriers in order to support wider transmission bandwidths.

Intra-band SL CA UE: UE that supports NR SL CA operation in a single band

NR SL inter-band con-current operating Band：Band combinations of NR Uu carrier and NR Sidelink carrier in different operating bands.

NR SL-U UE: UE that supports NR Sidelink operation in unlicensed bands (e.g. n46, n96, n102).

PC5: The interface for sidelink transmission.

PC3/PC5 UE: UE that supports Power class 3 or Power class 5

UE transmission bandwidth configuration: Set of resource blocks located within the UE channel bandwidth which may be used for transmitting or receiving by the UE.

Wideband operation: For a UE that supports shared spectrum channel access, wideband operation refers to operation within a channel larger than 20 MHz in which intra-cell guard bands may be configured to distinguish individual RB-sets

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

ΔfOOB	Δ Frequency of Out Of Band emission

ΔPPowerClass	Adjustment to maximum output power for a given power class

RB	The starting frequency offset between the allocated RB and the measured non-allocated RB

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔShift	Channel raster offset

TC	Allowed operating band edge transmission power relaxation

TC,c	Allowed operating band edge transmission power relaxation for serving cell c

∆LCRB	Difference RB size between NR Uu RMC and NR SL RMC

BWChannel	Channel bandwidth

BWinterferer	Bandwidth of the interferer

BWchannel CA 	The aggregated channel bandwidth of the wanted signal

FC	RF reference frequency on the channel raster

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FInterferer	Frequency of the interferer

FIoffset	Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

Foffset	Frequency offset from FC_high to the higher edge or FC_low to the lower edge.

FOOB	The boundary between the NR out of band emission and spurious emission domains

Fuw (offset)	The frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

Gpost connector	The supported post antenna connector gain

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

Max()	The largest of given numbers

Min()	The smallest of given numbers

Physical resource block number

NRACLR	NR ACLR

NR_SLREFSENS	The REFSENS value defined for NR SL

NR_UuREFSENS 	The REFSENS value defined for NR Uu

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg	The number of the aggregated RBs within the fully allocated aggregated channel bandwidth NRB_agg=1jNRBj*2μj for carrier 1 to j, where μ is defined in TS 38.211

NRB,c	The transmission bandwidth configuration of component carrier c, expressed in units of resource blocks NRB,cj=NRBj*2μj for carrier j, where μ is defined in TS 38.211

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

NFV2X	Noise figure for V2X

PCMAX	The configured maximum UE output power

PCMAX, c	The configured maximum UE output power for serving cell c

PCMAX, f, c	The configured maximum UE output power for carrier f of serving cell c in each slot

PEMAX	Maximum allowed UE output power signalled by higher layers

PEMAX, c	Maximum allowed UE output power signalled by higher layers for serving cell c

PInterferer	Modulated mean power of the interferer

Plargest BW	Power of the largest transmission bandwidth configuration of the component carriers in the bandwidth combination

PPowerClass	PPowerClass is the nominal UE power (i.e., no tolerance)

P-MPRc	Power Management Maximum Power Reduction for serving cell c

PRB	The transmitted power per allocated RB, measured in dBm

PUMAX	The measured configured maximum UE output power

Puw	Power of an unwanted DL signal

Pw	Power of a wanted DL signal

PREFSENS_V2X	The REFSENS power for V2X

PREFSENS_SL 	The REFSENS power for Sidelink

RBstart	Indicates the lowest RB index of transmitted resource blocks

RBend	Indicates the highest RB index of transmitted resource blocks

RX_BW	The bandwidth for receiving

REFSENSV2X	REFSENS value defined for V2X

SNRV2X	Signal-to-Noise Ratio for V2X

T(PCMAX, f, c)	Tolerance for applicable values of PCMAX, f, c for configured maximum UE output power for carrier f of serving cell c

SSREF	SS block reference frequency position

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AGC	Automatic Gain Control

A-MPR	Additional Maximum Power Reduction

BLER	BLock Error Rate

BS	Base Station

CBW	Channel Bandwidth

CC	Component Carriers

CDF	Cumulative Distribution Function

CP-OFDM	Cyclic Prefix-OFDM

C-IMD        [Counter Intermodulation Distortion]

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DMRS	Demodulation Reference Signal

EIRP	Equivalent Isotropically Radiated Power

EVM	Error Vector Magnitude

FDD	Frequency Division Duplex

FDM	Frequency Division Multiplexing

FR1	Frequency Range 1

FR2	Frequency Range 2

HD	Half Duplex

IM	Implementation Margin

ITS	Intelligent Transportation System

LTE	Long Term Evolution

LBT	Listen before Talk

MPR	Maximum Power Reduction

NF	Noise Figure

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

OLPC	Open Loop Power Control

PC	Power Control

PRB	Physical Resource Block

ProSe	Proximity-based Services

PSBCH	Physical Sidelink Broadcast CHannel

PSCCH	Physical Sidelink Control CHannel

PSFCH        Physical Sidelink Feedback CHannel

PSSCH	Physical Sidelink Shared CHannel

REFSENS	Reference Sensitivity

RF	Radio Frequency

SCS	Sub-Carrier Spacing

SINR	Signal to Interference plus Noise Ratio

SL	Sidelink

SL-U	Sidelink at unlicensed band

SNR	Signal-to-Noise Ratio

S-SSB         Sidelink Synchronization Signal Block

SEM	Spectrum emission mask

TDD	Time Division Duplex

TDM	Time Division Multiplexing

UE	User Equipment

UL	Uplink

V2V	Vehicle to Vehicle

V2X	Vehicle to Everything

============================ End of TP ============================