3GPP TSG-RAN WG4 Meeting #108bis	R4-2315876

Xiamen, Oct. 09 – 13, 2023

Source: 	Ericsson

Title:  	RRM requirements for electronically-steered beam UEs (Type 1)

Agenda Item:	5.26.6.1.1

Document for:	Discussion

# Introduction

In last meeting 3GPP TSG RAN WG4 Meeting #108, the issues in the objective of ‘NR-NTN deployment in above 10 GHz’ were collected in WF [1]. Later, in 3GPP TSG-RAN Meeting #101, further agreements were obtained in WF [2] to clarify the scenarios for the objective.

In this contribution, we herein present our views and proposals on the remaining issues.

# Discussion

Recap agreements in [2] in the below:

Proposal 1: As part of the NR-NTN-enh WID, RAN4 shall work on the definition of RRM requirements considering the following - cases: Case-1: Stationary UE for GSO; Case-2: Stationary UE for LEO; Case-3: Mobile UE for GSO - UE architectures: Fully electronically-steered beam UEs (Type 1); Fully mechanically-steered beam UEs (Type 2)

One interpretation of proposal 1 is that the three cases may be applied may be applied to each UE architectures, i.e. type 1 or type 2. For example, a type1 UE can be a stationary UE for GSO, a stationary UE for LEO or a mobile UE for GSO.

It is rational that the type 1 UE doesn’t need to do antenna/receive beam sweeping for measurements like how the FR2 hand-held UE does in TN, since the type 1 UE always knows its position and the positions of the satellites to be measured. The type 1 UE can switch its antenna/receive beam towards a neighboring satellite within negligible time (e.g. in order of microseconds) compared to the measurement period. Even the mobile type 1 UE (e.g. mounted on a vehicle) for GSO doesn’t require extra sweeping time because the mobile type 1 UE’s position and antenna angle to each satellite are almost static during the measurement delay.

Observation 1: A type 1 UE can be a stationary type 1 UE for GSO, a stationary type 1 UE for LEO or a mobile type 1 UE for GSO.

Proposal 1: No RX beam sweeping factor is needed for the type 1 UE. Reuse FR1 NTN requirements, except for the requirements relying on SCS, e.g., UE uplink timing accuracy.

## UE Uplink Timing Accuracy

In WF [3] it was agreed to further analyse the accuracy assumptions of UE GNSS for NTN evolution and UL SCS = 60 kHz and 120 kHz:

… ## Topic #1: NR-NTN deployment in above 10 GHz bands Agreement: - The scope of the work needed to define the RRM requirements (i.e. beam steering related) necessary for NR-NTN UEs operating in the NTN example band (i.e. Ka band) based on associated assumptions of the UE architecture - RRM requirements shall cover at least the following scenarios - Case-1: Stationary UE for GSO - Case-2: Stationary UE for LEO - Case-3: Mobile UE for GSO …

In this contribution we analyze issue 2-1, UE UL Timing Accuracy Requirements for UL SCSs of 60kHz and 120kHz.

### Baseline equation

In [2] the following dimensioning equation was proposed:

- Tg =  0.5*Tcp – (Td + Tp + Tr + Ta) > 0: an effective guard period in CP

  - Tcp: a length of CP for the given SCS of UL channel/signal

  - Td: UE downlink synchronization error for the given SCS of SSB (BW of PBCH DMRS, i.e. 20 PRBs)

  - Tp = Tp,ue + Tp,sat: a round trip propagation delay estimation error due to UE position and satellite position estimation errors

    - Tp,ue: a round trip propagation delay estimation error due to [X]m of UE position error

    - Tp,sat: a round trip propagation delay estimation error due to [Y]m of satellite position estimation error

  - Tr: TAC resolution error (from TS38.213)

  - Ta: TA adjustment accuracy error (from Table 7.3.2.2-1 of TS38.133)

The inequality of Tg is a good baseline for first order analysis, but it lacks a couple of additive terms in the total delay budget, namely:

- Tch: The radio channel delay spread [3]. In [3], we have Table 6.7.2-1b: Channel model parameters for Dense Urban Scenario (LOS) in Ka band, which states lgDS = -7.62 and lgDS = 0.78, where lgDS=log10(DS/1s) is log DS and DS is Delay Spread. This means that the  + 1* channel delay spread becomes -7.62 + 0.78 = -6.84 and DS = 10^(-6.84) = 144 ns.

- Tdr: Timing drift from DL timing estimation to UL transmission. Current RAN4 requirements in FR2-1 assume availability of an SSB in last 160 ms [4]. An allowed frequency offset at UE TX of 0.1 ppm from TS 38.101 corresponds to 16 ns.

If we add these terms the inequality Tg becomes:

Tcp – Tch – 2*(Td + Tp + Tr + Ta + Tdr) > 0

and

Te = Td + Tdr + Tp

For SSB SCS ∈ 120, 240 kHz and UL SCS  ∈ 60, 120 kHz we get:

| 120/60 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tcp | Tch | Td | Tr | Ta | Tdr | Tp |  | Te |
| 36,00 | 4,42 | 0,53 | 2,00 | 2,00 | 0,49 | 10,76 |  | 11,79 |
| 120/120 |  |  |  |  |  |  |  |  |
| Tcp | Tch | Td | Tr | Ta | Tdr | Tp |  | Te |
| 18,00 | 4,42 | 0,53 | 1,00 | 0,50 | 0,49 | 4,26 |  | 5,29 |
| 240/60 |  |  |  |  |  |  |  |  |
| Tcp | Tch | Td | Tr | Ta | Tdr | Tp |  | Te |
| 36,00 | 4,42 | 0,27 | 2,00 | 2,00 | 0,49 | 11,03 |  | 11,79 |
| 240/120 |  |  |  |  |  |  |  |  |
| Tcp | Tch | Td | Tr | Ta | Tdr | Tp |  | Te |
| 18,00 | 4,42 | 0,27 | 1,00 | 0,50 | 0,49 | 4,53 |  | 5,29 |

Observation 2: For UL SCS = 120 kHz, a UE with zero implementation margin when it comes to Te requirement has to have a total round trip error allowance of 4.3 to 4.5 Ts, which corresponds to a total position error of 21 to 22 m to distribute among satellite and UE. The Te value = 5.3 Ts.

Observation 3: For UL SCS = 120 kHz, a UE with zero implementation margin when it comes to Te requirement has to have a total round trip error allowance of 11 Ts, which corresponds to a total position error of 54 m to distribute among satellite and UE. The Te value = 12 Ts.

To summarize in a table of the same format as TS 38.133 section 7 we get:

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| 2-1 | 120 | 60 | 12*64*Tc |
|  |  | 120 | 5.3*64*Tc |
|  | 240 | 60 | 12*64*Tc |
|  |  | 120 | 5.3*64*Tc |

Proposal 2: Te_NTN Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| 2-1 | 120 | 60 | 12*64*Tc |
|  |  | 120 | 5.3*64*Tc |
|  | 240 | 60 | 12*64*Tc |
|  |  | 120 | 5.3*64*Tc |

# Conclusion

Observation 1: A type1 UE can be a stationary UE for GSO, a stationary UE for LEO or a mobile UE for GSO.

Proposal 1: No RX beam sweeping factor is needed for the type 1 UE. Reuse FR1 NTN requirements, except for the requirements relying on SCS, e.g., UE uplink timing accuracy.

Observation 2: For UL SCS = 120 kHz, a UE with zero implementation margin when it comes to Te requirement has to have a total round trip error allowance of 4.3 to 4.5 Ts, which corresponds to a total position error of 21 to 22 m to distribute among satellite and UE. The Te value = 5.3 Ts.

Observation 3: For UL SCS = 120 kHz, a UE with zero implementation margin when it comes to Te requirement has to have a total round trip error allowance of 11 Ts, which corresponds to a total position error of 54 m to distribute among satellite and UE. The Te value = 12 Ts.

Proposal 2: Te_NTN Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| 2-1 | 120 | 60 | 12*64*Tc |
|  |  | 120 | 5.3*64*Tc |
|  | 240 | 60 | 12*64*Tc |
|  |  | 120 | 5.3*64*Tc |

# References

- R4-2314365, WF on R18 NR NTN RRM requirements, Qualcomm

- RP-232694, WF on potential RRM objectives for NR-NTN deployment in above 10 GHz
 (NR-NTN-enh WID), Thales

- R4-2314447, WF on R18 NR NTN RRM requirements, Qualcomm Incorporated.