3GPP TSG-RAN WG4 Meeting # 108bis													R4-2316121

# Xiamen, China, October 09 – October 13, 2023

Source: 	OPPO

Title: 	TP on SL-U MPR and A-MPR

Agenda Item:	5.30.2.1.2

Document for:	Approval

Introduction

In the RAN4 #108-bis meeting, the MPR and A-MPR for SL-U has been fully discussed, In this TP, we capture the simulation results for MPR and A-MPR to the TP 38.786.

# References

# Text proposal

## <<Start of Change for TR 38.786>>

- 6.1.2	UE maximum output power reduction for NR SL-U

The following assumption can serve as a starting point for MPR simulation assumptions that are considered to derive MPR requirements for SL-U operation.

- Basic RF parameters for MPR of single carrier SL-U UE

  - Waveforms: CP-OFDM for SL-U

  - Supported CBW: 20/40/60/80/100MHz.

  - Modulation: QPSK/16-QAM/64-QAM/256-QAM

  - SCS: 15/30kHz

  - C-IMD: 45dBc or 60dBc

  - EVM and impairments

| Modulation | SystemEVM | PA only | Image |  |
| --- | --- | --- | --- | --- |
| QPSK | 17.5% | 10% | 34dB |  |
| 16QAM | 12.5% | 8% | 34dB |  |
| 64QAM | 8% | 4% | 34dB |  |
| 256QAM | 3.5% | 1.8% | 34dB |  |

  - IBE and impairment exceptions are same as NR-U

- Reuse IBE mask from TS38.10101 Table 6.4F.2.3-1

- Image/carrier exceptions position and specific test according to interlaces.

  - ACLR: 27dBc for both PC3/PC5 UE with NR MBW

  - SEM according to 38.101-1 clause 6.5F.2.2

  - PA calibration & PA configuration:

- Power class 5 (single PA): 1dB MPR and DFT-s-OFDM QPSK 100RB3 20MHz waveform with 27dB ACLR

  - MPR table format can be reused the MPR format of NR-U according to each transmitted channel e.g.PSSCH/PSCCH transmission, PSFCH transmission and PSBCH transmission if any issue is not seen..

  - The MPR will be applied to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel.

  - Follow the RB interlaced allocation based on Table 6.2F.2-1 and Table 6.2F.2-2 in TS38.101-1. Also refer the interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214. FFS on the inter-laced RB allocation for PSFCH, and on the repeated RB allocation for S-SSB by RAN1

For SL-U in single CC operation, the simultaneous transmission of PSCCH and PSSCH in the same sub-frame is supported and the following constraints in Table 6.1.2-1 can be assumed.

Table 6.1.2-1: SL-U operation’s MPR simulation assumptions

| Items | Assumption |
| --- | --- |
| Allowed sub-channel sizes | Support {10, 12, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size. |
| Allowed LCRB allocation | 10,12,15,20,24,25,30,36,40,45,48,50,60,70,72,75,80,84,90,96,100,105,108,110,120,125,130,132,135,140,144,150,156,160,165,168,170,175,180,190,192,195,200,204,210,216,220,225,228,230,240,250,252,255,260,264,270 The Allowed LCRB will be decided by the supported SCS. |
| Regarding PSCCH / PSSCH multiplexing |  |
| PSCCH size | 10RB*3 Symbols |
| PSD offset of X dB between PSCCH and PSSCH | 0dB |

For simultaneous transmission of PSFCH transmission for SL-U operation, RAN4 assumed as follow

Table 6.1.2-2: SL-U UE’s MPR simulation assumptions for PSFCH transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSSCH | QPSK |
| PSFCH | ZC sequence |
| Structure of Slot | Baseline is follow RAN1 agreements |
| RB allocation | - Power per RB is same in PSFCH for all users - Total power is 20dBm - Single RB-set and multiple RB-sets will be considered based on RAN1 decision. For single RB-set, RAN4 consider interlacing RBs for PSFCH For multiple RB-sets, RAN4 considers both contiguous RB sets and non-contiguous RB sets. - N gap from RBstart to RBend of interlaced transmission should meet at least 80% of channel bandwidth in a single RB-set [, Ngap = RBend – RBstart ]. |

For S-SSB transmission for SL-U operation, RAN4 assumed as follow

Table 6.1.2-3e: SL-U UE’s MPR simulation assumptions for S-SSB transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSBCH | QPSK |
| S-PSS | M-sequence |
| S-SSS | Golden-sequence |
| S-SSB structure |  |
| RB allocation | - Single RB-set and multiple RB-sets will be considered based on RAN1 decision. For single RB set, the 11RB will be repeated N time in a RB set. For the multiple RB-sets, RAN4 consider both contiguous RB sets and non-contiguous RB sets. - N gap from RBstart to RBend of repeated S-SSB transmission should meet at least 80% of channel bandwidth.in a single RB-set [,Ngap = RBend – RBstart]. |

6.1.2.1	MPR for SL-U operation

<Editor Note> Detail MPR requirements will be added

6.1.2.1.1	OPPO simulation results for MPR for SL-U operation

With the agreement above, the simulation cases are updated with 60kHz deleted. For each case, the QPSK, 16QAM, 64QAM and 256QAM will be simulated.

Table 6.1.2.1.1-1 simulation cases

|  | case | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | CP-OFDM | 20 | 105RB0 | 15 |
|  | 2 | CP-OFDM | 20 | 50RB0 | 30 |
|  | 3 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 4 | CP-OFDM | 40 | 105RB0 | 30 |
|  | 5 | CP-OFDM | 60 | 160RB0 | 30 |
|  | 6 | CP-OFDM | 80 | 216RB0 | 30 |
|  | 7 | CP-OFDM | 100 | 270RB0 | 30 |
| Interlaced Allocation Single CC | 8 | CP-OFDM | 20 | 1 RB0 every 10RBs | 15 |
|  | 9 | CP-OFDM | 20 | 1RB0 every 5RBs | 30 |
|  | 10 | CP-OFDM | 40 | 1RB0 every 10RBs | 15 |
|  | 11 | CP-OFDM | 40 | 1RB0 every 5RBs | 30 |
|  | 12 | CP-OFDM | 60 | 1RB0 every 5RBs | 30 |
|  | 13 | CP-OFDM | 80 | 1RB0 every 5RBs | 30 |
|  | 14 | CP-OFDM | 100 | 1RB0 every 5RBs | 30 |
| Wide band operation contiguous RB allocation | 15 | CP-OFDM | 40 | Bitmap 10 | 30 |
|  | 16 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 17 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 18 | CP-OFDM | 60 | Bitmap 010 | 30 |
|  | 19 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 20 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 21 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 22 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 23 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 24 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 25 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 26 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 27 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 28 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 29 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 30 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 31 | CP-OFDM | 100 | Bitmap 00100 | 30 |
| Wide band operation Interlaced RB allocation | 32 | CP-OFDM | 40 | Bitmap 10 | 30 |
|  | 33 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 34 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 35 | CP-OFDM | 60 | Bitmap 010 | 30 |
|  | 36 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 37 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 38 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 39 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 40 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 41 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 42 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 43 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 44 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 45 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 46 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 47 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 48 | CP-OFDM | 100 | Bitmap 00100 | 30 |

Corresponding MPR results are shown as below for case 1 to 14:

Table 6.1.2.1.1-2 MPR for single CC

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 3.2 | 3.1 | 3.2 | 3.2 | 3.2 | 3.2 | 3.2 | 3.4 | 3.1 | 3.3 | 3.2 | 3.2 | 3.3 | 3.2 |
| 16QAM | 3.7 | 3.6 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 3.4 | 3.1 | 3.3 | 3.2 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 4.1 | 4.3 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 6.6 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.2.1.1-3 MPR for Wideband operation

|  | Bitmap | 10 | 100 | 1000 | 10000 | 010 | 0100 | 01000 | 00100 | 110 | 1100 | 11000 | 0110 | 01100 | 1110 | 11100 | 01110 | 11110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 2.6 | 2.6 | 2.7 | 2.8 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.7 | 2.7 | 2.7 | 2.6 | 2.7 | 2.8 | 2.6 |
|  | 16QAM | 2.9 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 3.0 | 2.8 | 2.9 | 3.0 | 3.0 | 2.8 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

Figure 6.1.2.1.1- 1, Contiguous and Interlaced Single CC MPR

Figure 6.1.2.1.1-2, Contiguous and Interlaced Wideband MPR

The case 15, 16, 17, 19, 20, 22, 24, 25, 26, 28, 29 can apply 8dB limit while the rest of the cases should apply 9dB limit.

So the exception bitmap can be listed as below:

Table 6.1.2.1.1-4 exception cases mapping to bitmap

| 15 | CP-OFDM | 40 | Bitmap 10 | 30 |
| --- | --- | --- | --- | --- |
| 16 | CP-OFDM | 60 | Bitmap 100 | 30 |
| 17 | CP-OFDM | 60 | Bitmap 110 | 30 |
| 19 | CP-OFDM | 80 | Bitmap 1000 | 30 |
| 20 | CP-OFDM | 80 | Bitmap 1100 | 30 |
| 22 | CP-OFDM | 80 | Bitmap 0100 | 30 |
| 24 | CP-OFDM | 100 | Bitmap 10000 | 30 |
| 25 | CP-OFDM | 100 | Bitmap 11000 | 30 |
| 26 | CP-OFDM | 100 | Bitmap 11100 | 30 |
| 28 | CP-OFDM | 100 | Bitmap 01000 | 30 |
| 29 | CP-OFDM | 100 | Bitmap 01100 | 30 |

The Exception MPR mapping for wideband operation is proposed as below:

Table 6.1.2.1.1-5: Exception bitmap mapping

| Wideband operation channel bandwidth (MHz) | Sub-band configuration exceptions |
| --- | --- |
| 40 | 10, 01 |
| 60 | 110, 011 |
| 80 | 1000, 1100, 0100, 0010, 0011, 0001 |
| 100 | 10000, 11000, 11100, 01000, 01100, 00110, 00010, 00111, 00011, 00001 |

- 6.1.3	UE additional maximum output power reduction for NR SL-U

Based on the NR-U agreement for the additional spectrum emission mask in clause 6.5F.2.3 in TS38.101-1, RAN4 do not consider the additional spectrum emission mask.

For the additional spurious emission requirements for SL-U, RAN4 will consider the NR-U additional spurious emission requirements in clause 6.5F.3.3 in TS38.101-1.

To derive A-MPR requirements for SL-U operation in single CC, RAN4 will assume the basic simulation parameters and assumptions in section 6.1.2 and the additional spurious emission requirements in clause 6.5F.3.3 in TS38.101-1 will be considered.

Due to the limited time of Rel-18, it has been agreed to finish the A-MPR requirement for NS_31, NS_53, NS_58, NS_60 and NS_61 in Rel-18 and further NS values can be added in future release. Furthermore, to capture further different A-MPR requirements for PSSCH/PSCCH, PSFCH and S-SSB, new NS values for SL-U is needed. Below table shows the mapping of regional requirements between legacy NR-U and SL-U NS values.

Table 6.1.3-1 Regional requirement between SL-U and NR-U NS values

| SL-U NS values | NR-U NS values |
| --- | --- |
| NS_02S | NS_31 |
| NS_03S | NS_53 |
| NS_04S | NS_60 |
| NS_05S | NS_61 |
| NS_06S | NS_58 |

Table 6.1.3-2 Mapping of network signaling label for SL-U

| NR SL unlicensed band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n46 | NS_01 | NS_02S |  |  |  |  |  |  |
| n96 | NS_01 | NS_03S | NS_04S | NS_05S |  |  |  |  |
| n102 | NS_01 | NS_06S |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. |  |  |  |  |  |  |  |  |

6.1.3.1	A-MPR for SL-U with NS_02S

- 6.1.3.1.1	A-MPR for PSSCH/PSCCH transmission

- 6.1.3.1.1.1	OPPO simulation results for PSSCH/PSCCH transmission

For PSD = 10dBm/MHz：

Table 6.1.3.1.1.1-1 A-MPR for NS_02S single CC PSD=10 dBm/MHz

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 4.4 | 4.4 | 3.5 | 3.5 | 3.2 | 3.2 | 3.2 | 4.5 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 16QAM | 4.4 | 4.4 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 4.5 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 4.5 | 4.6 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 6.6 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.1.1.1-2 A-MPR for NS_02S Wideband operation PSD=10 dBm/MHz

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 3.5 | 3.0 | 3.0 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 3.7 | 3.1 | 3.1 | 2.8 | 2.7 | 2.8 | 2.6 | 2.6 | 2.6 | 2.6 | 2.7 | 2.7 | 2.7 | 2.6 | 2.7 | 2.8 | 2.6 |
|  | 16QAM | 3.7 | 3.1 | 3.1 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 3.0 | 2.8 | 2.9 | 3.0 | 3.0 | 2.8 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

For PSD = 7dBm/MHz：

Table 6.1.3.1.1.1-3 A-MPR for NS_02S single CC PSD=7 dBm/MHz

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 4.4 | 4.4 | 3.5 | 3.5 | 3.2 | 3.2 | 3.2 | 4.5 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 16QAM | 4.4 | 4.4 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 4.5 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 4.5 | 4.6 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 6.6 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.1.1.1-4 A-MPR for NS_02S Wideband operation PSD=7 dBm/MHz

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 3.5 | 3.0 | 3.0 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 3.7 | 3.1 | 3.1 | 3.0 | 2.7 | 2.9 | 2.6 | 2.9 | 2.6 | 2.9 | 2.7 | 2.7 | 2.7 | 2.9 | 2.7 | 2.8 | 3.0 |
|  | 16QAM | 3.7 | 3.1 | 3.1 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 2.8 | 3.0 | 3.0 | 3.0 | 3.0 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

For PSD = 4dBm/MHz：

Table 6.1.3.1.1.1-5 A-MPR for NS_02S single CC PSD=4 dBm/MHz

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 4.4 | 4.4 | 3.5 | 3.5 | 3.2 | 3.2 | 3.2 | 5.6 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 16QAM | 4.4 | 4.4 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 5.7 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 5.7 | 4.6 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 6.6 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.1.1.1-6 A-MPR for NS_02S Wideband operation PSD=4 dBm/MHz

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 3.5 | 3.5 | 3.0 | 3.6 | 2.8 | 3.5 | 2.8 | 3.5 | 2.8 | 3.5 | 2.8 | 2.8 | 2.8 | 3.5 | 2.8 | 2.8 | 3.6 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.6 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 5.9 | 5.9 | 3.1 | 6.0 | 2.9 | 5.9 | 2.6 | 5.9 | 2.9 | 5.9 | 2.9 | 2.7 | 2.7 | 5.9 | 3.0 | 2.8 | 6.0 |
|  | 16QAM | 6.0 | 6.0 | 3.1 | 6.0 | 3.0 | 6.0 | 3.0 | 6.0 | 3.0 | 5.9 | 3.0 | 3.0 | 2.8 | 6.0 | 3.0 | 3.0 | 6.0 |
|  | 64QAM | 6.0 | 6.0 | 4.2 | 6.0 | 4.2 | 5.9 | 4.1 | 5.9 | 4.2 | 5.9 | 4.2 | 4.1 | 4.0 | 6.0 | 4.2 | 4.1 | 5.9 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

From the table 1 to table 6 above, it can be found that even though the PSD requirement is different from 10 to 7 to 4 dBm/MHz, the A-MPR simulation result for both single CC and wide-band operation are the same. The reason here is that the PSD requirement is not the dominate factor and it is the SEM and EVM for some of the 256QAM. The figure 1 and figure 2 below show the A-MPR.

Figure 6.1.3.1.1.1-1 NS_02S Single CC A-MPR

Figure 6.1.3.1.1.1-2 NS_03S Wideband A-MPR

- 6.1.3.1.2	A-MPR for S-SSB transmission

- 6.1.3.1.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

6.1.3.2	A-MPR for SL-U with NS_03S

- 6.1.3.2.1	A-MPR for PSSCH/PSCCH transmission

- 6.1.3. 2.1.1	OPPO simulation results for PSSCH/PSCCH transmission

Case 1,2,8,9 are for 20MHz; 3,4,10,11 are for 40MHz; 5,12 are for 60MHz; 6,13 are for 80MHz and 7,14 are for 100MHz.

Table 6.1.3. 2.1.1-1 A-MPR for NS_03S single CC

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 8.4 | 8.6 | 5.4 | 5.5 | 3.8 | 3.2 | 3.2 | 9.8 | 10.9 | 6.6 | 7.8 | 6.0 | 4.5 | 3.9 |
| 16QAM | 8.4 | 8.6 | 5.4 | 5.6 | 3.9 | 3.9 | 3.8 | 9.8 | 10.9 | 6.6 | 7.8 | 6.0 | 4.5 | 3.9 |
| 64QAM | 8.3 | 8.5 | 5.5 | 5.4 | 5.6 | 5.5 | 5.5 | 9.8 | 10.9 | 6.7 | 7.8 | 6.1 | 4.6 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 9.8 | 10.9 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3. 2.1.1-2 A-MPR for NS_03S Wideband

|  | Bitmap | 10 | 100 | 010 | 1000 | 0100 | 10000 | 01000 | 00100 | 110 | 1100 | 0110 | 11000 | 01100 | 1110 | 11100 | 01110 | 11110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 8.5 | 8.5 | 8.6 | 8.5 | 8.5 | 8.5 | 8.5 | 8.6 | 5.8 | 5.5 | 5.5 | 5.5 | 5.8 | 4.0 | 4.0 | 4.0 | 2.8 |
|  | 16QAM | 8.5 | 8.5 | 8.6 | 8.5 | 8.5 | 8.5 | 8.5 | 8.6 | 5.8 | 5.6 | 5.6 | 5.6 | 5.7 | 4.0 | 4.0 | 4.0 | 3.6 |
|  | 64QAM | 8.5 | 8.5 | 8.6 | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 | 5.8 | 5.5 | 5.5 | 5.5 | 5.7 | 4.9 | 4.8 | 5.0 | 5.0 |
|  | 256QAM | 8.5 | 8.5 | 8.6 | 8.5 | 8.6 | 8.6 | 8.5 | 8.6 | 7.7 | 7.3 | 7.3 | 7.3 | 8.5 | 8.0 | 7.6 | 8.5 | 8.0 |
| Interlace | QPSK | 10.9 | 10.9 | 11.0 | 10.9 | 10.9 | 10.9 | 10.9 | 11.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 6.2 | 6.4 | 6.4 | 5.1 |
|  | 16QAM | 11.0 | 11.0 | 11.0 | 11.0 | 11.0 | 11.0 | 11.0 | 11.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 6.3 | 6.5 | 6.3 | 5.1 |
|  | 64QAM | 11.0 | 11.0 | 11.0 | 10.9 | 11.0 | 10.9 | 11.0 | 10.9 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 6.3 | 6.4 | 6.3 | 5.2 |
|  | 256QAM | 11.0 | 11.0 | 11.0 | 10.9 | 11.0 | 10.9 | 11.0 | 11.0 | 8.1 | 8.0 | 8.0 | 8.0 | 8.0 | 6.7 | 6.7 | 6.7 | 6.6 |

Figure 6.1.3. 2.1.1-1 NS_03S Single CC A-MPR

Figure 6.1.3. 2.1.1-2 NS_03S Wide-band A-MPR

- 6.1.3.2.2	A-MPR for S-SSB transmission

- 6.1.3.2.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

6.1.3.3	A-MPR for SL-U with NS_04S

- 6.1.3.3.1	A-MPR for PSSCH/PSCCH transmission

- 6.1.3.3.1.1	OPPO simulation results for PSSCH/PSCCH transmission

The A-MPR requirement is captured in sub-clause 6.2F.3.10 and the requirement is differentiated by different channel bandwidth as captured below:

Table 6.1.3.3.1.1-1 A-MPR for NS_04S single CC

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 5.4 | 5.6 | 3.5 | 3.5 | 3.2 | 3.2 | 3.2 | 6.7 | 8.0 | 3.5 | 4.7 | 3.2 | 3.3 | 3.2 |
| 16QAM | 5.4 | 5.7 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 6.8 | 8.0 | 3.5 | 4.8 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.3 | 5.6 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 6.8 | 8.0 | 4.1 | 4.7 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 7.9 | 8.0 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.3.1.1-2 A-MPR for NS_04S wideband

|  | Bitmap | 10 | 100 | 010 | 1000 | 0100 | 10000 | 01000 | 00100 | 110 | 1100 | 0110 | 11000 | 01100 | 1110 | 11100 | 01110 | 11110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 5.5 | 5.5 | 5.6 | 5.5 | 5.5 | 5.5 | 5.5 | 5.6 | 3.0 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 5.5 | 5.5 | 5.6 | 5.5 | 5.5 | 5.5 | 5.5 | 5.6 | 3.6 | 3.6 | 3.7 | 3.6 | 3.6 | 3.6 | 3.5 | 3.7 | 3.6 |
|  | 64QAM | 5.5 | 5.5 | 5.6 | 5.5 | 5.5 | 5.5 | 5.5 | 5.6 | 4.8 | 4.7 | 4.9 | 4.7 | 4.8 | 4.9 | 4.8 | 5.0 | 5.0 |
|  | 256QAM | 7.4 | 7.5 | 8.5 | 7.4 | 7.5 | 7.4 | 7.4 | 8.5 | 7.7 | 7.3 | 8.5 | 7.3 | 7.7 | 8.0 | 7.6 | 8.5 | 8.0 |
| Interlace | QPSK | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 5.0 | 5.0 | 5.0 | 5.0 | 5.0 | 3.1 | 3.3 | 3.4 | 2.7 |
|  | 16QAM | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 5.1 | 5.0 | 5.0 | 5.0 | 5.1 | 3.2 | 3.5 | 3.3 | 2.8 |
|  | 64QAM | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 5.1 | 5.1 | 5.1 | 5.1 | 5.1 | 4.1 | 4.1 | 4.1 | 4.0 |
|  | 256QAM | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 6.8 | 6.8 | 6.8 | 6.8 | 6.8 | 6.7 | 6.7 | 6.7 | 6.6 |

Figure 6.1.3.3.1.1-1 NS_04S Single CC A-MPR

Figure 6.1.3.3.1.1-2 NS_04S Wideband CC A-MPR

- 6.1.3.3.2	A-MPR for S-SSB transmission

- 6.1.3.3.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

6.1.3.4	A-MPR for SL-U with NS_05S

- 6.1.3.4.1	A-MPR for PSSCH/PSCCH transmission

- 6.1.3.4.1.1	OPPO simulation results for PSSCH/PSCCH transmission

The A-MPR requirement is captured in sub-clause 6.2F.3.10 and the requirement is differentiated by different channel bandwidth as captured below:

Table 6.1.3.4.1.1-1 A-MPR for NS_05S single CC

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 6.4 | 6.6 | 3.4 | 3.5 | 3.5 | 3.5 | 3.5 | 7.8 | 8.9 | 4.5 | 5.8 | 4.0 | 3.6 | 3.5 |
| 16QAM | 6.4 | 6.7 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 7.8 | 9.0 | 4.5 | 5.8 | 4.0 | 3.6 | 3.5 |
| 64QAM | 6.3 | 6.6 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 7.8 | 9.0 | 4.6 | 5.8 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 7.9 | 9.0 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.4.1.1-2 A-MPR for NS_05S wideband

|  | Bitmap | 10 | 100 | 010 | 1000 | 0100 | 10000 | 01000 | 00100 | 110 | 1100 | 0110 | 11000 | 01100 | 1110 | 11100 | 01110 | 11110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 6.5 | 6.5 | 6.6 | 6.5 | 6.5 | 6.5 | 6.5 | 6.6 | 3.7 | 3.5 | 3.7 | 3.5 | 3.7 | 3.0 | 2.8 | 2.8 | 3.2 |
|  | 16QAM | 6.5 | 6.5 | 6.6 | 6.5 | 6.6 | 6.5 | 6.5 | 6.7 | 3.7 | 3.6 | 3.7 | 3.6 | 3.7 | 3.6 | 3.5 | 3.7 | 3.6 |
|  | 64QAM | 6.6 | 6.5 | 6.6 | 6.6 | 6.5 | 6.6 | 6.6 | 6.6 | 4.8 | 4.7 | 4.9 | 4.7 | 4.8 | 4.9 | 4.8 | 5.0 | 5.0 |
|  | 256QAM | 7.4 | 7.5 | 8.5 | 7.4 | 7.5 | 7.4 | 7.4 | 8.5 | 7.7 | 7.3 | 8.5 | 7.3 | 7.7 | 8.0 | 7.6 | 8.5 | 8.0 |
| Interlace | QPSK | 8.9 | 9.0 | 9.0 | 9.0 | 8.9 | 9.0 | 8.9 | 9.0 | 6.0 | 5.9 | 5.9 | 5.9 | 6.0 | 4.2 | 4.4 | 4.4 | 3.2 |
|  | 16QAM | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 6.0 | 6.0 | 5.9 | 6.0 | 6.0 | 4.3 | 4.5 | 4.3 | 3.3 |
|  | 64QAM | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 6.0 | 6.0 | 6.0 | 6.0 | 6.0 | 4.3 | 4.4 | 4.3 | 4.0 |
|  | 256QAM | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 6.8 | 6.8 | 6.8 | 6.8 | 6.8 | 6.7 | 6.7 | 6.7 | 6.6 |

Figure 6.1.3.4.1.1-1 NS_05S Single CC A-MPR

Figure 6.1.3.4.1.1-2 NS_05S Wideband A-MPR

- 6.1.3.4.2	A-MPR for S-SSB transmission

- 6.1.3.4.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

6.1.3.5	A-MPR for SL-U with NS_06S

- 6.1.3.5.1	A-MPR for PSSCH/PSCCH transmission

- 6.1.3.5.1.1	OPPO simulation results for PSSCH/PSCCH transmission

The A-MPR requirement is captured in sub-clause 6.2F.3.10 and the requirement is differentiated by different channel bandwidth as captured below:

Table 6.1.3.5.1.1-1 A-MPR for NS_06S single CC

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 3.2 | 3.1 | 3.2 | 3.2 | 3.2 | 3.2 | 3.2 | 3.2 | 3.1 | 3.3 | 3.2 | 3.2 | 3.3 | 3.2 |
| 16QAM | 3.7 | 3.6 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 3.2 | 3.1 | 3.3 | 3.2 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 4.4 | 4.3 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 7.9 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.5.1.1-2 A-MPR for NS_06S wideband

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 2.6 | 2.6 | 2.7 | 2.8 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.7 | 2.7 | 2.7 | 2.6 | 2.7 | 2.8 | 2.6 |
|  | 16QAM | 2.9 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 3.0 | 2.8 | 2.9 | 3.0 | 3.0 | 2.8 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

Figure 6.1.3.5.1.1-1 NS_06S Single CC A-MPR

Figure 6.1.3.5.1.1-2 NS_06S Wideband A-MPR

- 6.1.3.5.2	A-MPR for S-SSB transmission

- 6.1.3.5.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

## <<End of Change>>