3GPP TSG-RAN WG4 Meeting #108bis              	R4-2316153

Xiamen, China, 9th Oct-13th Oct, 2023

Agenda item:	5.26.8.1

Source:	Samsung

Title:	View on BS demodulation requirements for NTN enhancement

Document for:	Discussion

- Introduction

In RAN#99 meeting, the WI on NR NTN (Non-Terrestrial Networks) enhancements was approved. The related objective of this WI was presented as following [1]

- Coverage enhancement - To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g., repetition) [RAN1, RAN4] - To specify, if necessary, enhancements to the Rel-17 procedures for DMRS bundling for PUSCH taking into account NTN-specifics (e.g., time-frequency pre-compensation) [RAN1] - NR NTN deployment in above 10GHz bands - Study and identify NTN example band: Analysis of regulations and adjacent channel co-existence scenarios. The example band shall be identified early in the WI. Additional bands can be introduced in a release-independent manner. [RAN4] - Specify Rx/Tx requirements for satellite access node and different VSAT UE class (not only 60 cm aperture) as appropriate for the identified example band [RAN4] - Network verified UE location - Based on RAN1 conclusions of the study phase, RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view [RAN1, 2, 3, 4]. DL-TDoA methods for verification may be considered as lower priority and if time permits and condition in Note is satisfied. - NTN-TN and NTN-NTN mobility and service continuity enhancement - Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements [RAN2, RAN3, RAN4] - For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point. [RAN2, RAN3, RAN4] - Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3] - Specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized). [RAN2, RAN3, RAN4] - Study and, if needed, specify enhancement to Xn[/NG] signalling to support feeder link switch-over, CHO, e.g., exchange of necessary information between gNBs. [RAN3]

According to the meeting agenda, this meeting is the first meeting to discuss the performance part of NTN enhancement.

In this contribution, the initial view on the test scope and test setup of BS demodulation requirement for NTN enhancement.

# 2	Test scope of BS demodulation requirement

In this section, the overview on impact of BS demodulation requirement based on the objective in WID was provided.

Coverage Enhancement

The Rel-18 NTN objectives are focused on the applicability of the “solutions developed by general NR coverage enhancement” (NR_cov_enh) to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement.

The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarization loss (per antenna port).

Based on RAN1 feasibility analysis as following

- For PUCCH for Msg4 HARQ-ACK with parameter set-1 for LEO-1200 operating at LOS, - One source observed that the existing specification can meet the performance requirement - Three sources observed that the existing specification cannot meet the performance requirement with a gap of 1.8 to 6 dB.

it was concluded that PUCCH for Msg4 HARQ-ACK should be enhanced to meet the coverage requirements for parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Therefore, PUCCH requirements for Msg4 HARQ-ACK under NTN deployment scenario could be introduced

Proposal 1: PUCCH requirement with Msg4 HARQ-ACK with repetition could be introduced

Excepting for PUCCH, enhancements to the Rel-17 procedures for DMRS bundling for PUSCH taking into account NTN-specifics was also considered.  In Rel-17, the PUSCH requirements with DMRS bundling for TN deployments have been specified. Although there is no new physical layer design, considering the test coverage and new deployment scenario of NTN channel, PUSCH requirement with DMRS bundling under the NTN channel condition could be introduced.

Proposal 2: PUSCH requirement with DMRS bundling can be introduced for NTN-specific UE

NR NTN deployment in above 10GHz bands

In Rel-17, the NTN WI is mainly targeting for FR1, where the related demodulation requirements under NTN deploy scenario were specified. In Rel-18, new scenarios to cover deployments in frequency bands over 10GHz is supported in Ka-band

Meanwhile, based on RAN4 agreement, the related on the band definition and parameters for above 10GHz was agreed

Proposed Table 5.1-1: Definition of NTN frequency ranges Frequency range designation; Corresponding frequency range. FR1-NTN1; 410 MHz – 7125 MHz. FR2-NTN2; 17300 MHz – 30000 MHz. NOTE 1: [NTN bands within this frequency range are regarded as a FR1 band when references from other specifications.] NOTE 2: [NTN bands within this frequency range are regarded as a FR2 band when references from other specifications.].

Considering BS demodulation requirement was introduced only for FR1 in Rel-17, considering the test coverage, it is necessary to introduce the demodulation requirements to verify the baseband performance under NTN channel in above 10GHz bands, including PUSCH, PUCCH and PRACH.

Meanwhile, only the basic requirements of TN were introduced in FR1 for NTN-specific UE, therefore, we would like to only define the basic requirements based on features introduced for TN in Rel-15

Proposal 3: BS demodulation requirement above 10GHz bands could be introduced. Only focus on the basic requirement based on features introduced for TN in Rel-15

Network verified UE location

This objective is related with UE location derivation, the related measurement and accuracy should be specified, which belongs to the RRM requirement. There is no demodulation performance impact

Proposal 4: No BS demodulation introduced related with Network verified UE location

NTN-TN and NTN-NTN mobility and service continuity enhancement

This objective is related with cell reelection, handover, and measurement for NTN mobility, which belongs to the RRM requirement. There is no demodulation performance impact.

Proposal 5: No BS demodulation requirement introduced for NTN-TN and NTN-NTN mobility and service continuity enhancement

# 3	Test setup of BS demodulation requirement

In this part, the detail test setup for specific demodulation requirements is discussed based on the test scope discussion in section 2.

## 3.1	Test setup of PUCCH requirement for Msg4 HARQ-ACK

PUCCH format

In Rel-15, PUCCH requirements with several formats were specified. For Msg4 HARQ-ACK is transmitted based on the structure of format 1. Therefore, only PUCCH format 1 requirement should be specified.

Proposal 6: Only PUCCH format 1 requirement should be specified

Number of Repetition

The number of transmissions supported for PUCCH transmission for Msg4 HARQ-ACK was agreed in RAN1

Agreement For PUCCH transmission for Msg4 HARQ-ACK, supported number of transmissions are 1, 2, 4, 8.

In Rel-17, multi-slot PUCCH with format 1 has been specified for NTN scenario with NLOS channel, where number of repetitions is 2. From the baseband processing, there is no different between LOS and NLOS channel. Considering the test coverage, and the coverage limitation of LEO-1200 satellite operating at LOS channel, additional test compared with Rel-17 multi-slots with format 1 under LOS channel can be introduced. For LOS channel, NTN-TDLC5-200 was introduced in Rel-17 for PUSCH requirement, then, it can be used for specifying the PUCCH format 1 with repetition requirement

Proposal 7: Introduce multi-slot PUCCH requirement with format 1 under 2 repetition for NTN-TDLC5-200 channel

Antenna configuration

For Antenna configuration, both 1Tx1Rx and 1Tx2Rx were considered for PUCCH requirement definition with repetition, where the related test applicability rule for different number of receiver antenna was introduced to reduce the test effort. Therefore, the same approach can be applied for performance test.

Proposal 8: Introduce PUCCH requirement with both 1Tx1Rx and 1Tx2Rx for NTN-specific UE with repetition. Apply the same test applicability for 1T1Rx and 1Tx2Rx performance test

Other test parameters

For other test parameters, the existing one for multi-slot PUCCH with format 1 can be used as

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |

Proposal 9: Using the following test parameters for specifying PUCCH requirement for Msg4 HARQ-ACK

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |

## 3.2	Test setup of PUSCH requirement with DMRS bundling

In Rel-17, DMRS bundling was introduced for coverage enhancement. To enable DMRS bunding feature, RAN4 has introduced the related phase difference, timing error and frequency error limitation requirement. For NTN-specific UE, the following work assumption was made in RAN1

Observation For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed. - FFS: whether/how to consider the initial timing error at the beginning - FFS: TA pre-compensation update is assumed - Frequency error limit (Section 6.4.1 in 38.101-5) can be satisfied over 32 slots if frequency pre-compensation update is not assumed. - FFS: impact of phase difference limit Observation For NTN-specific PUSCH DMRS bundling, - In LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots (for carrier bandwidth 5 MHz or larger), if the PRB allocation is not within 6 PRBs from the DC carrier, pre-compensation by UE and post-compensation by gNB are not assumed, and 70.5 (us/s) timing drift rate is assumed. - Note: this does not imply that UE shall be scheduled within 6 PRBs from the DC carrier. Working assumption For NTN-specific PUSCH DMRS bundling, to satisfy the phase difference limit without causing phase discontinuity, it is assumed that pre-compensation to keep phase rotation due to timing drift within the phase difference limit can be performed at UE side. - UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit. - FFS: how to determine the actual TDW - FFS: specification impact - Send an LS to RAN4

Based on RAN1 observation, the timing error and frequency error can be stratified within at most 13 slots, and 32 slots, separately, if TA pre-compensation update is not assumed.

Regarding the phase difference limit, as observed by RAN1, it cannot be satisfied over multiple slots (for carrier bandwidth 5 MHz or larger), if the PRB allocation is not within 6 PRBs from the DC carrier.

Timing drifting model

In Rel-17, the NTN requirements was defined based on UE can do the pre-compensation in very UL transmission. Therefore, there is no timing drift modeling. For NTT-specific UE, based on RAN1 WA, UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit. If TA pre-compensation is applied for the first transmission in the TDW, and there is no TA pre-compensation operation for other UL transmission slot, it may have some impact on demodulation requirement. Therefore, the related timing drift model should be considered for specifying the PUSCH requirements DMRS bunding.

Proposal 10: FFS on introduce the timing drift model for specifying the PUSCH requirement with DMRS bundling in NTN scenario.

Actual TDW and nominal TDW

Based on RAN1 agreement, the following is agreed for nominal TDW

For NTN-specific PUSCH DMRS bundling, reuse clause 6.1.7 in TS38.214 for nominal TDW determination, except for aspects related to UE capabilities and assistance information (if needed). - i.e., if PUSCH-TimeDomainWindowLength is configured, nominal TDW is determined by PUSCH-TimeDomainWindowLength; otherwise, nominal TDW is determined based on UE capability(ies) signaling. - FFS: which UE capability(ies) signaling is(are) used - FFS: whether/how to use UE assistance information, if supported

And the following is agreed for actual TDW

Agreement For NTN-specific PUSCH DMRS bundling, actual TDW is determined by the existing events and no additional event is defined

For Rel-17 CE with DMRS bundling, the value of nominal TDW is configured for 8 for FDD and 2 for TDD, and the values are same the configured PUSCH aggregation factor.  If the timing drifting model included, our preference is to consider the small value for nominal TDW for specifying PUSCH requirement with DMRS bundling.

Proposal 11: The following value for actual TDW and nominal TDW can be considered for PUSCH requirement with DMRS bundling in NTN scenario

- Actual TDW and PUSCH-TimeDomainWindowLength = 2 or 4 for FDD

- Actual TDW and PUSCH-TimeDomainWindowLength 2 for TDD

BW& SCS

In Rel-17, CE with DMRS bundling, the minimum CBW for each SCS was considered for requirement definition. As observed in RAN1, phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots (for carrier bandwidth 5 MHz or larger), if the PRB allocation is not within 6 PRBs from the DC carrier. Therefore, large CBWs or larger PRB may be not applied for NTN-specific UE with DMRS bundling, considering for satisfying the phase difference limit.

Proposal 12: The following PRB allocation can be considered for PUSCH requirement with DMRS bundling in NTN scenario

- PRB: 6RBs for both 15KHz and 30KHz

Mapping type

In Rel-17, both PUSCH mapping type A and mapping type B were considered for PUSCH requirement definition with DMRS bundling. To reduce the test effort, we prefer to specify the PUSCH DMRS bunding with mapping type A., while we are also open to further discuss whether the PUSCH mapping type B is necessary considering the test coverage.

Proposal 13: Introduce PUSCH requirement with mapping type A for NTN-specific UE with DMRS bundling

Waveform

Since the DMRS bundling introduced is mainly targeting for coverage enhancement, similar as Rel-17 CE, only CP-OFDM wave is considered for PUSCH requirement for NTN-specific UE with DMRS bundling.

Proposal 14: Introduce PUSCH requirement with CP-OFDM waveform for NTN-specific UE with DMRS bundling

Antenna configuration

For Antenna configuration, both 1Tx1Rx and 1Tx2Rx were considered for PUSCH requirement definition with DMRS bundling, where the related test applicability rule for different number of receiver antenna was introduced to reduce the test effort. Therefore, the same approach can be applied for performance test.

Proposal 15: Introduce PUSCH requirement with both 1Tx1Rx and 1Tx2Rx for NTN-specific UE with DMRS bundling. Apply the same test applicability for 1T1Rx and 1Tx2Rx performance test

Number Of DMRS

For number of DMRS, although both 1 DMRS symbol and 2 DMRS symbols were considered for PUSCH requirement definition with DMRS bundling, and minor performance between 1 DMRS and 2 DMRS symbols, we would like to select the typical DMRS configuration for specifying the PUSCH requirement for NTN-specific UE with 1+1 DMRS symbols.

Proposal 16: Introduce PUSCH requirement for NTN-specific UE with 1+1 DMRS symbols.

Other

For other test parameters, the existing test parameters for specifying PUSCH requirement with DMRS bundling for TN-specific UE can be reused for NTN-specific UE with DMRS bundling.

Proposal 17: Apply the existing test parameters for specifying PUSCH requirement with DMRS bundling for NTN-specific UE with DMRS bundling

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Example UL-DL pattern [Note 1] |  | 15 kHz SCS: FDD and TDD 7D1S2U, S=6D:4G:4U 30 kHz SCS: FDD and TDD 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence [Note 2] | 0, 3, 0, 3 for FDD 0, 3, 0, 3 for TDD |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | [n2] or [n8] for FDD n2 for TDD |
| pusch-TimeDomainWindowLength |  | [2] or [4] slots for FDD 2 slots for TDD |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The same TDD requirements are applicable to different UL-DL patterns with more than one consecutive UL slots when both pusch-TimeDomainWindowLength and PUSCH aggregation factor are configured as 2 slots. The UL (re)transmission of PUSCH is only scheduled for the actual TDW including 2 consecutive UL slots. Note 2: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

## 3.3	Test setup of BS demodulation requirement above 10GHz

In this subsection, the test setup for BS demodulation requirements above 10GHz, including PUSCH, PUCCH and PRACH are specified, separately.

SCS & BW

For numerologies in NR NTN above 10GHz bands, the following agreement are made in the RRM session

- RAN4 to consider the following numerologies for RRM requirement definition for NR-NTN above 10 GHz bands - Consider SSB SCS larger than 30KHz, i.e., 120KHz and 240KHz - Consider UL SCS larger than 30KHz, i.e., 60KHz and 120KHz - RAN4 to send an LS to RAN1 and RAN2

Meanwhile, regarding the spectral utilization, the following is agreed

- Agreement - Option 1: to use the same special unitization requirement of FR2 TN system for NTN Ka-band as starting point - Consider SSB SCS larger than 30KHz, i.e., 120KHz and 240KHz - Consider UL SCS larger than 30KHz, i.e., 60KHz and 120KHz - RAN4 to send an LS to RAN1 and RAN2

The supported channel bandwidth for NTN operation above 10GHz, is agreed

- For NTN operation above 10GHz, at least the channel bandwidth 50, 100 and 200 MHz are supported - Consider SSB SCS larger than 30KHz, i.e., 120KHz and 240KHz - Consider UL SCS larger than 30KHz, i.e., 60KHz and 120KHz - RAN4 to send an LS to RAN1 and RAN2

As the detail channel bandwidth for requirement definition, since RAN4 has already introduced the test applicability rule for different channel bandwidth, we would like to specify the BS demodulation requirement with minimum channel bandwidth per each SCS.

Proposal 18: The following SCS and CBW can be considered for specifying PUSCH requirements above 10GHz

- 60 KHz SCS, 50MHz

- 120 KHz SCS, 50MHz

Duplex

Regarding the duplex mode, FDD mode is assumed for satellite operation above 10GHz, while TDD mode is assumed for terrestrial operation in FR2, according to the WID.

Proposal 19: Introduce SAN demodulation requirement with FDD only

SAN Type

For SAN Type, whether to define both conductive and radiated RF requirement was under discussion. As agreed,

Agreement: Introduce Type 2-O radiated requirements and FFS for conductive requirements for Ka band/above 10GHz.

Proposal 20: Prioritize to define SAN demodulation requirement for Type 2-O. FFS to define BS demodulation requirement for Type 2-H, pending on conclusion of RF core requirement

Channel Model

In Rel-17 NTN WI, NTN-TDLA and NTN-TDLC were selected for demodulation requirement in S band to cover both NLOS and LOS scenario.

Regarding how to select the proper delay spread and doppler shift, RAN4 has discussion with several meeting in Rel-17 NTN WI. As for delay spread, small value is expected with considering high carrier frequency. The selection of delay spread is considering the elevation angle >=30 as the worst case from link budget perspective, also the timing limitation. Therefore, the same approach can be reused to select the proper delay spread. The following is the RMS delay spread for different scenarios NLOS and LOS in Ka band

The RMS delay spread is within range of 0.5~114.8ns regarding to elevation angle >= 30o.

Table 1 RMS delay spread for different scenarios NLOS in Ka band

| DS (ns) | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dense Urban NLOS | 138.0384 | 144.5440 | 100.0000 | 61.6595 | 38.0189 | 13.8038 | 17.3780 | 8.5114 | 11.2202 |
| Urban NLOS | 57.5440 | 19.9526 | 15.1356 | 9.1201 | 8.3176 | 7.9433 | 6.9183 | 9.3325 | 4.6774 |
| Suburban NLOS | 37.1535 | 23.4423 | 13.8038 | 10.9648 | 10.4713 | 3.5481 | 6.1660 | 2.0417 | 2.0417 |
| Rural NLOS | 0.7413 | 4.0738 | 7.9433 | 10.9648 | 10.2329 | 8.9125 | 9.7724 | 8.9125 | 12.3027 |

For LOS channel NTN-TDLC, the relevant parameters are K-factor and delay spread

Table 2 RMS delay spread for different scenarios LOS in Ka band

| DS (ns) | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dense Urban LOS | 37.1535 | 23.9883 | 17.3780 | 9.5499 | 7.4131 | 5.0119 | 4.5709 | 4.0738 | 3.5481 |
| Urban LOS | 3.0200 | 2.5704 | 3.0903 | 3.2359 | 3.3113 | 3.6308 | 3.9811 | 4.2658 | 4.4668 |
| Suburban LOS | 8.5114 | 2.4547 | 1.9055 | 2.3442 | 2.8840 | 3.3113 | 3.8019 | 4.0738 | 4.2658 |
| Rural LOS | 0.2089 | 1.3804 | 2.5704 | 3.4674 | 4.3652 | 5.0119 | 5.4954 | 6.0256 | 6.1660 |

As for delay spread, small value is preferred considering high frequency.  Based on the RMS delay spread, for NLOS scenario, the DS is around 10ns for most of cases, and for LOS scenario, the DS is around with 5ns. Considering that, we would like to apply TDLA10 and TDLC5 for specifying PUSCH requirement above 10GHz,

As for doppler spread, since assuming UE will do the doppler frequency tracking and pre-compensation, then, only the UE speed will be included for doppler spread calculation. Based on frequency range for above 10GHz in Ka-band as following, up to 30GHz is supported in UL.

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN1 | 410 MHz – 7125 MHz |
| FR2-NTN2 | 17300 MHz – 30000 MHz |
| NOTE 1: [NTN bands within this frequency range are regarded as a FR1 band when references from other specifications.] NOTE 2: [NTN bands within this frequency range are regarded as a FR2 band when references from other specifications.] |  |

Assuming the 0.1 ppm for residual Frequency error after UL pre-compensation, the related frequency offset is around 3000Hz.

Proposal 21: The following channel model can be considered for specifying BS demodulation as starting point

- [NTN-TDLA10-3000]

- [NTN-TDLC5-3000]

## 3.3.1	Test setup of PUSCH requirement above 10GHz

For other test parameters, the existing test parameters for specifying PUSCH requirement in FR2 can be reused as following

Proposal 22: The following test parameters could be considered for specifying PUSCH requirement for above 10GHz

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0, 1} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain | RB assignment | Full applicable test bandwidth |
| resource | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2, |
| configuration | Time density (LPT-RS) | 1, |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns |  |  |

## 3.3.2	Test setup of PUCCH requirement above 10GHz

In Rel-17 NTN WI, the PUCCH requirements with all the formats were considered for NTN-specific UE. For above 10GHz, the same PUCCH formats can be considered for PUCCH requirement. The same test parameters can be reused for requirement definition.

Proposal 23: The following test parameters could be considered for specifying PUCCH requirement for above 10GHz

Table 3:  Test parameters of PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |
| Test metric | DTX to ACK probability ACK missed detection probability |
| Channel Model | [NTN-TDLA10-3000] |

Table 4:  Test parameters of PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Test metric | NACK to ACK probability ACK missed detection probability |

Table 5   Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Starting RB location | 0 |
| Intra-slot frequency hopping | N/A |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Test metric | DTX to ACK probability ACK missed detection probability |

Table 6:  Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER |

Table 7:  Test parameters of PUCCH format 3

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |
| Test metric | BLER |  |

Table 8:  Test parameters of PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Test metric | BLER |

## 3.3.3	Test setup of PRACH requirement above 10GHz

PRACH formats

In Rel-17 NTN WI, the PRACH requirements with long short formats 0 and 2, short formats B4 and C2, were considered for NTN-specific UE. For above 10GHz, since the supported SCS is 60KHz and 120KHz, only short PRACH formats can be considered, where short format B4 and C2 can be selected as a starting point.

Test preamble configuration

Regarding the test preamble configuration, the existing value in Rel-15 FR2 can be reused as following

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
|  | 60 | 69 | 0 | 0 |
| B4, C2 | 120 | 69 | 0 | 0 |

Frequency offset

Regarding the frequency offset, the value in both AWGN and fading are considered. For AWGN, the corresponding frequency offset should be 0. For fading channel, assuming UE will do the pre-compensation, the residual frequency offset is 0.1ppm. Given the carrier frequency is 30GHz in uplink, then, the frequency offset can be calculated as 3000Hz.

Time error tolerance

Similar as Rel-15, the time error tolerance can be calculated as: 0.522μ+Tdelay where Tdelay is the largest delay of the propagation channel. The detail value of Tdelay pending on the selected channel model

In summary, the following test parameters could be considered for specifying PRACH requirement above 10GHz

- PRACH format: C2 and B4

- Test preamble configuration

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
|  | 60 | 69 | 0 | 0 |
| B4, C2 | 120 | 69 | 0 | 0 |

- Frequency offset

  - AWGN: 0Hz,

  - Fading: 3000Hz

- Time error tolerance

  - 0.522μ+Tdelay where Tdelay is the largest delay of the propagation channel

- Channel Model: [NTN-TDLA10-3000]

Proposal 24: The following test parameters could be considered for specifying PRACH requirement for above 10GHz

- PRACH format: C2 and B4

- Test preamble configuration

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
|  | 60 | 69 | 0 | 0 |
| B4, C2 | 120 | 69 | 0 | 0 |

- Frequency offset

  - AWGN: 0Hz,

  - Fading: 3000Hz

- Time error tolerance

  - 0.522μ+Tdelay where Tdelay is the largest delay of the propagation channel

- Channel Model: [NTN-TDLA10-3000]

# 4	Conclusion

In this contribution, the view on the test scope of BS demodulation requirement for NTN enhancement is provided.

Test scope

Proposal 1: PUCCH requirement with Msg4 HARQ-ACK with repetition could be introduced

Proposal 2: PUSCH requirement with DMRS bundling can be introduced for NTN-specific UE

Proposal 3: BS demodulation requirement above 10GHz bands could be introduced. Only focus on the basic requirement based on features introduced for TN in Rel-15

Proposal 4: No BS demodulation introduced related with Network verified UE location

Proposal 5: No BS demodulation requirement introduced for NTN-TN and NTN-NTN mobility and service continuity enhancement

Test setup for PUCCH requirement for Msg4 HARQ-ACK

Proposal 6: Only PUCCH format 1 requirement should be specified

Proposal 7: Introduce multi-slot PUCCH requirement with format 1 under 2 repetition for NTN-TDLC5-200 channel

Proposal 8: Introduce PUCCH requirement with both 1Tx1Rx and 1Tx2Rx for NTN-specific UE with repetition. Apply the same test applicability for 1T1Rx and 1Tx2Rx performance test

Proposal 9: Using the following test parameters for specifying PUCCH requirement for Msg4 HARQ-ACK

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |

Test setup for PUSCH requirement with DMRS bundling

Proposal 10: FFS on introduce the timing drift model for specifying the PUSCH requirement with DMRS bundling in NTN scenario.

Proposal 11: The following value for actual TDW and nominal TDW can be considered for PUSCH requirement with DMRS bundling in NTN scenario

- Actual TDW and PUSCH-TimeDomainWindowLength = 2 or 4 for FDD

- Actual TDW and PUSCH-TimeDomainWindowLength 2 for TDD

Proposal 12: The following PRB allocation can be considered for PUSCH requirement with DMRS bundling in NTN scenario

- PRB: 6RBs for both 15KHz and 30KHz

Proposal 13: Introduce PUSCH requirement with mapping type A for NTN-specific UE with DMRS bundling

Proposal 14: Introduce PUSCH requirement with CP-OFDM waveform for NTN-specific UE with DMRS bundling

Proposal 15: Introduce PUSCH requirement with both 1Tx1Rx and 1Tx2Rx for NTN-specific UE with DMRS bundling. Apply the same test applicability for 1T1Rx and 1Tx2Rx performance test

Proposal 16: Introduce PUSCH requirement for NTN-specific UE with 1+1 DMRS symbols.

Proposal 17: Apply the existing test parameters for specifying PUSCH requirement with DMRS bundling for NTN-specific UE with DMRS bundling

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Example UL-DL pattern [Note 1] |  | 15 kHz SCS: FDD and TDD 7D1S2U, S=6D:4G:4U 30 kHz SCS: FDD and TDD 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence [Note 2] | 0, 3, 0, 3 for FDD 0, 3, 0, 3 for TDD |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | [n2] or [n8] for FDD n2 for TDD |
| pusch-TimeDomainWindowLength |  | [2] or [4] slots for FDD 2 slots for TDD |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The same TDD requirements are applicable to different UL-DL patterns with more than one consecutive UL slots when both pusch-TimeDomainWindowLength and PUSCH aggregation factor are configured as 2 slots. The UL (re)transmission of PUSCH is only scheduled for the actual TDW including 2 consecutive UL slots. Note 2: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

Test setup for BS demodulation  requirement above 10GHz

Proposal 18: The following SCS and CBW can be considered for specifying PUSCH requirements above 10GHz

- 60 KHz SCS, 50MHz

- 120 KHz SCS, 50MHz

Proposal 19: Introduce SAN demodulation requirement with FDD only

Proposal 20: Prioritize to define SAN demodulation requirement for Type 2-O. FFS to define BS demodulation requirement for Type 2-H, pending on conclusion of RF core requirement

Proposal 21: The following channel model can be considered for specifying BS demodulation as starting point

- [NTN-TDLA10-3000]

- [NTN-TDLC5-3000]

Proposal 22: The following test parameters could be considered for specifying PUSCH requirement for above 10GHz

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0, 1} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain | RB assignment | Full applicable test bandwidth |
| resource | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2, |
| configuration | Time density (LPT-RS) | 1, |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns |  |  |

Proposal 23: The following test parameters could be considered for specifying PUCCH requirement for above 10GHz

Table 3:  Test parameters of PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |
| Test metric | DTX to ACK probability ACK missed detection probability |
| Channel Model | [NTN-TDLA10-3000] |

Table 4:  Test parameters of PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Test metric | NACK to ACK probability ACK missed detection probability |

Table 5   Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Starting RB location | 0 |
| Intra-slot frequency hopping | N/A |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Test metric | DTX to ACK probability ACK missed detection probability |

Table 6:  Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER |

Table 7:  Test parameters of PUCCH format 3

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |
| Test metric | BLER |  |

Table 8:  Test parameters of PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Test metric | BLER |

Proposal 24: The following test parameters could be considered for specifying PRACH requirement for above 10GHz

- PRACH format: C2 and B4

- Test preamble configuration

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
|  | 60 | 69 | 0 | 0 |
| B4, C2 | 120 | 69 | 0 | 0 |

- Frequency offset

  - AWGN: 0Hz,

  - Fading: 3000Hz

- Time error tolerance

  - 0.522μ+Tdelay where Tdelay is the largest delay of the propagation channel

- Channel Model: [NTN-TDLA10-3000]

# References

RP-230809, WID on NR NTN (Non-Terrestrial Networks) enhancements