3GPP TSG-RAN WG4 Meeting #108-bis 	R4-2316299

Xiamen, China, October 9-13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TR 38.858: Feasibility of FR1 Local Area BS

Agenda item:	5.19.2.2.3

Document for:	Approval

Introduction

In RAN4#108, big TP of TR 38.858 was agreed in R4-2314005 presenting FR1 BS feasibility aspects. This contribution presents a TP with Nokia’s view on FR1 local area BS feasibility.

- Consider the following text proposal into the TR 38.858.

TP to TR 38.858 on Feasibility of FR1 local area BS aspects

<<Start of Change for TR 38.858>>

## 9.4	Feasibility of FR1 local area BS aspects

### 9.4.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results

#### 9.4.1.1	Summary table for self-interference analysis

Table 9.4.1.1-1 summarizes the self-interference analysis results from companies.

Table 9.4.1.1-1: SI feasibility study for FR1 Local Area BS

| FR1 |  |  |  | Ericsson (preliminary) |  | ZTE | CATT | Nokia |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Local Area BS (3GPP minimum) | Local Area BS (Realistic RX) | LA | LA | LA |
| BS TX Power dBPTX = ① dBm |  |  |  | 24 | 24 | 23 | 24 | 23 |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 | 45 | 51 | 45 | 45 |
|  |  | Frequency isolation techniques used |  | Digital filtering, CFR, DPD |  | DPD utilized | CFR | Digital filtering or windowing to clean UL sub-band; DPD to suppress PA distortion |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 70 | 70 | 60 | 70 | 60 (if omnidirectional antennas are used, this would be less) |
|  |  | Spatial isolation techniques used |  | Physical distance, isolation structures |  | TX/RX panel separation | TX/RX panel separation, isolation structures, isolation material, cross polarization | Spatial separation between TX/RX panels; EM shielding structures between TX/RX panels |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 0 | 0 | 0 | 0 | 0 |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | TX beam nulling not assumed due to array size |  | N/A | TX Beam nulling is not assumed due to the array size | TX Beam nulling is not assumed due to the array size |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -91 | -91 | -101 | -104.01 | -88 (①-②-③-④-10*log10(80/20)) |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0 | 0 | 20 | 0 | 0 |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0 | 0 | 0 | 0 | 0 |
|  |  | RF IC techniques and other tech. (before LNA) |  | Analogue IC could be considered for this case, but is restrictive on pre-coding and multi-carrier. Digital IC has instead been assumed. |  | subband filtering |  | 0 |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | 0 | 0 |  | 0 | 0 |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -46 | -46 | -50 | -46 | -37 |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | xxx dBc |  | 60 | 55 |  |
|  |  | Frequency isolation techniques |  | Digital IC of TX. The impact of scattering / reflection in the environment has not been considered. It is assumed that RX ACS is very large due to time alignment and achieving orthogonality between the TX and RX signals in the digital domain. |  | Digital Filtering | Digital filter for ACS |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -24.6 | -14 | -5 | -16 | -10 |
|  |  |  | Rx IM3 contribution (dBm) | -88.8 | -110 | -140 | -106 | Negligible |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | No other significant impacts other than those mentioned above |  | Marginal | No other significant impacts | No other significant impacts |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -88.8 | -110 | -110 | -101 | -87 dBm (at 50 dB ACS) |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 0-3 dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  | 0 | 0 | 0 |
|  | RX sensitivity degradation caused by RX beam nulling |  |  |  |  | N/A |  |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 15 dBc on transmitter |  | 20 | 10 | 5 to 10 |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 112 dBc | 128 dBc | 130.5 | 122.33 | 110 to 115 |
| Noise floor ⑩dBm |  |  |  | -87 dBm/CBW | -87 dBm/CBW | -91.0 | -87.99 | -91.0 |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -93 dBm | -93 dBm | -97.0 | -93.99 | -97.00 |
| Required RSIC budget (①-⑪dBc) |  |  |  | 117 dBc | 117 dBc | 127.0 | 117.99 | 120.0 |
| SBFD configuration |  |  |  | 40-20-40 |  | 40-20-40 | 40-20-40 | 40-20-40 |
| Guardband assumption (if exist) |  |  |  | 5 PRB |  |  |  | 5 PRB |
| bandwidth over which suppression is achieved |  |  |  | <300MHz |  |  |  |  |
| Others |  |  |  | The conclusion does not take into account interference increase due to scattering effects, or the possibility for receiver algorihms to mitigate scattering, inter-sector and inter-site and self-interference (reference scenarios needed). |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |  |  |  |

#### 9.4.1.2	Feasibility study on self-interference

<<Omitted text>>

##### 9.4.1.2.x Nokia

The Nokia input in the Table 9.4.1.1-1 presents the company’s view on the self-interference mitigation analysis for a local area base station with 24 dBm total output power. The RSIC capability corresponds to 110 to 115 dBc for the Rx which is achieved using a combination of spatial isolation (60 dBc), frequency separation (45 dBc), and digital IC techniques (5-10 dBc). Under such considerations, the self-interference observed in the 20 MHz UL subband is dominated by transmitter ACLR and receiver selectivity with a combined magnitude of approximately -85 dBm prior to digital IC and -90 dBm to -95 dBm after digital IC, while the noise floor is -91.0 dBm. Even though there is a small desensitization of the receiver, it is feasible to operate the base station with acceptable performance as the coverage is not the primary target of local area deployments where the UL link budget and received UL power is generally very high.

A detailed description of some of the key assumptions is provided below.

Spatial isolation and beam nulling

Approximately 60 dB of spatial isolation can be achieved for ceiling- or wall-mounted base stations with directive antenna radiation patterns although at the expense of larger form-factor to reach sufficient physical separation between Tx and Rx. For typical LA base stations with omni-directional antennas, the spatial isolation would be significantly lower, e.g., 30 dB or less, thus SBFD operation would be much more challenging or potentially unfeasible.

Tx beam nulling is not assumed due to relatively low number of TRXs.

Digital Interference cancellation techniques

Since the number of TRX is significantly lowered compared to mMIMO, there is a possibility for digital IC to be feasible. Digital IC can be achieved by improving the DPD in the transmitter side to reduce the unwanted transmitter leakage components that fall in the UL subband, while receiver-side digital IC techniques are also required to supress the self-interference signal in gNB RX subband caused by non-ideal RX selectivity.

<<End of Change>>

Conclusions

This contribution presented a TP with Nokia’s view on FR1 local area BS feasibility.

- Consider the following text proposal into the TR 38.858.