3GPP TSG-RAN WG4 Meeting #108bis									                              R4- 2316521

Xiamen, China, Oct. 09 – 13, 2023

Agenda item:			5.34.1

Source:	Ericsson

Title:	Discussion on NES BS RF requirement

Document for:	Discussion

# Introduction

Network energy savings WI has been approved in RAN#98, and since then it has been discussed in multiple meetings in RAN1/2/3. Implementing the energy-saving features may impact the RF requirements of BSs and UEs, e.g., on network performance and user experience. In the last RAN4 meeting, various topics related NES RF were discussed, and this paper continues the discussion on the TAE requirement and other matters concerning the BS/UE.

# Discussion

During the last meeting, RAN4 agreed on the following WF [1][2].

RF agreement

Issue 1-1: TAE

Agreement:

- Alternative #1:

  - Do not specify the BS TAE requirements of SSB-less operation for FR1 co-located inter-band CA

  - Define the side condition of RTD to ensure UE performance in RRM part.

- Alternative #2:

  - Specify BS TAE requirements

    - FFS on values

### Issue 1-4: other UE related considerations

Agreement:

- No new UE RF requirements are defined for R18 NES feature.

Partial RRM agreement related to RTD

Issue 1-2-1: RTD conditions for scenario 1 (SCell without SSB transmission and with TRS transmission)

- Agreements

    - Define requirements for RTD ≤ CP

      - note: the CP corresponding to the [largest SCS across CCs or SCell SCS]

    - FFS if additional requirements shall be considered for RTD ≤ 260ns

    - Performance requirements (RRM and/or demodulation) need to be introduced for the scenario with RTD ≤ CP to guarantee proper performance on the SCell

Issue 2-1-2: RRM impacts of Cell DTX/DRX – SSB based measurement

Agreement:

RAN4 to reuse the legacy requirement for SSB-based measurement in Cell DTX/DRX

## TAE of SSB-less operation for inter-band CA co-located cells in FR1

This session aims to address the TAE values for achieving the optimal SSB-less performance in the context of NES SSB-less operation for inter-band CA co-located in FR1.

To determine the required TAE for achieving optimal performance of the SSB-less feature, it is essential to consider the following factors:

- The duration of the CP associated with various SCS,

- The channel dispersion,

- The UL transmit timing uncertainties for both the serving cell and SSB cell,

- The time of arrival difference of DL signals

The UL timing uncertainties include DL reference time estimation and UL timing errors (Te), timing advance quantization, and timing advance adjustment accuracy. For simplicity, the UL timing uncertainties are denoted as below:

UL_TX_time_uncertanties  = Te+TAqnt+TAadj

Then the TAE is derived as below:

TAE<CP-Ch_disp-2*(Te+TAqnt+TAadj)-∆RF_propg_diff

Where, in the above equation, CP is the duration of CP at a certain SCS and Ch_disp is listed in TS 38.101-4 Table B.2.1.1-1. Te represents the initial transmission timing error of the UE, which serves as an indicator of the SSB BW width (intrinsic accuracy proportional to 1/SSB_BW), combined with the UE’s internal clock drift over the periodicity of the SSB block (160 ms) and additional margins. This value can be found in Table 7.1.2-1 of TS 38.133. ∆RF_propg_diff is the time of arrival difference of DL signals, when assuming co-location, the value is nearly 0, but it becomes greater than zero when in the inter-band scenario and frequency are far apart from each other.

TAqnt= +/- 8 Ts @ SCS = 15 kHz and is divided by 2 for each doubling of SCS freq. (Ts=115000*2048).

TAadj is the timing advancement adjustment accuracies which are listed in TS 38.133 Table 7.3.2.2-1

Table B.2.1.1-1: Delay profiles for NR channel models

| Model | Number of channel taps | Delay spread (r.m.s.) | Maximum excess tap delay (span) | Delay resolution |
| --- | --- | --- | --- | --- |
| TDLA30 | 12 | 30 ns | 290 ns | 5 ns |
| TDLB100 | 12 | 100 ns | 480 ns | 5 ns |
| TDLC300 | 12 | 300 ns | 2595 ns | 5 ns |
| TDLD30 | 10 | 30 ns | 375 ns | 5 ns |

Table 7.1.2-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | +/-Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2-1 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| 2-2 | 120 | 120 | 3.5*64*Tc |
|  |  | 480 | [1.58]*64*Tc |
|  | 480 | 120 | 2.86*64*Tc |
|  |  | 480 | [1.35]*64*Tc |
|  |  | 960 | [0.90]*64*Tc |
|  | 960 | 120 | 2.80*64*Tc |
|  |  | 480 | [1.13]*64*Tc |
|  |  | 960 | [0.86]*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 7.3.2.2-1: UE Timing Advance adjustment accuracy

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| --- | --- | --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc | ±10 Tc | ±6 Tc |

It should be noted that even for co-located scenarios, RF propagation may vary across different bands due to distinct channel characteristics. Furthermore, this variation is dependent upon the deployment environment. When there is a significant frequency gap in FR1 inter-band frequencies, distinct delay spread channels emerge.​

Typical-case scenario

In the case where the serving cell (PCell) and SSB-less cell (SCell) use different SCS, the tightest TAE occurs when the SSB-less cell operating at higher SCS (i.e., 30 kHz) leads to a reduced CP duration, and the UE experiences a larger UL timing error at the serving cell with smaller SCS. (i.e., 15 kHz).

The TAE is determined by selecting the values corresponding to the typical-case scenario from the tables mentioned above, and it is as follows:

Note: the maximum excess tap delay accounts for the furthest and weakest reflex, and is considered as ‘not typical case’, therefore RMS spread delay is recommended to use in the formula instead.

However, we do not always have the tightest TAE combination of inaccurate DL timing due to SCS = 15 kHz narrow SSB bandwidth and small SCell UL SCS = 30 kHz. In practice, there are other SSB and UL SCS combinations. They are shown in Table 1 below, with all units in seconds:

Table 1 TAE values for different SSB SCS and UL SCS combinations.

| SSB/UL | CP | CH_disp | Te | TAqnt | TAadj | ∆RF_propg_diff | TAE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15/15 | 4,69E-06 | 1,00E-07 | 3,91E-07 | 2,60E-07 | 1,30E-07 | 0,00E+00 | 3,03E-06 |
| 15/30 | 2,34E-06 | 1,00E-07 | 3,91E-07 | 2,60E-07 | 1,30E-07 | 0,00E+00 | 6,81E-07 |
| 30/15 | 4,69E-06 | 1,00E-07 | 2,60E-07 | 1,30E-07 | 1,30E-07 | 0,00E+00 | 3,55E-06 |
| 30/30 | 2,34E-06 | 1,00E-07 | 2,60E-07 | 1,30E-07 | 1,30E-07 | 0,00E+00 | 1,20E-06 |

- It is difficult to specify and mandate a single fixed TAE value as the required TAE depends on actual deployment, RF characteristics, and UE relative position between transmit and receive points.

The necessary TAE depends on assumed deployment and many other parameters we’ve previously explored. These factors encompass diverse SCS combinations and corresponding CP durations, and UE UL timing errors used for both PCell and SCell, variations in RF propagation, and the presence of delay spread. Consequently, it should not be represented as a single, static value but rather as a dynamic and multifaceted parameter. The TAE values vary with different SCS combinations are listed in Table 2:

Table 2 TAE values summary for different SSB SCS and UL SCS combinations.

| SCS of SSB (kHz) | SCS of UL (kHz) | TAE |
| --- | --- | --- |
| 15 | 15 | 3.0 µs |
| 15 | 30 | 680 ns |
| 30 | 15 | 3.55 µs |
| 30 | 30 | 1.2 µs |

From Rel-15, the NW has the capability to configure SCell to be DL only or DL + UL. If the NW would like to configure SSB-less SCell to be DL-only, then the TAE value would be less stringent, as the UL timing uncertainties would not apply.

Moreover, in the RRM session, consensus was reached to define the RTD conditions for Scenario 1 within the CP duration. This consensus closely mirrored the proposal from the RF session, specifically, alternative 1 which does not specify the BS TAE requirement and define the side conditions of RTD to ensure UE performance in RRM part.

- Do not specify the BS TAE requirement for SSB-less operation for FR1 co-located inter-band CA.

## Cell DTX

Regarding the Cell DTX, RAN2 and RRM have made the following agreements which are relevant to RAN4 discussion:

1 Activation/deactivation is per serving cell. FFS if the configuration is per cell or per MAC entity 2 RAN2 will reuse the start timer formula of the onDurationTimer from UE C-DRX (including SlotOffset) to specify the start of cellDTX-onDurationTimer (and cellDRX-onDurationTimer) in 38.321. 3 The gNB should ensures that there is at least partial overlapping between UE C-DRX on-duration and cell DTX/DRX on-duration. It is up to network implementation to ensure the alignment. We will capture this in stage 2 specification. Understanding is that alignment means that the cell DTX/DRX and C-DRX periodicity should be multiple of each other. FFS if we anything needs to be specified in stage 3 (i.e. in IE description) 4 As a baseline legacy C-DRX reconfiguration is used to change UE C-DRX configuration once Cell DTX/DRX is activated/deactivated. 5 RAN2 specifies cellDTX-onDurationTimer (and cellDRX-onDurationTimer) to have the same value range as UE C-DRX on-duration timer. 6 RAN2 specifies cellDTX-Cycle (and cellDRX-Cycle) to have the same value range as UE C-DRX Long cycle. 7 Separate DTX and DRX configuration means that the features can be enabled separately (i.e. Cell DTX can be configured without Cell DRX) 8 On-duration and Cycle parameters are common between cell DTX and DRX, when both are configured. FFS if we have different start offset configuration for cell DTX and cell DRX 9 RAN2 will not introduce a MAC CE for cell DTX/DRX (de)activation. 10 Confirm working assumption, when the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy. It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX. 11 We focus on the case where DTX in RRC can only be configured when C-DRX is configured. We will not optimize for the case where C-DRX is not configured.

Issue 2-1-2: RRM impacts of Cell DTX/DRX – SSB based measurement Agreement: RAN4 to reuse the legacy requirement for SSB-based measurement in Cell DTX/DRX

Cell DTX is a power-saving technique that involves various hardware and software mechanisms. For hardware part, it may involve turning off the PA, transceiver module which includes other components within the transceiver, such as mixers, filters, and oscillators. In the current BS specification TS38.104, there are guidelines set for how long it should take for a transmitter to transition when the transmission/reception direction switches between UL and DL. This transient period accounts for the time it takes to turn off one path (including aforenoted hardware components) and activate the other path.

Based on the agreements in other WGs, we believe the mentioned transient period requirement can also enable the operation of cell DTX.

- The current transient period requirement can enable the cell DTX operation, and it is not necessary to define the new requirement for cell DTX.

## Techniques in spatial and power domain

Some RAN1 agreements regarding the spatial and power domain techniques are listed below:

For the purpose of further discussions in RAN1 on NES spatial domain adaptations, consider the following cases

- Type 1: all antenna elements associated to a logical antenna port is disabled/enabled

- Type 2: part/subset of antenna elements associated to a logical antenna port is disabled/enabled

This implies that the techniques will dynamically activate or deactivate the chosen antenna ports from gNB. How many ports will be deactivated depends on different deployments, loads, and UE coverage scenarios. Based on the current agreements reached in RAN1, it is proposed not to define any new RF requirement.

- Considering the current agreements in other WGs, it is recommended not to introduce any new RF requirements for spatial and power domain techniques for NES.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	It is difficult to specify and mandate a single fixed TAE value as the required TAE depends on actual deployment, RF characteristics, and UE relative position between transmit and receive points.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Do not specify the BS TAE requirement for SSB-less operation for FR1 co-located inter-band CA.

Proposal 2	The current transient period requirement can enable the cell DTX operation, and it is not necessary to define the new requirement for cell DTX.

Proposal 3	Considering the current agreements in other WGs, it is recommended not to introduce any new RF requirements for spatial and power domain techniques for NES.

# References

[1] R4-2314935 WF on RF requirements for NES

[2] R4-2314381 WF on RRM requirements for NR network energy saving