3GPP TSG-RAN WG4 Meeting # 108-bis									          R4-2316769

Xiamen, China, 9th October – 13th October, 2023

Title: 	CA_n34-n40 MSD

Agenda Item:	4.1.1.2

Source: 	Skyworks Solutions, Inc.,

Document for:	Approval

# 1	Background

This contribution proposes power class 3 (PC3) cross-band isolation MSD requirements for CA_n34-n40 based on power amplifier (PA) measurements.

# Discussion

## 2.1 Power Amplifier Calibration

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge with 1 dB MPR- PC3 ACLR -30 dBc

- Post PA losses: 4 dB

- Power Class 3 (PC3) operation

- Local Oscillator (LO) leakage: -28 dBc

- IQ Image rejection: -28 dB

## 2.2 Measurement Results and MSD Proposal

The cross-band isolation MSD of Band n34 and Band n40 is evaluated using the following parameters:

- PC3 operation, Tx power level: 23 dBm

- Post-PA insertion losses: 4 dB

- Primary antenna to diversity antenna isolation: 10 dB

- MRC combining and -1 dB Signal-to-Noise Ratio (SNR)

- Band n34 Tx filter rejection at n40 DL frequency range: 30 dB

- Band n40 Tx filter rejection at n34 DL frequency range: 32 dB

- Band n34 PA + RF transceiver noise floor at n40 Rx frequency range: -125 dBm/Hz

PA noise levels in DL frequency range:

- Band n34 15 MHz UL CBW Lcrb=75(RBstart=4) at n40 Rx: -125 dBm/Hz, i.e. -58.4 dBm/MBW

- Band n40 100MHz UL CBW Lcrb=270(RBstart=0) at n34 Rx: -37.5 dBm/MBW

The MSD analyses are summarized in Figure 1 below:

- The Band n40 5MHz CBW MSD is estimated at 3.2 dB (Figure 1-left)

- The Band n34 5MHz CBW MSD is estimated at 19.5 dB (Figure 1-right)

Figure 1: Cross-band isolation MSD analysis for Band n40 (left) and n34 (right) for CA_n34-n40

Proposal: Adopt the Band n34 and Band n40 PC3 cross-band isolation MSD test points of Table 1 below.

Table 1: Band n34 and Band n40 PC3 cross-band isolation MSD requirements

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n34 | n40 | 2017.5 | 15 | 15 | 75 (RBstart=4) | 2302.5 | 5 | 3.2 | >ACLR2 |
| n40 | n34 | 2350 | 100 | 30 | 270 (RBstart=0) | 2022.5 | 5 | 19.5 | >ACLR2 |

# Conclusion

In this contribution, we have presented measurements and calculations for CA_n34-n40 PC3 MSD due to cross-band isolation. We make the following proposal.

Proposal: Adopt the Band n34 and Band n40 PC3 cross-band isolation MSD test points of Table 1 below

Table 1: Band n34 and Band n40 PC3 cross-band isolation MSD requirements

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n34 | n40 | 2017.5 | 15 | 15 | 75 (RBstart=4) | 2302.5 | 5 | 3.2 | >ACLR2 |
| n40 | n34 | 2350 | 100 | 30 | 270 (RBstart=0) | 2022.5 | 5 | 19.5 | >ACLR2 |

# References