3GPP TSG-RAN WG4 Meeting #108bis					R4-2316957

Xiamen, China, October 09 – October 13, 2023

# Title: 	Ad-hoc meeting minutes for FS_NR_duplex_evo_Part3

# Agenda Item:	5.19.4

# Source: 	CMCC

# Document for:	Approval

- Introduction

This document captures the outcome and agreements after Ad-hoc meeting for FS_NR_duplex_evo_Part3.

- Statement in TR 38.828

## 2.1	Statement to be included in the TR regarding dynamic TDD  and SBFD flexible symbols

Agreement:

Dynamic TDD/ SBFD operation in symbols configured as Flexible symbols

RAN4, did not conduct any simulations with dynamic TDD [in symbols configured as flexible] in Rel-18 SI as similar discussions took place under Rel-16 and were captured in TR 38.828. The following recommendations were made in section 6.3.1.1 of TR 38.828

-    Concerning Urban Macro to Urban Macro scenario in FR1,  “Performance degradation was observed from the BS-to-BS interference for macro-macro scenario, which suggests that dynamic TDD should not be operated in such scenarios.”

-  	Concerning indoor network and a macro network scenario in FR1 and vice versa, “Performance degradations were not observed from operating dynamic TDD between an indoor network and a macro network and vice versa if there is sufficient isolation between them. No significant impact from operating dynamic TDD for the indoor scenario was observed as long as the BS and UE powers are similar and the operator’s co-ordinate so that base station positions are offset. If higher BS power is assumed, some throughput degradation in the indoor scenario was observed due to BS-to-BS interference. The observations imply that dynamic TDD can be used in indoors as long as care is taken.”

The recommendations for dynamic TDD in TR38.828 are still valid and should be taken into consideration.

Since SBFD operation in symbols configured as flexible can be considered as a specific case of dynamic TDD, Any mitigation techniques of dynamic TDD to address adjacent channel interference can apply to SBFD operation in symbols configured as flexible.

  - 2.2	Skeleton of co-existence part in TR

Way forward for further discussion:

- Proposals 1: final TR will include two parts, one for summary of all simulation analysis/results which mainly capture the excel file and word file as moderator provided in inbox and the other one for conclusion part. Detailed are listed as below:

- 11 Adjacent channel co-existence evaluation results. - 11.1 Introduction Editor's note: This section will capture adjacent channel co-existence simulation scenarios and cases. Besides, some brief introduction of what RAN4 have done in R18 for SBFD. - 11.2 Summary of all simulation results Editor's note: This section will capture the excel files and word file that moderator summarized based on all companies’ input with some description of these files. One example: We list three files. First excel file is the collection of all simulation results using the template format that companies use to provide simulation results. Word file is the summary of all simulation results categorized in terms of scenario, case, victim and parameters values of four simulation assumptions, i.e. antenna configuration 1 or 2, gNB Tx power value, grid shift value and enhanced NF or not. In addition to, we list the min value, max value and median for relative ACIR, ACIR enhanced by 2, 4, 6, and 8dB. We also list the numbers of companies that show simulation results for each scenario, each case and each victim band. Second excel file is the excel file of word file which is much straightforward compared with the word. [placeholder for clarifying text on ACIR enhancement definition for simulation and results interpretation] - 11.3 Conclusion Editor's note: This section will capture adjacent channel co-existence simulation results only categorized by cases and scenarios. Besides, for each case if the conclusions are much similar among some/all scenarios. Final conclusion would be merged together among these scenarios.

  - 2.3	Statement in TR based on simulation conclusion

Way forward for further discussion (already captured in conclusion part):

- Proposals

  - Option 1: It should be captured in the TR that even with degradation in the urban macro SBFD UL as a victim case,  latency reduction gains and UL coverage enhancements are expected with SBFD deployments. (Qualcomm)

  - Option 2: To protect the legacy TDD network from the detrimental BS-to-BS ACI introduced by SBFD, SBFD operation should only be considered in the legacy DL slot. (Cablelabs, Nokia, Charter Communications Inc, Spark, Ericsson, Korea Testing Lab)

- Recommended WF

  - At first focus on the simulation conclusion, after that we discuss above proposals one by one.

- Way forward on adjacent channel simulation assumption for SBFD

## 3.1	Sub-topic 1-3 Updating blocking model input power calculation formula

Issue 1-3-1: Blocking model input power calculation as below

Previous agreements:

The receiver input total power can be expressed as a sum of wanted signal and all interferer signals in linear scale as: Pin=Pw+Iintra-SB, inter-UE+Iinter-SB,inter-gNB+IACI+Iself+Ico-site,inter-sector IACI is the adjacent channel interference from gNB transmitting on the DL channel in the aggressor network. The interference level is calculated as: IACI=PTX,BSCLadjacent network gNB→serving gNB

- Proposals

- Option 1: IACI is the sum of the adjacent channel interference from gNB transmitting on the DL channel in the aggressor network and the adjacent channel interference from UEs transmitting on the UL channel in the aggressor network. The interference level is calculated as:(Nokia)

IACI=PTX,BSCLadjacent network gNB→serving gNB+PTX,UECLadjacent network UE→serving gNB

## 3.2 Sub-topic 1-4 Scenarios

Issue 1-4-1: how to consider case 3 and 4 in scenario 4

- Proposals

  - Option 1: To clarify the understanding of Case 3 and Case 4 under Scenario 4, as it contradicts with agreed WF R4-2305922. Further discuss whether or not, or how, to capture the results submitted to Case 3 and Case 4 under Scenario 4. (Samsung)

Moderator note: 3 companies also show simulation results for case 3 and case 4 under scenario 4.

- Recommended WF

  - It’s suggested to maintain case 3 and case 4 since 3 companies have show simulation results.

Issue 1-4-2: how to treat scenario 7 into final TR

- Proposals

  - Option 1: In the TR, to clearly note the Scenario 7 was down-selected in both Chapter 11 and Annex E, and to not capture any results and observations for Scenario 7 in Chapter 11. (Samsung)

- Conclusion of simulation results

  - 4.1 Case 1: aggressor SBFD DU victim NR TDD DL (high priority)

Case 1 considers legacy TDD in DL slot as a victim while SBFD is operating in the adjacent channel for both and FR1 and FR2-1. The following can be summarized:

- For Urban macro -> Urban Macro deployments for both FR1 and FR2-1: No DL throughput degradation on the victim legacy TDD DL network for both average throughput and cell edge throughput is observed for different Tx powers, ranging (46dBm to 53 dBm for FR1 and 30 dBm for FR2-1), Grid shifts (5% to 100%), and antenna configuration (single and double panels for SBFD operation).

- For Urban hotspot -> Urban hotspot deployments for FR1: DL throughput degradation is observed only at cell edge due to inter-UE CLI for different grid shifts (100% and 10%) and gNB Tx powers (49 dBm to 53 dBm).   [There is no observed DL throughput degradation with enhanced ACIR by [8] dB.]

- For Indoor ->Indoor deployments for both FR1 and FR2-1: No DL throughput degradation for both average throughput and cell edge throughput is observed.

- For Urban Micro -> Urban Micro deployments for both FR1 and FR2-1 (i.e., Urban dense): No observed DL throughput degradation for both average throughput and cell edge throughput.

  - 4.2 Case 2: aggressor SBFD DU victim NR TDD UL (low priority)

For case 2 (SBFD as aggressor operation is in UL slots, and legacy TDD as victim) in FR1 outdoor scenarios (1, 2, 4, and 5), the TDD UL throughput has significant throughput degradation regardless of average throughput or cell edge throughput, regardless of the BS antenna configuration, regardless of BS TX power options.

- The cell edge throughput degradation is worse than the average throughput degradation.

- [The throughput degradation is due to the BS-to-BS ACI introduced by SBFD, which increases as grid shift (BS-to-BS distance) decreases except in the UMa-to-UMi scenario.] The throughput degradation in the UMa-to-UMi scenario firstly increases as the grid shift decreases, then decreases with the grid shift, this is due to the relative elevation angle between UMa and UMi base stations change with the grid shift and the BS antenna gain reduces as the grid shift increases.

- The throughput degradation increases with SBFD BS TX power.

- SBFD BS antenna configuration slightly impacts the throughput degradation. On one hand, antenna config 2 has narrower beamwidth compared with config 1, which reduces the probability of causing strong interference. On the other hand, antenna config 2 has 3 dB higher BS TX power compared with antenna config 1, which increases interference.

- ACIR enhancement of [TBD 2 to 8 dB] reduces the throughput degradation. Note that ACIR could be improved on the SBFD [gNB] devices, but it does not apply to legacy TDD [gNB] devices. In case 2, ACIR is dominated by the legacy TDD ACS, improving ACLR on SBFD BS does not materially impact ACIR or throughput degradation.

For case 2 in FR2-1 outdoor scenarios (scenarios 6 and 8), TDD UL throughput degradation is observed at cell edge, no strong degradation is observed for the average throughput.

For case 2 in FR1 and FR2-1-1 indoor scenario (scenarios 3 and 9), no TDD UL throughput degradation is observed.

  - 4.3 Case 3: aggressor NR TDD DL victim SBFD DU (high priority)

Case 3 considers SBFD DU as a victim while NR TDD is operating DL in the adjacent channel for both and FR1 and FR2-1. The following can be summarized:

Impact on SBFD DL:

- For all the scenarios of FR1 and FR2-1: No observed throughput degradation on the SBFD DL for both average throughput and cell edge throughput for different gNB Tx powers, ranging (46dBm to 53 dBm for FR1 and 30 dBm for FR2-1), Grid shifts (5% to 100%), and antenna configuration (single and double panels for SBFD operation).

Impact on SBFD UL:

- For FR1 Urban macro -> Urban macro: Under baseline assumptions, observed SBFD UL throughput degradation is observed only for cell edge throughput and [no degradation is observed for average throughput]; and the degradation can be compensated reduced by the increased  enhanced ACIR. With higher gNB Tx power and lower grid shifts, the degradation is increased for cell edge throughput and [average throughput].

- FR2-1 Dense urban -> Dense urban deployments: Under baseline assumptions, SBFD UL throughput degradation is observed only for cell edge throughput and [no degradation is observed for average throughput].

- For Indoor ->Indoor deployments for both FR1 and FR2-1: No SBFD UL throughput degradation for both average throughput and cell edge throughput is observed.

- For FR1 Urban micro -> Urban micro: Under FR1 Urban micro 38dBm Tx power assumption, no degradation on the SBFD UL is observed for both cell edge throughput and cell center average throughput. Throughput loss is observed with higher gNB Tx power (46dBm) and lower grid shifts.

- FR2-1 Urban macro -> Urban macro deployments: Under baseline assumptions, no degradation on the SBFD UL is observed for both cell edge throughput and cell center average throughput. Throughput loss is observed with higher gNB Tx power and lower grid shifts.

- For FR1 Urban hotspot -> Urban hotspot deployments: Under baseline assumptions, observed throughput degradation is observed at cell edge throughput and average throughput. due to inter-UE CLI; and the cell edge degradation can be compensated by the increased ACIR. With higher gNB Tx power and lower grid shifts, the degradation is increased.

  - 4.4 Case 4: aggressor NR TDD UL victim SBFD DU  (low priority)

Case 4 considers SBFD DU as a victim while NR TDD is operating UL in the adjacent channel for both FR1 and FR2-1. The following can be summarized:

- Except for Urban hotspot -> Urban hotspot deployments for FR1, for all other scenarios of FR1 and FR2-1: No observed throughput degradation on the SBFD DL and UL for both average throughput and cell edge throughput for different gNB Tx powers, ranging (46dBm to 53 dBm for FR1 and 30 dBm for FR2-1), Grid shifts (5% to 100%), and antenna configuration (single and double panels for SBFD operation).

- For Urban hotspot -> Urban hotspot deployments for FR1: some companies’ results show DL throughput degradation is observed only for cell edge throughput due to inter-UE CLI for different grid shifts (5% to 100%), gNB Tx powers (46dBm to 53 dBm) and for all antenna configurations. the less grid shift, the larger degradation due to shorter UE-to-UE distance. However more companies show that there is no observed interference for cell edge throughput and cell average throughput for 100% grid shift, 49dBm gNB Tx power and antenna configuration 2.

  - 4.5 General remarks on coexistence findings

[For the above cases where no throughput degradation has been observed, no additional coexistence measures are required for SBFD deployment. However, for other cases where throughput degradation has been observed, interference mitigation techniques might need to be considered.]