3GPP TSG-RAN WG4 Meeting # 108bis                                      R4-2317381

Xiamen, China, October 09 – October 13, 2023

Agenda Item:	5.22.3

Source: 	CATT

Title:	Updated simulation assumptions for sidelink positioning

Document for:	Approval

- Introduction

In this contribution, the link level simulation assumption for SL-PRS based RSTD, RSRP, Rx-Tx time difference, and RSRPP measurement is captured. Companies are encouraged to provide simulation results based on the agreed performance metric in section 3.

- Simulation assumptions

Table 1: General parameters

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1 |
| Anchor UEs configuration | 3 anchor UEs at distinct locations: <UE 1, UE 2, UE 3>, where UE 1 is the reference anchor UE |
| Synchronization of anchor UEs | • Synchronous with time shifts <0, 0, 3 us> |
| Data and CCH load in PRS symbols | no other cell transmissions in its positioning symbols, except PRS |
| Data and CCH load in non-PRS symbols | - 50% utilization in time - 100% RE utilization |
| DRX | OFF |
| Carrier frequency / BW / SCS | - 2 GHz - 15 kHz - 4 GHz - 30 kHz, 60 kHz |
| Propagation conditions [TS 38.101-4] | For SL-PRS based RSTD, RSRP and Rx-Tx time difference: AWGN, TDL-A (30 ns delay spread, 5Hz Doppler shift), TDL-B (100 ns delay spread, 200Hz Doppler shift). |
|  | For SL-PRS based RSRPP: Two-tap channel defined in 38.101-4 Annex B.2.4, a = 1, τd=0.45 µs and fD=5 Hz |
| Es/Iot for three anchor UEs (UE 1, UE 2, UE 3), [dB] | For SL-PRS based RSTD/Rx-Tx/RSRP/RSRPP: (-3, -6, -6), (0, -6, -6) , (0, -3, -3), (0, 0, 0) |
| Number of UE receive antennas | 2 |
| UE measurement bandwidth | SL PRS BW |
| Number of transmit SL-PRS antennas | 1 |
| Power boosting | No power boosting |
| NOTE: Companies are encouraged to check suitability of the parameters and the values. |  |

Table 2: SL-PRS transmission configuration parameters

| Parameter | Value |  |  |
| --- | --- | --- | --- |
| SCS, RB num | SCS (kHz) | RB num | Sample rate (Tc) |
|  | 15 | 48 | 128 |
|  |  | 96 | 64 |
|  | 30 | 24 | 128 |
|  |  | 48 | 64 |
|  |  | 96 | 32 |
|  | 60 | 24 | 64 |
| SL-PRS comb size (N) | 4 |  |  |
| SL-PRS number of symbols (M) | 4 (for full staggering),2 (for partial staggering) |  |  |
| Number of samples | 1, 2,3,4 |  |  |
| SL-PRS periodicity | 40ms |  |  |
| TOA estimation | Realistic |  |  |
| Path # | First path |  |  |
| NOTE 1: Companies are encouraged to check suitability of the parameters and the values. |  |  |  |

- Performance metric

At least the following performance characteristics are to be provided for SL-PRS RSTD:

- SL-PRS RSTD error CDFs for the 2 anchor UEs (UE 2 and UE 3)

- 5%-ile and 95%-ile of the SL-PRS RSTD errors

At least the following performance characteristics are to be provided for SL-PRS RSRP:

- SL-PRS RSRP error CDFs for 3 anchor UEs

- 5%-ile and 95%-ile of the SL-PRS RSRP errors for each anchor UE

At least the following performance characteristics are to be provided for SL-PRS RSRPP:

- SL-PRS RSRPP error CDFs for 3 anchor UEs

- 5%-ile and 95%-ile of the SL-PRS RSRPP errors for each anchor UE

At least the following performance characteristics are to be provided for SL-PRS Rx-Tx:

- TSL-UE-RX error CDFs for the 3 anchor UEs

- 90%-ile of the TSL-UE-RX errors for each anchor UE

In the above SL PRS based measurement,

- RSTD error = estimated RSTD – ideal RSTD (based on perfect channel knowledge).

- RSRP error = estimated RSRP – ideal RSRP (based on perfect channel knowledge).

- RSRPP error = estimated RSRPP – ideal RSRPP (refer to 10.1.38.2.1 of TS 38.133).

- Rx-Tx error = abs(estimated TSL-UE-RX – ideal TSL-UE-RX ) (based on perfect channel and UE location knowledge).

- References

- R4-2009127, “Link-level simulation assumptions for UE Rx-Tx time difference measurements”, Qualcomm.

- R4-2211061, “Simulation assumption for PRS-RSRPP”, Huawei.

- R4-2002287, “Updated link level simulation assumption for RSTD and RSRP”, Huawei.