3GPP TSG-RAN WG4 Meeting # 108bis 														R4-2317204

Xiamen, China, October 09 - October 13, 2023

Agenda item:			5.13.8

Source:	Moderator (CMCC)

Title:	Topic summary for [108bis][212] NR_ATG

Document for:	Information

Introduction

This summary focuses on RRM core requirements maintenance and RRM performance requirements for Rel-18 NR ATG, including agenda 5.13.5 and 5.13.6. The agreed way forward in previous meetings are R4-2314307, R4-2310159, R4-2306344, R4-2303226, R4-2220361, R4-2217256 and R4-2214737.

Recommendation of prioritized topics for online discussion

Issue 2-2: Tsearch in HO requirements for UE with antenna array

Issue 4-1: Sharing factor for L1 measurements

Issue 5-1: Phase antenna array impact for L3 measurements

Issue 5-2:  If any additional UE capability is needed for ATG UEs with [antenna arrays]

Issue 6-2-1: General test configuration

Issue 6-2-2: Test scope

Issue 6-2-5: Test method for UE with antenna array

Issue 6-2-6: whether to define test cases for TCI switching delay requirements

Issue 6-2-10: Channel model

Topic #1: General

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2315160 | CMCC | Proposal 1: Introduce the ATG UE feature of uplink time and frequency pre-compensation and timing relationship enhancements as follows: Feature group; Components; Prerequisite feature groups; Field name in TS 38.331 [2]; Parent IE in TS 38.331 [2]; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. Uplink Time and Frequency pre-compensation and timing relationship enhancements; Support of UE specific TA calculation based on its GNSS-acquired position and the indicated BS location. For TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE autonomous TA estimation) and closed (i.e., received TA commands) control loops Support of pre-compensation of the calculated TA in its uplink transmissions Support of frequency pre-compensation to counter shift the Doppler experienced on the service link Support of determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated Support of UE receiving cell-specific K_offset in system information; TBD; BandNR; No; No; An ATG UE is required to at least support UE specific TA and frequency calculation based at least on its GNSS-acquired position and the indicated BS location Note: This UE feature group is applicable only for ATG operating bands; Optional with capability signalling For UE supports NR communication via ATG, UE must indicate this FG is supported.. Proposal 2: Introduce the ATG UE feature of UE reporting of TA information and UE-specific K_offset as follows: Feature group; Components; Prerequisite feature groups; Field name in TS 38.331 [2]; Parent IE in TS 38.331 [2]; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. UE reporting of TA information; Support UE reporting of TA information; TBD [Uplink Time and Frequency pre-compensation and timing relationship enhancements]; TBD; BandNR; No; No; Note: The exact content of UE reporting of information about the TA pre-compensation is up to RAN2 Note: This UE feature group is applicable only for ATG operating bands; Optional with capability signalling. UE-specific K_offset; Support of reception of UE-specific K_offset via MAC-CE Support of determining the timing of PUSCH, PUCCH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset; TBD [Uplink Time and Frequency pre-compensation and timing relationship enhancements And UE reporting of information related to TA pre-compensation]; TBD; BandNR; No; No; Note: This UE feature group is applicable only for ATG operating bands; Optional with capability signalling. Proposal 3: Introduce the ATG UE feature of Increasing the number of HARQ processes and K1 range extension as follows: Feature group; Components; Prerequisite feature groups; Field name in TS 38.331 [2]; Parent IE in TS 38.331 [2]; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. Increasing the number of HARQ processes; The maximal supported HARQ process number is X for UL and Y for DL; TBD; BandNR; No; No; Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} Note: This UE feature group is applicable only for ATG operating bands; Optional with capability signalling. K1 range extension; Support of extended K1 value range of (0..31) for unpaired spectrum; TBD; BandNR; No; No; Note: This UE feature group is applicable only for ATG operating bands; Optional with capability signalling. Proposal 4: Introduce the ATG UE feature of Location based CHO, Event A4 based CHO and Location-based measurement report trigger as follows: Feature group; Components; Prerequisite feature groups; Field name in TS 38.331 [2]; Parent IE in TS 38.331 [2]; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. Location based CHO; Indicates whether the UE supports location based CHO; condHandover-r16 is set for ATG bands.; TBD; BandNR; No; No; UE shall set the capability value consistently for all ATG operating bands.; Optional with capability signalling. Event A4 based CHO; Indicates whether the UE supports Event A4 based CHO; condHandover-r16 is set for ATG bands.; TBD; BandNR; No; No; UE shall set the capability value consistently for all ATG operating bands.; Optional with capability signalling. Location-based measurement report trigger; Indicates whether the UE supports location-based triggered measurement reporting (i.e., event D1); TBD [Location based CHO]; TBD; MeasAndMobParametersCommon; No; No; Optional with capability signalling. Proposal 5: Introduce the ATG UE feature of SR triggered by a TA report and TA reporting during initial access as follows: Feature group; Components; Prerequisite feature groups; Field name in TS 38.331 [2]; Parent IE in TS 38.331 [2]; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. SR triggered by a TA report; Indicates whether the UE supports triggering of SR when a TA report is triggered and there are no available UL-SCH resources.; TBD; MAC-ParametersCommon; No; No; Optional with capability signalling. TA reporting during initial access; It is mandatory to support TA reporting during initial access for UEs supporting uplink-TA-Reporting-r17 as specified in TS 38.321 [10].; n/a; n/a; n/a; n/a; Conditional mandatory without capability signalling. Proposal 6: Introduce the ATG UE feature of RRC_INACTIVE in ATG as follows: Feature group; Components; Prerequisite feature groups; Field name in TS 38.331 [2]; Parent IE in TS 38.331 [2]; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. RRC_INACTIVE in ATG; Indicates whether the UE supports RRC_INACTIVE in ATG; TBD; TBD; No; No; Conditional mandatory with capability signalling. Feature group; Components; Prerequisite feature groups; Field name in TS 38.331 [2]; Parent IE in TS 38.331 [2]; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. Enhanced RRM requirements for measurements in IDLE and INACTIVE modes; If UE does not support the capability, other ATG measurement requirements (as specified in TS 38.133, clause 4.2X.2) are applied.; n/a; n/a; No; No; Optional without capability signaling. Proposal 7: Introduce the ATG UE feature of Enhanced RRM requirements for measurements in IDLE and INACTIVE modes as follows: Proposal 8: For the ATG requirements, revise the current ATG clauses which could directly refer to legacy clauses with additional note (if needed) to clarify that the legacy requirements shall apply only for FR1 and NR SA operation mode without considering CA/DC and inter RAT measurement. Such kind of ATG Clauses including: 4.2X.2.5 Maximum interruption in paging reception 6.1X.2.2.3 Preparation time 6.1X.2.2.4 Interruption time 6.2X.2.2 Requirements for 4-step RA type 6.2X.2.3 Requirements for 2-step RA type 8.1X.4 Minimum requirement at transitions 8.1X.5 Minimum requirement for UE turning off the transmitter 8.1X.6 Minimum requirement for L1 indication 8.5X.4 Minimum requirement for L1 indication 8.5X.9 Minimum requirement at transitions for beam failure detection 8.13X.2 UE-specific CBW change delay 8.14X.3 MAC-CE based pathloss reference signal switch delay 8.19X.2.1 Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC 8.19X.3 Pre-configured measurement gap activation/deactivation upon RRC reconfiguration 9.1X.2 Measurement gap 9.1X.2.1a SA: Measurement Gap Sharing 9.1X.4 Capabilities for Support of Event Triggering and Reporting Criteria 9.1X.6 Minimum requirement at transitions 9.1X.7.2 Requirements applicability 9.1X.7.3 Requirements 9.1X.7.3.1 Requirements for autonomous activation/deactivation mechanism 9.1X.7.3.2 Requirements for network-controlled activation/deactivation mechanism 9.1X.7.3.3 Requirements for reception/transmission during activation/deactivation 9.2X.3.1 Requirements for FR1 9.2X.5.3 Scheduling availability of UE during intra-frequency measurements 9.2X.5.3.2 Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 9.3X.3.1 Requirements for FR1 9.3X.9.3.2 Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 |

Open issues summary

Issue 1-1: ATG UE feature

- Proposals

  - Option 1: Introduce following ATG UE features: (CMCC)

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uplink Time and Frequency pre-compensation and timing relationship enhancements | Support of UE specific TA calculation based on its GNSS-acquired position and the indicated BS location. For TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE autonomous TA estimation) and closed (i.e., received TA commands) control loops Support of pre-compensation of the calculated TA in its uplink transmissions Support of frequency pre-compensation to counter shift the Doppler experienced on the service link Support of determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated Support of UE receiving cell-specific K_offset in system information |  | TBD | BandNR | No | No | An ATG UE is required to at least support UE specific TA and frequency calculation based at least on its GNSS-acquired position and the indicated BS location Note: This UE feature group is applicable only for ATG operating bands | Optional with capability signalling For UE supports NR communication via ATG, UE must indicate this FG is supported. |

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE reporting of TA information | Support UE reporting of TA information | TBD [Uplink Time and Frequency pre-compensation and timing relationship enhancements] | TBD | BandNR | No | No | Note: The exact content of UE reporting of information about the TA pre-compensation is up to RAN2 Note: This UE feature group is applicable only for ATG operating bands | Optional with capability signalling |
| UE-specific K_offset | Support of reception of UE-specific K_offset via MAC-CE Support of determining the timing of PUSCH, PUCCH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset | TBD [Uplink Time and Frequency pre-compensation and timing relationship enhancements And UE reporting of information related to TA pre-compensation] | TBD | BandNR | No | No | Note: This UE feature group is applicable only for ATG operating bands | Optional with capability signalling |

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Increasing the number of HARQ processes | The maximal supported HARQ process number is X for UL and Y for DL |  | TBD | BandNR | No | No | Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} Note: This UE feature group is applicable only for ATG operating bands | Optional with capability signalling |
| K1 range extension | Support of extended K1 value range of (0..31) for unpaired spectrum |  | TBD | BandNR | No | No | Note: This UE feature group is applicable only for ATG operating bands | Optional with capability signalling |

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location based CHO | Indicates whether the UE supports location based CHO | condHandover-r16 is set for ATG bands. | TBD | BandNR | No | No | UE shall set the capability value consistently for all ATG operating bands. | Optional with capability signalling |
| Event A4 based CHO | Indicates whether the UE supports Event A4 based CHO | condHandover-r16 is set for ATG bands. | TBD | BandNR | No | No | UE shall set the capability value consistently for all ATG operating bands. | Optional with capability signalling |
| Location-based measurement report trigger | Indicates whether the UE supports location-based triggered measurement reporting (i.e., event D1) | TBD [Location based CHO] | TBD | MeasAndMobParametersCommon | No | No |  | Optional with capability signalling |

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SR triggered by a TA report | Indicates whether the UE supports triggering of SR when a TA report is triggered and there are no available UL-SCH resources. |  | TBD | MAC-ParametersCommon | No | No |  | Optional with capability signalling |
| TA reporting during initial access | It is mandatory to support TA reporting during initial access for UEs supporting uplink-TA-Reporting-r17 as specified in TS 38.321 [10]. |  | n/a | n/a | n/a | n/a |  | Conditional mandatory without capability signalling |

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RRC_INACTIVE in ATG | Indicates whether the UE supports RRC_INACTIVE in ATG |  | TBD | TBD | No | No |  | Conditional mandatory with capability signalling |

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Enhanced RRM requirements for measurements in IDLE and INACTIVE modes | If UE does not support the capability, other ATG measurement requirements (as specified in TS 38.133, clause 4.2X.2) are applied. |  | n/a | n/a | No | No |  | Optional without capability signaling |

- Recommended WF

  - To be discussed

Issue 1-2: Specification documentation

  - Option 1: For the ATG requirements, revise the current ATG clauses which could directly refer to legacy clauses with additional note (if needed) to clarify that the legacy requirements shall apply only for FR1 and NR SA operation mode without considering CA/DC and inter RAT measurement. (CMCC)

- Recommended WF

  - To be discussed

  - Check the CR.

Topic #2: Mobility requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2315161 | CMCC | Proposal 1: For HO requirements, when network assistance on ATG unknown target cell’s reference BS locations is not provided, the Tsearch should be scaled with scaling factor N=3. When network assistance on ATG unknown target cell’s reference BS locations is provided, no need to scale the Tsearch Proposal 2: For CHO requirements, when network assistance on ATG cells reference BS locations is provided to UE with antenna array, the scaling factor of Tidentify intra with index or Tidentify_intra_without_index should be decreased to N=2, and N=3 otherwise. Proposal 3: For CHO requirements, it should be applicable even if UE doesn’t have the valid and applicable parameters of BS location information and Koffset for target NR cell. Proposal 4: For UE with antenna array, the scaling factor should be considered for unknown cells of intra-frequency and inter-frequency in RRC Re-establishment delay requirement requirements. The scaling factor of L3 measurements should be used. For intra-frequency, N = 3 for the case when network assistance on ATG cells reference BS locations is provided, N = 4 otherwise. For inter-frequency, N = 2 for the case when network assistance on ATG cells reference BS locations is provided, N = 3 otherwise. Proposal 5: For UE with antenna array, the scaling factor of L3 inter-frequency measurements are suggested to be used. N = 2 for the case when network assistance on ATG cells reference BS locations is provided, N = 3 otherwise. |
| R4-2315162 | CMCC | - Draft CR to TS 38.133 on ATG mobility requirements for RRC_IDLE and CONNECTED mode |
| R4-2316357 | Ericsson | Corrections to the ATG mobility requirements |
| R4-2316438 | ZTE Corporation | Proposal 1: The consideration of phase array antenna should be extended to the requirement of Tsearch of unknown case during handover, i.e. Tsearch = N*Trs ms or Tsearch = 3N*Trs ms. - If [omnidirectional antennas] is assumed at UE, N = 1; - If [Phase antenna array] is assumed at UE, - When [network assistance on ATG cells reference location] of the target cell is provided to UE, N = 1; - Otherwise, N = [3 or 4]. Proposal 2: The consideration of phase array antenna should be extended to the requirement of Tidentify_intra_NR and Tidentify_inter_NR, i within RRC re-establishment as: Table 6.2X.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x N2 x TSMTC); MAX (800 ms, 10 x N2 x TSMTC). < -8; FR1; N/A; 800Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: If [omnidirectional antennas] is assumed at UE, N = 1. If [Phase antenna array] is assumed at UE, - When [network assistance on ATG cells reference location] of the target cell is provided to UE, N = 3; - Otherwise, N = [3 or 4].. Table 6.2X.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x N2 x TSMTC, i); MAX (800 ms, 13 x N2 x TSMTC, i). < -8; FR1; N/A; 800Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: If [omnidirectional antennas] is assumed at UE, N = 1. If [Phase antenna array] is assumed at UE, - When [network assistance on ATG cells reference location] of the target cell is provided to UE, N = 2; - Otherwise, N = [3 or 4].. Proposal 3: The consideration of phase array antenna should be extended to the requirement of Tidentify-NR within RRC connection release with redirection to NR as below: Table 6.2X.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; MAX (680 ms, 11 x N2 x Trs). Note 1: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell. Note 2: If [omnidirectional antennas] is assumed at UE, N = 1. If [Phase antenna array] is assumed at UE, - When [network assistance on ATG cells reference location] of the target cell is provided to UE, N = 2; - Otherwise, N = [3 or 4].. |
| R4-2316454 | ZTE | Draft CR on CONNECTED state mobility for ATG |

Open issues summary

Issue 2-1: Remove the square brackets

- Proposals

  - Option 1: Remove the square brackets for UE with antenna array and UE with omnidirectional antennas (Ericsson)

- Recommended WF

  - Option 1 can be agreed

Issue 2-2: Tsearch in HO requirements for UE with antenna array

- Proposals

  - For HO requirements, when network assistance on ATG unknown target cell’s reference BS locations is provided,

    - Option 1-1:  no need to scale the Tsearch (CMCC, ZTE)

  - For HO requirements, when network assistance on ATG unknown target cell’s reference BS locations is not provided,

    - Option 1-2: the Tsearch should be scaled with scaling factor N=3. (CMCC)

    - Option 1-3: the Tsearch should be scaled with scaling factor N=[3 or 4]. (ZTE)

- Recommended WF

  - Option 1-1 can be agreed.

  - Further discuss Option 1-2 and Option 1-3

Issue 2-3: Tmeasure in CHO requirements for UE with antenna array

- Proposals

  - Option 1: For intra-frequency CHO requirements, when network assistance on ATG cells reference BS locations is provided to UE with antenna array, the scaling factor of Tidentify intra with index or Tidentify_intra_without_index should be decreased to N=2, and N=3 otherwise. (CMCC)

- Recommended WF

  - To be discussed

Issue 2-4: Requirement applicability for CHO requirement

- Proposals

  - Option 1: For CHO requirements, it should be applicable even if UE doesn’t have the valid and applicable parameters of BS location information and Koffset for target NR cell. (CMCC)

- Recommended WF

  - To be discussed

Issue 2-5: Tidentify_intra_NR in RRC Re-establishment delay requirement for UE with antenna array

- Proposals

  - Option 1: For UE with antenna array, the scaling factor should be considered for unknown cells of intra-frequency and inter-frequency in RRC Re-establishment delay requirement requirements. The scaling factor of L3 measurements should be used. (CMCC)

    - For intra-frequency, N = 3 for the case when network assistance on ATG cells reference BS locations is provided, N = 4 otherwise.

    - For inter-frequency, N = 2 for the case when network assistance on ATG cells reference BS locations is provided, N = 3 otherwise.

  - Option 2: The consideration of phase array antenna should be extended to the requirement of Tidentify_intra_NR and Tidentify_inter_NR, i within RRC re-establishment as: (ZTE)

Table 6.2X.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x N2 x TSMTC) | MAX (800 ms, 10 x N2 x TSMTC) |
| < -8 | FR1 | N/A | 800Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: If [omnidirectional antennas] is assumed at UE, N = 1. If [Phase antenna array] is assumed at UE, - When [network assistance on ATG cells reference location] of the target cell is provided to UE, N = 3; - Otherwise, N = [3 or 4]. |  |  |  |

Table 6.2X.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Serving cell SSB Ês/Iot (dB) | FR of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x N2 x TSMTC, i) | MAX (800 ms, 13 x N2 x TSMTC, i) |
| < -8 | FR1 | N/A | 800Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: If [omnidirectional antennas] is assumed at UE, N = 1. If [Phase antenna array] is assumed at UE, - When [network assistance on ATG cells reference location] of the target cell is provided to UE, N = 2; - Otherwise, N = [3 or 4]. |  |  |  |

- Recommended WF

  - Firstly, for the UE with antenna array, discuss whether the scaling factor is needed for known NR cell and unknown NR cell separately

  - Secondly, discuss the scaling number

Issue 2-6: Tidentify_NR in RRC connection release with redirection requirement for UE with antenna array

- Proposals

  - Option 1: For UE with antenna array, the scaling factor of L3 inter-frequency measurements are suggested to be used. N = 2 for the case when network assistance on ATG cells reference BS locations is provided, N = 3 otherwise. (CMCC)

  - Option 2: The consideration of phase array antenna should be extended to the requirement of Tidentify-NR within RRC connection release with redirection to NR as below: (ZTE)

Table 6.2X.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR

| FR of target NR cell | Tidentify-NR |
| --- | --- |
| FR1 | MAX (680 ms, 11 x N2 x Trs) |
| Note 1: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell. Note 2: If [omnidirectional antennas] is assumed at UE, N = 1. If [Phase antenna array] is assumed at UE, - When [network assistance on ATG cells reference location] of the target cell is provided to UE, N = 2; - Otherwise, N = [3 or 4]. |  |

- Recommended WF

  - Firstly, for the UE with antenna array, discuss whether the scaling factor is needed

  - Secondly, discuss the scaling number

Topic #3: Timing

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2316490 | Ericsson | Proposal 1: We therefore propose to remove the brackets and keep the validity timer statement, since running validity timer for is a sign that the UE has received data in SIB regarding ATG gNB positions and is ready to transmit in UL. |
| R4-2316491 | Ericsson | Draft CR: UE transmit timing for ATG (removal of brackets) |

Open issues summary

Issue 3-1: validity timer for ATG

- Proposals

  - Option 1:  remove the brackets and keep the validity timer statement, since running validity timer for NTA,adjUE is a sign that the UE has received data in SIB regarding ATG gNB positions and is ready to transmit in UL. (Ericsson)

The ATG UE shall meet the Te_ATG requirement for an initial transmission provided that at least one SSB is available at the ATG UE during the last 160 ms. [and the ATG UE has a validity time running for NTA,UE-specific.] The reference point for the ATG UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus NTA+NTA-offset+NTA,adjUE×Tc.

The timing advance is initiated by ATG UE configured with only PCell [served by gNB upon initiating a validity timer for NTA,adjUE. ] The timing advance can be adjusted with MAC message that implies the adjustment of the timing advance, as defined in clause 5.2 of TS 38.321 [7].

- Recommended WF

  - To be discussed

Topic #4: Signaling characteristics

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2315163 | CMCC, Huawei, HiSilicon | - Draft CR to TS 38.133 on ATG signaling characteristics requirements |
| R4-2316358 | Ericsson | Corrections to the ATG signaling characteristics requirements |

Open issues summary

Issue 4-1: Sharing factor for L1 measurements

- Proposals

  - Option 1: P value for an L1-RS resource to be measured is defined as (CMCC)

    - Psharing factor * Ntotal / Noutside_MG with Navailable = 0

    - Ntotal / Navailable with Navailable > 0

      - For a window W of duration max(TL1,  MGRPmax), where MGRPmax is the maximum MGRP across all configured per-UE measurement gaps and per-FR1 measurement gaps, and starting at the beginning of any L1-RS resource occasion:

      - Ntotal is the total number of L1-RS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

      - Noutside_MG is the number of L1-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

      - Navailable is

        - the number of L1-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

      - TL1 is periodicity of the target L1-RS

      - Psharing factor = 3.

- Recommended WF

  - Option 1 is from last meeting’s WF, recommended to be agreed for L1 measurements.

Topic #5: Measurement

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2315109 | CATT | - Draft CR on L3 measurement procedure requirements for ATG |
| R4-2315164 | CMCC | Proposal 1: The scaling factor should be applied for L3 measurements with MG. Proposal 2: - For intra-frequency L3 measurements without MG of ATG UE with antenna array - N = 3 for the case when network assistance on ATG cells reference BS locations is provided - N = 4 otherwise - For inter-frequency L3 measurements (also for RRC IDLE/INACTIVE mode) of ATG UE with antenna array - N = 2 for the case when network assistance on ATG cells reference BS locations is provided - N = 3 otherwise |
| R402315165 | CMCC | Draft CR to TS 38.133 on ATG measurement requirements for RRC_CONNECTED mode |
| R4-2315513 | LG Electronics | Draft CR on directional beam for NR ATG |
| R4-2315693 | Ericsson | Proposal 1. No capability is needed for ATG UE to support antenna arrays in FR1 4GHz. |
| R4-2316065 | Huawei, HiSilicon | Proposal 1: When reference location for neighbor cell is provided, for inter-frequency measurement, the scaling factor is reduced from 3 to 2. Proposal 2: RAN4 to define a UE capability to differentiate UE with [omnidirectional antennas] and UE with [antenna arrays]. |
| R4-2316437 | ZTE Corporation | Proposal 1: For intra-frequency RRM measurement, keep the agreements achieved in last meeting in terms of scaling factor N for phase antenna array. While for inter-frequency RRM measurement, reduce the N by one, i.e. N = 2 for the case when network assistance on ATG cells reference locations is provided, otherwise N = 3. Proposal 2: Regarding the antenna assumption, phase antenna array is tied with band n79, and omni-antenna is tied with other ATG bands. For the band n79 capable UE, it works with phase antenna array by default at band n79. |

Open issues summary

Issue 5-1: Phase antenna array impact for L3 measurements

- Proposals

  - For the applicability of scaling factor

  - Option 1: The scaling factor should be also applied for L3 measurements with MG. (CMCC)

  - For the specific value of scaling factor

  - Option 1: (CMCC, HW, ZTE)

    - For intra-frequency L3 measurements without MG of ATG UE with antenna array

      - N = 3 for the case when network assistance on ATG cells reference BS locations is provided

      - N = 4 otherwise

    - For inter-frequency L3 measurements (also for RRC IDLE/INACTIVE mode) of ATG UE with antenna array

      - N = 2 for the case when network assistance on ATG cells reference BS locations is provided

      - Option 1-1: N = 3 otherwise (CMCC, ZTE)

      - Option 1-2: N =4 otherwise (HW)

- Recommended WF

  - To be discussed

Issue 5-2:  If any additional UE capability is needed for ATG UEs with [antenna arrays]

- Proposals

  - Option 1: No capability is needed for ATG UE to support antenna arrays in FR1 4GHz. (Ericsson)

  - Option 2: Define a UE capability to differentiate UE with [omnidirectional antennas] and UE with [antenna arrays]. (HW)

  - Option 3: For the band n79 capable UE, it works with phase antenna array by default at band n79. (ZTE)

    - Regarding the antenna assumption, phase antenna array is tied with band n79, and omni-antenna is tied with other ATG bands.

- Recommended WF

  - To be discussed

Issue 5-3: Clear description of  behavior for UE with antenna arrays when network assistance on ATG cells reference location is provided

- Proposals

  - Option 1: UE shall be able to form a beam based on the reference location (LGE)

- Recommended WF

  - To be discussed, whether the UE behavior need to be explicitly specified in spec.

Topic #6: ATG RRM performance requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-231xxxx | CMCC | Proposal 1: Reuse the Es/Iot side condition and measurement accuracy requirement of legacy TN network for ATG. Proposal 2: Consider two general test configurations as follows: Configuration; Description. 1; 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode. 2; 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode. Proposal 3: Define FDD and TDD test cases and test requirements for UE with omnidirectional antennas. Define TDD test cases and test requirements for UE with antenna array. Proposal 4: Define following test cases for ATG: Test scope RRC_IDLE state mobility; Test number; Note. Cell reselection to intra-frequency neighbour cell; ATG 1-1; Neighbour cell reference BS location is provided. Location based cell reselection to intra-frequency neighbour cell; ATG 1-2. Cell reselection to inter-frequency neighbour cell; ATG 1-3; Neighbour cell reference BS location is not provided. Cell reselection to inter-frequency NR cell for UE configured with [enhanced requirements]; ATG 1-4; Neighbour cell reference BS location is provided. Location based cell reselection to inter-frequency neighbour cell; ATG 1-5. Test scope RRC_CONNECTED state mobility; Test number; Note. Intra-frequency HO with known target cell; ATG 2-1; N/A. Intra-frequency HO with unknown target cell; ATG 2-2; Neighbour cell reference BS location is provided. Inter-frequency HO with known target cell; ATG 2-3; N/A. Inter-frequency HO with unknown target cell; ATG 2-4; Neighbour cell reference BS location is not provided. Intra-frequency location based CHO; ATG 2-5; Neighbour cell reference BS location is not provided. Inter-frequency location based CHO; ATG 2-6; Neighbour cell reference BS location is provided. Intra-frequency RRC Re-establishment with known target cell; ATG 2-7; N/A. Intra-frequency RRC Re-establishment with unknown target cell; ATG 2-8; Neighbour cell reference BS location is not provided. Inter-frequency RRC Re-establishment with known target cell; ATG 2-9. Inter-frequency RRC Re-establishment with unknown target cell; ATG 2-10; Neighbour cell reference BS location is provided. 4-step RA type contention based random access test; ATG 2-11; N/A. 4-step RA type Non-Contention based random access test; ATG 2-12; N/A. 2-step RA type contention based random access test; ATG 2-13; N/A. 2-step RA type Non-Contention based random access test; ATG 2-14. RRC Connection Release with Redirection; ATG 2-15; Neighbour cell reference BS location is provided. Test scope Timing; Test number; Note. UE transmit timing test; ATG 3-1; N/A. Timing advance adjustment accuracy; ATG 3-2; N/A. Test scope Signalling characteristics; Test number; Note. Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode; ATG 4-1; N/A. Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode; ATG 4-2; N/A. Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode; ATG 4-3; N/A. Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode; ATG 4-4; N/A. Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode; ATG 4-5; N/A. Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode; ATG 4-6; N/A. Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode; ATG 4-7; N/A. Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode; ATG 4-8; N/A. Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode; ATG 4-9; N/A. Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode; ATG 4-10; N/A. Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode; ATG 4-11; N/A. Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode; ATG 4-12; N/A. DCI-based and Timer-based Active BWP Switch; ATG 4-13; N/A. RRC-based Active BWP Switch; ATG 4-14; N/A. UE specific CBW change; ATG 4-15; N/A. Pathloss reference signal switching delay; ATG 4-16; N/A. Test scope Measurement procedure; Test number; Note. Intra-frequency measurements event triggered reporting tests without gap under non-DRX; ATG 5-1; Neighbour cell reference BS location is provided. Intra-frequency measurements event triggered reporting tests without gap under DRX; ATG 5-2; Neighbour cell reference BS location is not provided. Intra-frequency measurements event triggered reporting tests with per-UE gaps under non-DRX; ATG 5-3; Neighbour cell reference BS location is not provided. Intra-frequency measurements event triggered reporting tests with per-UE gaps under DRX; ATG 5-4; Neighbour cell reference BS location is provided. Intra-frequency measurements event triggered reporting tests without gap under non-DRX with SSB index reading; ATG 5-5; Neighbour cell reference BS location is provided. Intra-frequency measurements SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading; ATG 5-6; Neighbour cell reference BS location is provided. Inter-frequency measurements event triggered reporting tests for FR1 without SSB time index detection when DRX is not used; ATG 5-7; Neighbour cell reference BS location is not provided. Inter-frequency measurements event triggered reporting tests for FR1 without SSB time index detection when DRX is used; ATG 5-8; Neighbour cell reference BS location is provided. Inter-frequency measurements event triggered reporting tests for FR1 with SSB time index detection when DRX is not used; ATG 5-9; Neighbour cell reference BS location is provided. Inter-frequency measurements event triggered reporting tests for FR1 with SSB time index detection when DRX is used; ATG 5-10; Neighbour cell reference BS location is not provided. Inter-frequency measurements event triggered reporting tests for FR1 without gap when DRX is used; ATG 5-11; Neighbour cell reference BS location is provided. Inter-frequency measurements event triggered reporting tests for FR1 without gap when DRX is not used; ATG 5-12; Neighbour cell reference BS location is provided. SSB based L1-RSRP measurement when DRX is not used; ATG 5-13; N/A. SSB based L1-RSRP measurement when DRX is used; ATG 5-14; N/A. CSI-RS based L1-RSRP measurement when DRX is not used; ATG 5-15; N/A. CSI-RS based L1-RSRP measurement when DRX is used; ATG 5-16; N/A. L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used; ATG 5-17; N/A. L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used; ATG 5-18; N/A. L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used; ATG 5-19; N/A. SA intra-frequency CGI identification of NR neighbor cell in FR1; ATG 5-20; N/A. Test scope Measurement Performance requirements; Test number; Note. SS-RSRP intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell; ATG 6-1; N/A. SS-RSRP inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell; ATG 6-2; N/A. SS-RSRQ intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell; ATG 6-3; N/A. SS-RSRQ inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell; ATG 6-4; N/A. SS-SINR intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell; ATG 6-5; N/A. SS-SINR inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell; ATG 6-6; N/A. L1-RSRP measurement SSB based L1-RSRP measurement; ATG 6-7; N/A. L1-RSRP measurement CSI-RS based L1-RSRP measurement on resource set with repetition off; ATG 6-8; N/A. L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off; ATG 6-9; N/A. L1-SINR measurement with SSB based CMR and dedicated IMR; ATG 6-10; N/A. L1-SINR measurement with CSI-RS based CMR and dedicated IMR; ATG 6-11; N/A. CSI-RSRP intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell; ATG 6-12; N/A. CSI-RSRP inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell; ATG 6-13; N/A. CSI-RSRQ Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell; ATG 6-14; N/A. CSI-RSRQ Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell; ATG 6-15; N/A. CSI-SINR intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell; ATG 6-16; N/A. CSI-SINR Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell; ATG 6-17; N/A. Proposal 5: For the location-based cell re-selection tests, location-based CHO tests and UL transmit timing tests, the UE mobility should be assumed with 1200km/h. For the other tests, UE could be assumed with no mobility. Proposal 6: For the FDD cell re-selection test and intra/inter-frequency measurement test for UE with omnidirectional antenna, configure 1 neighbour cell. For the TDD cell re-selection test and and intra/inter-frequency measurement test for UE with antenna array, choose some test cases to configure 2 neighbour cells. - Proposal 7: Use the AWGN with residual doppler channel model for RRM test cases. |
| R4-2315648 | Huawei, HiSilicon | Proposal 1: Only define test case for non-DRX mode in connected mode. Proposal 2: GNSS is viable via AT command for all test cases, and GNSS only changed during the test for location-based CHO. Observation 1: The test methodology for FR1 OTA RRM test is not supported so far. Observation 2: The mapping between AoA and reference location of serving/neighbour cell is not clear. Proposal 3: RAN4 to discuss whether and how to define test cases for an ATG UE with antenna array. Proposal 4: Define RRM test cases for ATG UE based on Table I. Table I. Test cases for ATG UE. Cell re-selection; Cell reselection to FR1 intra-frequency NR. Cell reselection to FR1 inter-frequency NR. Cell reselection to FR1 intra-frequency NR with enhanced requirements [Set2]. Cell reselection to FR1 inter-frequency NR with enhanced requirements [Set2]. Handover; Intra-frequency handover from FR1 to FR1. Inter-frequency handover from FR1 to FR1. Conditional Handover; Intra-frequency conditional handover from FR1 to FR1. Intra-frequency conditional handover from FR1 to FR1. RRC Re-establishment; Intra-frequency RRC Re-establishment in FR1. Inter-frequency RRC Re-establishment in FR1. Intra-frequency RRC Re-establishment in FR1 without serving cell timing. RACH; 4-step RA type contention based random access test in FR1 for NR standalone. 4-step RA type non-contention based random access test in FR1 for NR standalone. RRC release with redirection; Redirection from NR in FR1 to NR in FR1. RLM; Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS. Radio Link Monitoring in-sync Test for FR1 PCell configured with SSB-based RLM RS. Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM RS. Radio Link Monitoring in-sync Test for FR1 PCell configured with CSI-RS-based RLM RS. BFD and CBD; Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR. Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR. Active BWP switch; DCI-based and Timer-based Active BWP Switch. RRC-based Active BWP Switch. UE specific CBW change; UE specific CBW change on PCell in FR1. Measurement; Intra-frequency event triggered reporting tests without gap. Inter-frequency event triggered reporting tests with gap. Inter-frequency event triggered reporting tests with gap. Inter-frequency event triggered reporting tests without gap. L1-RSRP; SSB based L1-RSRP measurement. CSI-RS based L1-RSRP measurement. L1-SINR; L1-SINR measurement with SSB based CMR and dedicated IMR. L1-SINR measurement with CSI-RS based CMR and dedicated IMR. |
| R4-2316356 | Ericsson | - The legacy NR RMCs defined in section A.3.1 of TS 38.133 are reused for ATG test cases. Following legacy NR OCNGs are reused for ATG test cases: - Generic OCNG pattern for all unused REs defined in A.3.2.1.1 in TS 38.133. - Generic OCNG pattern for unused REs in the same bandwidth as CORESET defined in A.3.2.1.3 of TS 38.133. - Generic OCNG pattern for all unused REs outside SSB slot(s) defined in A.3.2.1.4 in TS 38.133 - RAN4 not to define test cases for SCC. - RAN4 to define test cases with non-DRX only. - RAN4 to discuss additional test cases considering the new TDD pattern for ATG (if agreed). - RAN4 to introduce new IDLE mode cell reselection test cases to verify that UE can meet the set of cell reselection requirements indicated by the new network indicator. - The new ATG cell reselection test cases shall verify following two types of cell reselections: - Cell reselection to FR1 intra-frequency - Cell reselection to FR1 inter-frequency - RAN4 to introduce new test cases to verify the handover and conditional handover requirements as follows: - Intra-frequency handover from FR1 to FR1; known target cell - Intra-frequency handover from FR1 to FR1; unknown target cell - Inter-frequency handover from FR1 to FR1; unknown target cell - Intra-frequency conditional handover from FR1 to FR1 - Inter-frequency conditional handover from FR1 to FR1 - RAN4 to introduce new test cases to verify the RRC re-establishment requirements: - Intra-frequency RRC Re-establishment in FR1 - Inter-frequency RRC Re-establishment in FR1 - RAN4 to introduce new test cases to verify the random access requirements: - 4-step RA type contention based random access test in FR1 for NR standalone - 4-step RA type non-contention based random access test in FR1 for NR standalone - 2-step RA type contention based random access test in FR1 for NR standalone - 2-step RA type non-contention based test in FR1 for NR standalone - RAN4 to introduce new test cases to verify the RRC connection release with redirection requirements: - Redirection from NR in FR1 to NR in FR1 RAN4 to introduce new test cases to verify the ATG timing requirements as follows: - NR UE Transmit Timing Test for FR1: to verify that the UE can follow frame timing change of the reference cell and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. - Timing advance for ATG test is to verify UE Timing Advance adjustment delay and accuracy requirement. - RAN4 to introduce new test cases to verify the ATG RLM requirements as follows: - Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode - Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode - Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode - Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode - RAN4 to introduce new test cases to verify the ATG link recovery requirements as follows: - Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode - Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode - RAN4 to introduce new test cases to verify the ATG active BWP switch delay requirements: - DCI-based and Timer-based Active BWP Switch - RRC-based Active BWP Switch - RAN4 to introduce new test cases to verify the ATG UE specific CBW change requirements as follows: - UE specific CBW change on PCell in FR1 in non-DRX - RAN4 to introduce new test cases to verify the pathloss reference signal switching delay requirements as follows: - MAC-CE based pathloss reference signal switch delay - RAN4 to discuss the test cases for TCI switching delay requirements for MAC-, DCI and RRC-based TCI state switching delay requirements. - RAN4 to follow the legacy FR1 test case configuration for ATG UE measurement requirements in FR1 2GHz. - RAN4 to discuss how to model the L3 measurement test with beam sweeping in FR1 4GHz. - RAN4 to introduce the following new test cases to verify the ATG UE measurement requirements in FR1. - Intra-frequency measurement(2GHz) - SA event-triggered reporting tests without gaps under non-DRX - SA event-triggered reporting tests with per-UE gaps under non-DRX - Inter-frequency measurement(2GHz) - SA event triggered reporting tests without SSB time index detection when DRX is not used - Intra-frequency measurement(4GHz) - SA event-triggered reporting tests without gaps under non-DRX - SA event-triggered reporting tests with per-UE gaps under non-DRX - Inter-frequency measurement(4GHz) - SA event triggered reporting tests without SSB time index detection when DRX is not used - L1-RSRP measurement(2GHz and 4GHz) - SSB based L1-RSRP measurement when DRX is not used - CSI-RS based L1-RSRP measurement when DRX is not used |
| R4-2316439 | ZTE Corporation | Proposal 1: The general configuration of the test are suggested: Testing band: Band n79 should be configured for the sake of phase array antenna relevant enhancements testing; At least one band within {n1, n3, n34, n39, n41, n78} is considered to test other ATG enhancement besides the phase array antenna relevant enhancements testing; Duplex: Similar as in legacy, both FDD and TDD should be configured. SCS: Similar as in legacy, both 15kHz, 30kHz should be configured. Not consider 60kHz since some RRM requirement is vacant for 60kHz. TDD pattern: TDD pattern ‘30D4S6U’, S=’14GP’ should be prioritized in the test. Proposal 2: Ignoring the frequency compensation when considering the measurement accuracy since no RRM impact. The accuracy of timing compensation consists of GNSS error and BS location error consideration, both have been addressed. No need to introduce additional measurement accuracy. Proposal 3: The accuracy requirement of L1 measurement should be identified for ATG. Observation 1: The requirement of L3 measurement is divided into two cases: phase array antenna case and non-phase array antenna i.e. omni antenna case. For the former, reuse the requirement of legacy FR1 TN. For the latter, additional scaling factor N is introduced to address the beam issue during L3 measurement. Proposal 4: Regarding L3 measurement accuracy, reuse the accuracy requirement of legacy FR1 TN for the non-phase array antenna assumption; it is better to identify the accuracy requirement for the phase array antenna assumption. Proposal 5: A general principle is to only define the test cases for those requirements that are different from the TN deployment or not involved in the TN deployment. Proposal 6: The following test cases are preferred for ATG test: TC index; TC; Purpose. 1; ATG UE transmit timing; To verify the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits.. 2; ATG UE timing advance; To verify the UE Timing Advance adjustment delay and accuracy requirement, including the Koffset into the consideration.. 3; Inter-frequency measurement for cell re-selection; To verify whether the UE would perform set 1 or set 2 requirements based on the NW indicator. Furthermore, the Set 1 requirements is used by default if without any NW indicator.. 4; Intra-frequency measurement for CONNECTED mode under phase antenna array assumption; To verify whether the UE perform the intra-frequency measurement with the newly introduced scaling factor N. Both cases of with and without ATG cells location provision should be verified.. 5; Inter-frequency measurement for CONNECTED mode under phase antenna array assumption; To verify whether the UE perform the inter-frequency measurement with the newly introduced scaling factor N. Both cases of with and without ATG cells location provision should be verified.. 6; L1-RSRP measurement accuracy under phase antenna array assumption; To verify whether the L1-RSRP measurement accuracy is within the specified limits under the assumption of phase antenna array. 7; L1-SINR measurement accuracy under phase antenna array assumption; To verify whether the L1-SINR measurement accuracy is within the specified limits under the assumption of phase antenna array. 8; Intra-frequency measurement accuracy for CONNECTED mode under phase antenna array assumption; To verify whether the Intra-frequency measurement accuracy is within the specified limits under the assumption of phase antenna array. Both the cases with and without NW assistance on ATC cells reference location provision should be verified.. 9; Inter-frequency measurement accuracy for CONNECTED mode under phase antenna array assumption; To verify whether the Inter-frequency measurement accuracy is within the specified limits under the assumption of phase antenna array. Both the cases with and without NW assistance on ATC cells reference location provision should be verified.. |

Measurement accuracy

Issue 6-1-1: Measurement accuracy

- Proposals

  - Option 1: Reuse the Es/Iot side condition and measurement accuracy requirement of legacy TN network for ATG.(CMCC)

  - Option 2: The accuracy requirement of L1 measurement should be identified for ATG. Regarding L3 measurement accuracy, reuse the accuracy requirement of legacy FR1 TN for the non-phase array antenna assumption; it is better to identify the accuracy requirement for the phase array antenna assumption. (ZTE)

- Recommended WF

  - reuse the L3 and L1 accuracy requirement of legacy FR1 TN for the non-phase array antenna assumption

  - Further discuss whether to identify the incremental accuracy requirement for the phase array antenna assumption

Test cases and test configurations

Issue 6-2-1: General test configuration

- Proposals

  - Option 1:  (CMCC)

    - Configuration 1:15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, only for UE with omnidirectional antennas

    - Configuration 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, for both UE with omnidirectional antennas and for UE with antenna array

  - Option 2: (ZTE)

    - Testing band:

      - Band n79 should be configured for the sake of phase array antenna relevant enhancements testing;

      - At least one band within {n1, n3, n34, n39, n41, n78} is considered to test other ATG enhancement besides the phase array antenna relevant enhancements testing;

    - Duplex:

      - Similar as in legacy, both FDD and TDD should be configured.

    - SCS:

      - Similar as in legacy, both 15kHz, 30kHz should be configured.

      - Not consider 60kHz since some RRM requirement is vacant for 60kHz.

    - TDD pattern:

      - TDD pattern ‘30D4S6U’, S=’14GP’ should be prioritized in the test.

  - Option 3: RAN4 to discuss additional test cases considering the new TDD pattern for ATG (if agreed). (Ericsson)

- Recommended WF

  - To be discussed based on Option 1, and further discuss whether and how to configure the testing band and TDD pattern

Issue 6-2-2: Test scope

- Proposals

  - Option 1: Only define test case for non-DRX mode in connected mode (HW, Ericsson)

  - Option 2: Define test cases for both DRX mode and non-DRX mode. (CMCC)

  - Option 3: RAN4 not to define test cases for SCC. (Ericsson)

  - Option 4: A general principle is to only define the test cases for those requirements that are different from the TN deployment or not involved in the TN deployment. (ZTE)

- Recommended WF

  - RAN4 not to define test cases for SCC

  - For DRX configuration: To be discussed

  - Further discuss whether to define the test cases which has same core requirements as TN

  - Companies can show your support in the table below

| Test scope RRC_IDLE state mobility | Test number | Supporter |
| --- | --- | --- |
| Cell reselection to intra-frequency neighbour cell | ATG 1-1 | CMCC (Neighbour cell reference BS location is provided) HW, Ericsson |
| Location based cell reselection to intra-frequency neighbour cell | ATG 1-2 | CMCC (Neighbour cell reference BS location is provided) |
| Cell reselection to inter-frequency neighbour cell | ATG 1-3 | CMCC (Neighbour cell reference BS location is not provided) HW, Ericsson |
| Cell reselection to inter-frequency NR cell for UE configured with [enhanced requirements] | ATG 1-4 | CMCC (Neighbour cell reference BS location is provided) HW, Ericsson, ZTE |
| Location based cell reselection to inter-frequency neighbour cell | ATG 1-5 | CMCC (Neighbour cell reference BS location is provided) |

| Test scope RRC_CONNECTED state mobility | Test number | Supporter |
| --- | --- | --- |
| Intra-frequency HO with known target cell | ATG 2-1 | CMCC, HW, Ericsson |
| Intra-frequency HO with unknown target cell | ATG 2-2 | CMCC (Neighbour cell reference BS location is provided) HW, Ericsson |
| Inter-frequency HO with known target cell | ATG 2-3 | CMCC, [HW] |
| Inter-frequency HO with unknown target cell | ATG 2-4 | CMCC (Neighbour cell reference BS location is not provided) HW, Ericsson |
| Intra-frequency location based CHO | ATG 2-5 | CMCC (Neighbour cell reference BS location is not provided) HW, Ericsson |
| Inter-frequency location based CHO | ATG 2-6 | CMCC (Neighbour cell reference BS location is provided) HW, Ericsson |
| Intra-frequency RRC Re-establishment with known target cell | ATG 2-7 | CMCC HW, Ericsson |
| Intra-frequency RRC Re-establishment with unknown target cell | ATG 2-8 | CMCC (Neighbour cell reference BS location is not provided) |
| Inter-frequency RRC Re-establishment with known target cell | ATG 2-9 | CMCC |
| Inter-frequency RRC Re-establishment with unknown target cell | ATG 2-10 | CMCC (Neighbour cell reference BS location is provided) HW, Ericsson |
| Inter-frequency RRC Re-establishment in FR1 without serving cell timing | ATG 2-11 | HW |
| 4-step RA type contention based random access test | ATG 2-12 | CMCC, HW, Ericsson |
| 4-step RA type Non-Contention based random access test | ATG 2-13 | CMCC, HW, Ericsson |
| 2-step RA type contention based random access test | ATG 2-14 | CMCC, Ericsson |
| 2-step RA type Non-Contention based random access test | ATG 2-15 | CMCC, Ericsson |
| RRC Connection Release with Redirection | ATG 2-16 | CMCC (Neighbour cell reference BS location is provided) HW, Ericsson |

| Test scope Timing | Test number | Supporter |
| --- | --- | --- |
| UE transmit timing test | ATG 3-1 | CMCC, Ericsson, ZTE |
| Timing advance adjustment delay and accuracy | ATG 3-2 | CMCC, Ericsson, ZTE |

| Test scope Signalling characteristics | Test number | Supporter |
| --- | --- | --- |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode | ATG 4-1 | CMCC, HW, Ericsson |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode | ATG 4-2 | CMCC, HW, Ericsson |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode | ATG 4-3 | CMCC |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode | ATG 4-4 | CMCC |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode | ATG 4-5 | CMCC, HW, Ericsson |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode | ATG 4-6 | CMCC, HW, Ericsson |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode | ATG 4-7 | CMCC |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode | ATG 4-8 | CMCC |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode | ATG 4-9 | CMCC, HW, Ericsson |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode | ATG 4-10 | CMCC |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode | ATG 4-11 | CMCC, HW, Ericsson |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode | ATG 4-12 | CMCC |
| DCI-based and Timer-based Active BWP Switch | ATG 4-13 | CMCC, HW, Ericsson |
| RRC-based Active BWP Switch | ATG 4-14 | CMCC, HW, Ericsson |
| UE specific CBW change | ATG 4-15 | CMCC, HW, Ericsson |
| Pathloss reference signal switching delay | ATG 4-16 | CMCC |

| Test scope Measurement procedure | Test number | Supporter |
| --- | --- | --- |
| Intra-frequency measurements event triggered reporting tests without gap under non-DRX | ATG 5-1 | CMCC (Neighbour cell reference BS location is provided) HW, Ericsson, ZTE (under phase antenna array assumption) |
| Intra-frequency measurements event triggered reporting tests without gap under DRX | ATG 5-2 | CMCC (Neighbour cell reference BS location is not provided), ZTE (under phase antenna array assumption) |
| Intra-frequency measurements event triggered reporting tests with per-UE gaps under non-DRX | ATG 5-3 | CMCC (Neighbour cell reference BS location is not provided) HW, Ericsson, ZTE (under phase antenna array assumption) |
| Intra-frequency measurements event triggered reporting tests with per-UE gaps under DRX | ATG 5-4 | CMCC (Neighbour cell reference BS location is provided), ZTE (under phase antenna array assumption) |
| Intra-frequency measurements event triggered reporting tests without gap under non-DRX with SSB index reading | ATG 5-5 | CMCC (Neighbour cell reference BS location is provided), ZTE (under phase antenna array assumption) |
| Intra-frequency measurements SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | ATG 5-6 | CMCC (Neighbour cell reference BS location is provided), ZTE (under phase antenna array assumption) |
| Inter-frequency measurements event triggered reporting tests for FR1 without SSB time index detection when DRX is not used | ATG 5-7 | CMCC (Neighbour cell reference BS location is not provided) HW, Ericsson, ZTE (under phase antenna array assumption) |
| Inter-frequency measurements event triggered reporting tests for FR1 without SSB time index detection when DRX is used | ATG 5-8 | CMCC (Neighbour cell reference BS location is provided), ZTE (under phase antenna array assumption) |
| Inter-frequency measurements event triggered reporting tests for FR1 with SSB time index detection when DRX is not used | ATG 5-9 | CMCC (Neighbour cell reference BS location is provided) HW, ZTE (under phase antenna array assumption) |
| Inter-frequency measurements event triggered reporting tests for FR1 with SSB time index detection when DRX is used | ATG 5-10 | CMCC (Neighbour cell reference BS location is not provided), ZTE (under phase antenna array assumption) |
| Inter-frequency measurements event triggered reporting tests for FR1 without gap when DRX is used | ATG 5-11 | CMCC (Neighbour cell reference BS location is provided), ZTE (under phase antenna array assumption) |
| Inter-frequency measurements event triggered reporting tests for FR1 without gap when DRX is not used | ATG 5-12 | CMCC (Neighbour cell reference BS location is provided) HW, ZTE (under phase antenna array assumption) |
| SSB based L1-RSRP measurement when DRX is not used | ATG 5-13 | CMCC, HW, Ericsson, ZTE (under phase antenna array assumption) |
| SSB based L1-RSRP measurement when DRX is used | ATG 5-14 | CMCC, ZTE (under phase antenna array assumption) |
| CSI-RS based L1-RSRP measurement when DRX is not used | ATG 5-15 | CMCC, HW, Ericsson, ZTE (under phase antenna array assumption) |
| CSI-RS based L1-RSRP measurement when DRX is used | ATG 5-16 | CMCC, ZTE (under phase antenna array assumption) |
| L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used | ATG 5-17 | CMCC, ZTE (under phase antenna array assumption) |
| L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used | ATG 5-18 | CMCC, HW, ZTE (under phase antenna array assumption) |
| L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used | ATG 5-19 | CMCC, HW, ZTE (under phase antenna array assumption) |
| SA intra-frequency CGI identification of NR neighbor cell in FR1 | ATG 5-20 | CMCC |

| Test scope Measurement Performance requirements | Test number | Supporter |
| --- | --- | --- |
| SS-RSRP intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-1 | CMCC, ZTE (under phase antenna array assumption) |
| SS-RSRP inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-2 | CMCC, ZTE (under phase antenna array assumption) |
| SS-RSRQ intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-3 | CMCC, ZTE (under phase antenna array assumption) |
| SS-RSRQ inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-4 | CMCC, ZTE (under phase antenna array assumption) |
| SS-SINR intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-5 | CMCC, ZTE (under phase antenna array assumption) |
| SS-SINR inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-6 | CMCC, ZTE (under phase antenna array assumption) |
| L1-RSRP measurement SSB based L1-RSRP measurement | ATG 6-7 | CMCC |
| L1-RSRP measurement CSI-RS based L1-RSRP measurement on resource set with repetition off | ATG 6-8 | CMCC |
| L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off | ATG 6-9 | CMCC |
| L1-SINR measurement with SSB based CMR and dedicated IMR | ATG 6-10 | CMCC |
| L1-SINR measurement with CSI-RS based CMR and dedicated IMR | ATG 6-11 | CMCC |
| CSI-RSRP intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-12 | CMCC |
| CSI-RSRP inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-13 | CMCC |
| CSI-RSRQ Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-14 | CMCC |
| CSI-RSRQ Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-15 | CMCC |
| CSI-SINR intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-16 | CMCC |
| CSI-SINR Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-17 | CMCC |

Issue 6-2-3: RMC

- Proposals

  - Option 1: The legacy NR RMCs defined in section A.3.1 of TS 38.133 are reused for ATG test cases. (Ericsson)

- Recommended WF

  - Option 1 can be the baseline

Issue 6-2-4: OCNG

- Proposals

  - Option 1: Following legacy NR OCNGs are reused for ATG test cases: (Ericsson)

    - Generic OCNG pattern for all unused REs defined in A.3.2.1.1 in TS 38.133.

    - Generic OCNG pattern for unused REs in the same bandwidth as CORESET defined in A.3.2.1.3 of TS 38.133.

    - Generic OCNG pattern for all unused REs outside SSB slot(s) defined in A.3.2.1.4 in TS 38.133

- Recommended WF

  - Option 1 can be the baseline.

Issue 6-2-5: Test method for UE with antenna array

- Proposals

  - Option 1: RAN4 to discuss whether and how to define test cases for an ATG UE with antenna array (HW)

  - Option 2: RAN4 to discuss how to model the L3 measurement test with beam sweeping in FR1 4GHz. (Ericsson)

- Recommended WF

  - To be discussed

Issue 6-2-6: whether to define test cases for TCI switching delay requirements

- Proposals

  - Option 1: RAN4 to discuss the test cases for TCI switching delay requirements for MAC-, DCI and RRC-based TCI state switching delay requirements.  (Ericsson)

- Recommended WF

  - To be discussed

Issue 6-2-7: GNSS setup

- Proposals

  - Option 1: GNSS is viable via AT command for all test cases, and GNSS only changed during the test for location-based CHO.  (HW)

- Recommended WF

  - To be discussed

Issue 6-2-8: UE mobility assumption

- Proposals

  - Option 1: For the location-based cell re-selection tests, location-based CHO tests and UL transmit timing tests, the UE mobility should be assumed with 1200km/h. For the other tests, UE could be assumed with no mobility. (CMCC)

- Recommended WF

  - To be discussed

Issue 6-2-9: Neighbour cell configuration

- Proposals

  - Option 1: For the FDD cell re-selection test and intra/inter-frequency measurement test for UE with omnidirectional antenna, configure 1 neighbour cell. For the TDD cell re-selection test and and intra/inter-frequency measurement test for UE with antenna array, choose some test cases to configure 2 neighbour cells. (CMCC)

- Recommended WF

  - To be discussed

Issue 6-2-10: Channel model

- Proposals

  - Option 1: Use the AWGN with residual doppler channel model for RRM test cases.  (CMCC)

- Recommended WF

  - To be discussed

Topic #6: CR and TP

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2315109 | CATT | Draft CR on L3 measurement procedure requirements for ATG |
| R4-2315162 | CMCC | CR to TS 38.133 on ATG mobility requirements for RRC_IDLE and CONNECTED mode |
| R4-2316357 | Ericsson | Corrections to the ATG mobility requirements |
| R4-2316454 | ZTE | Draft CR on CONNECTED state mobility for ATG |
| R4-2316491 | Ericsson | Draft CR: UE transmit timing for ATG (removal of brackets) |
| R4-2315163 | CMCC | CR to TS 38.133 on ATG signaling characteristics requirements |
| R4-2316358 | Ericsson | Corrections to the ATG signaling characteristics requirements |
| R4-2315165 | CMCC | CR to TS 38.133 on ATG measurement requirements for RRC_CONNECTED mode |
| R4-2315513 | LG Electronics | Draft CR on directional beam for NR ATG |

Open issues summary

Sub-topic 5-1 Draft CRs and TP to review

| T-doc number | Company | Proposals / Observations | Recommended WF |
| --- | --- | --- | --- |
| R4-2315162 | CMCC | CR to TS 38.133 on ATG mobility requirements for RRC_IDLE and CONNECTED mode | CRs can be merged Further revise the CR according to the agreements of Issue 1-2, Issue 2-1 to Issue 2-6 |
| R4-2316357 | Ericsson | Corrections to the ATG mobility requirements |  |
| R4-2316454 | ZTE | Draft CR on CONNECTED state mobility for ATG |  |
| R4-2316491 | Ericsson | Draft CR: UE transmit timing for ATG (removal of brackets) | Further revise the CR according to the agreements of Issue 1-2, Issue 3-1 |
| R4-2315163 | CMCC | CR to TS 38.133 on ATG signaling characteristics requirements | CRs can be merged Further revise the CR according to the agreements of Issue 1-2, Issue 4-1 |
| R4-2316358 | Ericsson | Corrections to the ATG signaling characteristics requirements |  |
| R4-2315165 | CMCC | CR to TS 38.133 on ATG measurement requirements for RRC_CONNECTED mode | CRs can be merged Further revise the CR according to the agreements of Issue 1-2, Issue 5-1 and 5-3 |
| R4-2315109 | CATT | Draft CR on L3 measurement procedure requirements for ATG |  |
| R4-2315513 | LG Electronics | Draft CR on directional beam for NR ATG |  |