3GPP TSG-RAN WG4 Meeting # 108bis											          R4-2317210

Xiamen, China, October 09 – October 13, 2023

Agenda item:			5.23.5

Source:	Moderator (Huawei, HiSilicon)

Title:	Topic summary for [108bis][218] NR_MC_enh

Document for:	Information

Introduction

This email thread discusses the RRM performance requirements of WI on R18 Multi-carrier enhancements.

Topic #1:	Test cases for DL interruption for Tx switching across 3/4 bands

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2315313 | Qualcomm, Inc. | Proposal: The legacy RRM tests for Tx switching on one or two UL bands can be considered as a starting point and RAN4 can incorporate the additional signaling on interruption length introduced in R18. |
| R4-2315663 | Huawei, HiSilicon | Proposal 1: The following test cases are to be defined to verify the interruption due to UE dynamic switching across 3/4 band uplink carriers: For single TAG - DL interruptions at Tx switching across three uplink bands in TDD-TDD CA with different UL/DL pattern in SA - DL interruptions at Tx switching across three uplink bands in FDD-TDD CA in SA - DL interruptions at Tx switching across four uplink bands in TDD-TDD CA with different UL/DL pattern in SA - DL interruptions at Tx switching across four uplink bands in FDD-TDD CA in SA For Two TAG - DL interruptions at Tx switching across three uplink bands in TDD-TDD CA with different UL/DL pattern in SA - DL interruptions at Tx switching across three uplink bands in FDD-TDD CA in SA - DL interruptions at Tx switching across four uplink bands in TDD-TDD CA with different UL/DL pattern in SA - DL interruptions at Tx switching across four uplink bands in FDD-TDD CA in SA Proposal 2: For TX switching among 3 bands, For FDD-TDD uplink CA cases, FR1 FDD PCell (Cell 1), FR1 FDD SCell (Cell 2) and FR1 TDD SCell (Cell 3) where cell 1 with 1TX is on band A, cell 2 with 1TX is on band B, and cell 3 with 2TX is on band C. TX switching is from band A to band C, and from band B to band C. Dynamic Tx switches between case1 and case 2. Case 1; 1 Tx on FDD band A and 1 Tx on FDD band B. Case 2; 2 Tx on TDD band C. TDD UL/DL pattern on band C is 3D1S4U, S = 10DL: 2GP: 2UL. Symbol level DL interruption verification: Triggering an aperiodic CSI-RS L1-RSRP reporting with CSI-RS resources on the OFDM symbol right before the interruption and check UE’s aperiodic L1-RSRP report with corresponding measurement accuracy. Which symbol to be verified? - Symbol #8 or symbol #9 or symbol #10 in the slot overlapping with the special slot of the NR TDD carrier depending on UE capability uplinkTxSwitchingPeriod on NR FDD carrier (Cell 1, Cell 2) - Symbol #4 or symbol #5 or symbol #8 on the special slot depending on UE capability uplinkTxSwitchingPeriod on NR TDD carrier (Cell 3) Proposal 3: For TX switching among 3 bands, for TDD-TDD uplink CA cases, FR1 TDD PCell (Cell 1), FR1 FDD SCell (Cell 2) and FR1 TDD SCell (Cell 3) where cell 1 with 1TX is on band A, cell 2 with 1TX is on band B, and cell 3 with 2TX is on band C. - band#A and band B TDD UL/DL pattern is 3D1S4U, S = 10DL: 2GP: 2UL; - band#C TDD UL/DL pattern is 1D1S2U, S = 10DL: 2GP: 2UL TX switching is from band A to band C, and from B to band C. Dynamic Tx switches between case1 and case 2. Case 1; 1 Tx on TDD band A and 1 Tx on TDD band B. Case 2; 2 Tx on TDD band C. Which symbol to be verified? - symbol #4 or symbol #5 or symbol #8 on the special slot depending on UE capability uplinkTxSwitchingPeriod on NR TDD PCell (Cell 1 and Cell 2) - symbol #4 or symbol #5 or symbol #8 on the 2nd special slot of every 8 slots depending on UE capability uplinkTxSwitchingPeriod NR TDD SCell (Cell 3) Proposal 4: For TX switching among 4 bands, For TDD-TDD uplink CA cases, the similar test configuration for TX switching among 4 bands can be similar as 3band, where Dynamic Tx switches are switched between case 1 and case 2 Case 1; 1 Tx on FDD band A and 1 Tx on FDD band B. Case 2; 1 Tx on TDD band C and 1 Tx on TDD band D. Or Case 1; 1 Tx on TDD band A and 1 Tx on TDD band B. Case 2; 1 Tx on TDD band C and 1 Tx on TDD band D. Proposal 5: For 1-TAG, MRTD is set as 0us; For 2-TAG, MRTD is set as 9us. |
| R4-2316395 | Nokia, Nokia Shanghai Bell | - A test case would be required for DL interruptions at switching across 3/4 bands with two transmit antenna for single TAG in TDD-TDD CA A test case would be required for DL interruptions at switching across 3/4 bands with two transmit antenna for single TAG in FDD-TDD CA A test case would be required for DL interruptions at switching across 3/4 bands with two transmit antenna for two TAGs in TDD-TDD CA A test case would be required for DL interruptions at switching across 3/4 bands with two transmit antenna for two TAGs in TDD-TDD CA |
| R4-2316396 | Nokia, Nokia Shanghai Bell | Draft CR on test cases |

Open issues summary

Sub-topic 1: Test list and test principle

Issue 1-1: Test case list for UL Tx switching across 3/4 bands

- Proposals

  - Option 1a(Huawei):

For single TAG

- DL interruptions at Tx switching across three uplink bands in TDD-TDD CA with different UL/DL pattern in SA

- DL interruptions at Tx switching across three uplink bands in FDD-TDD CA in SA

- DL interruptions at Tx switching across four uplink bands in TDD-TDD CA with different UL/DL pattern in SA

- DL interruptions at Tx switching across four uplink bands in FDD-TDD CA in SA

For Two TAG

- DL interruptions at Tx switching across three uplink bands in TDD-TDD CA with different UL/DL pattern in SA

- DL interruptions at Tx switching across three uplink bands in FDD-TDD CA in SA

- DL interruptions at Tx switching across four uplink bands in TDD-TDD CA with different UL/DL pattern in SA

- DL interruptions at Tx switching across four uplink bands in FDD-TDD CA in SA

  - Option 1b (Nokia):

For single TAG

- DL interruptions at Tx switching across three/four uplink bands in TDD-TDD CA with different UL/DL pattern in SA

- DL interruptions at Tx switching across three/four uplink bands in FDD-TDD CA in SA

For Two TAG

- DL interruptions at Tx switching across three/four uplink bands in TDD-TDD CA with different UL/DL pattern in SA

- DL interruptions at Tx switching across three/four uplink bands in FDD-TDD CA in SA

- Recommended WF

Moderator note: The difference between option 1a and option 1b is whether to define separate tests for 3 bands and 4 bands. Moderator suggests to first agree on the general principle that both 3 bands and 4 bands are needed to be verified and further discuss the test configurations to check whether there is difference for 3 bands and 4 bands switching case. And then we can come back to this point latter.

Is option 1a agreeable with a note “FFS on whether to define separate tests for 3 bands and 4 bands:

Further discussion”?

Issue 1-2: Principle of verifying symbol level DL interruption for TX switching across 3 or 4 bands

- Proposals

  - Option 1(QC, Huawei, Nokia): The legacy RRM tests for Tx switching on one or two UL bands can be considered as a starting point, that is,

Triggering an aperiodic CSI-RS L1-RSRP reporting with CSI-RS resources on the OFDM symbol right before the interruption and check UE’s aperiodic L1-RSRP report with corresponding measurement accuracy.

- Recommended WF

Is option 1 agreeable?

Issue 1-3: Whether to verify the DL interruption when optional capability [uplinkTxSwitchingPeriod1T1Tto2T] is reported

- Proposals

  - Option 1(Nokia): yes

- Recommended WF

Further discussion

Sub-topic 2: Tx switching Scenarios

Issue 2-1: Tx switching scenario for FDD-TDD uplink CA cases for TX switching across 3 bands

- Proposals

  - Option 1(Huawei):

- FR1 FDD PCell (Cell 1), FR1 FDD SCell (Cell 2) and FR1 TDD SCell (Cell 3) where cell 1 with 1TX is on band A, cell 2 with 1TX is on band B, and cell 3 with 2TX is on band C.

- TX switching is from band A to band C, and from band B to band C. Dynamic Tx switches between case1 and case 2.

| Case 1 | 1 Tx on FDD band A and 1 Tx on FDD band B |
| --- | --- |
| Case 2 | 2 Tx on TDD band C |

  - Option 2(Nokia):

- FR1 FDD PCell (Cell 1), FR1 TDD SCell (Cell 2) and FR1 TDD SCell (Cell 3) where cell 1, cell2 and cell 3 are respectively in band A, B and C respectively with 2Tx.

- TX switching is from band A to band B. Dynamic Tx switches between case1 and case 2.

| Case 1 | 2 Tx on FDD band A |
| --- | --- |
| Case 2 | 2 Tx on TDD band B, 2 Tx on TDD band C |

- Recommended WF

Further discussion

Issue 2-2: Tx switching scenario for TDD-TDD uplink CA cases for TX switching across 3 bands

- Proposals

  - Option 1(Huawei):

- FR1 TDD PCell (Cell 1), FR1 TDD SCell (Cell 2) and FR1 TDD SCell (Cell 3) where cell 1 with 1TX is on band A, cell 2 with 1TX is on band B, and cell 3 with 2TX is on band C.

- TX switching is from band A to band C, and from band B to band C. Dynamic Tx switches between case1 and case 2.

| Case 1 | 1 Tx on TDD band A and 1 Tx on TDD band B |
| --- | --- |
| Case 2 | 2 Tx on TDD band C |

  - Option 2(Nokia):

- FR1 TDD PCell (Cell 1), FR1 TDD SCell (Cell 2) and FR1 TDD SCell (Cell 3) where cell 1, cell2 and cell 3 are respectively in band A, B and C respectively with 2Tx.

- TX switching is from band A to band B. Dynamic Tx switches between case1 and case 2.

| Case 1 | 2 Tx on TDD band A |
| --- | --- |
| Case 2 | 2 Tx on TDD band B, 2 Tx on TDD band C |

- Recommended WF

Further discussion

Issue 2-3: Tx switching scenario for FDD-TDD uplink CA cases for TX switching across 4 bands

- Proposals

  - Option 1(Huawei):

- TX switching scenario for TX switching across 4 bands can be similar as 3 bands, where Dynamic Tx switches are switched between case 1 and case 2

- For FDD+TDD CA

| Case 1 | 1 Tx on FDD band A and 1 Tx on FDD band B |
| --- | --- |
| Case 2 | 1 Tx on TDD band C and 1 Tx on TDD band D |

- For TDD+TDD CA

| Case 1 | 1 Tx on TDD band A and 1 Tx on TDD band B |
| --- | --- |
| Case 2 | 1 Tx on TDD band C and 1 Tx on TDD band D |

- Recommended WF

Further discussion

Sub-topic 3: Test configurations

Issue 3-1: TDD UL/DL pattern for FDD-TDD uplink CA cases for TX switching across 3 or 4 bands

- Proposals

  - Option 1(QC, Huawei, Nokia): The legacy RRM tests for Tx switching on one or two UL bands can be considered as a starting point, that is

- TDD UL/DL pattern is 3D1S4U, S = 10DL: 2GP: 2UL

- Recommended WF

Is option 1 agreeable?

Issue 3-2: Different TDD UL/DL patterns for TDD-TDD uplink CA cases for TX switching across 3 or 4 bands

- Proposals

  - Option 1(QC, Huawei, Nokia): The legacy RRM tests for Tx switching on one or two UL bands can be considered as a starting point, that is

- TDD UL/DL pattern is 3D1S4U, S = 10DL: 2GP: 2UL;

- TDD UL/DL pattern is 1D1S2U, S = 10DL: 2GP: 2UL

- Recommended WF

Is option 1 agreeable?

Issue 3-3: MRTD setup for single TAG and dual TAG

- Proposals

  - Option 1(Huawei):

- For 1-TAG, MRTD is set as 0us;

- For 2-TAG, MRTD is set as 9us.

- Recommended WF

Further discussion

Issue 3-4: Which symbol to be verified for FDD-TDD uplink CA cases for TX switching across 3 or 4 bands for single TAG

- Proposals

  - Option 1(Huawei, Nokia):

- Symbol #8 or symbol #9 or symbol #10 in the slot overlapping with the special slot of the NR TDD carrier depending on UE capability uplinkTxSwitchingPeriod on NR FDD carrier

- Symbol #4 or symbol #5 or symbol #8 on the special slot depending on UE capability uplinkTxSwitchingPeriod on NR TDD carrier

- Recommended WF

Further discussion

Issue 3-5: Which symbol to be verified for TDD-TDD uplink CA cases for TX switching across 3 or 4 bands for single TAG

- Proposals

  - Option 1(Huawei, Nokia):

- symbol #4 or symbol #5 or symbol #8 on the special slot depending on UE capability uplinkTxSwitchingPeriod on NR TDD PCell

- symbol #4 or symbol #5 or symbol #8 on the 2nd special slot of every 8 slots depending on UE capability uplinkTxSwitchingPeriod NR TDD SCell

- Recommended WF

Further discussion

Issue 3-6: Which symbol to be verified for FDD-TDD uplink CA cases for TX switching across 3 or 4 bands for Dual TAG

- Proposals

  - Option 1(Nokia):

Symbol #8 or symbol #9 or symbol #10 in the slot overlapping with the special slot of the NR TDD carrier depending on UE capability uplinkTxSwitchingPeriod on NR FDD carrier

Symbol #4 or symbol #5 or symbol #8 on the special slot depending on UE capability uplinkTxSwitchingPeriod on NR TDD carrier

- Recommended WF

Further discussion

Issue 3-7: Which symbol to be verified for TDD-TDD uplink CA cases for TX switching across 3 or 4 bands for Dual TAG

- Proposals

  - Option 1(Nokia):

Symbol #4 or symbol #5 or symbol #8 in the slot overlapping with the special slot of the NR TDD carrier depending on UE capability uplinkTxSwitchingPeriod on NR TDD carrier

Symbol #4 or symbol #5 or symbol #8 on the special slot depending on UE capability uplinkTxSwitchingPeriod on NR TDD carrier

- Recommended WF

Further discussion