3GPP TSG-RAN WG4 Meeting #108bis												R4-2317219

Xiamen, China, 9 – 13 October, 2023

Agenda item:			5.31.2

Source:	Moderator (Ericsson)

Title:	Topic summary for [108bis][227] NR_redcap_enh

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

RRM contributions from agenda item 5.31.2, related to Rel-18 eRedCap WI, are discussed here.

# Topic #1: Extended eDRX for RRC INACTIVE

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2315117 | CATT | Proposal 1: No need to define new transition requirements for transition between short INACTIVE eDRX (≤10.24s) and long INACTIVE eDRX (20.48s). - Reuse legacy requirements for transition. |
| R4-2315426 | Xiaomi | Proposal 1: RAN4 to reuse transition requirements from IDLE eDRX for INACTIVE eDRX, where UE is required to meet the transition requirements which is less stringent requirement of the two requirements corresponding to the first- and second states during the transition time interval. |
| R4-2315578 | ZTE Corporation | Proposal 1: Support to reuse legacy requirements for the transition period between short INACTIVE eDRX (≤10.24sec) and long INACTIVE eDRX(20.48sec). |
| R4-2315664 | Huawei, HiSilicon | Proposal 1: UE needs to satisfy the less stringent requirements for the transition between short INACTIVE eDRX (≤10.24sec) and long INACTIVE eDRX(>10.24s). |
| R4-2315665 | Huawei, HiSilicon | CR on measurements of inter-RAT E-UTRAN cells for eRedCap UE |
| R4-2316244 | MediaTek inc. | Observation 1: In previous releases the requirements for the UE transiting from one state to another has the following requirements: ‘the UE follow the less stringent requirement of the two requirements corresponding to the first state and the second state’. Proposal 1: RAN4 should reuse the legacy requirements for this scenario, i.e., ‘the UE follow the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state’. |
| R4-2316245 | MediaTek inc. | Draft CR for measurement and evaluation of serving cell measurements for RedCap enhancements |
| R4-2316349 | Ericsson | Observation 1: Legacy eDRX requirements contain general transition requirements covering changes in eDRX cycle, eDRX cycele length and PTW configuration. - RAN4 to revise the legacy transition requirements to cover the transition when any of the eDRX parameters of eDRX_IDLE and eDRX_INACTIVE are changed. |
| R4-2316350 | Ericsson | Draft CR for introducing intra-frequency neighbour cell measurement requirements for releae 18 RedCap UE |
| R4-2316583 | Apple | Proposal 1: RAN4 to define transition requirement for case 1 below: - transition between Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX - transition between Rel-18 INACTIVE eDRX and INACTIVE RAN DRX. Proposal 2: Legacy principle can be reused to design the transition requirement for case 1: - UE needs to satisfy the less stringent requirements for the transition between Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, and between Rel-18 INACTIVE eDRX and INACTIVE RAN DRX. Proposal 3: No requirement is specified for case 2. |
| R4-2316584 | Apple | Draft CR on measurements of inter-frequency NR cells for eRedCap R18 |
| R4-2316720 | Nokia, Nokia Shanghai Bell | - RAN4 to consider both cases 1 and 2 to define transition requirements. RAN4 to use the legacy requirements as the baseline to define the transition requirements between short INACTIVE eDRX (≤10.24sec) and long INACTIVE eDRX (³20.48sec). |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1 Extended RRC INACTIVE eDRX cycle

Sub-topic description:

Under this sub-topic the requirements for eDRX cycle > 10.24s with PTW for Rel-18 RedCap UE in RRC INACTIVE state is discussed.

Open issues and candidate options before meeting:

Issue 1-1: Transition requirements: transition between long short INACTIVE eDRX (≤10.24s) and long INACTIVE eDRX (20.48s)

- Proposals

  - Option 1 (CATT, Xiaomi, ZTE, HW, MTK, Apple, Nokia): No new transition requirements are needed.

    - Legacy transition requirements are reused.

  - Option 1a (Ericsson): RAN4 to revise the legacy transition requirements to cover the transition when any of the eDRX parameters of eDRX_IDLE and eDRX_INACTIVE are changed.

  - Option 1b (Apple): Legacy principle can be reused to design the transition requirement for case 1:

      - transition between Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX

      - transition between Rel-18 INACTIVE eDRX and INACTIVE RAN DRX.

- Recommended WF

    - All companies support reusing the legacy principle for defining the transition requirements.

    - Option 1a suggest to revise the wording in legacy requirements to cover change of parameters of eDRX_IDLE and eDRX_INACTIVE.

    - Option 1c suggest to also include the transition between Rel-18 INACTIVE eDRX and INACTIVE RAN DRX which is also a valid case.

Moderator therefore suggests following revised option:

“If UE is configured with eDRX_INACTIVE ≥ 20.48s, when the UE transitions between any two states when changing eDRX_IDLE cycle length, eDRX_INACTIVE cycle length, INACTIVE RAN DRX length or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.”

Issue 1-2: Transition requirements: UE moves from a cell that supports and configures Rel-18 INACTIVE eDRX to a cell that supports only Rel-17 INACTIVE eDRX and vice versa.

- Proposals

  - Option 1 (Apple): No requirement is specified.

- Recommended WF

Check if option 1 can be agreed.