# 3GPP TSG-RAN WG4 Meeting # 108bis	R4-2317254

# Xiamen, China, October 09 – October 13, 2023

Agenda item:			8.13.8

Source:	Moderator (CMCC)

Title:	Topic summary for [108bis][131] NR_ATG_UERF_part1

Document for:	Information

Introduction

RAN#96 meeting approved RP-221369 Revised WID on Air-to-ground network for NR in Rel-18.

This thread focuses on adjacent channel co-existence evaluation for Rel-18 ATG and corresponds to agenda 5.13.1 and 5.13. According to the agreed timeline during last meeting, the target of this meeting is to finish the collection of simulation results for non-synchronization case.

Topic #1: Simulation assumption

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2315097 | CATT | This contribution provides more simulation results for ATG non-synchronized scenarios. ATG gNB -to TN gNB model: RMa with hUT=30m in 38.901. |
| R4-2315191 | CMCC | Observation 1: we have following three options for path loss model when evaluate ATG gNB-to-TN gNB CLI. - Option 1: Free space as listed in current TR - Option 2: Using RMa in TR 38.901 by updating hUT as 30m - Option 3: Using model in ITU-R P.452.P Proposal 1: there is no mandatory path loss model when evaluating ATG gNB-to-TN gNB CLI. All simulation results will be captured into final TR with explicitly state its corresponding path loss model provided final non-synchronized simulation results will only for information without impacting RF requirements. Observation 2: for 4GHz ATG DL interfering TN UL case, isolation distance is in the range [6, 17]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 1. Observation 3: for 4GHz TN DL interfering ATG UL case, isolation distance is in the range [5, 16]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 2. Observation 4: for 2GHz TN DL interfering ATG UL case, isolation distance is in the range [6, 18]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 3. Observation 5: for 4GHz ATG DL interfering TN UL case, isolation distance is in the range [15, >400]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 4. Observation 6: for 4GHz TN DL interfering ATG UL case, isolation distance is in the range [25, >600]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 5. Observation 7: for 2GHz TN DL interfering ATG UL case, isolation distance is in the range [35, >500]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 6. |
| R4-2315444 | Ericsson | Proposal 1 RAN4 to adopt the proposed network layout models and align on the orientation, to make sure the ATG BS, ATG UE and nearest TN BS sector point in the right directions. |
| R4-2315445 | Ericsson | Observation 1 For Scenario 5, where TN BS is the victim, the isolation is unreasonably large for real world deployments. Observation 2 For scenario 7 and 14 where ATG BS is the victim, the isolation distance that needs to be maintained for the 5% throughput degradation criteria between TN and ATG BS is substantial, making the non-synchronized scenarios real world deployments unserviceable. Based on the discussion in the previous sections we propose the following: Proposal 1 TN and ATG BS synchronization are needed to avoid such unserviceable large isolation distances. |
| R4-2315446 | Ericsson | Observation 1 The applicable range of User Terminal antenna height- hUT in the proposed RMa Pathloss model is . Using the model with hUT of 30m might likely lead to inaccuracy of the results. Observation 2 The applicable maximum distance - d2D in the proposed RMa Pathloss model is 10 km for LoS and 5km for NLoS. This again questions the validity and accuracy of the model, if applied to the ATG non-synchronized scenarios. Based on the discussion in the previous sections we propose the following: Proposal 1 RAN4 to further investigate the rationality of the proposed RMa Pathloss model for the ATG non-synchronized scenarios and provide further analysis. Proposal 2 There is a need for TN and ATG BS synchronization to avoid large isolation distances |
| R4-2315974 | Qualcomm CDMA Technologies | Observation 1: Inter-gNB cross-link interference assuming FSPL model is higher compared to the RMa 38.901 assumption. Observation 2: The application of RMa 38.901 model to derive the isolation distance between TN and ATG need adjustments to account for higher antenna heights and 2D distances. Observation 3: For case 5, where TN UL is a victim and ATG DL is aggressor, isolation distance equal 5km or higher is required to ensure throughput loss below the 5% threshold mark. For RMa model, smaller isolation distances are expected. Observation 4: For cases 7, where ATG UL is a victim and TN DL is aggressor, isolation distance larger than 9km and 14km for 4GHz and 2GHz, respectively, is required to ensure throughput loss below the 5% threshold mark assuming RMa pathloss model between ATG and BS gNBs. Observation 5: For cases 7, where ATG UL is a victim and TN DL is aggressor, isolation distance larger than 203km and 220km for 4GHz and 2GHz, respectively, is required to ensure throughput loss below the 5% threshold mark assuming FSPL pathloss model between ATG and BS gNBs. Observation 6: Observed significant difference between the isolation distances for RMa 38.901 and FSPL model is more impactful for the case when TN DL is aggressor as we have 57 interfering BSs compared to a single interfering gNB in the case of ATG DL as interferer. Proposal 1: RAN4 to consider only synchronized operation of TN and ATG deployments to avoid either large isolation distance or higher ACIR values. |

Open issues summary

Sub-topic 1-1 more clarification on non-synchronized layout

Issue 1-1: more clarification on non-synchronized layout

- Proposals

  - Option 1: RAN4 to adopt the proposed network layout models and align on the orientation, to make sure the ATG BS, ATG UE and nearest TN BS sector point in the right directions. Detailed as listed as below.

- Recommended WF

  - Moderator note: thanks for more clarification for this layout, it seems we are aligned with above option 1 and we just need to state it explicitly in final TR

  - Option 1

Case 1 - Angle between ATG BS boresight and nearest TN BS boresight in azimuth is 0 degree.

This is the case where the TN and ATG network coverage is overlapping. Figure 1 depicts this case with the agreed assumptions in WFs [1] [2], also highlighted below –

- The nearest TN BS sector points at the ATG BS in azimuth, with angle between the ATG BS boresight and nearest TN boresight as 0 degree ((In Figure 1, highlighted through the orange dotted line)

- ATG BS points at the ATG UE (In Figure 1, highlighted through the orange dotted line).

- ATG BS, ATG UE and TN cluster center are in a straight line.

- Isolation distance is the between the ATG BS and nearest TN BS.

- ATG UE is dropped between the maximum and minimum distance assumption depending on the ATG/ TN BS antenna configuration.

Figure 1: Angle between ATG BS boresight and nearest TN BS boresight in azimuth is 0 degree

Case 2 - Angle between ATG BS boresight and nearest TN BS boresight in azimuth is 30 degrees.

This is the case where the TN and ATG network coverage is non-overlapping. Figure 2 depicts this case with the agreed assumptions in WFs [1] [2], also highlighted below –

- The nearest TN BS sector points at the ATG BS in azimuth, with angle between the ATG BS boresight and nearest TN boresight as 30 degrees ((In Figure 2, highlighted through the orange dotted line)

- ATG BS points at the ATG UE ((In Figure 2, highlighted through the orange dotted line)

- Isolation distance is the between the ATG BS and nearest TN BS.

- ATG UE is dropped between the maximum and minimum distance assumption depending on the ATG/ TN BS antenna configuration.

Figure 2: Angle between ATG BS boresight and nearest TN BS boresight in azimuth is 30 degrees

Case 3 - Angle between ATG BS boresight and nearest TN BS boresight in azimuth is 60 degrees.

This is the case where the TN and ATG network coverage is non-overlapping. Figure 3 depicts this case with the agreed assumptions in WFs [1] [2], also highlighted below –

- The nearest TN BS sector points at the ATG BS in azimuth, with angle between the ATG BS boresight and nearest TN boresight as 60 degrees ((In Figure 3, highlighted through the orange dotted line)

- ATG BS points at the ATG UE ((In Figure 3, highlighted through the orange dotted line)

- Isolation distance is the between the ATG BS and nearest TN BS.

- ATG UE is dropped between the maximum and minimum distance assumption depending on the ATG/ TN BS antenna configuration.

Figure 3: Angle between ATG BS boresight and nearest TN BS boresight in azimuth is 60 degrees

Sub-topic 1-2 the conclusion of non-synchronized scenario

Issue 1-1: the conclusion of non-synchronized scenario

- Proposals

  - Option 1: TN and ATG BS synchronization are needed to avoid either large isolation distance or higher ACIR values. (Ericsson, Qualcomm)

  - Option 2: doesn’t conclude any conclusion based on isolation distance into final TR, just capture the isolation distance into TR for information.

- Recommended WF

  - TBA

Sub-topic 1-3 ATG gNB-to-TN gNB path-loss modelling

Issue 1-1: ATG gNB-to-TN gNB path-loss modelling

- Proposals

  - Option 1: there is no mandatory path loss model when evaluating ATG gNB-to-TN gNB CLI. All simulation results will be captured into final TR with explicitly state its corresponding path loss model provided final non-synchronized simulation results will only for information without impacting RF requirements. (CMCC)

  - Option 2: Proposal 1	RAN4 to further investigate the rationality of the proposed RMa Pathloss model for the ATG non-synchronized scenarios and provide further analysis. (Ericsson)

    - Observation 1 - The applicable range of User Terminal antenna height- hUT in the proposed RMa Pathloss model is . Using the model with hUT of 30m might likely lead to inaccuracy of the results.

    - Observation 2 - The applicable maximum distance - d2D in the proposed RMa Pathloss model is 10 km for LoS and 5km for NLoS. This again questions the validity and accuracy of the model, if applied to the ATG non-synchronized scenarios.

- Recommended WF

  - TBA

Some other observations from Qualcomm are listed as below for information

Observation 6: Observed significant difference between the isolation distances for RMa 38.901 and FSPL model is more impactful for the case when TN DL is aggressor as we have 57 interfering BSs compared to a single interfering gNB in the case of ATG DL as interferer.

- Topic #3: Simulation results of non-synchronized scenario

  - Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2315097 | CATT | This contribution provides more simulation results for ATG non-synchronized scenarios. ATG gNB -to TN gNB model: RMa with hUT=30m in 38.901. |
| R4-2315189 | CMCC | This TP update some agreements and editorial errors, besides, adding some missing agreements from clause 1 to clause 6. |
| R4-2315191 | CMCC | Observation 1: we have following three options for path loss model when evaluate ATG gNB-to-TN gNB CLI. - Option 1: Free space as listed in current TR - Option 2: Using RMa in TR 38.901 by updating hUT as 30m - Option 3: Using model in ITU-R P.452.P Proposal 1: there is no mandatory path loss model when evaluating ATG gNB-to-TN gNB CLI. All simulation results will be captured into final TR with explicitly state its corresponding path loss model provided final non-synchronized simulation results will only for information without impacting RF requirements. Observation 2: for 4GHz ATG DL interfering TN UL case, isolation distance is in the range [6, 17]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 1. Observation 3: for 4GHz TN DL interfering ATG UL case, isolation distance is in the range [5, 16]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 2. Observation 4: for 2GHz TN DL interfering ATG UL case, isolation distance is in the range [6, 18]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 3. Observation 5: for 4GHz ATG DL interfering TN UL case, isolation distance is in the range [15, >400]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 4. Observation 6: for 4GHz TN DL interfering ATG UL case, isolation distance is in the range [25, >600]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 5. Observation 7: for 2GHz TN DL interfering ATG UL case, isolation distance is in the range [35, >500]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 6. |
| R4-2315192 | CMCC | Collection of non-synchronized scenario calibration data |
| R4-2315197 | CMCC | This TP add the calibration assumptions and results which is approved in RAN4 #106 bis meeting. Detailed excel file for calibration results are captured into R4-2304204(excel file). |
| R4-2315443 | Ericsson | TP for TR 38.876 Addition of Summary for synchronized scenarios simulation results |
| R4-2315444 | Ericsson | Proposal 1 RAN4 to adopt the proposed network layout models and align on the orientation, to make sure the ATG BS, ATG UE and nearest TN BS sector point in the right directions. |
| R4-2315445 | Ericsson | Observation 1 For Scenario 5, where TN BS is the victim, the isolation is unreasonably large for real world deployments. Observation 2 For scenario 7 and 14 where ATG BS is the victim, the isolation distance that needs to be maintained for the 5% throughput degradation criteria between TN and ATG BS is substantial, making the non-synchronized scenarios real world deployments unserviceable. Based on the discussion in the previous sections we propose the following: Proposal 1 TN and ATG BS synchronization are needed to avoid such unserviceable large isolation distances. |
| R4-2315446 | Ericsson | Observation 1 The applicable range of User Terminal antenna height- hUT in the proposed RMa Pathloss model is . Using the model with hUT of 30m might likely lead to inaccuracy of the results. Observation 2 The applicable maximum distance - d2D in the proposed RMa Pathloss model is 10 km for LoS and 5km for NLoS. This again questions the validity and accuracy of the model, if applied to the ATG non-synchronized scenarios. Based on the discussion in the previous sections we propose the following: Proposal 1 RAN4 to further investigate the rationality of the proposed RMa Pathloss model for the ATG non-synchronized scenarios and provide further analysis. Proposal 2 There is a need for TN and ATG BS synchronization to avoid large isolation distances |
| R4-2315904 | CMCC | For post meeting email discussion |
| R4-2315974 | Qualcomm CDMA Technologies | Observation 1: Inter-gNB cross-link interference assuming FSPL model is higher compared to the RMa 38.901 assumption. Observation 2: The application of RMa 38.901 model to derive the isolation distance between TN and ATG need adjustments to account for higher antenna heights and 2D distances. Observation 3: For case 5, where TN UL is a victim and ATG DL is aggressor, isolation distance equal 5km or higher is required to ensure throughput loss below the 5% threshold mark. For RMa model, smaller isolation distances are expected. Observation 4: For cases 7, where ATG UL is a victim and TN DL is aggressor, isolation distance larger than 9km and 14km for 4GHz and 2GHz, respectively, is required to ensure throughput loss below the 5% threshold mark assuming RMa pathloss model between ATG and BS gNBs. Observation 5: For cases 7, where ATG UL is a victim and TN DL is aggressor, isolation distance larger than 203km and 220km for 4GHz and 2GHz, respectively, is required to ensure throughput loss below the 5% threshold mark assuming FSPL pathloss model between ATG and BS gNBs. Observation 6: Observed significant difference between the isolation distances for RMa 38.901 and FSPL model is more impactful for the case when TN DL is aggressor as we have 57 interfering BSs compared to a single interfering gNB in the case of ATG DL as interferer. Proposal 1: RAN4 to consider only synchronized operation of TN and ATG deployments to avoid either large isolation distance or higher ACIR values. |
| R4-2316200 | Huawei, HiSilicon | Observation 1: There is no serious throughput loss for scenario 5. Observation 2: For scenario 7 and 14, throughput loss larger than 5% can be observed assuming FSPL model between ATG BS and TN BS, even if 100km isolation distance is set. Observation 3: For scenario 14, the reason why such large throughput loss can be observed is that ATG single carrier scenario without adjacent channel interference has a good throughput performance. As the UL Tx power for ATG UE is restricted, ATG UE can transmit higher power to ease the interference from TN network. |
| R4-2316526 | ZTE Corporation | For RMA channel model: - Observation 1: for Case 5 with overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 20km. - Observation 2a: for Case 5 with non-overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 3km with 30 degree angle shift. - Observation 2b: for Case 5 with non-overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 17km with 60 degree angle shift. - Observation 3: for Case 7 with overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 20km. - Observation 4a: for Case 7 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 9km with 30 degree angle shift. - Observation 4b: for Case 7 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 6km with 60 degree angle shift. - Observation 5: for Case 11 with overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 17km. - Observation 6a: for Case 11 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 10km with 30 degree angle shift. - Observation 6b: for Case 11 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 17km with 60 degree angle shift. For Free space channel model: - Observation 7: for Case 5 with overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 100km. - Observation 8a: for Case 5 with non-overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 10km with 30 degree angle shift. - Observation 8b: for Case 5 with non-overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 80km with 60 degree angle shift. - Observation 9: for Case 7 with overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 550km. - Observation 10a: for Case 7 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 50km with 30 degree angle shift. - Observation 10b: for Case 7 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 40km with 60 degree angle shift. - Observation 11: for Case 11 with overlapping coverage in Rural channel model case, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 700km. - Observation 12a: for Case 11 with non-overlapping coverage in Rural channel model case, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 90km with 30 degree angle shift. - Observation 12b: for Case 11 with non-overlapping coverage in Rural channel model case, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 70km with 60 degree angle shift. |

  - Open issues summary

Following is approved simulation scenarios.

Table 6.1-1: Simulation scenarios for ATG coexistence study

| No. | Combination | Aggressor |  | Victim |  | Simulation frequency | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | deployment scenario UL/DL | CBW duplex mode | deployment scenario UL/DL | CBW duplex mode |  |  |  |
| 1 | TN with ATG | ATG DL | 100MHz TDD | TN rural DL | 100MHz /TDD | 4GHz |  | Phase 1 |
| 2 | TN with ATG | ATG UL | 100MHz TDD | TN rural UL | 100MHz TDD | 4GHz |  | Phase 1 |
| 3 | TN with ATG | TN rural DL | 100MHz TDD | ATG DL | 100MHz TDD | 4GHz |  | Phase 1 |
| 4 | TN with ATG | TN rural UL | 100MHz TDD | ATG UL | 100MHz TDD | 4GHz |  | Phase 1 |
| 5 | TN with ATG | ATG DL | 100MHz TDD | TN rural UL | 100MHz /TDD | 4GHz |  | FFS |
| 6 | TN with ATG | ATG UL | 100MHz TDD | TN rural DL | 100MHz TDD | 4GHz |  | FFS |
| 7 | TN with ATG | TN rural DL | 100MHz TDD | ATG UL | 100MHz TDD | 4GHz |  | FFS |
| 8 | TN with ATG | TN rural UL | 100MHz TDD | ATG DL | 100MHz TDD | 4GHz |  | FFS |
| 9 | TN with ATG | ATG DL | 20MHz FDD | TN rural DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 10 | TN with ATG | ATG UL | 20MHz FDD | TN rural UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 11 | TN with ATG | TN rural DL | 20MHz FDD | ATG DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 12 | TN with ATG | TN rural UL | 20MHz FDD | ATG UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 13 | TN with ATG | ATG UL | 20MHz FDD | TN rural DL | 20MHz TDD | 2 GHz | n1/n39 | FFS |
| 14 | TN with ATG | TN rural DL | 20MHz TDD | ATG UL | 20MHz FDD | 2 GHz | n39/n1 | FFS |

Sub-topic 3-1 Calibration data for non-synchronized co-existence scenario

CMCC, Ericsson, Qualcomm, ZTE, CATT, Huawei provide non-synchronized co-existence scenario for one or two kinds of path loss modeling, one for free space modeling and the other for RMa model in TR 38.901 by updating hUT as 30m. detailed calibration data are captured into R4-2315192.

Thanks all for the efforts.

Sub-topic 3-2 Detailed simulation results based on RMa modeling in TR 38.901 with updating hUT as 30m

For non-synchronized scenario, gNB-to-gNB interference is the dominant interference type. Non-synchronization scenario will only focus on scenario 5, 7, 14.

2.2.2.1	Simulation scenario 5: ATG DL -> TN UL (4GHz)

Recommended WF:

- For ATG and TN overlapping network, at most 20km is enough to meet 5% throughput loss criteria when Angle between ATG BS boresight and nearest TN BS boresight in azimuth is zero

- For ATG and TN non-overlapping network, at most 17km is enough to meet 5% throughput loss criteria when Angle between ATG BS boresight and nearest TN BS boresight in azimuth is 30 or 60 degree.

Summary of simulation results:

Table 1: Simulation results for Scenario 5 – 4GHz ATG DL interfering TN UL based on RMa model in 38.901

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 17 | 11 | 9 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim | 12 | 7 | 6 |
|  | Subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
| CATT | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 19 | 13 | 8 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim | <5 | <5 | <5 |
| ZTE | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 20 | 3 | 17 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |

Detailed observation

Following list companies’ simulation results.

- Observation: Simulation result from CATT revision of R4-2315097.

The isolation distances are summarized in Table 1.

Table 1: Simulation results for Scenario 5 – 4GHz ATG DL interfering TN UL

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CATT | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 19 | 13 | 8 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim | <5 | <5 | <5 |

- Observation: Simulation result from Ericsson R4-2311264.

In scenario 5 (below), the isolation distance needed to maintain the throughput degradation of the TN (most impacted cell) at 5% is more than 32 km for the zero-degree case, more than 11 km for the thirty-degree case, and more than 3 km for the sixty-degree case.

- Observation: Simulation result from CMCC R4-2315191.

Table 1: Simulation results for Scenario 5 – 4GHz ATG DL interfering TN UL based on RMa model in 38.901

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 17 | 11 | 9 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim | 12 | 7 | 6 |
|  | Subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |

Observation 2: for 4GHz ATG DL interfering TN UL case, isolation distance is in the range [6, 17]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 1.

- Observation: Simulation result from ZTE R4-2316526

- Observation 1: for Case 5 with overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 20km.

- Observation 2a: for Case 5 with non-overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 3km with 30 degree angle shift.

- Observation 2b: for Case 5 with non-overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 17km with 60 degree angle shift.

2.2.2.2	Simulation scenario 7: TN rural DL -> ATG UL (4GHz)

Recommended WF:

- For ATG and TN overlapping network, at most 20km is enough to meet 5% throughput loss criteria when Angle between ATG BS boresight and nearest TN BS boresight in azimuth is zero

- For ATG and TN non-overlapping network, at most 13km is enough to meet 5% throughput loss criteria when Angle between ATG BS boresight and nearest TN BS boresight in azimuth is 30 or 60 degree.

Summary of simulation results:

Table 2: Simulation results for Scenario 7 – 4GHz TN DL interfering ATG UL based on RMa model in 38.901

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 16 | 9 | 8 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim | 11 | 7 | 5 |
|  | Subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
| CATT | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 18 | 13 | 11 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim | 15 | 10 | 7 |
| Qualcomm | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 9 |  |  |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
| ZTE | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 20 | 9 | 6 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |

Detailed observation

Following list companies’ simulation results.

- Observation: Simulation result from Ericsson R4-2311264.

In scenario 7 (below), the isolation distance needed to maintain the throughput degradation of the ATG network at 5% is around 150 km for the zero-degree case, around 16 km for the thirty-degree case, and around 6 km for the sixty-degree case.

- Observation: Simulation result from CATT revision of R4-2315097.

The isolation distances are summarized in Table 2.

Table 2: Simulation results for Scenario 7 – 4GHz TN DL interfering ATG UL

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CATT | Non-Subarray | 5% in the whole network | 18 | 13 | 11 |
|  |  | Average of all users in the whole network | 15 | 10 | 7 |

- Observation: Simulation result from CMCC R4-2315191.

Table 2: Simulation results for Scenario 7 – 4GHz TN DL interfering ATG UL

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-Subarray | 5% in the whole network | 16 | 9 | 8 |
|  |  | Average of all users in the whole network | 11 | 7 | 5 |
|  | Subarray | 5% in the whole network |  |  |  |
|  |  | Average of all users in the whole network |  |  |  |

Observation 3: for 4GHz TN DL interfering ATG UL case, isolation distance is in the range [5, 16]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 2.

- Observation: Simulation result from ZTE R4-2316526

- Observation 3: for Case 7 with overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 20km.

- Observation 4a: for Case 7 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 9km with 30 degree angle shift.

- Observation 4b: for Case 7 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 6km with 60 degree angle shift.

- Observation: Simulation result from Qualcomm R4-2315974

Observation 4: For cases 7, where ATG UL is a victim and TN DL is aggressor, isolation distance larger than 9km and 14km for 4GHz and 2GHz, respectively, is required to ensure throughput loss below the 5% threshold mark assuming RMa pathloss model between ATG and BS gNBs.

Figure 4 Throughput loss as a function of isolation distance for case 7 (left) and case 14 (right) assuming RMa 38.901 model.

2.2.2.3	Simulation scenario 14: TN rural DL -> ATG UL (2GHz)

Recommended WF:

- For ATG and TN overlapping network, at most 26km is enough to meet 5% throughput loss criteria when Angle between ATG BS boresight and nearest TN BS boresight in azimuth is zero

- For ATG and TN non-overlapping network, at most 19km is enough to meet 5% throughput loss criteria when Angle between ATG BS boresight and nearest TN BS boresight in azimuth is 30 or 60 degree.

Summary of simulation results:

Table 3: Simulation results for Scenario 14 – 2GHz TN DL interfering ATG UL based on RMa model in 38.901

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 18 | 12 | 10 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim | 13 | 0 | 6 |
|  | Subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
| CATT | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 26 | 19 | 19 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim | 22 | 18 | 16 |
| Qualcomm | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 14 |  |  |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
| ZTE | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 17 | 10 | 6 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |

Detailed observation

Following list companies’ simulation results.

- Observation: Simulation result from Ericsson R4-2311264.

In scenario 14 (below), the isolation distance needed to maintain the throughput degradation of the ATG network at 5% is around 220 km for the zero-degree case, around 26 km for the thirty-degree case, and around 6 km for the sixty-degree case.

- Observation: Simulation result from CATT revision of R4-2315097.

The isolation distances are summarized in Table 3.

Table 3: Simulation results for Scenario 14 – 2GHz TN DL interfering ATG UL

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CATT | Non-Subarray | 5% in the whole network | 26 | 19 | 19 |
|  |  | Average of all users in the whole network | 22 | 18 | 16 |

20 km isolation distance is needed.

- Observation: Simulation result from CMCC R4-2315191..

Following table list our simulation results.

Table 3: Simulation results for Scenario 13 – 2GHz TN DL interfering ATG UL

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-Subarray | 5% in the whole network | 18 | 12 | 10 |
|  |  | Average of all users in the whole network | 13 | 0 | 6 |
|  | Subarray | 5% in the whole network |  |  |  |
|  |  | Average of all users in the whole network |  |  |  |

Noted: the reason that 30 degree will lead to less isolation distance is because at 30degree direction, antenna pattern is almost null and lead to less interference in our simulation.

Observation 4: for 2GHz TN DL interfering ATG UL case, isolation distance is in the range [6, 18]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 3.

- Observation : Simulation result from ZTE R4-2316526

- Observation 5: for Case 14 with overlapping coverage in Rural channel model case, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 17km.

- Observation 6a: for Case 14 with non-overlapping coverage in Rural channel model case, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 10km with 30 degree angle shift.

- Observation 6b: for Case 14 with non-overlapping coverage in Rural channel model case, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 6km with 60 degree angle shift.

- Observation: Simulation result from Qualcomm R4-2315974

Observation 4: For cases 7, where ATG UL is a victim and TN DL is aggressor, isolation distance larger than 9km and 14km for 4GHz and 2GHz, respectively, is required to ensure throughput loss below the 5% threshold mark assuming RMa pathloss model between ATG and BS gNBs.

Figure 4 Throughput loss as a function of isolation distance for case 7 (left) and case 14 (right) assuming RMa 38.901 model.

Sub-topic 3-3 Detailed simulation results based on free space modeling

For non-synchronized scenario, gNB-to-gNB interference is the dominant interference type. Non-synchronization scenario will only focus on scenario 5, 7, 14.

2.2.3.1	Simulation scenario 5: ATG DL -> TN UL (4GHz)

The simulation results are not aligned. No recommended WF.

Summary of simulation results:

Table 4: Simulation results for Scenario 5 – 4GHz ATG DL interfering TN UL based on free space modeling

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | >400 | 50 | 35 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim | 200 | 30 | 15 |
|  | Subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
| ZTE | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 100 | 10 | 80 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
| Ericsson | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 230 | 33 | 21 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
| Qualcomm | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | Equal or higher 5km |  |  |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
| Huawei | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | <5km |  |  |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |

Detailed observation

Following list companies’ simulation results.

- Observation: Simulation result from Ericsson R4-2315445.

We have only focused on the FSPL model as agreed in the TR [3] and questioned on the validity of the RMa model proposed in offline email discussions and in our companion contribution [1].

In Scenario 5, Figure 1 shows the results for the non-subarray antenna configuration. As can be seen from the figure, isolation distance needed to maintain the 5% throughput degradation criteria for ATG network is 230 km for the zero-degree case, more than 33 km for the thirty-degree case, and more than 21 km for the sixty-degree case.

Figure 1: Isolation distance for Scenario 5, non-subarray model

For Scenario 5, where TN BS is the victim, the isolation is unreasonably large for real world deployments.

.

- Observation: Simulation result from CMCC R4-2315191.

Table 4: Simulation results for Scenario 5 – 4GHz ATG DL interfering TN UL based on free space modeling

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | >400 | 50 | 35 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim | 200 | 30 | 15 |
|  | Subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |

Observation 5: for 4GHz ATG DL interfering TN UL case, isolation distance is in the range [15, >400]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 4.

- Observation: Simulation result from ZTE R4-2316526

- Observation 7: for Case 5 with overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 100km.

- Observation 8a: for Case 5 with non-overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 10km with 30 degree angle shift.

- Observation 8b: for Case 5 with non-overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 80km with 60 degree angle shift.

- .

- Observation: Simulation result from Qualcomm R4-2315974

This scenario aims at identifying the isolation distance between TN cluster and ATG gNB when the TN gNBs are the victim while experiencing interference from the ATG gNB. Figure 3 presents the throughput loss as a function of the isolation distance for case 5 considering FSPL model. It can be observed that an isolation distance in the greater than 5km is needed to ensure the throughput loss is below the 5% loss threshold.

Figure 3 Throughput loss as a function of isolation distance for case 5 assuming FSPL.

Observation 3: For case 5, where TN UL is a victim and ATG DL is aggressor, isolation distance equal 5km or higher is required to ensure throughput loss below the 5% threshold mark. For RMa model, smaller isolation distances are expected.

- Observation: Simulation result from Huawei R4-2316200

The simulation results for scenario 5 were provided below assuming 5km isolation distance. The throughput loss is 2.6% which is less than 5%. Thus, there is no serious throughput loss for scenario 5.

2.2.3.2	Simulation scenario 7: TN rural DL -> ATG UL (4GHz)

Recommended WF:

- For ATG and TN overlapping network, hundreds of kilometers is required to meet 5% throughput loss criteria when Angle between ATG BS boresight and nearest TN BS boresight in azimuth is zero

- For ATG and TN non-overlapping network, at most 75km is enough to meet 5% throughput loss criteria when Angle between ATG BS boresight and nearest TN BS boresight in azimuth is 30 or 60 degree.

Summary of simulation results:

Table 5: Simulation results for Scenario 5 – 4GHz TN DL interfering ATG UL based on free space modeling

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | >500 | 75 | 55 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim | >300 | 45 | 25 |
|  | Subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
| ZTE | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 550 | 50 | 40 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
| Ericsson | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | >300 | 70 | 37 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
| Qualcomm | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 203 |  |  |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
| Huawei | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | >1000 |  |  |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |

- When boresight is zero,

  - some companies show tens of km is enough even for the case when boresight is zero (CATT 11km, CMCC 9km, ZTE 20km)

  - whereas other companies show 100-300km is required(Qualcomm larger than 200km, Ericsson 150km) is required.

- When boresight is 30 degree or 60 degree,

  - companies show tens of km is enough even for the case when boresight is zero (CATT 11km, CMCC 5km, ZTE 9km, Ericsson 16km)

Detailed observation

Following list companies’ simulation results.

- Observation: Simulation result from Ericsson R4-2315445.

We have only focused on the FSPL model as agreed in the TR [3] and questioned on the validity of the RMa model proposed in offline email discussions and in our companion contribution [1].

In Scenario 7, Figure 2 shows the results for the non-subarray antenna configuration. As can be seen from the figure, isolation distance needed to maintain the 5% throughput degradation criteria for ATG network is more than 300 km for the zero-degree case, around 70 km for the thirty-degree case, and around 37 km for the sixty-degree case.

Figure 2: Isolation distance for Scenario 7, non-subarray model

- Observation: Simulation result from CMCC R4-2315191.

.

Table 2: Simulation results for Scenario 7 – 4GHz TN DL interfering ATG UL

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-Subarray | 5% in the whole network | >500 | 75 | 55 |
|  |  | Average of all users in the whole network | >300 | 45 | 25 |
|  | Subarray | 5% in the whole network |  |  |  |
|  |  | Average of all users in the whole network |  |  |  |

Noted: the reason that 30 degree will lead to less isolation distance is because at 30degree direction, antenna pattern is almost null steering and lead to less interference in our simulation.

Observation 6: for 4GHz TN DL interfering ATG UL case, isolation distance is in the range [25, >600]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 5.

- Observation: Simulation result from ZTE R4-2316526

- Observation 9: for Case 7 with overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 550km.

- Observation 10a: for Case 7 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 50km with 30 degree angle shift.

- Observation 10b: for Case 7 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 40km with 60 degree angle shift.

- .

- Observation: Simulation result from Qualcomm R4-2315974

Observation 5: For cases 7, where ATG UL is a victim and TN DL is aggressor, isolation distance larger than 203km and 220km for 4GHz and 2GHz, respectively, is required to ensure throughput loss below the 5% threshold mark assuming FSPL pathloss model between ATG and BS gNBs.

Observation 6: Observed significant difference between the isolation distances for RMa 38.901 and FSPL model is more impactful for the case when TN DL is aggressor as we have 57 interfering BSs compared to a single interfering gNB in the case of ATG DL as interferer.

Figure 5 Throughput loss as a function of isolation distance for case 7 (left) and case 14 (right) assuming FSPL model.

- Observation: Simulation result from Huawei R4-2316200

Table 2 the performance loss for different isolation distance

| Isolation distance | 5km | 100km | 1000km |
| --- | --- | --- | --- |
| SINR at 50% CDF point | -8dB | 2 | 7dB |
| SINR at 5% CDF point | -30dB | -10 | 1dB |
| Throughput loss at 5% CDF point | 100% (51.589Mbps loss) | 85.8% (43.698Mbps loss) | 6.7% (3.48Mbps loss) |

Simulation scenario 14: TN rural DL -> ATG UL (2GHz)

Recommended WF:

- For ATG and TN overlapping network, hundreds of kilometers is required to meet 5% throughput loss criteria when Angle between ATG BS boresight and nearest TN BS boresight in azimuth is zero

- For ATG and TN non-overlapping network, over 100km is required to meet 5% throughput loss criteria when Angle between ATG BS boresight and nearest TN BS boresight in azimuth is 30 or 60 degree.

Summary of simulation results:

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | >500 | >100 | 80 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim | 500 | 75 | 35 |
|  | Subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
| ZTE | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 700 | 90 | 70 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
| Ericsson | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 300 | 120 | 90 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
| Qualcomm | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 220 |  |  |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
| Huawei | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | >1000 |  |  |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |

- When boresight is zero,

  - some companies show tens of km is enough even for the case when boresight is zero (CATT 20km, CMCC 12km, ZTE 20km)

  - whereas other companies show 100-300km or even larger is required(Qualcomm larger than 200km, Ericsson 220km, Huawei larger distance) is required.

- When boresight is 30 degree or 60 degree,

  - companies show tens of km is enough even for the case when boresight is zero (CMCC 3-5km, ZTE 10-17km, Ericsson 6-26km)

Detailed observation

Following list companies’ simulation results.

- Observation: Simulation result from Ericsson R4-2315445.

We have only focused on the FSPL model as agreed in the TR [3] and questioned on the validity of the RMa model proposed in offline email discussions and in our companion contribution [1].

In Scenario 14, Figure 2 shows the results for the non-subarray antenna configuration the isolation distance needed to maintain the throughput degradation of the ATG network at 5% is, around 300 km for the zero-degree case, around 120 km for the thirty-degree case, and around 90 km for the sixty-degree case.

Figure 3: Isolation distance for Scenario 14, non-subarray model

- Observation: Simulation result from CMCC R4-2315191.

Table 6: Simulation results for Scenario 13 – 2GHz TN DL interfering ATG UL

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-Subarray | 5% in the whole network | >500 | >100 | 80 |
|  |  | Average of all users in the whole network | 500 | 75 | 35 |
|  | Subarray | 5% in the whole network |  |  |  |
|  |  | Average of all users in the whole network |  |  |  |

Observation 7: for 2GHz TN DL interfering ATG UL case, isolation distance is in the range [35, >500]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 6.

- Observation : Simulation result from ZTE R4-2316526

- Observation 11: for Case 11 with overlapping coverage in Rural channel model case, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 700km.

- Observation 12a: for Case 11 with non-overlapping coverage in Rural channel model case, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 90km with 30 degree angle shift.

- Observation 12b: for Case 11 with non-overlapping coverage in Rural channel model case, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 70km with 60 degree angle shift

- Observation: Simulation result from Qualcomm R4-2315974

Observation 4: For cases 7, where ATG UL is a victim and TN DL is aggressor, isolation distance larger than 9km and 14km for 4GHz and 2GHz, respectively, is required to ensure throughput loss below the 5% threshold mark assuming RMa pathloss model between ATG and BS gNBs.

Figure 4 Throughput loss as a function of isolation distance for case 7 (left) and case 14 (right) assuming RMa 38.901 model.

Observation 5: For cases 7, where ATG UL is a victim and TN DL is aggressor, isolation distance larger than 203km and 220km for 4GHz and 2GHz, respectively, is required to ensure throughput loss below the 5% threshold mark assuming FSPL pathloss model between ATG and BS gNBs.

Observation 6: Observed significant difference between the isolation distances for RMa 38.901 and FSPL model is more impactful for the case when TN DL is aggressor as we have 57 interfering BSs compared to a single interfering gNB in the case of ATG DL as interferer.

Figure 5 Throughput loss as a function of isolation distance for case 7 (left) and case 14 (right) assuming FSPL model.

- Observation: Simulation result from Huawei R4-2316200

Table 2 the performance loss for different isolation distance

| Isolation distance | 10km | 100km | 1000km |
| --- | --- | --- | --- |
| SINR at 50% CDF point | -8dB | 7dB | 14.5dB |
| SINR at 5% CDF point | -25dB | -2.5dB | 11dB |
| Throughput loss at 5% CDF point | 100% (40.22Mbps loss) | 87.7% (32.295Mbps loss) | 24.8% (9.99Mbps loss) |

Observation 3: For scenario 14, the reason why such large throughput loss can be observed is that ATG single carrier scenario without adjacent channel interference has a good throughput performance. As the UL Tx power for ATG UE is restricted, ATG UE can transmit higher power to ease the interference from TN network.