| 3GPP TR 37.718-00-00 V0.6.0 (2023-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; Band combinations for SA NR Supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP); (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	9

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Background	10

4.1	TR Maintenance	10

5	SA NR SUL band combination: Specific Band Combination Part	10

5.1	CA_n1_SUL_n78-n81	10

5.1.1	Operating bands	10

5.1.2	Channel bandwidths per operating band	10

5.1.3	Maximum output power	11

5.1.4	Spurious emission band UE co-existence	11

5.1.5	REFSENS requirements	11

5.1.6	∆TIB and ∆RIB values	11

5.2	CA_n3_SUL_n41-n80	12

5.2.1	Operating bands	12

5.2.2	Channel bandwidths per operating band	12

5.2.3	Maximum output power	12

5.2.4	Spurious emission band UE co-existence	12

5.2.5	REFSENS requirements	12

5.2.6	∆TIB and ∆RIB values	13

5.3	CA_n41_SUL_n79-n98	13

5.3.1	Operating bands	13

5.3.2	Channel bandwidths per operating band	13

5.3.3	Maximum output power	14

5.3.4	Spurious emission band UE co-existence	14

5.3.5	REFSENS requirements	14

5.3.6	∆TIB and ∆RIB values	14

5.4	CA_n79_SUL_n41-n98	15

5.4.1	Operating bands	15

5.4.2	Channel bandwidths per operating band	15

5.4.3	Maximum output power	15

5.4.4	Spurious emission band UE co-existence	16

5.4.5	REFSENS requirements	16

5.4.6	∆TIB and ∆RIB values	16

5.5	CA_n41_SUL_n79-n95	17

5.5.1	Operating bands	17

5.5.2	Channel bandwidths per operating band	17

5.5.3	Maximum output power	17

5.5.4	Spurious emission band UE co-existence	17

5.5.5	REFSENS requirements	18

5.5.6	∆TIB and ∆RIB values	18

5.6	CA_n79_SUL_n41-n95	19

5.6.1	Operating bands	19

5.6.2	Channel bandwidths per operating band	19

5.6.3	Maximum output power	19

5.6.4	Spurious emission band UE co-existence	19

5.6.5	REFSENS requirements	20

5.6.6	∆TIB and ∆RIB values	20

5.7	CA_n8_SUL_n78-n81	20

5.7.1	Operating bands	20

5.7.2	Channel bandwidths per operating band	21

5.7.3	Maximum output power	21

5.7.4	Spurious emission band UE co-existence	21

5.7.5	REFSENS requirements	21

5.7.6	∆TIB and ∆RIB values	22

5.8	 SUL_n1-n81	22

5.8.1	Operating bands	22

5.8.2	Channel bandwidths per operating band	23

5.8.3 	Maximum output power	23

5.8.4	 Spurious emission band UE co-existence	23

5.8.5	∆TIB and ∆RIB values	24

5.9	 SUL_n3-n84	24

5.9.1	Operating bands	24

5.9.2	Channel bandwidths per operating band	25

5.9.3 	Maximum output power	25

5.9.4	 Spurious emission band UE co-existence	25

5.9.5	REFSENS requirements	26

5.10	 SUL_n1-n89	26

5.10.1	Operating bands	26

5.10.2	Channel bandwidths per operating band	26

5.10.3 	Maximum output power	27

5.10.4	 Spurious emission band UE co-existence	28

5.10.5	∆TIB and ∆RIB values	29

5.11	 SUL_n1-n80	29

5.11.1	Operating bands	29

5.11.2	Channel bandwidths per operating band	30

5.11.3 	Maximum output power	30

5.11.4	 Spurious emission band UE co-existence	30

5.11.5	REFSENS requirements	31

5.11.6	∆TIB and ∆RIB values	31

5.12	 SUL_n78-n89	31

5.12.1	Operating bands	31

5.12.2	Channel bandwidths per operating band	32

5.12.3 	Maximum output power	32

5.12.4	 Spurious emission band UE co-existence	32

5.12.5	REFSENS requirements	32

5.12.6	∆TIB and ∆RIB values	32

5.13	 CA_n78_SUL_n1-n81	33

5.13.1	Operating bands	33

5.13.2	Channel bandwidths per operating band	34

5.13.3 	Maximum output power	34

5.13.4	 Spurious emission band UE co-existence	34

5.13.5	REFSENS requirements	34

5.13.6	∆TIB and ∆RIB values	35

5.14	 CA_n78_SUL_n3-n84	35

5.14.1	Operating bands	35

5.14.2	Channel bandwidths per operating band	36

5.14.3 	Maximum output power	36

5.14.4	 Spurious emission band UE co-existence	36

5.14.5	REFSENS requirements	36

5.14.6	∆TIB and ∆RIB values	37

5.15	 CA_n78_SUL_n1-n89	37

5.15.1	Operating bands	37

5.15.2	Channel bandwidths per operating band	38

5.15.3 	Maximum output power	38

5.15.4	 Spurious emission band UE co-existence	38

5.15.5	REFSENS requirements	38

5.15.6	∆TIB and ∆RIB values	39

5.16	 CA_n78_SUL_n1-n80	39

5.16.1	Operating bands	39

5.16.2	Channel bandwidths per operating band	40

5.16.3 	Maximum output power	40

5.16.4	 Spurious emission band UE co-existence	40

5.16.5	REFSENS requirements	40

5.16.6	∆TIB and ∆RIB values	41

5.Y	SUL_nX-nY/CA_nX_SUL_nY-nZ	41

5.Y.1	Operating bands	41

5.Y.2	Configuration	41

5.Y.3	Maximum output power	41

5.Y.4	Spurious emission band UE co-existence	41

5.Y.5	REFSENS requirements	41

5.Y.6	∆TIB and ∆RIB values	42

5.Y.7	Out-of-band blocking exception	42

6	NSA NR SUL band combination: Specific Band Combination Part	42

6.Y	DC_X_SUL_nY-nZ	42

6.Y.1	Operating bands	42

6.Y.2	Configuration	42

6.Y.3	Maximum output power	42

6.Y.4	Spurious emission band UE co-existence	42

6.Y.5	REFSENS requirements	42

6.Y.6	∆TIB and ∆RIB values	43

7	NSA NR SUL with UL sharing from ULSUP band combination: Specific Band Combination Part	43

7.Y	DC_X_SUL_nY-nZ	43

7.Y.1	Configuration	43

7.Y.2	Maximum output power	43

7.Y.3	Spurious emission band UE co-existence	43

7.Y.4	MSD	43

7.Y.5	∆TIB and ∆RIB values	44

Annex <A> (informative): Change history	45

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for SA NR Supplementary uplink (SUL), NSA NR SUL and NSA NR SUL with UL sharing from the UE perspective (ULSUP) under Rel-18 time frame. The purpose is to gather the relevant background information and studies in order to address NR SUL for the Rel-18 bands/band combinations in Table 1-1 to Table 1-4.

Table 1-1: Release 18 SUL bands

| Band number | UL | DL | Duplex mode |
| --- | --- | --- | --- |

Table 1-2: Release 18 SA SUL band combinations

| SA SUL band combination | REL independent from |
| --- | --- |
| CA_n1_SUL_n78-n81 | Rel-15 |
| CA_n3_SUL_n41-n80 | Rel-15 |
| CA_n41_SUL_n79-n98 | Rel-15 |
| CA_n79_SUL_n41-n98 | Rel-15 |
| CA_n41_SUL_n79-n95 | Rel-15 |
| CA_n79_SUL_n41-n95 | Rel-15 |
| CA_n8_SUL_n78-n81 | Rel-15 |
| SUL_n1-n81 | Rel-15 |
| SUL_n3-n84 | Rel-15 |
| SUL_n1-n89 | Rel-15 |
| SUL_n1-n80 | Rel-15 |
| SUL_n78-n89 | Rel-15 |
| CA_n78_SUL_n1-n81 | Rel-15 |
| CA_n78_SUL_n3-n84 | Rel-15 |
| CA_n78_SUL_n1-n89 | Rel-15 |
| CA_n78_SUL_n1-n80 | Rel-15 |

Table 1-3: Release 18 NSA SUL band combinations

| NSA SUL band combination | REL independent from |
| --- | --- |

Table 1-4: Release 18 NSA SUL with ULSUP band combinations

| NSA SUL with ULSUP band combination | REL independent from |
| --- | --- |

This TR contains a general part and specific band combination part. The actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-221839, "New WID: Rel-18 Band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP)", RAN#96

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, for serving cell c.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

BS				Base Station

BCS				Bandwidth Combination Set

CA				Carrier Aggregation

CC				Component Carriers

DC				Dual Connectivity

DL				Downlink

E-UTRA 				Evolved UMTS Terrestrial Radio Access

FDD				Frequency Division Duplex

MPR 				Allowed maximum power reduction

MSD				Maximum Sensitivity Degradation

NR				New Radio

NSA				Non-Stand Alone

REFSENS				Reference Sensitivity power level

SA				Stand Alone

SCS				Subcarrier spacing

SUL				Supplementary uplink

TDD				Time Division Duplex

UE				User Equipment

UL				Uplink

ULSUP				UL sharing from the UE perspective

4	Background

The present document is a technical report for SA NR SUL, NSA NR SUL and NSA NR SUL with UL sharing from ULSUP under Rel-17 time frame. It covers both the UE and BS side. The document is divided in different parts:

- 	Specific SA NR SUL part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

- 	Specific NSA NR SUL part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

- 	Specific NSA NR SUL with UL sharing from ULSUP part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band/band combination to ensure that the TPs related to the band/band combination have been implemented.

5	SA NR SUL band combination: Specific Band Combination Part

5.1	CA_n1_SUL_n78-n81

5.1.1	Operating bands

Table 5.1.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1_SUL_n78-n81 | n1, n78, n81 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

5.1.2	Channel bandwidths per operating band

Table 5.1.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A_SUL_n78A-n81A | SUL_n78A-n81A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n81 | 5, 10, 15, 20 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

5.1.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.1.4	Spurious emission band UE co-existence

There is only single UL in uplink so the requirement for each band in clause 6.5.3.2 from 38.101-1 is applicable.

5.1.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.1.5-1: Supplementary uplink configuration for reference sensitivity

|  | NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | n81 | 15 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |
| n78 | n81 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |

Since the 4th harmonic interference of SUL band n81 may fall into the Rx band n78, the MSD due to harmonic interference should be took care of. The REFSENS requirements can refer to the MSD due to harmonic interference between SUL band n81 and n78 which has been specified in Table 7.3C.2-2, if harmonic interference need to be considered.

5.1.6	∆TIB and ∆RIB values

For CA_n1_SUL_n78-n81, the TIB,c and RIB,c values are given in the tables below referring to CA_n1-n8-n78.

Table 5.1.6-1: ΔTIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1_SUL_n78-n81 | 0.3 | 0.8 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

Table 5.1.6-2: ΔRIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1_SUL_n78-n81 | - | 0.5 | - |
| NOTE *: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

5.2	CA_n3_SUL_n41-n80

5.2.1	Operating bands

Table 5.2.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n3_SUL_n41-n80 | n3, n41, n80 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

5.2.2	Channel bandwidths per operating band

Table 5.2.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A_SUL_n41C-n80A | SUL_n41C-n80A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

5.2.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 or 6.2A.1.1 from 38.101-1 is applicable.

5.2.4	Spurious emission band UE co-existence

The requirements have been studied in clause 5.20.4 from TR 37.717-00-00.

5.2.5	REFSENS requirements

The requirements for UL configurations have been studied in clause 5.20.5 from TR 37.717-00-00 and MSD due to cross band isolation has been specified in the spec. For UL configuration SUL_n41C-n80A of CA_n3A_SUL_n41C-n80A, we can get the IMD order for contiguous UL CA as below referring to clause 7.3.2.2 from TR 38.862.

WGap = lowest band edge of n41 - highest DL band edge of n3 = 2496 – 1880 = 616 MHz.

Min(maxUL aggregated BW, UL band bandwidth) = Min(190, 194) = 190 MHz.

IMD order = 2*ceil(WGap /Min(maxUL aggregated BW, UL band bandwidth) + 1 = 9.

Since IMD order is < 11, MSD should be considered.

The Reference sensitivity exception due to intermodulation interference can be same as DL_n3A-n41C_UL_n41C. Thus, there is no need to specify it into specification again assuming that the fall back configuration would specify the same requirements.

Table 5.2.5-1: Reference sensitivity exception due to intermodulation interference

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
|  | n3 | N/A | 5 | N/A | 1830 | 5.2 | FDD | IMD9 |
| CA_n3A_SUL_n41C-n80A | n41 | 2550 | 100 | 1 (RBSTART=24) | 2550 | N/A | TDD | N/A |
|  |  | 2645 | 90 | 1 (RBSTART=218) | 2645 |  |  |  |
|  | n80 | 1735 | 5 | 25 | N/A | N/A | SUL | N/A |

5.2.6	∆TIB and ∆RIB values

The requirements have been studied in clause 5.20.6 from TR 37.717-00-00.

5.3	CA_n41_SUL_n79-n98

5.3.1	Operating bands

Table 5.3.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n41_SUL_n79-n98 | n41, n79, n98 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

5.3.2	Channel bandwidths per operating band

Table 5.3.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
|  |  | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A_SUL_n79A-n98A | SUL_n79A-n98A | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A_SUL_n79C-n98A | SUL_79A-n98A SUL_n79C-n98A CA_n41A-n79A CA_n79C | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 | 0 |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |
| CA_n41C_SUL_n79A-n98A | SUL_n79A-n98A CA_n41C CA_n41A-n79A | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |
| CA_n41C_SUL_n79C-n98A | CA_n41C CA_n79C SUL_n79A-n98A SUL_n79C-n98A CA_n41A-n79A | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 | 0 |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1 from 38.101-1. |  |  |  |  |

5.3.3	Maximum output power

There is only single UL for CA_n41A_SUL_n79A-n98A in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable. For the SUL band combination with intra-band contiguous CA and inter-band CA, the requirements have been specified in clause 6.2A.1.1 and 6.2A.1.3 from 38.101-1 of CA_n41C, CA_n79C and CA_n41A-n79A.

5.3.4	Spurious emission band UE co-existence

There is only single UL in uplink for CA_n41A_SUL_n79A-n98A so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.3.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n41_SUL_n79-n98.

Table 5.3.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n41 | UL/DL frequency range | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n79 | DL frequency range | 4400 | 5000 | 8800 | 10000 | 13200 | 15000 | 17600 | 20000 |
| n98 | UL frequency range | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 | 7520 | 7680 |

There is no harmonic/harmonic mixing issue for this combination

5.3.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified in 7.3C.2-1 from TS 38.101-1. The requirement for CA_n41A-n79A specified in clause 7.3A.6 from 38.101-1.

Since there are two downlink bands, MSD studies are covered by fallback band combinations.

5.3.6	∆TIB and ∆RIB values

For CA_n41_SUL_n79-n98, the TIB,c and RIB,c values are given in the tables below.

Table 5.3.6-1: ΔTIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n41_SUL_n79-n98 | 0.3 | 0.8 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

Table 5.3.6-2: ΔRIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n41_SUL_n79-n98 | - | 0.5 | - |
| NOTE *: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

5.4	CA_n79_SUL_n41-n98

5.4.1	Operating bands

Table 5.4.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n79_SUL_n41-n98 | n41, n79, n98 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

5.4.2	Channel bandwidths per operating band

Table 5.4.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
|  |  | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n79A_SUL_n41A-n98A | SUL_n41A-n98A | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n79C_SUL_n41A-n98A | SUL_n41A-n98A CA_n41A-n79A CA_n79C | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 | 0 |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |
| CA_n79A_SUL_n41C-n98A | SUL_n41A-n98A SUL_n41C-n98A CA_n41A-n79A CA_n41C | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |
| CA_n79C_SUL_n41C-n98A | CA_n41C CA_n79C SUL_n41A-n98A SUL_n41C-n98A CA_n41A-n79A | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 | 0 |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1 from 38.101-1. |  |  |  |  |

5.4.3	Maximum output power

There is only single UL for CA_n79A_SUL_n41A-n98A in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable. For the SUL band combination with intra-band contiguous CA and inter-band CA, the requirements have been specified in clause 6.2A.1.1 and 6.2A.1.3 from 38.101-1 of CA_n41C, CA_n79C and CA_n41A-n79A.

5.4.4	Spurious emission band UE co-existence

There is only single UL in uplink for CA_n79A_SUL_n41A-n98A so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.4.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n79_SUL_n41-n98.

Table 5.4.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n41 | UL/DL frequency range | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n79 | DL frequency range | 4400 | 5000 | 8800 | 10000 | 13200 | 15000 | 17600 | 20000 |
| n98 | UL frequency range | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 | 7520 | 7680 |

There is no harmonic/harmonic mixing issue for this combination

5.4.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified in 7.3C.2-1 from TS 38.101-1. The requirement for CA_n41A-n79A specified in clause 7.3A.6 from 38.101-1.

Since there are two downlink bands, MSD studies are covered by fallback band combinations.

5.4.6	∆TIB and ∆RIB values

For CA_n79_SUL_n41-n98, the TIB,c and RIB,c values are given in the tables below.

Table 5.4.6-1: ΔTIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n79_SUL_n41-n98 | 0.3 | 0.8 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

Table 5.4.6-2: ΔRIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n79_SUL_n41-n98 | - | 0.5 | - |
| NOTE *: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

5.5	CA_n41_SUL_n79-n95

5.5.1	Operating bands

Table 5.5.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n41_SUL_n79-n95 | n41, n79, n95 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

5.5.2	Channel bandwidths per operating band

Table 5.5.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
|  |  | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A_SUL_n79A-n95A | SUL_n79A-n95A | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n95 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A_SUL_n79C-n95A | SUL_79A-n95A SUL_n79C-n95A CA_n41A-n79A CA_n79C | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 | 0 |
|  |  | n95 | 5, 10, 15 |  |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |
| CA_n41C_SUL_n79A-n95A | SUL_n79A-n95A CA_n41C CA_n41A-n79A | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n95 | 5, 10, 15 |  |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |
| CA_n41C_SUL_n79C-n95A | CA_n41C CA_n79C SUL_n79A-n95A SUL_n79C-n95A CA_n41A-n79A | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 | 0 |
|  |  | n98 | 5, 10, 15 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1 from 38.101-1. |  |  |  |  |

5.5.3	Maximum output power

There is only single UL for CA_n41A_SUL_n79A-n95A in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable. For the SUL band combination with intra-band contiguous CA and inter-band CA, the requirements have been specified in clause 6.2A.1.1 and 6.2A.1.3 from 38.101-1 of CA_n41C, CA_n79C and CA_n41A-n79A.

5.5.4	Spurious emission band UE co-existence

There is only single UL in uplink for CA_n41A_SUL_n79A-n95A so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.5.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n41_SUL_n79-n95.

Table 5.5.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n41 | UL/DL frequency range | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n79 | DL frequency range | 4400 | 5000 | 8800 | 10000 | 13200 | 15000 | 17600 | 20000 |
| n95 | UL frequency range | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 | 8040 | 8100 |

There is no harmonic/harmonic mixing issue for this combination

5.5.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified in 7.3C.2-1 from TS 38.101-1. The requirement for CA_n41A-n79A specified in clause 7.3A.6 from 38.101-1.

Since there are two downlink bands, MSD studies are covered by fallback band combinations.

5.5.6	∆TIB and ∆RIB values

For CA_n41_SUL_n79-n95, the TIB,c and RIB,c values are given in the tables below.

Table 5.5.6-1: ΔTIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n41_SUL_n79-n95 | 0.3 | 0.8 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

Table 5.5.6-2: ΔRIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n41_SUL_n79-n95 | - | 0.5 | - |
| NOTE *: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

5.6	CA_n79_SUL_n41-n95

5.6.1	Operating bands

Table 5.6.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n79_SUL_n41-n95 | n41, n79, n95 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

5.6.2	Channel bandwidths per operating band

Table 5.6.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
|  |  | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n79A_SUL_n41A-n95A | SUL_n41A-n95A | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n95 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n79C_SUL_n41A-n95A | SUL_n41A-n95A CA_n41A-n79A CA_n79C | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 | 0 |
|  |  | n95 | 5, 10, 15 |  |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |
| CA_n79A_SUL_n41C-n95A | SUL_n41A-n95A SUL_n41C-n95A CA_n41A-n79A CA_n41C | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n95 | 5, 10, 15 |  |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |
| CA_n79C_SUL_n41C-n95A | CA_n41C CA_n79C SUL_n41A-n95A SUL_n41C-n95A CA_n41A-n79A | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 | 0 |
|  |  | n95 | 5, 10, 15 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1 from 38.101-1. |  |  |  |  |

5.6.3	Maximum output power

There is only single UL for CA_n79A_SUL_n41A-n95A in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable. For the SUL band combination with intra-band contiguous CA and inter-band CA, the requirements have been specified in clause 6.2A.1.1 and 6.2A.1.3 from 38.101-1 of CA_n41C, CA_n79C and CA_n41A-n79A.

5.6.4	Spurious emission band UE co-existence

There is only single UL in uplink for CA_n79A_SUL_n41A-n95A so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.6.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n79_SUL_n41-n95.

Table 5.6.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n41 | UL/DL frequency range | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n79 | DL frequency range | 4400 | 5000 | 8800 | 10000 | 13200 | 15000 | 17600 | 20000 |
| n95 | UL frequency range | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 | 8040 | 8100 |

There is no harmonic/harmonic mixing issue for this combination

5.6.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified in 7.3C.2-1 from TS 38.101-1. The requirement for CA_n41A-n79A specified in clause 7.3A.6 from 38.101-1.

Since there are two downlink bands, MSD studies are covered by fallback band combinations.

5.6.6	∆TIB and ∆RIB values

For CA_n79_SUL_n41-n95, the TIB,c and RIB,c values are given in the tables below.

Table 5.6.6-1: ΔTIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n79_SUL_n41-n95 | 0.3 | 0.8 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

Table 5.6.6-2: ΔRIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n79_SUL_n41-n95 | - | 0.5 | - |
| NOTE *: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

5.7	CA_n8_SUL_n78-n81

5.7.1	Operating bands

Table 5.7.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n8_SUL_n78-n81 | n8, n78, n81 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

5.7.2	Channel bandwidths per operating band

Table 5.7.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A_SUL_n78A-n81A | SUL_n78A-n81A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n81 | 5, 10, 15, 20 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

5.7.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.7.4	Spurious emission band UE co-existence

There is only single UL in uplink so the requirement for each band in clause 6.5.3.2 from 38.101-1 is applicable.

5.7.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.7.5-1: Supplementary uplink configuration for reference sensitivity

|  | NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n8 | n811 | 15 | 25 | 25 | 20 | 20 |  |  |  |  |  |  |  |  |  |
| n78 | n81 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| NOTE 1: The Tx-Rx carrier center frequency separation between SUL band and DL band is the same as the Tx-Rx carrier center frequency separation of DL band specified in table 5.4.4-1 from TS 38.101-1. The channel bandwidth of SUL band is the same as DL band. This restriction of REFSENS configurations applies also for these carriers when applicable SUL configuration is part of a higher order configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Since the 4th harmonic interference of SUL band n81 may fall into the Rx band n78, the MSD due to harmonic interference should be took care of. The REFSENS requirements can refer to the MSD due to harmonic interference between SUL band n81 and n78 which has been specified in Table 7.3C.2-2, if harmonic interference need to be considered.

5.7.6	∆TIB and ∆RIB values

For CA_n8_SUL_n78-n81, the TIB,c and RIB,c values are given in the tables below referring to CA_n8-n78.

Table 5.7.6-1: ΔTIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n8_SUL_n78-n81 | 0.6 | 0.8 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

Table 5.7.6-2: ΔRIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n8_SUL_n78-n81 | 0.2 | 0.5 | - |
| NOTE *: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

5.8	 SUL_n1-n81

5.8.1	Operating bands

Table 5.8.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n1-n81 | n1, n81 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

5.8.2	Channel bandwidths per operating band

Table 5.8.2-1: Supported bandwidths per SUL band combination

| SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- |
| SUL_n1A-n81A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | n81 | 5, 10, 15, 20 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |

5.8.3 	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.8.4	 Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.8.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for SUL_n1A-n81A.

Table 5.8.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n1 | DL frequency range | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 |
| n81 | UL frequency range | 880 | 915 | 1760 | 1830 | 2640 | 2745 | 3520 | 3660 |

For SUL_n1-n81, there is no harmonic or harmonic mixing product generated by Band n81 that may fall into the RX band of Band n1. Therefore, MSD due to harmonic or harmonic mixing is not needed.

### 5.8.5	REFSENS requirements

For SUL operation, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.8.5-1: Supplementary uplink configuration for reference sensitivity

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | n81 | 15 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |

5.8.6	∆TIB and ∆RIB values

For SUL_n1-n81, the TIB,c values are given in the tables below referring to CA_n1-n8. RIB,c is 0 for both the bands.

Table 5.8.6-1: ΔTIB,c due to SUL(two bands)

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_n1-n81 | 0.3 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |

5.9	 SUL_n3-n84

5.9.1	Operating bands

Table 5.9.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n3-n84 | n3, n84 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

5.9.2	Channel bandwidths per operating band

Table 5.9.2-1: Supported bandwidths per SUL band combination

| SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- |
| SUL_n3A-n84A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | n84 | 5, 10, 15, 20 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |

5.9.3 	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.9.4	 Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.9.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for SUL_n3A-n84A.

Table 5.9.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n3 | DL frequency range | 1805 | 1880 | 3610 | 3760 | 5415 | 5640 | 7220 | 7520 |
| n84 | UL frequency range | 1920 | 1980 | 3840 | 3960 | 5760 | 5940 | 7680 | 7920 |

For SUL_n3-n84, there is no harmonic or harmonic mixing product generated by Band n84 that may fall into the RX band of Band n3. Therefore, MSD due to harmonic or harmonic mixing is not needed.

5.9.5	REFSENS requirements

For SUL operation, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.9.5-1: Supplementary uplink configuration for reference sensitivity

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n3 | n84 | 15 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |

The cross band isolation for CA_n1-n3 has been specified in TS38.101-1, and it can be the reference to that of SUL_n3-n84 with the same uplink frequency range between band n1 and band n84. Therefore, the reference sensitivity for SUL_n3-n84 is as follows.

Table 5.9.5-2: Reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to cross band isolation)

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n84 | n3 | 1945 | 50 | 15 | 128 (RBstart=0) | 1877.5 | 5 | 19.7 | ACLR1 |

### 5.9.6	∆TIB and ∆RIB values

For SUL_n3-n84, the TIB,c values are given in the tables below referring to CA_n1-n3. RIB,c is 0 for both the bands.

Table 5.9.6-1: ΔTIB,c due to SUL(two bands)

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_n3-n84 | 0.3 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |

5.10	 SUL_n1-n89

5.10.1	Operating bands

Table 5.10.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n1-n89 | n1, n89 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

5.10.2	Channel bandwidths per operating band

Table 5.10.2-1: Supported bandwidths per SUL band combination

| SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- |
| SUL_n1A-n89A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | n89 | 5, 10, 15, 20 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |

5.10.3 	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.10.4	 Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.10.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for SUL_n1A-n89A.

Table 5.10.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n1 | DL frequency range | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 |
| n89 | UL frequency range | 824 | 849 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 |

For SUL_n1-n89, there is no harmonic or harmonic mixing product generated by Band n89 that may fall into the RX band of Band n1. Therefore, MSD due to harmonic or harmonic mixing is not needed.

### 5.10.5	REFSENS requirements

For SUL operation, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.10.5-1: Supplementary uplink configuration for reference sensitivity

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | n89 | 15 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |

5.10.6	∆TIB and ∆RIB values

For SUL_n1-n89, the TIB,c values are given in the tables below referring to CA_n1-n5. RIB,c is 0 for both the bands.

Table 5.10.6-1: ΔTIB,c due to SUL(two bands)

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_n1-n89 | 0.3 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |

5.11	 SUL_n1-n80

5.11.1	Operating bands

Table 5.11.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n1-n80 | n1, n80 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

5.11.2	Channel bandwidths per operating band

Table 5.11.2-1: Supported bandwidths per SUL band combination

| SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- |
| SUL_n1A-n80A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | n80 | 5, 10, 15, 20,25,30 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |

5.11.3 	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.11.4	 Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.11.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for SUL_n1A-n80A.

Table 5.11.4-1: Supplementary Uplink configuration for reference sensitivity

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n1 | DL frequency range | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 |
| n80 | UL frequency range | 1710 | 1785 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 |

For SUL_n1-n80, there is no harmonic or harmonic mixing product generated by Band n81 that may fall into the RX band of Band n1. Therefore, MSD due to harmonic or harmonic mixing is not needed.

5.11.5	REFSENS requirements

The intermodulation interference for CA_n1-n3 has been specified in TS38.101-1, and it can be the reference to that of SUL_n1-n80 with the same uplink frequency range between band n3 and band n80. Therefore, the reference sensitivity on band n1 for SUL_n1-n80 is as follows.

Table 5.11.5-1: Reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to intermodulation interference)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR SUL band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| SUL_n1-n80 | n1 | 1950 | 5 | 25 | 2140 | 23 | FDD | IMD3 |

5.11.6	∆TIB and ∆RIB values

For SUL_n1-n80, the TIB,c values are given in the tables below referring to CA_n1-n3. RIB,c is 0 for both the bands.

Table 5.11.6-1: ΔTIB,c due to SUL(two bands)

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_n1-n80 | 0.3 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |

5.12	 SUL_n78-n89

5.12.1	Operating bands

Table 5.12.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n78-n89 | n78, n89 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

5.12.2	Channel bandwidths per operating band

Table 5.12.2-1: Supported bandwidths per SUL band combination

| SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- |
| SUL_n78A-n89A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  | n89 | 5, 10, 15, 20 |  |
| SUL_n78C-n89A | n78 | See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.1-1 | 0 |
|  | n89 | 5, 10, 15, 20 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |

5.12.3 	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.12.4	 Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

5.12.5	REFSENS requirements

For SUL operation, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.12.5-1: Supplementary uplink configuration for reference sensitivity

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n78 | n89 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |

Table 5.12.5-2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for SUL_n78A-n89A.

Table 5.12.5-2: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n78 | DL frequency range | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |
| n89 | UL frequency range | 824 | 849 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 |

The 4th harmonic interference of band n89 may fall into Rx band of n78.

The MSD exception for SUL_n78-n89 are specified below. MSD values for Band n78 due to 4th harmonic of Band n89 in SUL_n78A-n89A are captured in Table 5.12.5-2 according to CA_n5-n78 values which also had a similar 4th harmonic of n89 falling in n78 DL frequency range.

Table 5.12.5-3: Reference sensitivity for SUL operation (exceptions due to harmonic issue)

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n89 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.5 | NOTE 4 | UL4/DL1 direct-hit |
|  | n78 | 20 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| NOTE 4: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

5.12.6	∆TIB and ∆RIB values

For SUL_n78-n89, the TIB,c values are given in the tables below referring to CA_n5-n78. RIB,c is 0 for both the bands.

Table 5.12.6-1: ΔTIB,c due to SUL (two bands)

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_n78-n89 | 0.8 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |

5.13	 CA_n78_SUL_n1-n81

5.13.1	Operating bands

Table 5.13.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n78_SUL_n1-n81 | n1, n78, n81 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

5.13.2	Channel bandwidths per operating band

Table 5.13.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n78A_SUL_n1A-n81A | SUL_n1A-n81A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n81 | 5, 10, 15, 20 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

5.13.3 	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.13.4	 Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

5.13.5	REFSENS requirements

For SUL operation, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.13.5-1: Supplementary uplink configuration for reference sensitivity

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n78 | n81 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n1 | n81 | 15 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |

Table 5.13.5-2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n78_SUL_n1-n81.

Table 5.13.5-2: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n78 | UL/DL frequency range | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |
| n1 | DL frequency range | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 |
| n81 | UL frequency range | 880 | 915 | 1760 | 1830 | 2640 | 2745 | 3520 | 3660 |

4th harmonic interference of SUL band n81 may fall into the Rx band n78. The REFSENS requirements for UL4/DL1 direct-hit have been defined in Table 7.3C.2-2 of TS38.101-1.

5.13.6	∆TIB and ∆RIB values

For CA_n78_SUL_n1-n81, the TIB,c and RIB,c values are given in the tables below referring to CA_n1-n8-n78.

Table 5.13.6-1: ΔTIB,c due to NR CA (three bands)

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n78_SUL_n1-n81 | 0.8 | 0.3 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

Table 5.13.6-2: ΔRIB,c due to NR CA (three bands)

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n78_SUL_n1-n81 | 0.5 | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

5.14	 CA_n78_SUL_n3-n84

5.14.1	Operating bands

Table 5.14.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n78_SUL_n3-n84 | n3, n78, n84 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

5.14.2	Channel bandwidths per operating band

Table 5.14.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n78A_SUL_n3A-n84A | SUL_n3A-n84A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n84 | 5, 10, 15, 20 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

5.14.3 	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.14.4	 Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

5.14.5	REFSENS requirements

For SUL operation, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.14.5-1: Supplementary uplink configuration for reference sensitivity

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n78 | n84 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n3 | n84 | 15 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |

Table 5.14.5-2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n78_SUL_n3-n84.

Table 5.14.5-2: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n78 | UL/DL frequency range | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |
| n3 | DL frequency range | 1805 | 1880 | 3610 | 3760 | 5415 | 5640 | 7220 | 7520 |
| n84 | UL frequency range | 1920 | 1980 | 3840 | 3960 | 5760 | 5940 | 7680 | 7920 |

There is no harmonic or harmonic mixing product generated by Band n84 that may fall into the RX band of Band n3 or Band n78. And other MSD analysis of harmonic/harmonic mixing between Band n3 and Band n78 has been completed by CA_n3-n78. Therefore, further MSD due to harmonic or harmonic mixing is not needed. The cross band isolation interference analysis was covered by fallback combinations.

5.14.6	∆TIB and ∆RIB values

For CA_n78_SUL_n3-n84, the TIB,c and RIB,c values are given in the tables below referring to CA_n1-n3-n78.

Table 5.14.6-1: ΔTIB,c due to NR CA (three bands)

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n78_SUL_n3-n84 | 0.8 | 0.6 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

Table 5.14.6-2: ΔRIB,c due to NR CA (three bands)

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n78_SUL_n3-n84 | 0.5 | 0.2 | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

5.15	 CA_n78_SUL_n1-n89

5.15.1	Operating bands

Table 5.15.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n78_SUL_n1-n89 | n1, n78, n89 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

5.15.2	Channel bandwidths per operating band

Table 5.15.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n78A_SUL_n1A-n89A | SUL_n1A-n89A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n89 | 5, 10, 15, 20 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

5.15.3 	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.15.4	 Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

5.15.5	REFSENS requirements

For SUL operation, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.15.5-1: Supplementary uplink configuration for reference sensitivity

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n78 | n89 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n1 | n89 | 15 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |

Table 5.15.5-2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n78_SUL_n1-n89.

Table 5.15.5-2: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n78 | UL/DL frequency range | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |
| n1 | DL frequency range | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 |
| n89 | UL frequency range | 824 | 849 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 |

4th harmonic interference of SUL band n89 may fall into the Rx band n78. The REFSENS requirements for UL4/DL1 direct-hit have been defined in SUL_n78-n89.

5.15.6	∆TIB and ∆RIB values

For CA_n78_SUL_n1-n89, the TIB,c and RIB,c values are given in the tables below referring to CA_n1-n5-n78.

Table 5.15.6-1: ΔTIB,c due to NR CA (three bands)

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n78_SUL_n1-n89 | 0.8 | 0.6 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

Table 5.15.6-2: ΔRIB,c due to NR CA (three bands)

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n78_SUL_n1-n89 | 0.5 | 0.2 | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

5.16	 CA_n78_SUL_n1-n80

5.16.1	Operating bands

Table 5.16.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n78_SUL_n1-n80 | n1, n78, n80 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

5.16.2	Channel bandwidths per operating band

Table 5.16.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n78A_SUL_n1A-n80A | SUL_n1A-n80A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n80 | 5, 10, 15, 20,25,30 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

5.16.3 	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.16.4	 Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

5.16.5	REFSENS requirements

For SUL operation, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.16.5-1: Supplementary uplink configuration for reference sensitivity

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n78 | n80 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n1 | n80 | 15 | 160 | 160 | 160 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |

Table 5.16.5-2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n78_SUL_n1-n80.

Table 5.16.5-2: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n78 | UL/DL frequency range | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |
| n1 | DL frequency range | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 |
| n80 | UL frequency range | 1710 | 1785 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 |

2nd harmonic interference of SUL band n80 may fall into the Rx band n78. The REFSENS requirements for UL2/DL1 direct-hit have been defined in CA_n3-n78. The intermodulation interference of IMD3 analysis was covered by fallback combinations.

5.16.6	∆TIB and ∆RIB values

For CA_n78_SUL_n1-n80, the TIB,c and RIB,c values are given in the tables below referring to CA_n1-n3-n78.

Table 5.16.6-1: ΔTIB,c due to SUL (three bands)

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n78_SUL_n1-n80 | 0.8 | 0.6 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

Table 5.16.6-2: ΔRIB,c due to SUL (three bands)

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n78_SUL_n1-n80 | 0.5 | 0.2 | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

5.Y	SUL_nX-nY/CA_nX_SUL_nY-nZ

5.Y.1	Operating bands

Table 5.Y.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1 from TS 38.101-1) |
| --- | --- |

5.Y.2	Configuration

Table 5.Y.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
|  |  |  |  | 0 |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1 from TS 38.101-1. |  |  |  |  |

5.Y.3	Maximum output power

<Editor’s note: This requirement is only applicable when there is simultaneous transmission in the band combination.>

5.Y.4	Spurious emission band UE co-existence

<Editor’s note: This requirement is only applicable when there is simultaneous transmission in the band combination.>

5.Y.5	REFSENS requirements

5.Y.6	∆TIB and ∆RIB values

Table 5.Y.6-1: ΔTIB,c due to SUL band combination

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_nX-nY |  |  |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for SUL_n41-n81 the order of band is n41 and n81. |  |  |

Table 5.Y.6-2: ΔRIB,c due to SUL band combination

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_nX-nY |  |  |
| NOTE *: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for SUL_n41-n80 the order of band is n41 and n80. |  |  |

5.Y.7	Out-of-band blocking exception

<Editor’s note: This requirement is only applicable when there is Out-of-band blocking exception in the band combination.>

6	NSA NR SUL band combination: Specific Band Combination Part

6.Y	DC_X_SUL_nY-nZ

6.Y.1	Operating bands

6.Y.2	Configuration

6.Y.3	Maximum output power

6.Y.4	Spurious emission band UE co-existence

6.Y.5	REFSENS requirements

6.Y.6	∆TIB and ∆RIB values

7	NSA NR SUL with UL sharing from ULSUP band combination: Specific Band Combination Part

7.Y	DC_X_SUL_nY-nZ

7.Y.1	Configuration

Table 7.Y.2-1: Inter-band EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |

7.Y.2	Maximum output power

Table 7.Y.2-1: Maximum output power for inter-band EN-DC

| DC configuration | Power class X (dBm) | Tolerance (dB) |
| --- | --- | --- |

7.Y.3	Spurious emission band UE co-existence

Table 7.Y.3-1: Spurious emissions for inter-band EN-DC

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |

7.Y.4	MSD

7.Y.5	∆TIB and ∆RIB values

Table 7.Y.5-1: ΔTIB,c due to EN-DC

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |

Table 7.Y.5-2: ΔRIB,c due to EN-DC

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |

Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | 3GPP RAN4#104-e | R4-2213057 |  |  |  | Initial TR skeleton | 0.0.1 |
| 2022-08 | 3GPP RAN4#104-e | R4-2213058 |  |  |  | Implemented TP’s from RAN4#104e R4-2213128 TP for 37.718-00-00 to introduce SUL configuration CA_n1A_SUL_n78A-n81A Huawei, HiSilicon R4-2213129 TP for 37.718-00-00 to add SUL configuration SUL_n41C-n80A for CA_n3A_SUL_n41C-n80A Huawei, HiSilicon | 0.1.0 |
| 2022-10 | 3GPP RAN4#104bis-e | R4-2216045 |  |  |  | Implemented TP’s from RAN4#104bis-e R4-2216622 TP for TR 37.718-00-00 on table templates and error corrections ZTE Corporation R4-2217059 TP for TR 37.718-00-00 CA_n41A_SUL_n79A-n98A and CA_n79A_SUL_n41A-n98A Huawei, HiSilicon, CMCC R4-2217060 TP for TR 37.718-00-00 CA_n41A_SUL_n79A-n95A and CA_n79A_SUL_n41A-n95A Huawei, HiSilicon, CMCC | 0.2.0 |
| 2022-11 | 3GPP RAN4#105 | R4-2219565 |  |  |  | Implemented TP’s from RAN4#105 R4-2219757 TP for TR 37.718-00-00 on delta TIB and RIB template for SUL combinations ZTE Corporation | 0.3.0 |
| 2023-03 | 3GPP RAN4#106 | R4-2302033 |  |  |  | Implemented TP’s from RAN4#106 R4-2303571 TP for TR 37.718-00-00 to introduce CA_n8A_SUL_n78A-n81A Huawei, HiSilicon R4-2302042 TP for TR 37.718-00-00 to improve wording in Tib and Rib tables Huawei, HiSilicon R4-2302055 TP for TR 37.718-00-00 SUL_n1A-n81A Huawei, Hisilicon, China Telecom R4-2303587 TP for TR 37.718-00-00 SUL_n3A-n84A Huawei, Hisilicon, China Telecom R4-2302057 TP for TR 37.718-00-00 SUL_n1A-n89A Huawei, Hisilicon, China Telecom R4-2303588 TP for TR 37.718-00-00 SUL_n1A-n80A Huawei, Hisilicon, China Telecom R4-2302059 TP for TR 37.718-00-00 SUL_n78A-n89A Huawei, Hisilicon, China Telecom R4-2302060 TP for TR 37.718-00-00 CA_n78A_SUL_n1A-n81A Huawei, Hisilicon, China Telecom R4-2303589 TP for TR 37.718-00-00 CA_n78A_SUL_n3A-n84A Huawei, Hisilicon, China Telecom R4-2302062 TP for TR 37.718-00-00 CA_n78A_SUL_n1A-n89A Huawei, Hisilicon, China Telecom R4-2303590 TP for TR 37.718-00-00 CA_n78A_SUL_n1A-n80A Huawei, Hisilicon, China Telecom | 0.3.0 |
| 2023-04 | 3GPP RAN4#106bis-e | R4-2305366 |  |  |  | Implemented TP’s from RAN4#106bis-e R4-2305445 TP for TR 37.718-00-00: correction on the Note for delta_Tib and delta_Rib, Huawei, HiSilicon R4-2305446 TP on TR37.718-00-00: SUL_n78C-n89A, Huawei, HiSilicon R4-2306475 TP on TR37.718-00-00: correction on SUL_n1A-n80A Huawei, HiSilicon | 0.4.0 |
| 2023-05 | 3GPP RAN4#107 | R4-2307544 |  |  |  | Implemented TP’s from RAN4#107 R4-2307166 TP for TR 37.718-00-00: CA_n41_SUL_n79-n98, CA_n79_SUL_n41-n98, CA_n41_SUL_n79-n95 and CA_n79_SUL_n41-n95, Huawei, HiSilicon | 0.5.0 |
| 2023-10 | 3GPP RAN4#108bis | R4-2317785 |  |  |  | Implemented TP’s from RAN4#108bis R4-2316199 TP for TR 37.718-00-00 to maintain the channel bandwidth table, Huawei, HiSilicon | 0.6.0 |