3GPP TSG-RAN WG4 Meeting # 108-bis												R4-2317934

Xiamen, China, October 09 – October 13, 2023

Agenda item:			5.14.7

Source:	Man Hung Ng (Nokia)

Title:	Topic summary for [108-bis][302] NR_FR1_lessthan_5MHz_BW_BSRF

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

Summary for contributions submitted under agenda item 5.14.3 for NR support for dedicated spectrum less than 5MHz for FR1.

List of candidate target of discussion for 1st round and 2nd round:

- 1st round: Discussion and agreement on open issues listed below.

- 2nd round: Continue discussion and agreement on open issues listed below.

# Topic #1: Core requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2315269 | Nokia, Nokia Shanghai Bell, ZTE Corporation, Ericsson | Proposal 1: Draft CR to TS 38.104. Observation 1: |
| R4-2315596 | NTT DOCOMO, INC., SoftBank Corp., KDDI Corporation, Rakuten mobile, Inc | Proposal 1: Add Note3 to TS38.104 as follows, - Option 1: Add Note3 in the Table 6.7.2.1-1 for BS type 1-C Co-location Tx intermodulation requirements to set 5MHz as BWChannel in band n100, n106, n26, n28, n85 when BS doesn’t support 3MHz bandwidth as shown in table 2. Table 2 Parameter; Value. Wanted signal type; NR single carrier, or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers, with NB-IoT operation in NR in-band if supported.. Interfering signal type; NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5.. Interfering signal level; Rated total output power (Prated,t,AC) in the operating band – 30 dB. Interfering signal centre frequency offset from the lower/upper edge of the wanted signal or edge of sub-block inside a sub-block gap; , for n=1, 2 and 3. NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the base station are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. In case that none of the interfering signal positions fall completely within the frequency range of the downlink operating band, TS 38.141-1 [5] provides further guidance regarding appropriate test requirements. NOTE 2: In Japan, NOTE 1 is not applied in Band n77, n78, n79. NOTE3: In Band n100, n106, n26, n28, n85, each BWChannel in interfering signal type and interfering signal centre frequency is set to 5MHz when BS doesn’t support 3MHz bandwidth.. - Option 2: Add Note3 in the Table 6.7.2.1-1 for BS type 1-C Co-location Tx intermodulation requirements to set 5MHz as BWChannel in band n26, n28 in Japan when BS doesn’t support 3MHz bandwidth as shown in table3. Table3 Parameter; Value. Wanted signal type; NR single carrier, or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers, with NB-IoT operation in NR in-band if supported.. Interfering signal type; NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5.. Interfering signal level; Rated total output power (Prated,t,AC) in the operating band – 30 dB. Interfering signal centre frequency offset from the lower/upper edge of the wanted signal or edge of sub-block inside a sub-block gap; , for n=1, 2 and 3. NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the base station are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. In case that none of the interfering signal positions fall completely within the frequency range of the downlink operating band, TS 38.141-1 [5] provides further guidance regarding appropriate test requirements. NOTE 2: In Japan, NOTE 1 is not applied in Band n77, n78, n79. NOTE3: In Band n26 and n28, each BWChannel in interfering signal type and interfering signal centre frequency is set to 5MHz when BS doesn’t support 3MHz bandwidth in Japan.. Observation 1: Tx Intermodulation shall be tested with 3MHz BW even if the BS doesn’t support 3MHz BW in current TS38.104 specification. Observation 2: In current TS38.104 Tx intermodulation, the interfering signal type and interfering signal frequency position are not match Japanese regulation. |
| R4-2316316 | NEC | Proposal 1: Draft CR to TS 38.104. Observation 1: |
| R4-2316360 | NTT DOCOMO, INC., SoftBank Corp., KDDI Corporation, Rakuten mobile, Inc | Proposal 1: Draft CR to TS 38.104 using option 1 in R4-2315596. Observation 1: |
| R4-2316852 | Huawei, HiSilicon | Proposal 1: Draft CR to TS 38.104. Observation 1: |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

Sub-topic description:

Open issues and candidate options before meeting:

Issue 1-1: R4-2315269

- Proposals

  - Option 1: Endorse the draft CR

  - Option 2: Revise the draft CR

  - Option 3: Note the draft CR

- Recommended WF

  - Option 1: Endorse the draft CR

### Sub-topic 1-2

Sub-topic description:

Open issues and candidate options before meeting:

Issue 1-2: R4-2315596 and R4-2316360

- Proposals

  - Option 1: Add Note3 in the Table 6.7.2.1-1 for BS type 1-C Co-location Tx intermodulation requirements to set 5MHz as BWChannel in band n100, n106, n26, n28, n85 when BS doesn’t support 3MHz bandwidth as shown in table 2.

Table 2

| Parameter | Value |
| --- | --- |
| Wanted signal type | NR single carrier, or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers, with NB-IoT operation in NR in-band if supported. |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5. |
| Interfering signal level | Rated total output power (Prated,t,AC) in the operating band – 30 dB |
| Interfering signal centre frequency offset from the lower/upper edge of the wanted signal or edge of sub-block inside a sub-block gap | , for n=1, 2 and 3 |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the base station are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. In case that none of the interfering signal positions fall completely within the frequency range of the downlink operating band, TS 38.141-1 [5] provides further guidance regarding appropriate test requirements. NOTE 2: In Japan, NOTE 1 is not applied in Band n77, n78, n79. NOTE3: In Band n100, n106, n26, n28, n85, each BWChannel in interfering signal type and interfering signal centre frequency is set to 5MHz when BS doesn’t support 3MHz bandwidth. |  |

  - Option 2: Add Note3 in the Table 6.7.2.1-1 for BS type 1-C Co-location Tx intermodulation requirements to set 5MHz as BWChannel in band n26, n28 in Japan when BS doesn’t support 3MHz bandwidth as shown in table3.

Table3

| Parameter | Value |
| --- | --- |
| Wanted signal type | NR single carrier, or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers, with NB-IoT operation in NR in-band if supported. |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5. |
| Interfering signal level | Rated total output power (Prated,t,AC) in the operating band – 30 dB |
| Interfering signal centre frequency offset from the lower/upper edge of the wanted signal or edge of sub-block inside a sub-block gap | , for n=1, 2 and 3 |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the base station are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. In case that none of the interfering signal positions fall completely within the frequency range of the downlink operating band, TS 38.141-1 [5] provides further guidance regarding appropriate test requirements. NOTE 2: In Japan, NOTE 1 is not applied in Band n77, n78, n79. NOTE3: In Band n26 and n28, each BWChannel in interfering signal type and interfering signal centre frequency is set to 5MHz when BS doesn’t support 3MHz bandwidth in Japan. |  |

- Recommended WF

  - TBD

### Sub-topic 1-3

Sub-topic description:

Open issues and candidate options before meeting:

Issue 1-3: R4-2316316

- Proposals

  - Option 1: Endorse the draft CR

  - Option 2: Revise the draft CR

  - Option 3: Note the draft CR

- Recommended WF

  - Option 1: Endorse the draft CR

### Sub-topic 1-4

Sub-topic description:

Open issues and candidate options before meeting:

Issue 1-4: R4-2316852

- Proposals

  - Option 1: Endorse the draft CR

  - Option 2: Revise the draft CR

  - Option 3: Note the draft CR

- Recommended WF

  - Option 2: Revise the draft CR (Big CR already approved in RAN#101, this CR should only contain changes to updated clauses 7.7.2 and 7.7.3 but not the other unchanged clauses)

# Topic #2: Test requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company |  | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2315145 | ZTE Corporation |  | Proposal 1: Draft CR to TS 38.141-1. Observation 1: |
| R4-2315270 | Nokia, Nokia Shanghai Bell |  | Proposal 1: Draft CR to TS 38.141-1. Observation 1: |
| R4-2315597 | NTT DOCOMO, INC., SoftBank Corp., KDDI Corporation, Rakuten mobile, Inc | Proposal 1: Add Note3 to TS38.104 as follows, - Option 1: Add Note3 in the Table 6.7.2.1-1 for BS type 1-C Co-location Tx intermodulation requirements to set 5MHz as BWChannel in band n100, n106, n26, n28, n85 when BS doesn’t support 3MHz bandwidth as shown in table 2. Table 2 Parameter; Value. Wanted signal type; NR single carrier, or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers, with NB-IoT operation in NR in-band if supported.. Interfering signal type; NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5.. Interfering signal level; Rated total output power (Prated,t,AC) in the operating band – 30 dB. Interfering signal centre frequency offset from the lower/upper edge of the wanted signal or edge of sub-block inside a sub-block gap; , for n=1, 2 and 3. NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the base station are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. In case that none of the interfering signal positions fall completely within the frequency range of the downlink operating band, TS 38.141-1 [5] provides further guidance regarding appropriate test requirements. NOTE 2: In Japan, NOTE 1 is not applied in Band n77, n78, n79. NOTE3: In Band n100, n106, n26, n28, n85, each BWChannel in interfering signal type and interfering signal centre frequency is set to 5MHz when BS doesn’t support 3MHz bandwidth.. - Option 2: Add Note3 in the Table 6.7.2.1-1 for BS type 1-C Co-location Tx intermodulation requirements to set 5MHz as BWChannel in band n26, n28 in Japan when BS doesn’t support 3MHz bandwidth as shown in table3. Table3 Parameter; Value. Wanted signal type; NR single carrier, or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers, with NB-IoT operation in NR in-band if supported.. Interfering signal type; NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5.. Interfering signal level; Rated total output power (Prated,t,AC) in the operating band – 30 dB. Interfering signal centre frequency offset from the lower/upper edge of the wanted signal or edge of sub-block inside a sub-block gap; , for n=1, 2 and 3. NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the base station are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. In case that none of the interfering signal positions fall completely within the frequency range of the downlink operating band, TS 38.141-1 [5] provides further guidance regarding appropriate test requirements. NOTE 2: In Japan, NOTE 1 is not applied in Band n77, n78, n79. NOTE3: In Band n26 and n28, each BWChannel in interfering signal type and interfering signal centre frequency is set to 5MHz when BS doesn’t support 3MHz bandwidth in Japan.. Observation 1: Tx Intermodulation shall be tested with 3MHz BW even if the BS doesn’t support 3MHz BW in current TS38.104 specification. Observation 2: In current TS38.104 Tx intermodulation, the interfering signal type and interfering signal frequency position are not match Japanese regulation. |
| R4-2315770 | Ericsson |  | Proposal 1: NB-IoT operation in NR in-band should be added for 3 MHz channel bandwidth. Observation 1: RAN4 didn’t specify BS RF requirements for NB-IoT operation in NR in-band for 3 MHz channel bandwidth. Observation 2: From the WID and the RAN LS reply, it’s unclear if NB-IoT operation in NR in-band shall be supported for 3MHz channel BW. Observation3: Bands n25, 26 and n85 are supporting NB-IoT operations and 3 MHz channel bandwidth. Observation4: NB-IoT in-band operation is supported for E-UTRA 3 MHz channel bandwidth. Observation5: If NB-IoT is not supported in 3 MHz channel bandwidth, BS RF testing effort (additional tests) and complexity (procedure) would increase significantly. |
| R4-2315771 | Ericsson |  | Proposal 1: Draft CR to TS 38.141-1 without NB-IoT support Observation 1: |
| R4-2315772 | Ericsson |  | Proposal 1: Draft CR to TS 38.141-1 with NB-IoT support Observation 1: |
| R4-2316361 | NTT DOCOMO, INC., SoftBank Corp., KDDI Corporation, Rakuten mobile, Inc | Proposal 1: Draft CR to TS 38.141-1 using option 1 in R4-2315597. Observation 1: |  |
| R4-2316844 | Huawei, HiSilicon |  | Proposal 1: Draft CR to TS 38.141-1. Observation 1: |
| R4-2316851 | Huawei, HiSilicon |  | Proposal 1: Draft CR to TS 38.141-1. Observation 1: |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description:

Open issues and candidate options before meeting:

Issue 2-1: R4-2315145

- Proposals

  - Option 1: Endorse the draft CR

  - Option 2: Revise the draft CR

  - Option 3: Note the draft CR

- Recommended WF

  - Option 1: Endorse the draft CR

### Sub-topic 2-2

Sub-topic description

Open issues and candidate options before meeting:

Issue 2-2: R4-2315270

- Proposals

  - Option 1: Endorse the draft CR

  - Option 2: Revise the draft CR

  - Option 3: Note the draft CR

- Recommended WF

  - Option 1: Endorse the draft CR

### Sub-topic 2-3

Sub-topic description:

Open issues and candidate options before meeting:

Issue 2-3: R4-2315597 and R4-2316361

- Proposals

  - Option 1: Add Note3 in the Table 6.7.2.1-1 for BS type 1-C Co-location Tx intermodulation requirements to set 5MHz as BWChannel in band n100, n106, n26, n28, n85 when BS doesn’t support 3MHz bandwidth as shown in table 2.

Table 2

| Parameter | Value |
| --- | --- |
| Wanted signal type | NR single carrier, or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers, with NB-IoT operation in NR in-band if supported. |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5. |
| Interfering signal level | Rated total output power (Prated,t,AC) in the operating band – 30 dB |
| Interfering signal centre frequency offset from the lower/upper edge of the wanted signal or edge of sub-block inside a sub-block gap | , for n=1, 2 and 3 |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the base station are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. In case that none of the interfering signal positions fall completely within the frequency range of the downlink operating band, TS 38.141-1 [5] provides further guidance regarding appropriate test requirements. NOTE 2: In Japan, NOTE 1 is not applied in Band n77, n78, n79. NOTE3: In Band n100, n106, n26, n28, n85, each BWChannel in interfering signal type and interfering signal centre frequency is set to 5MHz when BS doesn’t support 3MHz bandwidth. |  |

  - Option 2: Add Note3 in the Table 6.7.2.1-1 for BS type 1-C Co-location Tx intermodulation requirements to set 5MHz as BWChannel in band n26, n28 in Japan when BS doesn’t support 3MHz bandwidth as shown in table3.

Table3

| Parameter | Value |
| --- | --- |
| Wanted signal type | NR single carrier, or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers, with NB-IoT operation in NR in-band if supported. |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5. |
| Interfering signal level | Rated total output power (Prated,t,AC) in the operating band – 30 dB |
| Interfering signal centre frequency offset from the lower/upper edge of the wanted signal or edge of sub-block inside a sub-block gap | , for n=1, 2 and 3 |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the base station are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. In case that none of the interfering signal positions fall completely within the frequency range of the downlink operating band, TS 38.141-1 [5] provides further guidance regarding appropriate test requirements. NOTE 2: In Japan, NOTE 1 is not applied in Band n77, n78, n79. NOTE3: In Band n26 and n28, each BWChannel in interfering signal type and interfering signal centre frequency is set to 5MHz when BS doesn’t support 3MHz bandwidth in Japan. |  |

- Recommended WF

  - TBD

### Sub-topic 2-4

Sub-topic description:

Open issues and candidate options before meeting:

Issue 2-4: R4-2315770, R4-2315771 and R4-2315772

- Proposals

  - NB-IoT operation in NR in-band should be added for 3 MHz channel bandwidth

- Recommended WF

  - Discuss the proposal in R4-2315770, and endorse R4-2315771 (if proposal is not agreed) or R4-2315772 (if proposal is agreed)

### Sub-topic 2-5

Sub-topic description:

Open issues and candidate options before meeting:

Issue 2-5: R4-2316844

- Proposals

  - Option 1: Endorse the draft CR

  - Option 2: Revise the draft CR

  - Option 3: Note the draft CR

- Recommended WF

  - Option 1: Endorse the draft CR

### Sub-topic 2-6

Sub-topic description

Open issues and candidate options before meeting:

Issue 2-6: R4-2316851

- Proposals

  - Option 1: Endorse the draft CR

  - Option 2: Revise the draft CR

  - Option 3: Note the draft CR

- Recommended WF

  - Option 1: Endorse the draft CR