3GPP TSG-RAN WG4 Meeting # 108bis 													R4-2317950

Xiamen, China, October 09 - October 13, 2023

Agenda item:			5.13.8

Source:	Moderator (CMCC)

Title:	Topic summary for [108bis][319] NR_ATG_Demod

Document for:	Information

Introduction

This summary focuses on demodulation requirements for Rel-18 NR ATG, including agenda 5.13.7. The agreed way forward in previous meetings are R4-2313875, R4-2309796, R4-2302996 and R4-2305899. Companies are encouraged to be concise.

Recommendation of prioritized topics for online discussion

Issue 1-1: TDD pattern

Issue 2-1-1: Test scope for PDSCH:

Issue 2-1-2: Test scope for PDCCH

Issue 2-1-3: Test scope for PBCH

Issue 2-2-1: Test scope for CSI reporting

Issue 3-1-1:  Test Scope

Topic #1: General

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2315583 | Ericsson | Observation 1: Demodulation tests with new TDD pattern need ATG UE support extra capabilities. Observation 2: If new TDD pattern is not confirmed to be used in real deployment, two different configurations would be needed for newly defined requirements. Proposal 1 : RAN4 make decision on TDD pattern used in ATG deployment to reduce implementation effort on ATG UE. Proposal 2 : Take following applicability rule for TDD pattern: Only one TDD pattern will be tested according to ATG UE capability. |
| R4-2315696 | ZTE Corporation | Observation 1. For PDSCH demodulation requirements and PUSCH demodulation requirements we can find the separate section defined for specific deployment such as high speed train. Proposal 1. To introduce new sections for ATG PDSCH demodulation requirements and ATG PUSCH demodulation requirements. Proposal 2. To clarify the values of the channel matrix used for demodulation requirements is the static channel defined in B.1 in TS 38.101-4. |
| R4-2316011 | Huawei, HiSilicon | Proposal 1:Define corresponding demodulation requirements with the new TDD pattern and note should be added in the specification that this pattern is for ATG scenario only, these requirements should be optional based on UE capability. Proposal 2:Define the applicability rule that the case with legacy TDD pattern can be skipped if the test of the case with the new TDD pattern is passed, only when the two cases are with same configuration except the TDD pattern. Proposal 3:For ATG UE demodulation requirements, introduce a new section under each physical channel for ATG UE demodulation requirements. Proposal 4:For ATG BS demodulation requirements, add a new section for ATG PUSCH demodulation requirements. In this section, clarifications for how to reuse legacy applicability rules and requirements should be added, and new defined PUSCH demodulation requirements could be captured. |
| R4-2316734 | Qualcomm Incorporated | For the introduction of ATG UE Demod requirements, RAN4 should follow the approach used for RedCap UE Demod requirement, introducing sections based on [5.1.1.11, 5.2.2.1.17, …]; |

Open issues summary

Issue 1-1: TDD pattern

- Proposals

  - Option 1: RAN4 make decision on TDD pattern used in ATG deployment to reduce implementation effort on ATG UE.  (Ericsson)

  - Option 2: Define corresponding demodulation requirements with the new TDD pattern and note should be added in the specification that this pattern is for ATG scenario only, these requirements should be optional based on UE capability. (HW)

  - Option 3: For new incremental PDSCH and PUSCH requirements, introduce test cases of new TDD pattern configuration 30D4S6U. These requirements is optional based on UE capability. (CMCC)

- Recommended WF

  - For new incremental ATG PDSCH and PUSCH requirements, introduce test cases of new TDD pattern configuration 30D4S6U. These requirements is optional based on UE capability.

Issue 1-2: Applicability rule for TDD pattern

- Proposals

  - Option 1: Only one TDD pattern will be tested according to ATG UE capability.  (Ericsson)

  - Option 2: legacy TDD pattern can be skipped if the test of the case with the new TDD pattern is passed, only when the two cases are with same configuration except the TDD pattern. (HW, CMCC, Samsung)

- Recommended WF

  - legacy TDD pattern can be skipped if the test of the case with the new TDD pattern is passed, only when the two cases are with same configuration except the TDD pattern.

Issue 1-3: Specification impact

- Proposals

- For UE demodulation requirements

  - Option 1: introduce a new section under each physical channel for ATG UE demodulation requirements (HW, CMCC)

  - Option 2:  introduce new sections for ATG PDSCH demodulation requirements (ZTE, CMCC)

  - Option 3: For the introduction of ATG UE Demod requirements, RAN4 should follow the approach used for RedCap UE Demod requirement, introducing sections based on [5.1.1.11, 5.2.2.1.17, …]; (QC)

- For BS demodulation requirements

  - Option 1: add a new section for ATG PUSCH demodulation requirements. In this section, clarifications for how to reuse legacy applicability rules and requirements should be added, and new defined PUSCH demodulation requirements could be captured. (HW)

  - Option 2:  introduce new sections for ATG PUSCH demodulation requirements.  (ZTE)

  - Option 3: Capture ATG demodulation requirement into the same section with legacy requirement to minimize the effort of specification modification with adding the referring statement as “The following requirements in sections of 8.2.1 and 8.2.3 of 38.104, can be applied for BS declared to support ATG scenario”. New dedicated requirement can be added into the corresponding table in section 8.2.1 of 38.104.  As for ATG test applicability rule in 38.141-1 and 38.141-2, new clause can be introduced (Samsung)

  - Option 4: (CMCC)

    - New clause for ATG new incremental PUSCH requirements should be introduced in 38.104

    - New clause for ATG applicability of PUSCH requirements and Applicability of requirements for TDD with different UL-DL patterns should be introduced in 38.141-1 and 38.141-2

    - New clause for ATG applicability of PUCCH requirements should be introduced in 38.141-1 and 38.141-2

    - New clause for ATG applicability of PUCCH requirements should be introduced in 38.141-1 and 38.141-2

  - Option 5: (Ericsson)

    - Adding new clause for ATG BS requirements and applicability rules.

- Recommended WF

  - For UE demodulation requirements

    - introduce a new section under each physical channel for ATG UE demodulation requirements

  - For BS demodulation requirements

    - Add new sections for ATG BS demodulation requirements and applicability rules.

Issue 1-4: Channel model

- Proposals

  - Option 1: To clarify the values of the channel matrix used for demodulation requirements is the static channel defined in B.1 in TS 38.101-4.  (ZTE)

- Recommended WF

  - Option 1 can be agreed

Topic #2: UE demodulation

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2315168 | CMCC | Proposal 1: For new incremental PDSCH requirements, introduce test cases of new TDD pattern configuration 30D4S6U. These requirements is optional based on UE capability. Proposal 2: The legacy TDD pattern can be skipped if the test of new TDD pattern is passed. Proposal 3: Select following legacy test cases for ATG UEs 1T2R FDD: Test number 2, 3 and 4 in 5.3.2.1.1 2T2R FDD: All test cases in 5.3.2.1.2 1T2R TDD: Test number 1 and 2 in 5.3.2.2.1 2T2R TDD: All test cases in 5.3.2.2.2 1T4R FDD: Test number 2, 3 and 4 in 5.3.3.1.1 2T4R FDD: All test cases in 5.3.3.1.2 1T4R TDD: Test number 1 and 2 in 5.3.3.2.1 2T4R TDD: All test cases in 5.3.3.2.2 Proposal 4: Not to introduce PMI requirements for ATG UE. Proposal 5: Merge ATG requirements to current specification through one of methods as follows: Option 1: Introduce a new section under each physical channel for ATG UE demodulation requirements. For PDCCH and CQI requirements, refer to the current clauses. Option 2: Only introduce new clauses for ATG PDSCH requirements. Proposal 6: Introduce new clause for ATG applicability of requirements as follows, the test list column and the wording of applicability notes shall be further selected according to how to introduce ATG requirements in the spec: UE Type/capability; Test type; Test list (Option 1); Test list (Option 2); Applicability notes. ATG UE with 2RX; FR1 FDD; PDSCH; All tests in Clause 5.2.2.1.x; All tests in Clause 5.2.2.1.x. PDCCH; All tests in Clause 5.3.2.1.x; Test number 2, 3 and 4 in Clause 5.3.2.1.1, and All tests in Clause 5.3.2.1.2. CQI; All tests in Clause 6.2.2.1.1.x; All tests in Clause 6.2.2.1.1.1. FR1 TDD; PDSCH; Tests [x-x] in Clause 5.2.2.2.x; Tests in Clause 5.2.2.2.x except TDD pattern 30D4S6U; Option 1: If the Tests [y-y] are passed, the test coverage can be considered fulfilled without executing Tests [x-x]. Option 2: If the Tests in Clause 5.2.2.2.x with TDD pattern 30D4S6U are passed, the test coverage can be considered fulfilled without executing Tests in Clause 5.2.2.2.x except TDD pattern 30D4S6U.. PDCCH; All tests in Clause 5.3.2.2.x; Test number 1 and 2 in Clause 5.3.2.2.1, and All tests in Clause 5.3.2.2.2. CQI; All tests in Clause 6.2.2.2.1.x; All tests in Clause 6.2.2.2.1.1. ATG UE with 4RX; FR1 FDD; PDSCH; All tests in Clause 5.2.3.1.x; Tests in Clause 5.2.2.1.x except TDD pattern 30D4S6U. PDCCH; All tests in Clause 5.3.3.1.x; Test number 2, 3 and 4 in Clause 5.3.3.1.1, and All tests in Clause 5.3.3.1.2. CQI; All tests in Clause 6.2.3.1.1.x; All tests in Clause 6.2.3.1.1.1. FR1 TDD; PDSCH; Test [x-x] in Clause 5.2.3.2.x; Tests in Clause 5.2.3.2.x except TDD pattern 30D4S6U; Option 1: If the Tests [y-y] are passed, the test coverage can be considered fulfilled without executing Tests [x-x]. Option 2: If the Tests in Clause 5.2.3.2.x with TDD pattern 30D4S6U are passed, the test coverage can be considered fulfilled without executing Tests in Clause 5.2.3.2.x except TDD pattern 30D4S6U.. PDCCH; All tests in Clause 5.3.3.2.x; Test number 1 and 2 in Clause 5.3.3.2.1, and All tests in Clause 5.3.3.2.2. CQI; All tests in Clause 6.2.3.2.1.x; All tests in Clause 6.2.3.2.1.1. ATG UE support ATG feature (K1 and HARQ process extension introduced by RAN1) with 2Rx; FR1 TDD; PDSCH; Tests [y-y] in Clause 5.2.2.2.x; Tests in Clause 5.2.2.2.x with TDD pattern 30D4S6U. ATG UE support ATG feature (K1 and HARQ process extension introduced by RAN1) with 4Rx; FR1 TDD; PDSCH; Test [y-y] in Clause 5.2.3.2.x; Tests in Clause 5.2.3.2.x with TDD pattern 30D4S6U. |
| R4-2315169 | CMCC | Simulation results for ATG PDSCH demodulation |
| R4-2315697 | ZTE Corporation | Observation 1. CSI reporting of PMI is feasible for ATG scenario. Proposal 1. To consider option1 for ATG PDCCH requirements. |
| R4-2315698 | ZTE Corporation | Simulation results for ATG UE demodulation |
| R4-2315853 | Ericsson | Proposal 1: Define following new test cases for PDSCH for ATG UEs: FDD 10 MHz 15kHz SCS; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). TDD 40MHz 30kHz SCS; 7D1S2U; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). Select following PDSCH legacy test cases for ATG UEs: - Test num 1-3 and 1-4 in Table 5.2.2.1.1-3 (2R FDD) - Test num 1-3 and 1-4 in Table 5.2.2.2.1-3 (2R TDD) - Test num 1-3 and 1-4 in Table 5.2.3.1.1-3 (4R FDD) - Test num 1-3 and 1-4 in Table 5.2.3.2.1-3 (4R TDD) Proposal 2: Select following PDCCH legacy test cases for ATG UEs: - 2T2R FDD: All test cases in 5.3.2.1.2 - 2T4R FDD: All test cases in 5.3.3.1.2 - 2T2R TDD: All test cases in 5.3.2.2.2 - 2T4R TDD: All test cases in 5.3.3.2.2 Proposal 3: Select following legacy PBCH test cases for ATG UEs: - Test num 1 in table 5.4.2.1-2 (1T2R, FDD) - Test num 1 in table 5.4.2.1-3 (1T2R, FDD) - Test num 1 in table 5.4.2.2-2 (1T2R, TDD) - Test num 1 in table 5.4.2.2-3 (1T2R, TDD) - Test num 1 in table 5.4.3.1-2 (1T4R, FDD) - Test num 1 in table 5.4.3.1-3 (1T4R, FDD) - Test num 1 in table 5.4.3.2-2 (1T4R, TDD) - Test num 1 in table 5.4.3.2-3 (1T4R, TDD) Proposal 4: Not to consider SDR requirement for ATG UEs Proposal 5: Not to consider PMI requirements for ATG UEs Proposal 6: Propose applicability rule as described above in the section 2.2 |
| R4-2315854 | Ericsson | Updated simulation results for ATG PDSCH demodulation requirements |
| R4-2316012 | Huawei, HiSilicon | Proposal 1:Cover all aggregation level and select one case per aggregation level. Proposal 2:Select the “yellow-marked” cases in the following Table 2.2.1-1 and Table 2.2.1-2 for ATG UE requirements, i.e. Option 3. Observation 1:The CSI reporting is still useful for ATG scenario. Proposal 3:Reuse the CSI reporting cases for ATG scenario from the existing legacy CSI reporting cases. |
| R4-2316013 | Huawei, HiSilicon | Simulation results on NR UE ATG demodulation requirements |
| R4-2316731 | Qualcomm Incorporated | Simulation Results for ATG UE Demod |
| R4-2316735 | Qualcomm Incorporated | Proposal 1: RAN4 to choose the following legacy PDSCH requirements as applicable to ATG UEs. No additional PDSCH tests should be considered; Test num 1-3 and 1-4 in Table 5.2.2.1.1-3 (2R FDD) Test num 1-3 and 1-4 in Table 5.2.2.2.1-3 (2R TDD) Test num 1-3 and 1-4 in Table 5.2.3.1.1-3 (4R FDD) Test num 1-3 and 1-4 in Table 5.2.3.2.1-3 (4R TDD) Proposal 2: RAN4 to choose 1 test per each 1TX and 2TX configuration, with different AL to provide sufficient coverage, for 2 and 4 RX separately. The tables for are: -2RX: FDD (5.3.2.1.[1/2]), TDD (5.3.2.2.[1/2]); -4RX: FDD (5.3.3.1.[1/2]), TDD (5.3.3.2.[1/2]); Proposal 3: RAN4 to reuse legacy applicability rule for PDCCH requirements based on the number of RX, according to which only 4RX requirements are applicable if UE supports both 2 and 4 RX; Proposal 4: RAN4 not to further consider PMI reporting requirements in the scope of ATG UE Demod; |

Open issues summary

Sub-topic 2-1 UE demodulation requirement

Issue 2-1-1: Test scope for PDSCH:

Agreements in last meeting:

- For new incremental requirements:

  - introduces following test cases for new PDSCH requirements

| FDD 10 MHz 15kHz SCS | 2T2R, 2T4R | 16QAM (MCS [13] in table 1) |
| --- | --- | --- |
|  |  | 64QAM (MCS [22] in table 1) |
|  |  | 256QAM (MCS [24] in table 2) |

| TDD 40MHz 30kHz SCS | 7D1S2U & New TDD pattern for ATG specific scenario e.g. flexible TDD configuration or 30D4S6U(if introduced) | 2T2R, 2T4R | 16QAM (MCS [13] in table 1) |
| --- | --- | --- | --- |
|  |  |  | 64QAM (MCS [22] in table 1) |
|  |  |  | 256QAM (MCS [24] in table 2) |

- Proposals

  - Option 1: Select following PDSCH legacy test cases for ATG UEs: (QC, Ericsson)

    - Test num 1-3 and 1-4 in Table 5.2.2.1.1-3 (2R FDD)

    - Test num 1-3 and 1-4 in Table 5.2.2.2.1-3 (2R TDD)

    - Test num 1-3 and 1-4 in Table 5.2.3.1.1-3 (4R FDD)

    - Test num 1-3 and 1-4 in Table 5.2.3.2.1-3 (4R TDD)

- Recommended WF

  - To be discussed, whether legacy test cases are needed.

Issue 2-1-2: Test scope for PDCCH

- Proposals

  - Option 1: To consider legacy PDCCH requirements for ATG PDCCH requirements. (ZTE, Ericsson)

    - 2T2R FDD: All test cases in 5.3.2.1.2

    - 2T4R FDD: All test cases in 5.3.3.1.2

    - 2T2R TDD: All test cases in 5.3.2.2.2

    - 2T4R TDD: All test cases in 5.3.3.2.2

  - Option 2: Select following legacy test cases for ATG UEs (CMCC)

    - 1T2R FDD: Test number 2, 3 and 4 in 5.3.2.1.1

    - 2T2R FDD: All test cases in 5.3.2.1.2

    - 1T2R TDD: Test number 1 and 2 in 5.3.2.2.1

    - 2T2R TDD: All test cases in 5.3.2.2.2

    - 1T4R FDD: Test number 2, 3 and 4 in 5.3.3.1.1

    - 2T4R FDD: All test cases in 5.3.3.1.2

    - 1T4R TDD: Test number 1 and 2 in 5.3.3.2.1

    - 2T4R TDD: All test cases in 5.3.3.2.2

  - Option 3: (HW)

    - 1T2R FDD: Test number 1, 3 and 5 in 5.3.2.1.1

    - 2T2R FDD: Test number 3 in 5.3.2.1.2

    - 1T2R TDD: All test cases in 5.3.2.2.1

    - 2T2R TDD: All test cases in 5.3.2.2.2

    - 1T4R FDD: Test number 1, 3 and 5 in 5.3.3.1.1

    - 2T4R FDD: Test number 3 in 5.3.3.1.2

    - 1T4R TDD: All test cases in 5.3.3.2.1

    - 2T4R TDD: All test cases in 5.3.3.2.2

  - Option 4: RAN4 to choose 1 test per each 1TX and 2TX configuration, with different AL to provide sufficient coverage, for 2 and 4 RX separately. The tables for are: (QC)

    - 2RX: FDD (5.3.2.1.[1/2]), TDD (5.3.2.2.[1/2]);

    - 4RX: FDD (5.3.3.1.[1/2]), TDD (5.3.3.2.[1/2]);

- Recommended WF

  - With the spirit of compromise, please check whether Option 1 can be agreed.

Issue 2-1-3: Test scope for PBCH

- Proposals

  - Option 1:  Select following legacy PBCH test cases for ATG UE: (Ericsson)

    - Test num 1 in table 5.4.2.1-2 (1T2R, FDD)

    - Test num 1 in table 5.4.2.1-3 (1T2R, FDD)

    - Test num 1 in table 5.4.2.2-2 (1T2R, TDD)

    - Test num 1 in table 5.4.2.2-3 (1T2R, TDD)

    - Test num 1 in table 5.4.3.1-2 (1T4R, FDD)

    - Test num 1 in table 5.4.3.1-3 (1T4R, FDD)

    - Test num 1 in table 5.4.3.2-2 (1T4R, TDD)

    - Test num 1 in table 5.4.3.2-3 (1T4R, TDD)

- Recommended WF

  - To be discussed.

Issue 2-1-4: SDR test cases

- Proposals

  - Option 1:  Not to consider SDR requirement for ATG UE (Ericsson)

- Recommended WF

  - Option 1 can be agreed.

Sub-topic 2-2 CSI reporting requirement

Issue 2-2-1: Test scope for CSI reporting

- Proposals

  - Option 1: Not to introduce PMI requirements for ATG UE.(CMCC, Ericsson, QC)

  - Option 2: Reuse the CSI reporting cases for ATG scenario from the existing legacy CSI reporting cases. (ZTE, HW)

- Recommended WF

  - To be discussed

Sub-topic 2-3 General

Issue 2-3-1: Applicability rule for UE demodulation requirements

- Proposals

  - Option 1:  Introduce new clause for ATG applicability of requirements as follows, the test list column and the wording of applicability notes shall be further selected according to how to introduce ATG requirements in the spec (CMCC)

| UE Type/capability | Test type |  | Test list(Option 1) | Test list(Option 2) | Applicability notes |
| --- | --- | --- | --- | --- | --- |
| ATG UE with 2RX | FR1 FDD | PDSCH | All tests in Clause 5.2.2.1.x | All tests in Clause 5.2.2.1.x |  |
|  |  | PDCCH | All tests in Clause 5.3.2.1.x | Test number 2, 3 and 4 in Clause 5.3.2.1.1, and All tests in Clause 5.3.2.1.2 |  |
|  |  | CQI | All tests in Clause 6.2.2.1.1.x | All tests in Clause 6.2.2.1.1.1 |  |
|  | FR1 TDD | PDSCH | Tests [x-x] in Clause 5.2.2.2.x | Tests in Clause 5.2.2.2.x except TDD pattern 30D4S6U | Option 1: If the Tests [y-y] are passed, the test coverage can be considered fulfilled without executing Tests [x-x]. Option 2: If the Tests in Clause 5.2.2.2.x with TDD pattern 30D4S6U are passed, the test coverage can be considered fulfilled without executing Tests in Clause 5.2.2.2.x except TDD pattern 30D4S6U. |
|  |  | PDCCH | All tests in Clause 5.3.2.2.x | Test number 1 and 2 in Clause 5.3.2.2.1, and All tests in Clause 5.3.2.2.2 |  |
|  |  | CQI | All tests in Clause 6.2.2.2.1.x | All tests in Clause 6.2.2.2.1.1 |  |
| ATG UE with 4RX | FR1 FDD | PDSCH | All tests in Clause 5.2.3.1.x | Tests in Clause 5.2.2.1.x except TDD pattern 30D4S6U |  |
|  |  | PDCCH | All tests in Clause 5.3.3.1.x | Test number 2, 3 and 4 in Clause 5.3.3.1.1, and All tests in Clause 5.3.3.1.2 |  |
|  |  | CQI | All tests in Clause 6.2.3.1.1.x | All tests in Clause 6.2.3.1.1.1 |  |
|  | FR1 TDD | PDSCH | Test [x-x] in Clause 5.2.3.2.x | Tests in Clause 5.2.3.2.x except TDD pattern 30D4S6U | Option 1: If the Tests [y-y] are passed, the test coverage can be considered fulfilled without executing Tests [x-x]. Option 2: If the Tests in Clause 5.2.3.2.x with TDD pattern 30D4S6U are passed, the test coverage can be considered fulfilled without executing Tests in Clause 5.2.3.2.x except TDD pattern 30D4S6U. |
|  |  | PDCCH | All tests in Clause 5.3.3.2.x | Test number 1 and 2 in Clause 5.3.3.2.1, and All tests in Clause 5.3.3.2.2 |  |
|  |  | CQI | All tests in Clause 6.2.3.2.1.x | All tests in Clause 6.2.3.2.1.1 |  |
| ATG UE support ATG feature (K1 and HARQ process extension introduced by RAN1) with 2Rx | FR1 TDD | PDSCH | Tests [y-y] in Clause 5.2.2.2.x | Tests in Clause 5.2.2.2.x with TDD pattern 30D4S6U |  |
| ATG UE support ATG feature (K1 and HARQ process extension introduced by RAN1) with 4Rx | FR1 TDD | PDSCH | Test [y-y] in Clause 5.2.3.2.x | Tests in Clause 5.2.3.2.x with TDD pattern 30D4S6U |  |

  - Option 2: (Ericsson)

| UE capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| ATG UE with 2RX | FR1 FDD | PDSCH | All tests in Clause 5.2.1.1.x (New tests) Test num 1-3, 1-4 in Table 5.2.2.1.1-3 (legacy tests) | If the tests of test num 1-3, 1-4 in Table 5.2.2.1.1-3 are passed, all tests in 5.2.1.1.x (new tests) can be skipped. |
|  |  | PDCCH | All tests in Clause 5.3.2.1.2 |  |
|  |  | PBCH | Test num 1 in Table 5.4.2.1-2/3 |  |
|  |  | CQI | All tests in Clause 6.2.2.1.1.1 |  |
|  | FR1 TDD | PDSCH | Tests in Clause 5.2.1.2.x (New tests) Test num 1-3, 1-4 in Table 5.2.2.2.1-3 (legacy tests) | If the tests of test num 1-3, 1-4 in Table 5.2.2.1.1-3 are passed, all tests in 5.2.1.1.x (new tests) can be skipped. |
|  |  | PDCCH | All tests in Clause 5.3.2.2.2 |  |
|  |  | PBCH | Test num 1 in Table 5.4.2.2-2/3 |  |
|  |  | CQI | All tests in Clause 6.2.2.2.1.1 |  |
| ATG UE with 4RX | FR1 FDD | PDSCH | Tests in Clause 5.2.2.1.x (New tests) Test num 1-3, 1-4 in Table 5.2.3.1.1-3 (legacy tests) | If the tests of test num 1-3, 1-4 in Table 5.2.3.1.1-3 are passed, all tests in 5.2.2.1.x (new tests) can be skipped. |
|  |  | PDCCH | All tests in Clause 5.3.3.1.2 |  |
|  |  | PBCH | Test num 1 in Tabble 5.4.3.1-2/3 |  |
|  |  | CQI | All tests in Clause 6.2.3.1.1.1 |  |
|  | FR1 TDD | PDSCH | Tests in Clause 5.2.2.2.x (New tests) Test num 1-3, 1-4 in Table 5.2.3.2.1-3 (legacy tests) | If the tests of test num 1-3, 1-4 in Table 5.2.3.2.1-3 are passed, all tests in 5.2.2.2.x (new tests) can be skipped. |
|  |  | PDCCH | All tests in Clause 5.3.3.2.2 |  |
|  |  | PBCH | Test num 1 in Table 5.4.3.2-2/3 |  |
|  |  | CQI | All tests in Clause 6.2.3.2.1.1 |  |

- Recommended WF

  - Depends on the agreements in Topic 1, Section 2-1 to 2-2, to be discussed later.

Topic #3: BS Demodulation

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2315170 | CMCC | Proposal 1: For new incremental PUSCH requirements, introduce test cases of new TDD pattern configuration 30D4S6U. Proposal 2: The legacy TDD pattern can be skipped if the test of new TDD pattern is passed. Proposal 3: Introduce following new dedicated requirements for PUSCH, which are separately for FDD and TDD cases. FDD 5 MHz 15kHz SCS AWGN+200Hz doppler; 1T2R; 64QAM (MCS 28 in table 1). 256QAM (MCS 22 in table 2). TDD 10MHz 30kHz SCS AWGN+500Hz doppler; 7D1S2U & 30D4S6U; 1T2R; 64QAM (MCS 28 in table 1). 256QAM (MCS 22 in table 2). Proposal 4: New clause for ATG new incremental PUSCH requirements should be introduced in 38.104 New clause for ATG applicability of PUSCH requirements and Applicability of requirements for TDD with different UL-DL patterns should be introduced in 38.141-1 and 38.141-2 New clause for ATG applicability of PUCCH requirements should be introduced in 38.141-1 and 38.141-2 New clause for ATG applicability of PUCCH requirements should be introduced in 38.141-1 and 38.141-2 |
| R4-2315171 | CMCC | Simulation results for ATG PUSCH demodulation |
| R4-2315584 | Ericsson | Observation 1 It would be feasible to test 64QAM MCS28 and 256QAM MCS22 under ATG channel model. Based on the discussion in the previous sections we propose the following: Proposal 1 Consider 64QAM MCS28 for ATG PUSCH demodulation requirements. Proposal 2 If 64QAM MCS28 and 256QAM MCS22 are introduced for ATG PUSCH demodulation requirements, RAN4 introduce a new applicability rule: If an ATG BS passes newly defined requirement tests, it could skip the legacy tests with the same modulation level.. Proposal 3 Adding new clause for ATG BS requirements and applicability rules. |
| R4-2315699 | ZTE Corporation | Simulation results on NR BS ATG demodulation requirements |
| R4-2316014 | Huawei, HiSilicon | Simulation results on NR BS ATG demodulation requirements |
| R4-2316144 | Samsung | Proposal 1: The legacy TDD pattern can be skipped if the test of new TDD pattern is passed Proposal 2: Capture ATG demodulation requirement into the same section with legacy requirement to minimize the effort of specification modification with adding the referring statement as “The following requirements in sections of 8.2.1 and 8.2.3 of 38.104, can be applied for BS declared to support ATG scenario”. New dedicated requirement can be added into the corresponding table in section 8.2.1 of 38.104. As for ATG test applicability rule in 38.141-1 and 38.141-2, new clause can be introduced Proposal 3: For the new dedicated ATG requirements, cover 256QAM MCS 20 (Table 2) for PUSCH requirement |

Open issues summary

Sub-topic 3-1 PUSCH requirements

Issue 3-1-1:  Test Scope

- Proposals

  - Option 1:  Introduce following new dedicated requirements for PUSCH, which are separately for FDD and TDD cases. (CMCC)

| FDD 5 MHz 15kHz SCS AWGN+200Hz doppler | 1T2R | 64QAM (MCS 28 in table 1) |
| --- | --- | --- |
|  |  | 256QAM (MCS 22 in table 2) |

| TDD 10MHz 30kHz SCS AWGN+500Hz doppler | 7D1S2U & 30D4S6U | 1T2R | 64QAM (MCS 28 in table 1) |
| --- | --- | --- | --- |
|  |  |  | 256QAM (MCS 22 in table 2) |

  - Option 2: Consider 64QAM MCS28 for ATG PUSCH demodulation requirements. (Ericsson)

  - Option 3: For the new dedicated ATG requirements, cover 256QAM MCS 20 (Table 2) for PUSCH requirement (Samsung)

- Recommended WF

  - Confirm to introduce new dedicated test cases for  64QAM MCS28 and 256QAM MCS 22

  - Second check whether to separate the FDD and TDD test cases

Issue 3-1-2: Applicability rule among legacy requirements and new ATG requirements

- Proposals

  - Option 1: If 64QAM MCS28 and 256QAM MCS22 are introduced for ATG PUSCH demodulation requirements, RAN4 introduce a new applicability rule: If an ATG BS passes newly defined requirement tests, it could skip the legacy tests with the same modulation level. (Ericsson)

- Recommended WF

  - Check whether Option 1 can be agreed.