3GPP TSG-RAN WG4 Meeting #108bis                                        R4-2315865

Xiamen, China, October 9 – October 13, 2023

Title:	LS on the new channel bandwidth class for FR2-2

Response to:	-

Release:	Rel-17

Work Item:	NR_ext_to_71GHz-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact person:

#### Name:	Chunying Gu

#### E-mail Address:	guchunying@huawei.com

Attachments: None

# 1		Overall description

New CA bandwidth classes for FR2-2 have been discussed in past RAN4 meetings. Considering the approved channel bandwidth for FR2-2 was up to 2000MHz, the new CA bandwidth classes V and W within FRG1 (4*400MHz and 5*400MHz) were approved to support up to 2000MHz along with the NOTE 4 in TS 38.101-2 v17.8.0 by CR R4-2220821.

Table 5.3A.4-1: CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4,5 |
| B | 400 MHz < BWChannel_CA ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < BWChannel_CA ≤ 1200 MHz | 3 |  |
| V (Note 4) | 1200 MHz < BWChannel_CA ≤ 1600 MHz | 4 |  |
| W (Note 4) | 1600 MHz < BWChannel_CA ≤ 2000 MHz | 5 |  |
| NOTE 1: Maximum supported component carrier bandwidths for fallback groups 1, 2, 3, 4 and 5 are 400 MHz, 200 MHz, 100 MHz, 100 MHz and 200 MHz respectively except for CA bandwidth class A. For CA bandwidth classes of fallback group 5, requirements apply for non-interlaced 100 MHz and 200 MHz channel bandwidths (each CA bandwidth class consisting of up to two contiguous sub-blocks each with component carriers of a single channel bandwidth). NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group. NOTE 3: In this release of the specification, the minimum requirements for intra-band contiguous CA configurations apply for aggregated channel bandwidths up to 1600 MHz for FR2-1 (this note is not relevant for UE capability parsing by the network). NOTE 4: This bandwidth class is applicable only for operating bands within FR2-2. |  |  |  |

RAN4 respectfully ask RAN2 to take above into consideration in the UE capability signaling design.

# 2		Actions

To RAN2

ACTION: 	RAN4 respectfully ask RAN2 to take the new CA channel bandwidth classes for FR2-2 into consideration in the UE capability signaling design.

# 3		Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #109			November 13 – November 17, 2023				Chicago, USA

TSG RAN WG4 Meeting #110			February 26 – March 1, 2024						Athens, Greece