3GPP TSG-RAN WG4 Meeting #109	R4-2318682

Chicago, US, November 13 – 17, 2023

Source:	Apple

Title:	TX power requirement for STxMP

Agenda item:	8.29.1.1

Document for:	Discussion

Introduction

In the WID on Rel-18 MIMO evolution for downlink and uplink, the following objectives are relevant to the feature simultaneous transmission with multi-panel (STxMP) [1]. At the last two RAN4 meetings, discussion on the UE power requirements continued for STxMP and RAN4 agreements were captured in WFs [2][3].

This paper provides our views on the open issues:

- Configured power

- MPR/A-MPR

Discussion

Configured power for STxMP

In the WF [3], the following was agreed for further study.

<Agreement> PCMAXf,c,k

-	LS is sent to RAN1 to inform that RAN4 will introduce PCMAXf,c,k for STxMP (See R4-2314698)

-	How to incorporate the  PCMAXf,c,k in to the spec will be discussed in RAN4#108-bis

With the agreement of defining per-panel ‘k (k=0,1)’ for PCMAXf,c,k as per TCI state, the next task is how to revise the inequality for “per-panel” Pumax. For convenience, the proposed change from [3] is copied here.

There are two issues to be discussed, namely PPowerclass and the relaxation factor ∆TSTxMP. Based on previous agreement, the existing PPowerclass, i.e., the UE minimum peak EIRP, can be reused. In addition, it is necessary to have the relaxation factor as a placeholder to account for possible RF impairments or other design constraint. If there is a per-panel min. peak EIRP requirement defined, in other words, a different Ppowerclass, which considers the relaxation, an explicit relaxation factor would not be needed.

Proposal 1: Relaxation factor in the per-TCI state configured power formulation is needed to account for RF impairments or design constraints.

MPR/A-MPR

In the WF [3], it was agreed to further investigate MPR/A-MPR as follows:

## <Sub-topic 1-3> MPRf,c,k/A-MPRf,c,k

<Way forward>: MPRf,c,k/A-MPRf,c,k derivation without overlapped beam indication

-	Option 1: MAX(MPRf,c, A- MPRf,c,) + X dB, where X is

-	Option 1a: 10*log (number of UL TCI-states indicated for STxMP) dB

-	Option 1b: [3 dB] for STxMP

-	Option 2: MAX(X, MPRf,c, A- MPRf,c,), where X is

-	Option 2a: 10*log (number of UL TCI-states indicated for STxMP) dB

-	Option 2b: [3 dB] for STxMP

<Way forward>: Additional relaxation (TSTxMP)

-	Whether to leave additional relaxation, outside of MAX(MPR) to the lower bound, will be further discussed together with MPRf,c,k and/or for future implementation constraints

MPR allows a UE to back off its power to meet requirements like out-of-band emission requirement or EVM requirement or in-band emission (IBE). A-MPR allows the UE to further back off its power to meet some additional emission limits for a band.

It is believed that when two panels are used for UL transmission, each panel should meet the existing EVM requirement to ensure communication quality. Therefore, it is expected that the current MPR for single panel would serve as the lower bound of MPR needed to meet the existing EVM requirement. What is a bit unclear is the interaction between the two active panels may further degrade the transmission signal quality, thereby requiring further investigation.

On out-of-band emission requirement, we believe because of the agreed minimum requirement on STxMP UE architecture, i.e., the ability to steer two UL beams independently is a minimum capability, it is likely the two panels will have separate phase shifters and PAs after that, also argued in [5]. Therefore, there is no IMD generated by the two transmissions from the same PA. However, it remains to be seen if any interaction between the two active panel will further deteriorate the out of band emission.

At the same time, it is worth mentioning that the relaxation factor in Proposal 1 could be used to deal with such undesired effects due to two simultaneously active panels.

Overall, we think Option 2 for MPR/A-MPR can be a starting point, i.e., MAX(X, MPRf,c,k, A-MPRf,c,k), X = 10*log10(number of UL TCI-states indicated for [STxMP]) dB in lower bound, subject to further checking of whether MPR is gated by EVM/IBE/OOBE. If OOBE is a gating factor in some cases, then additional MPR would be needed. IBE is a directional requirement so only when the two UL beams overlap to a large extent, say two beam peak directions are close in space, additional MPR may be needed. For EVM, the existing MPR should be sufficient.

We also agree that Option 1 is workable, but may lead to unnecessarily lower TX power.

Proposal 2: Option 2 for MPR/A-MPR can be a starting point, i.e., MAX(X, MPRf,c,k, A-MPRf,c,k), X = 10*log10(number of UL TCI-states indicated for [STxMP]) dB in lower bound, subject to further checking of whether MPR is gated by EVM/IBE/OOBE, and further investigation of interaction between the two active panels.

Observation 1: Option 1 is workable, but may lead to unnecessarily lower TX power.

Conclusion

In this contribution, we make the following proposals on STxMP：

Proposal 1: Relaxation factor in the per-TCI state configured power formulation is needed to account for RF impairments or design constraints.

Proposal 2: Option 2 for MPR/A-MPR can be a starting point, i.e., MAX(X, MPRf,c,k, A-MPRf,c,k), X = 10*log10(number of UL TCI-states indicated for [STxMP]) dB in lower bound, subject to further checking of whether MPR is gated by EVM/IBE/OOBE, and further investigation of interaction between the two active panels.

Observation 1: Option 1 is workable, but may lead to unnecessarily lower TX power.

- 4. References

- RP-223276, “WID Update: MIMO Evolution for Downlink and Uplink,” RAN#98-e

- R4-2314911, “WF on NR MIMO evo DL UL UERF,” Samsung

- R4-2317604, “WF on STxMP UE RF requirements,” Samsung

- R4-2311281, “LS to RAN1 on configured Tx power for STxMP in FR2,” Qualcomm