# 3GPP TSG-RAN WG4 Meeting # 109	R4-2320382

# Chicago, US, November 13 – 17, 2023

Agenda Item:			8.2.2

Source:					Qualcomm Incorporated

Title:                        Views on RF test method for FR2 multi-Rx UE

Document for:		Approval

# Introduction

In RAN4#108bis, the test methods for FR2 multi-Rx UE were discussed and the WF was approved in [1]. In this meeting, we provide our views on remaining issues of the test method for UE RF.

# Discussion

## Step size of measurement grid

In RAN4#108bis, RAN4 discussed the step size of measurement grid for the multi-Rx RF testing. The following WF was captured in [1].

Issue 1-1-1: Step size of the measurement grid - Proposals - Proposal 1 (vivo, Qualcomm, CAICT): Take 10deg as the starting point of step size for measurement grid. Testing time comparison between 10deg and 15deg step size should be considered. - Proposal 2 (Samsung): 15deg step size can be considered as starting point and further check 30deg step size. - Proposal 3 (OPPO): 10-degree and 15-degree step sizes can be considered as down-selected options for further discussion. - Agreement: - Take the 10deg and 15deg as the starting point of step size for measurement grid. - Encourage companies to evaluate the worst case antenna configuration of 6x2 for further analysis.

In this section, we provide the simulation results in Table 1 and Table 2 with 10deg and 15deg step size for 6x2 antenna configuration. As the comparison, the simulation results for 4x2 antenna configuration are also provided in Table 3 and Table 4. The data highlighted by Green is the candidate declared AoA offset.

Table 1: Absolute gap compared to 1deg step size for 6*2 Antenna configuration

| Adjacent modules with 6*2 antenna configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 30 | 60 |  | 90 |  |  | 120 | 150 |  | 180 |  |
| OR | 10deg |  |  | 0.10% |  | -0.20% |  |  | 0.40% | 0.50% |  | 0.00% |  |
| OR | 15deg |  |  | 1.40% |  | 0.50% |  |  | 2.70% | 2.70% |  | 1.80% |  |
| Arithmetic | 10deg |  |  | -0.10% |  | -0.30% |  |  | 0.20% | 0.20% |  | 0.00% |  |
| Arithmetic | 15deg |  |  | 1.10% |  | 0.60% |  |  | 1.30% | 2.00% |  | 1.80% |  |
| Opposite sides modules with 6*2 antenna configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | 60 |  | 90 | 120 |  |  | 150 |  | 180 |
| OR | 10deg |  |  |  | 0.80% |  | -1.30% | -2.60% |  |  | -1.60% |  | -2.00% |
| OR | 15deg |  |  |  | 1.90% |  | 2.50% | -1.30% |  |  | -2.10% |  | 1.00% |
| Arithmetic | 10deg |  |  |  | 0.40% |  | -0.60% | -1.00% |  |  | -1.40% |  | -2.00% |
| Arithmetic | 15deg |  |  |  | 0.90% |  | 1.30% | 0.30% |  |  | -0.90% |  | 1.00% |

Table 2: Relative ratio compared to 1deg step size for 6*2 Antenna configuration

| Adjacent modules with 6*2 antenna configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 30 | 60 | 90 | 120 |  | 150 | 180 |  |
| OR | 10deg |  | 0.39% | -0.60% | 1.13% |  | 2.00% | 0.00% |  |
| OR | 15deg |  | 5.49% | 1.49% | 7.63% |  | 10.80% | 13.95% |  |
| Arithmetic | 10deg |  | -0.70% | -1.76% | 1.13% |  | 1.46% | 0.00% |  |
| Arithmetic | 15deg |  | 7.75% | 3.53% | 7.34% |  | 14.60% | 13.95% |  |
| Opposite sides modules with 6*2 antenna configuration |  |  |  |  |  |  |  |  |  |
|  |  | 30 | 60 | 90 | 120 | 150 |  |  | 180 |
| OR | 10deg |  | 57.14% | -9.22% | -6.63% | -3.29% |  |  | -4.02% |
| OR | 15deg |  | 135.70% | 17.73% | -3.32% | -4.32% |  |  | 2.01% |
| Arithmetic | 10deg |  | 57.14% | -8.57% | -4.93% | -3.89% |  |  | -4.02% |
| Arithmetic | 15deg |  | 128.60% | 18.57% | 1.48% | -2.50% |  |  | 2.01% |

Table 3: Absolute gap compared to 1deg step size for 4*2 Antenna configuration

| Adjacent modules with 4*2 antenna configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 30 | 60 |  | 90 |  |  | 120 | 150 |  | 180 |  |
| OR | 10deg |  |  | 0.10% |  | 0.20% |  |  | 0.80% | 0.60% |  | 0.00% |  |
| OR | 15deg |  |  | 1.60% |  | 0.30% |  |  | 2.60% | 2.60% |  | 1.50% |  |
| Arithmetic | 10deg |  |  | -0.10% |  | 0.10% |  |  | 0.40% | 0.20% |  | 0.00% |  |
| Arithmetic | 15deg |  |  | 1.20% |  | 0.10% |  |  | 1.30% | 1.90% |  | 1.50% |  |
| Opposite sides modules with 4*2 antenna configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | 60 |  | 90 | 120 |  |  | 150 |  | 180 |
| OR | 10deg |  |  |  | 1.20% |  | 0.10% | -1.10% |  |  | -0.70% |  | -1.30% |
| OR | 15deg |  |  |  | 1.50% |  | 0.40% | -3.20% |  |  | -2.80% |  | -2.20% |
| Arithmetic | 10deg |  |  |  | 0.60% |  | 0.10% | -0.20% |  |  | -0.50% |  | -1.30% |
| Arithmetic | 15deg |  |  |  | 0.70% |  | 0.30% | -0.90% |  |  | -1.90% |  | -2.20% |

Table 4: Relative ratio compared to 1deg step size for 4*2 Antenna configuration

| Adjacent modules with 4*2 antenna configuration |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 30 | 60 | 90 |  |  | 120 |  | 150 | 180 |  |
| OR | 10deg |  | 0.40% | 0.60% |  |  | 2.25% |  | 2.39% | 0.00% |  |
| OR | 15deg |  | 6.32% | 0.90% |  |  | 7.32% |  | 10.36% | 11.36% |  |
| Arithmetic | 10deg |  | -0.71% | 0.59% |  |  | 2.26% |  | 1.45% | 0.00% |  |
| Arithmetic | 15deg |  | 8.51% | 0.59% |  |  | 7.34% |  | 13.77% | 11.36% |  |
| Opposite sides modules with 4*2 antenna configuration |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 | 60 |  | 90 | 120 |  | 150 |  |  | 180 |
| OR | 10deg |  | 171.00% |  | 0.70% | -2.70% |  | -1.40% |  |  | -2.60% |
| OR | 15deg |  | 214.00% |  | 2.70% | -7.90% |  | -5.80% |  |  | -4.40% |
| Arithmetic | 10deg |  | 150.00% |  | 1.37% | -0.97% |  | -1.38% |  |  | -2.61% |
| Arithmetic | 15deg |  | 175.00% |  | 4.10% | -4.40% |  | -5.20% |  |  | -4.40% |

Observation 1: For 6*2 antenna configuration, the biggest gap between 10deg and 15deg step size is 2.3% for absolute value and 5.5% for the relative ratio.

Observation 2: For multi-Rx UE RF testing, the DL power is not decreasing which is to measure with a fixed step size that will significantly reduce the testing time compared to legacy EIS/EIS spherical coverage testing.

Proposal 1: RAN4 to adopt 10deg as the step size of measurement grid for multi-Rx UE RF testing.

## UE orientation options for multi-Rx UE RF testing

Based on the simulation assumptions and decisions in RF core requirements session, 9 UE orientations in J.3.X of TS 38.101-2 need to be considered for the UE RF testing [1]. But we noticed that the following alignment is missing in the current testing discussion which will limit the candidate UE orientations. The additional alignment option shown in Figure 2.2-1 would bring the total number of UE orientations to 12, and would give UEs more flexibility to find the optimal orientation with different implementation.

Figure 2.2-1: The missing alignment option

Meanwhile, to avoid the potential blocking by fixture or positioner, the re-positioning concept should be adapted for multi-Rx testing that means the re-positioning should be able to apply for each of the 12 UE orientations.

Proposal 2: The additional alignment option shown in Figure 2.2-1 should be considered together with 9 UE orientation in J.3.X of TS 38.101-2. The total number of UE orientations is 12.

Proposal 3: Re-positioning should be able to apply for each of the 12 UE orientations.

# Conclusions

In this paper, we discuss the test methods for UE RF for multi-Rx UE. The following observations and proposals are made:

Observation 1: For 6*2 antenna configuration, the biggest gap between 10deg and 15deg step size is 2.3% for absolute value and 5.5% for the relative ratio.

Observation 2: For multi-Rx UE RF testing, the DL power is not decreasing which is to measure with a fixed step size that will significantly reduce the testing time compared to legacy EIS/EIS spherical coverage testing.

Proposal 1: RAN4 to adopt 10deg as the step size of measurement grid for multi-Rx UE RF testing.

Proposal 2: The additional alignment option shown in Figure 2.2-1 should be considered together with 9 UE orientation in J.3.X of TS 38.101-2. The total number of UE orientations is 12.

Proposal 3: Re-positioning should be able to apply for each of the 12 UE orientations.

# References

- R4-2317002, WF on FS_NR_FR2_OTA_en, Qualcomm, RAN4#108bis, October 2023.

- R4-2316507, Discussion on RF test method for FR2 multi-Rx UE, Qualcomm, RAN4#108bis, October 2023