3GPP TSG-RAN WG4 Meeting #109	R4-2321599

Chicago, US, 13 – 17 November, 2023

Title:	Reply LS on measurement definitions for positioning with bandwidth aggregation

Response to:	R1-2308449

Release:	Release 18

Work Item:	NR_pos_enh2

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

Name:	Li Zhang

E-mail Address:	zhangli164@huawei.com

Attachments: -

1. Overall Description:

RAN4 thanks RAN1 for the LS R1-2308449. RAN4 discussed how to capture the condition of single RF chain (same antenna) in RAN4’s specification, and reached the following agreements.

- RAN4 to take “single Rx chain” assumption for defining accuracy requirements for positioning measurements based on PRS/SRS bandwidth aggregation.

- RAN4 to define the following side conditions for the UE RSTD and Rx-Tx measurement period (core) and accuracy (performance) requirements.

  - For PRS resources on multiple PFLs linked for aggregation, the UE RSTD/Rx-Tx (core) and accuracy (performance) requirements apply provided that the channel over which a symbol on one PFL for PRS transmission is conveyed can be inferred from the channel over which the same symbol of another PFL or the aggregated PFL is conveyed.

Note: this does not imply the radio propagation conditions are the same for aggregated PFLs

- RAN4 to define the following side conditions for the gNB Rx-Tx measurement accuracy (performance) requirements.

  - For positioning SRS resources on multiple carriers linked for aggregation, the gNB Rx-Tx measurement accuracy (performance) requirements apply provided that the channel over which a symbol on one carrier for SRS transmission is conveyed can be inferred from the channel over which the same symbol of another carrier or the aggregated carrier is conveyed.

Note: this does not imply the radio propagation conditions are the same for aggregated carriers

Based on the agreements, RAN4 believes it is better to capture the “single Tx chain” assumption in RAN1 spec.

2. Actions:

To RAN1:

RAN4 respectfully asks RAN1 to take the above information into account and to capture the “single Tx chain” (same Tx antenna) assumption in RAN1 spec.

3. Date of Next TSG-RAN4 Meetings:

RAN WG4 Meeting #110		Feb 26 – Mar 1, 2024	         Athens, Greece

RAN WG4 Meeting #110-bis		Apr 15 – Apr 19, 2024	         TBD, China