3GPP TSG RAN Meeting #102							draft RP-232792

Edinburgh, Scotland, December 11-15, 2023

Agenda Item:		9.3.4.3

Source:			Nokia, Xiaomi

Title:				Summary for WI NR RF requirements enhancement for frequency range 2 (FR2), Phase 3

WI code(s):			WI code NR_RF_FR2_req_Ph3

leading WG:		RAN4

Release:			Rel-18

### 1	Introduction

This Rel-18 work item introduces UL 256 QAM requirement and UE beam correspondence requirement in inactive and initial states as FR2 UE RF requirement enhancement features.

NR FR2-1 bands are new frequency bands introduced for the first time in Rel-15 specifications. Since then, UE RF requirement enhancements for FR2-1 bands have been introduced in Rel-16 WI on NR RF Requirement Enhancements for FR2 (WI code: NR_RF_FR2_req_enh) and Rel-17 WI on Further enhancements of NR RF requirements for frequency range 2 (WI code: NR_RF_FR2_req_enh2).

In earlier work items in Rel-16 and Rel-17, to enhance uplink data throughput, wider bandwidth by carrier aggregation was targeted but is now uplink higher modulation up to 256QAM is in Rel-18. The beam correspondence requirement enhancement was targeted for the connected state only earlier and is now for the inactive and initial states in Rel-18.

### 2	Description

For power class 1/2/5 UE in FR2-1 bands, the following UL 256 QAM requirements are specified.

- EVM requirement is 3.5%.

- MPR requirement for single CC and intra-band CC

  - MPRWT is 3 larger than 64QAM for single carrier or inter-band CA and 3.5 to 4 dB larger than 64QAM for intra-band CA for 28GHz bands

  - additionally, 1 dB extra for 39 GHz bands as ∆MPR.

For power class 3 UE in FR2-1 bands, the following beam correspondence requirements in initial access and RRC_Inactive are specified.

- For spherical coverage EIRP in beam correspondence, minimum EIRP requirement at 50%-tile CDF is specified 2 dB lower in initial access and RRC_INACTIVE than RRC_CONNECTED.

- Side conditions are the same as SSB based enhanced beam correspondence requirement introduced in Rel-16.

- Minimum peak EIRP requirement is not applicable in initial access and RRC_INACTIVE.

### 3	References

[1] R4-232791Revised WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3

[2] TR38.891 NR RF requirements enhancement for frequency range 2 (FR2), Phase 3

[3] RP-232790 Status report for WI NR RF requirements enhancement for frequency range 2 (FR2), Phase 3; rapporteur: Nokia, Xiaomi

[4] R4-2321984 Introducing beam correspondence requirement for initial access and RRC_INACTIVE, Nokia, Nokia Shanghai Bell	, Apple, CMCC, Huawei, HiSilicon, MediaTek inc., OPPO, Qualcomm, Samsung, Sony, Vivo, Xiaomi, ZTE Corporation

[5] R4-2318632 On beam correspondence requirement for EN-DC/NE-DC, Apple

[6] R4-2321718 CR for Rel-18 38.101-2 to introduce FR2-1 UL 256 QAM RF requirements, Xiaomi, Nokia, vivo, LG Electronics, ZTE, Qualcomm, Sony, MediaTek, Huawei, Apple, Ericsson