3GPP TSG-RAN WG4 Meeting #110                                                                                         R4-2400040

Athens, GR, February 26 – March 01, 2024

Source: 	CATT

Title: 	Further discussion on co-existence study for above 10GHz bands

Agenda item:	8.18.2

Document for:	Approval

- Introduction

In RAN4#109 meeting, a WF [1] for NTN co-existence study was agreed, some remaining issues in WF need further discussion.

This contribution provides our views on the remaining issues for NTN co-existence study for above 10GHz bands.

- Discussion

Updated co-existence simulation result

The co-existence simulation result for case 2 has been updated, see excel [2]. The updated ACIR curves of case 2 for fixed VSAT are shown as following.

Figure 2-2: Case 2 Aggressor TN UL to Victim NTN UL (GEO, LEO-1200, LEO-600)

Based on above updated simulation result for case 2, the required ACIR for SAN ACS for GEO, LEO-1200 and LEO-600 are 16.64, 12.8, and 12dB, respectively. The ACIR at 5% throughput loss of the updated simulation result for case 2 is more in line with the average ACIR for case 2 in WF in RAN4#109.

Observation 1: The ACIR at 5% throughput loss of the updated simulation result for case 2 is more in line with  average ACIR for case 2 in WF in RAN4#109.

VSAT ACS

As per the WF [1], the remaining open issue concerning VSAT ACS is shown as below:

For VSAT ACS: - Considering below candidate values as starting point for VSAT ACS - Option 1: 23 dBc - Option 2: 30 dBc - Option 3: 35 dBc - FFS for the additional means to address the co-existence issue, candidate options for further discussion - Option 1: Limit the elevation angles on DL side for VSAT UE - Option 2: Configure additional guard-band - Option 3: Consider different channel model (e.g. UMa instead of free space loss) between VSAT UE and TN BS. - Other options not precluded

In WF [1], based on collected results till RAN4#109, ACIR for GEO and ACIR for LEO for VSAT ACS in WF are [39.2] and [42.9], respectively. So we think that 35dBc ACS is closer to ACIR for VSAT ACS, and can meet the Ka-band NTN co-existence scenarios where NTN UEs are randomly dropped in TN clusters. So we prefer to option 3 (35dBc)

Proposal 1: Adopt option 3(35dBc) for VSAT ACS.

For the additional means to address the co-existence issue, the option 1and option 3 may require an additional round of co-existence simulation to verify the effect. However, for option 2, guard band has always proven to be a helpful approach in solving the coexistence issues (it can reduce adjacent frequency interference to achieve co-existence). We think that Option 2 can be as additional mean to address the co-existence issue.

Observation 2: Option 2 might be helpful to address the co-existence issue.

- Conclusion

This contribution discussed VSAT ACS for above 10GHz bands. The following proposal and observation are provided as follows:

Observation 1: The ACIR at 5% throughput loss of the updated simulation result for case 2 is more in line with  average ACIR for case 2 in WF in RAN4#109.

Proposal 1: Adopt option 3(35dBc) for VSAT ACS.

Observation 2: Option 2 might be helpful to address the co-existence issue.

- Reference

- R4-2321115, WF for NTN co-existence study, Samsung, RAN4#109

- R4-2400039, Updated co-existence simulation result for above 10GHz bands, CATT, RAN4#110

- R4-2318298, Co-existence simulation result for above 10GHz bands, CATT, RAN4#109

- R4-2318493, Discussion on co-existence study for above 10GHz bands, CATT, RAN4#109