3GPP TSG-RAN WG4 Meeting #110	R4-2400148

# Athens, Greece,  February 26th - March 1st 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 6030 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for the LTE based 5G terrestrial broadcast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_bands_part2 |  |  |  |  |  | Date: |  |  | 2024-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During the previous meetings it was discussed which ACS values should be specified for the 6, 7 and 8MHz PMCH channel in presence of the blocker that may fall into the UE 10MHz filter. No conclusion was reached, but instead some tentative values were put in […]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The ACS values for the PMCH channel 6,7 and 8MHz bandwidth are corrected in accordance with the baseline assumption of using the 10MHz filter at the UE side. Case 2 interferer level is corrected to -25dBm. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ACS values will remain unspecified in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.1H |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 7.3.1H	Minimum requirements for LTE based 5G terrestrial broadcast

The throughput shall be ≥ 95% of the maximum throughput as represented by a reported BLER of <5% for the reference measurement channels as specified in Annexes A.3.18 with parameters specified in Table 7.3.1H-1.

Table 7.3.1H-1: Reference sensitivity for LTE based 5G terrestrial broadcast

| Operating Band | PMCH bandwidth |  |  | Duplex Mode |
| --- | --- | --- | --- | --- |
|  | 6 MHz (dBm) | 7 MHz (dBm) | 8 MHz (dBm) |  |
| 107 | -94 | -94 | -94 | SDO |
| 108 | -94 | -94 | -94 | SDO |
| NOTE: The signal power is specified per port |  |  |  |  |

******************** next changed section ********************

### 7.5.1H	Minimum requirements for LTE based 5G terrestrial broadcast

The UE shall fulfil the minimum requirement specified in Table 7.5.1H-1 for all values of an adjacent channel interferer up to –22 dBm. However it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5.1H-2 and Table 7.5.1H-3 where the throughput shall be ≥ 95% of the maximum throughput as represented by a reported BLER of <5% for the reference measurement channels as specified in Annex A.3.18.

Table 7.5.1H-1: Adjacent channel selectivity for LTE based 5G terrestrial broadcast

| Rx Parameter | Units | PMCH bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | 6 MHz | 7 MHz | 8 MHz |
| ACS | dB | 20.5 | 22.5 | 24.5 |

Table 7.5.1H-2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | PMCH bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | 6 MHz | 7 MHz | 8 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |
| PInterferer | dBm | REFSENS +41.5dB | REFSENS +43.0dB | REFSENS +44.0dB |
| BWInterferer | MHz | 5 |  |  |
| FInterferer (offset) | MHz | 5.5+0.0125 / -5.5-0.0125 | 6.0+0.0075 / -6.0-0.0075 | 6.5+0.0025 / -6.5-0.0025 |
| NOTE: The interferer consists of the Reference measurement channel specified in Annex A.3.18. |  |  |  |  |

Table 7.5.1H-3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | PMCH bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | 6 MHz | 7 MHz | 8 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -49.5 | -51 | -52 |
| PInterferer | dBm | -25 |  |  |
| BWInterferer | MHz | 5 |  |  |
| FInterferer (offset) | MHz | 5.5+0.0125 / -5.5-0.0125 | 6.0+0.0075 / -6.0-0.0075 | 6.5+0.0025 / -6.5-0.0025 |
| NOTE: The interferer consists of the Reference measurement channel specified in Annex A.3.18. |  |  |  |  |