3GPP TSG-RAN4 Meeting #110	R4- 2400181

# Athens, Greece, February 26th – March 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 1110 | rev |  | Current version: | 15.24.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-3 on adding the missing UE transmitter requirements for inter-band NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2024-01-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Configurations for inter-band NR-DC between FR1 and FR2 have been introduced since Rel-15 while the essential UE transmitter power requirements have been missing except for configured output power which could be an oversight in RAN4 specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the following missing requirements: - UE maximum output power for inter-band NR-DC between FR1 and FR2 (sub-clause 6.2B.1.6) - UE maximum output power reduction for inter-band NR-DC between FR1 and FR2 (sub-clause 6.2B.2.6) - UE additional maximum output power reduction for inter-band NR-DC between FR1 and FR2 (sub-clause 6.2B.3.6) - Change the clause 6.2B.3 title from “UE additional maximum output power reduction for EN-DC” to “UE additional maximum output power reduction for DC”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE transmitter power requirements for inter-band NR-DC between FR1 and FR2 remain missing in RAN4 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.1, 6.2B.2, 6.2B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changed sections >>>

### 6.2B.1	UE maximum output power for DC

<<< Unchanged sections omitted >>>

#### 6.2B.1.5	Inter-band EN-DC including both FR1 and FR2

UE maximum output power requirement for E-UTRA single carrier and CA operation specified in clauses 6.2.2 and 6.2.2A of TS 36.101 [4] and for NR single carrier specified in clause 6.2.1 of TS 38.101-1 [2] and for NR single carrier, CA operation and UL-MIMO specified in clause 6.2.1, 6.2A.1, and 6.2D.1 of TS 38.101-2 [3] apply. When uplink is EN-DC mode within FR1 only then UE maximum output power requirement is specified in clause 6.2B.1.3 of this specification.

#### 6.2B.1.6	Inter-band NR-DC between FR1 and FR2

For inter-band NR-DC in FR1 and FR2 combined, the UE shall meet each transmitter power requirement specified in clause 6.2.1 of TS 38.101-1 [2] for NR single carrier and clause 6.2.1 and clause 6.2.1D of TS 38.101-2 [3] for NR single carrier and UL-MIMO independently.

<<< Unchanged sections omitted >>>

### 6.2B.2	UE maximum output power reduction for DC

<<< Unchanged sections omitted >>>

#### 6.2B.2.5	Inter-band EN-DC including both FR1 and FR2

UE maximum output power reduction requirement for E-UTRA single carrier and CA operation specified in clauses 6.2.3 and 6.2.3A of TS 36.101 [4] and for NR single carrier specified in clause 6.2.2 of TS 38.101-1 [2] and for NR single carrier, CA operation and UL-MIMO specified in clause 6.2.2, 6.2A.2, and 6.2D.2 of TS 38.101-2 [3] apply.

#### 6.2B.2.6	Inter-band NR-DC between FR1 and FR2

For inter-band NR-DC between FR1 and FR2, UE maximum output power reduction specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for each frequency range respectively.

<<< Unchanged sections omitted >>>

### 6.2B.3	UE additional maximum output power reduction for DC

<<< Unchanged sections omitted >>>

#### 6.2B.3.5	Inter-band EN-DC including both FR1 and FR2

UE additional maximum output power reduction requirement for E-UTRA single carrier and CA operation specified in clauses 6.2.4 and 6.2.4A of TS 36.101 [4] and for NR single carrier specified in clause 6.2.3 of TS 38.101-1 [2] and for NR single carrier, CA operation and UL-MIMO specified in clause 6.2.3, 6.2A.3, and 6.2D.3 of TS 38.101-2 [3] apply.

#### 6.2B.3.6	Inter-band NR-DC between FR1 and FR2

For inter-band NR-DC between FR1 and FR2, UE additional maximum output power reduction specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for each frequency range respectively.

<<< End of changed sections >>>