# 3GPP TSG-RAN WG4 Meeting #110	R4-2400208

# Athens, Greece, 26 Feb - 01 Mar, 2024

Source: 	Samsung, Bell Mobility, TELUS

Title:	TP for TR 38.880: Input for CA_n7A-n77A with 3Tx

Agenda Item:	7.25.2

Document for:	Approval

- Introduction

This contribution provides a text proposal for CA_n7A-n77A 3Tx with 2Tx(UL MIMO and TxD) on n77 for TR 38.880.

Both PC2 and PC1.5 for this combo with 3Tx transmitting simultaneously are proposed in this contribution.

Please note that the harmonic mixing requirements for 2Tx CA_n7A-n77A (PC2 and PC1.5) which are supposed to be specified is unfortunately missing from Rel-18, a correction CR R4-2400207 is submitted in this meeting to address this issue, hence 3Tx harmonic mixing has corresponding referable requirements.

Reference: RP-234046	WID revision for Core part 4Rx handheld UE for low NR bands (1GHz) and or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC

# Text Proposal

<< Beginning of Text Proposal >>

## 5.x CA_n7-n77

### 5.x.1	Operating bands for CA

Table 5.x.1-1:  CA band combination of band n7+n77

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n7-n77 | n7, n77 |

### 5.x.2	Maximum output power for inter-band CA

Table 5.x.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n7A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. NOTE 4: The UE supports PC2 with UL MIMO in either one of the TDD bands and PC2 in the other TDD band. |  |  |  |  |  |  |

Table 5.x.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n7A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.x.3	MSD scenario studies

Table 5.x.3-1 and 5.x.3-2 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_ n7-n77.

Table 5.x.3-1: MSD scenarios for CA_n7-n77 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n77(UL2/DL3) | n7 | Yes |
| Cross band leakage | n77 | n7 | Yes |
| IMD | IMD 4 (2nd order of n7 UL + 2nd order of n77 UL) | n7 | Yes |

Table 5.x.3-2: MSD scenarios for CA_n7-n77 with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n77(UL2/DL3) | n7 | Yes |
| Cross band leakage | n77 | n7 | Yes |
| IMD | IMD 4 (2nd order of n7 UL + 2nd order of n77 UL) | n7 | No |

### 5.x.4	REFSENS requirements

#### 5.x.4.1	REFSENS requirements for total power class 2

- The 2Tx MSD due to harmonic mixing has already been specified, which is applicable for 3Tx.

- The 2Tx MSD due to cross band isolation has already been specified, which is applicable for 3Tx.

- The 2Tx MSD due to IMD has already been specified, which is applicable for 3Tx.

#### 5.x.4.2	REFSENS requirements for total power class 1.5

- The 2Tx MSD due to harmonic mixing has already been specified, which is applicable for 3Tx.

- The 2Tx MSD due to cross band isolation has already been specified, which is applicable for 3Tx.

- The MSD due to IMD4 for PC1.5 3Tx is specified in the following table.

Table 5.x.4.2-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n77 | n7 | 2540 | 5 | 25 | 2660 | 29.7 | FDD | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | TDD | N/A |

<< End of Text Proposal >