3GPP TSG-RAN WG4 Meeting # 110		R4-2400339

# Athens, GR, 26 Feb – 01 Mar, 2024

Agenda Item:	8.19.1.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Necessity of UE capability associated with dpc-Reporting-FR1

Document for:	Approval

# Introduction

This contribution discusses necessity of a UE capability for dpc-Reporting-FR1.

# Discussion

RAN2#124 agreed a CR [1] for TS38.321 and another CR [2] for TS38.331 via post RAN2#124 e-mail discussion. A corresponding UE capability, however, was not defined in TS38.306. The situation may come from the fact that the approved RAN4 LS of [3] included UE capability aspects for the following Q4, but the question was actually, not about the granularity of UE capability (filed), but rather information to be reported via MAC.

Observation: RAN2#124 agreed a CR [R2-2314028] for TS38.321 and another CR [R2-2314064] for TS38.331 via post RAN2#124 e-mail discussion. A corresponding UE capability, however, was not defined in TS38.306. A corresponding UE capability is, however, not defined in TS38.306.

Proposal 1: Send an LS to RAN2 to introduce a corresponding UE capability for dpc-Reporting-FR1.

Note: a companion draft LS can be seen in [4]. Final decision of sending the LS also depends on RAN2 status over the week of RAN4#110.

On Feature (capability)

Proposal 2: Followings are proposed as feature

Feature group : dpc (∆PPowerClass) report for FR1

Components: Indicates whether UE can report ∆PPowerClass or report ∆PPowerClass as well as ΔPPowerClass,CA/∆PPowerClass,EN-DC/∆PPowerClass,NR-DC when the UE is configured with dpc-Reporting-FR1 and the reporting is triggered only by uplink duty cycle exceedance or by return to the ue-PowerClass or powerClass after the duty cycle exceedance.

Prerequisite feature groups: NONE (The UE supports at least a band or band combination with power class higher than 23 dBm (power class 3))

Need for the gNB to know if the feature is supported: YES

Applicable to the capability signalling exchange between UEs: No

Consequence if the feature is not supported by the UE:  UE cannot obtain optimized resource schedule by the network

Type:Per UE

Need of FDD/TDD differentiation : No

Need of FR1/FR2 differentiation: Yes (applicable to only FR1)

Capability interpretation for mixture of FDD/TDD and/or FR1/FR2: N/A

Mandatory/Optional: Optional with capability signalling

# Conclusion

We obtained following one observation and two proposals.

Observation: RAN2#124 agreed a CR [R2-2314028] for TS38.321 and another CR [R2-2314064] for TS38.331 via post RAN2#124 e-mail discussion. A corresponding UE capability, however, was not defined in TS38.306. A corresponding UE capability is, however, not defined in TS38.306.

Proposal 1: Send an LS to RAN2 to introduce a corresponding UE capability for dpc-Reporting-FR1.

Note: a companion draft LS can be seen in [4]. Final decision of sending the LS also depends on RAN2 status over the week of RAN4#110.

Proposal 2: Followings are proposed as feature.

Feature group : dpc (∆PPowerClass) report for FR1

Components Indicates whether UE can report ∆PPowerClass or report ∆PPowerClass as well as ΔPPowerClass,CA/∆PPowerClass,EN-DC/∆PPowerClass,NR-DC when the UE is configured with dpc-Reporting-FR1 and the reporting is triggered only by uplink duty cycle exceedance or by return to the ue-PowerClass or powerClass after the duty cycle exceedance.

Prerequisite feature groups: NONE (The UE supports at least a band or band combination with power class higher than 23 dBm (power class 3))

Need for the gNB to know if the feature is supported: YES

Applicable to the capability signalling exchange between UEs: No

Consequence if the feature is not supported by the UE:  UE cannot obtain optimized resource schedule by the network

Type:Per UE

Need of FDD/TDD differentiation : No

Need of FR1/FR2 differentiation: Yes (applicable to only FR1)

Capability interpretation for mixture of FDD/TDD and/or FR1/FR2: N/A

Mandatory/Optional: Optional with capability signalling

# Reference

- R2-2314028, Introduction of Further NR Coverage Enhancements in MAC spec, ZTE Corporation, TSG-RAN WG2 #124

- R2-2314064, Introduction of Further NR coverage enhancements in RRC, Huawei, HiSilicon, TSG-RAN WG2 #124

- R4-2321998, LS reply on further clarifications on enhancements to realize increasing UE power high limit for CA and DC, Huawei, HiSilicon,, TSG-RAN WG4 #109

- R4-2400340, draft LS for dpc-Reporting-FR1, Nokia, , Nokia Shanghai Bell, TSG-RAN WG4 #110