3GPP TSG-RAN WG4 Meeting # 110 								                 R4-2400364

Athens, Greece, 26th February – 1st March 2024

Title: 	NR-U Nominal channel spacing

Agenda Item:	4.1

Source: 	Skyworks Solutions, Nokia

Document for:	Approval

# 1	Introduction

At RAN4 meeting #109, we observed that the intra-band contiguous CA (IB-CA) Nominal Channel Spacing (NCS) equations for NR may not always be met for certain NR-U configurations [1]. In this paper, we extend the analysis by covering a larger set of component carrier (CC) channel bandwidth (CBW) permutations. We present a set of rules for NR-U NCS deviations from the NR IB-CA NCS and the NR adjacent carrier (AC) NCS equations. Release 16 (Rel-16) companion change request (CRs) are proposed accordingly [2,3], as well as mirror CRs for Rel-17/Rel-18.

# Discussion

Contrary to NR, operation in unlicensed bands n46, n96 and n102, is supported using a discrete set of NR-ARFCN channels which have been selected to serve a dual purpose:

- To enable listen-before-talk, the NR-U channels shall match as closely as possible the WiFi channel grid;

- To enable intra-band contiguous CA operation, the NR-U channels shall meet the nominal channel spacing (NCS) specified in clause 5.4A.1.

Due to the large bandwidth of NR-U bands, it is impossible to meet both requirements across the whole band. This is because the NR nominal channel spacing for intra-band contiguous operation may be less than the Wi-Fi channel spacing. For example, two 20MHz adjacent channels are separated by 20MHz for Wi-Fi, but for intra-band contiguous CA (IB-CA), the channels are separated by an IB-CA NCS of 19.98MHz. To mitigate this mismatch, the NR-U channels (ARFCN) have been corrected at regular intervals across the band. In turn, these NR-U ARFCN corrections introduce deviations from:

- the NR IB-CA NCS specified in clause 5.4A.1 and,

- the NR “adjacent channel” NCS (AC-NCS) specified in clause 5.4.1.1.

The NR-U ARFCN correction pattern, and both deviations, are highlighted in light blue in Figure 1. This is to illustrate the example of band n96 using adjacent 20MHz component carriers (CCs). For the sake of simplicity, Figure 1 only depicts the configurations of the lowermost, mid, and uppermost channels. The comprehensive list is captured in the Annex Table 1.

Lowermost channels

Mid channels

Uppermost channels

Figure 1: Analysis for band n96 20+20MHz CCs

From Figure 1, we observe that for two adjacent 20MHz CCs in band n96:

- The 20 MHz + {-5, 0, 5} [kHz] AC-NCS specified in clause 5.4.1.1 is never achieved. For this example, the configured AC-NCS is 20 MHz + {-20, 40} [kHz],

- The 19.98MHz IB-CA NCS specified in clause 5.4A.1 is:

  - satisfied for some pairs of channels, for example {79700;798332}, {799668;801000}, {801000;802332} etc...

  - exceeded by 60kHz for pairs where an ARFCN correction is applied, for example {798332;799668}, {802332 ;803668} etc. (see blue highlighted cells). The configured channel spacing for these channels is 20.04MHz.

In this example, the configured IB-CA NCS can be expressed as 19.98 MHz + {0, 60} [kHz].

- Since the NR-ARFCN correction pattern is regular, the deviation from the NR NCS can be detected by reducing the analysis to several channels, either the lowermost or the uppermost channels. For example, Figure 2 shows that the lowermost 20+20MHz channels of band n46 exhibit the same correction pattern than that for band n96, and therefore the deviation from the IB-CA NCS and the AC-NCS is identical for band n46 than that exhibited for band n96.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 20 | 745332 | 746668 | 5179.98 | 5200.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 746668 | 748000 | 5200.02 | 5220 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 748000 | 749332 | 5220 | 5239.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 749332 | 750668 | 5239.98 | 5260.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 750668 | 752000 | 5260.02 | 5280 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 752000 | 753332 | 5280 | 5299.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 753332 | 754668 | 5299.98 | 5320.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 754668 | 756000 | 5320.02 | 5340 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 756000 | 765332 | 5340 | 5480 | 9332 | 139.98 | 20 MHz + {-5, 0, 5} | 19.98 | 120000 UNII-2B gap | 119980 UNII-2B gap |
| 20 | 20 | 765332 | 766668 | 5479.98 | 5500.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 766668 | 768000 | 5500.02 | 5520 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 768000 | 769332 | 5520 | 5539.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |

Figure 2: Analysis for band n46 20+20MHz CCs

We also submit (cf Annex) analyses for other CC CBW permutations for bands n46 and n96 in Table 1 to Table 19. While this study is not exhaustive, it is sufficiently complete to establish a set of rules for NR-U NCS deviations from the NR NCS equations.

From this analysis, we present the following observation:

Observation 1: For unlicensed bands:

- the deviations from the clause 5.4.1.1 adjacent carrier NCS never exceeds +- 40kHz and can be summarized as {-40 kHz, -20 kHz, 0 kHz, 20 kHz, 40 kHz}.

- the deviations from the clause 5.4A.1 intra-band contiguous NCS never exceeds 300kHz and can be summarized as + {0 kHz, 60 kHz, 120 kHz, 180kHz, 240kHz, 300kHz}

Observation 2 summarizes the key concern regarding these deviations.

Observation 2: From clause 5.4A.1, some UEs designed to support intra-band contiguous CA operation, such as CA_n96B, may conclude that network configurations for which the configured channel spacing exceeds the nominal channel bandwidth are not valid configurations. This observation applies to both DL-CA configurations and to UL-CA configurations. The UE initial attach behavior may be unreliable, and the UE may fail the initial attach procedure.

To address these observations, we propose to adopt the changes of proposal 1 for TS 38.101-1, and those of proposal 2 for TS 38.104. Companion CRs are presented at this meeting, [2] and [3] are the Rel-16 CRs.

Proposal 1: Adopt the following changes to TS 38.101-1 – see proposed Rel-16 CR [2].

- Add the following text to Rel-16 clause 5.4.1.1:

For NR bands restricted to operation with shared-spectrum channel access,

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2 + {-40 kHz, -20 kHz, 0 kHz, 20 kHz, 40 kHz}.

- Add the following text into a new clause 5.4F:

## 5.4F	Channel arrangement for shared spectrum channel access

### 5.4F.1A	Channel spacing for CA

For intra-band contiguous carrier aggregation with two or more component carriers, the nominal channel spacing between two adjacent shared-spectrum access component carriers is defined as the following, unless stated otherwise:

For NR bands restricted to operation with shared-spectrum channel access,

+ {0 kHz, 60 kHz, 120 kHz, 180kHz, 240kHz, 300kHz}

with

n = µ0

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective shared-spectrum access component carriers according to Table 5.3.2-1 with values in MHz, μ0  is the largest μ value among the subcarrier spacing configurations supported in the operating band for both of the channel bandwidths according to Table 5.3.5-1 and GBChannel(i) is the minimum guard band for channel bandwidth i according to Table 5.3.3-1 for the said μ value with μ as defined in TS 38.211. In case there is no common μ value for both of the channel bandwidths, μ0=1 is selected and GBChannel(i) is the minimum guard band for channel bandwidth i according to Table 5.3.3-1 for μ=1 with μ as defined in TS 38.211.

For intra-band non-contiguous carrier aggregation, the channel spacing between two shared-spectrum access component carriers in different sub-blocks shall be larger than the nominal channel spacing defined in this clause.

Mirror Cat-A CRs are submitted at this meeting to reflect these changes in Rel-17 and in Rel-18 [3,4].

Proposal 2: Adopt the following changes to TS 38.104 – see proposed Rel-16 CR [5].

- Add the following text to clause 5.4.1.1

For NR bands restricted to operation with shared-spectrum channel access,

▪	Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2 + {-40 kHz, -20 kHz, 0 kHz, 20 kHz, 40 kHz}

- Add the following text to clause 5.4.1.2

For NR bands restricted to operation with shared-spectrum channel access:

+ {0 kHz, 60 kHz, 120 kHz, 180kHz, 240kHz, 300kHz}

with n = µ0.

Mirror Cat-A CRs are submitted at this meeting to reflect these changes in Rel-17 and in Rel-18 [6,7].

# Conclusion

In this contribution, we observe that the NR-U nominal channel spacing for adjacent carriers and for intra-band contiguous CA operation may be exceeded for certain pairs of component carriers. For such cell configurations, the initial attach UE behavior may be unreliable, and the UE may fail its initial attach procedure.

Observation 1: For unlicensed bands:

- the deviations from the clause 5.4.1.1 adjacent carrier NCS never exceeds +- 40kHz and can be summarized as {-40 kHz, -20 kHz, 0 kHz, 20 kHz, 40 kHz}.

- the deviations from the clause 5.4A.1 intra-band contiguous NCS never exceeds 300kHz and can be summarized as + {0 kHz, 60 kHz, 120 kHz, 180kHz, 240kHz, 300kHz}

Observation 2: From clause 5.4A.1, some UEs designed to support intra-band contiguous CA operation, such as CA_n96B, may conclude that network configurations for which the configured channel spacing exceeds the nominal channel bandwidth are not valid configurations. This observation applies to both DL-CA configurations and to UL-CA configurations. The UE initial attach behavior may be unreliable, and the UE may fail the initial attach procedure.

To address these observations, we propose to adopt the Rel-16 changes of CR [2] for TS 38.101-1 and CR [5] for TS 38.104.

Rel-17 and Rel-18 mirror CRs are also presented at this meeting [3,4,6,7].

# References

[1] 	R4-2321677 On nominal channel spacing for NR-U, 3GPP TSG-RAN WG4 Meeting # 109, Chicago, USA, 13th November – 17th November, 2023, Skyworks Solutions, Inc.

[2]	R4-2400361 CR to TS 38.101-1 Rel-16 NR-U Nominal channel spacing, 3GPP TSG-RAN WG4 Meeting # 110, Athens, Greece, 26th February – 1st March, 2024, Skyworks Solutions, Inc., Nokia.

[3]	R4-2400360 CR to TS 38.101-1 Rel-17 NR-U Nominal channel spacing, 3GPP TSG-RAN WG4 Meeting # 110, Athens, Greece, 26th February – 1st March, 2024, Skyworks Solutions, Inc., Nokia.

[4] 	R4-2400359 CR to TS 38.101-1 Rel-18 NR-U Nominal channel spacing, 3GPP TSG-RAN WG4 Meeting # 110, Athens, Greece, 26th February – 1st March, 2024, Skyworks Solutions, Inc., Nokia.

[5]	R4-2402238 (NR_unlic_enh-Core) CR to TS 38.104 Rel-16 NR-U Nominal channel spacing, 3GPP TSG-RAN WG4 Meeting # 110, Athens, Greece, 26th February – 1st March, 2024, Nokia, Skyworks Solutions, Inc.

[6]	R4-2402239 (NR_unlic_enh-Core) CR to TS 38.104 Rel-17 NR-U Nominal channel spacing, 3GPP TSG-RAN WG4 Meeting # 110, Athens, Greece, 26th February – 1st March, 2024, Nokia, Skyworks Solutions, Inc.

[7]	R4-2402240 (NR_unlic_enh-Core) CR to TS 38.104 Rel-18 NR-U Nominal channel spacing, 3GPP TSG-RAN WG4 Meeting # 110, Athens, Greece, 26th February – 1st March, 2024, Nokia, Skyworks Solutions, Inc.

# Annex

NR-U deviations from the NR NCS, specified in clause 5.4.1.1 and 5.4A.1, are highlighted in light blue for band n46 and band n96, in Table 1 to Table 19.

Table 1: Analysis for band n96 20+20MHz CCs.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 20 | 797000 | 798332 | 5955.00 | 5974.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 798332 | 799668 | 5974.98 | 5995.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 799668 | 801000 | 5995.02 | 6015.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 801000 | 802332 | 6015.00 | 6034.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 802332 | 803668 | 6034.98 | 6055.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 803668 | 805000 | 6055.02 | 6075.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 805000 | 806332 | 6075.00 | 6094.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 806332 | 807668 | 6094.98 | 6115.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 807668 | 809000 | 6115.02 | 6135.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 809000 | 810332 | 6135.00 | 6154.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 810332 | 811668 | 6154.98 | 6175.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 811668 | 813000 | 6175.02 | 6195.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 813000 | 814332 | 6195.00 | 6214.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 814332 | 815668 | 6214.98 | 6235.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 815668 | 817000 | 6235.02 | 6255.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 817000 | 818332 | 6255.00 | 6274.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 818332 | 819668 | 6274.98 | 6295.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 819668 | 821000 | 6295.02 | 6315.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 821000 | 822332 | 6315.00 | 6334.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 822332 | 823668 | 6334.98 | 6355.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 823668 | 825000 | 6355.02 | 6375.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 825000 | 826332 | 6375.00 | 6394.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 826332 | 827668 | 6394.98 | 6415.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 827668 | 829000 | 6415.02 | 6435.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 829000 | 830332 | 6435.00 | 6454.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 830332 | 831668 | 6454.98 | 6475.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 831668 | 833000 | 6475.02 | 6495.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 833000 | 834332 | 6495.00 | 6514.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 830332 | 831668 | 6454.98 | 6475.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 831668 | 833000 | 6475.02 | 6495.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 833000 | 834332 | 6495.00 | 6514.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 830332 | 831668 | 6454.98 | 6475.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 831668 | 833000 | 6475.02 | 6495.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 833000 | 834332 | 6495.00 | 6514.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 830332 | 831668 | 6454.98 | 6475.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 831668 | 833000 | 6475.02 | 6495.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 833000 | 834332 | 6495.00 | 6514.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 830332 | 831668 | 6454.98 | 6475.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 831668 | 833000 | 6475.02 | 6495.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 833000 | 834332 | 6495.00 | 6514.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 830332 | 831668 | 6454.98 | 6475.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 831668 | 833000 | 6475.02 | 6495.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 833000 | 834332 | 6495.00 | 6514.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 830332 | 831668 | 6454.98 | 6475.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 831668 | 833000 | 6475.02 | 6495.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 833000 | 834332 | 6495.00 | 6514.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 830332 | 831668 | 6454.98 | 6475.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 831668 | 833000 | 6475.02 | 6495.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 833000 | 834332 | 6495.00 | 6514.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 830332 | 831668 | 6454.98 | 6475.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 831668 | 833000 | 6475.02 | 6495.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 833000 | 834332 | 6495.00 | 6514.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 830332 | 831668 | 6454.98 | 6475.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 831668 | 833000 | 6475.02 | 6495.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 833000 | 834332 | 6495.00 | 6514.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 830332 | 831668 | 6454.98 | 6475.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 831668 | 833000 | 6475.02 | 6495.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 833000 | 834332 | 6495.00 | 6514.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 830332 | 831668 | 6454.98 | 6475.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 831668 | 833000 | 6475.02 | 6495.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 833000 | 834332 | 6495.00 | 6514.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 834332 | 835668 | 6514.98 | 6535.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 835668 | 837000 | 6535.02 | 6555.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 837000 | 838332 | 6555.00 | 6574.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 838332 | 839668 | 6574.98 | 6595.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 839668 | 841000 | 6595.02 | 6615.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 841000 | 842332 | 6615.00 | 6634.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 842332 | 843668 | 6634.98 | 6655.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 843668 | 845000 | 6655.02 | 6675.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 845000 | 846332 | 6675.00 | 6694.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 846332 | 847668 | 6694.98 | 6715.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 847668 | 849000 | 6715.02 | 6735.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 849000 | 850332 | 6735.00 | 6754.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 850332 | 851668 | 6754.98 | 6775.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 851668 | 853000 | 6775.02 | 6795.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 853000 | 854332 | 6795.00 | 6814.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 854332 | 855668 | 6814.98 | 6835.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 855668 | 857000 | 6835.02 | 6855.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 857000 | 858332 | 6855.00 | 6874.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 858332 | 859668 | 6874.98 | 6895.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 859668 | 861000 | 6895.02 | 6915.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 861000 | 862332 | 6915.00 | 6934.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 862332 | 863668 | 6934.98 | 6955.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 863668 | 865000 | 6955.02 | 6975.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 865000 | 866332 | 6975.00 | 6994.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 866332 | 867668 | 6994.98 | 7015.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 867668 | 869000 | 7015.02 | 7035.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 869000 | 870332 | 7035.00 | 7054.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 870332 | 871668 | 7054.98 | 7075.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 871668 | 873000 | 7075.02 | 7095.00 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 873000 | 874332 | 7095.00 | 7114.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |

Table 2: Analysis for band n96 20+40MHz CCs.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 40 | 798332 | 800332 | 5974.98 | 6004.98 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 20 | 40 | 801000 | 803000 | 6015.00 | 6045.00 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 20 | 40 | 803668 | 805668 | 6055.02 | 6085.02 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 20 | 40 | 806332 | 808332 | 6094.98 | 6124.98 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 40 | 20 | 797668 | 799668 | 5965.02 | 5995.02 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 40 | 20 | 800332 | 802332 | 6004.98 | 6034.98 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 40 | 20 | 803000 | 805000 | 6045.00 | 6075.00 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 20 | 40 | 803668 | 805668 | 6055.02 | 6085.02 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 20 | 40 | 870332 | 872332 | 7054.98 | 7084.98 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 40 | 20 | 872332 | 874332 | 7084.98 | 7114.98 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |

Table 3: Analysis for band n96 20+60MHz CCs.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 20 | 798332 | 801000 | 5974.98 | 6015.00 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 60 | 20 | 799668 | 802332 | 5995.02 | 6034.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.78 | 180 | -40 |
| 60 | 20 | 803668 | 806332 | 6055.02 | 6094.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.78 | 180 | -40 |
| 60 | 20 | 805000 | 807668 | 6075.00 | 6115.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 20 | 60 | 797000 | 799668 | 5955.00 | 5995.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 20 | 60 | 801000 | 803668 | 6015.00 | 6055.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 20 | 60 | 802332 | 805000 | 6034.98 | 6075.00 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 20 | 60 | 865000 | 867668 | 6975.00 | 7015.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 20 | 60 | 866332 | 869000 | 6994.98 | 7035.00 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 20 | 60 | 870332 | 873000 | 7054.98 | 7095.00 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 60 | 20 | 863668 | 866332 | 6955.02 | 6994.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.78 | 180 | -40 |
| 60 | 20 | 867668 | 870332 | 7015.02 | 7054.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.78 | 180 | -40 |
| 60 | 20 | 869000 | 871668 | 7035.00 | 7075.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |

Table 4: Analysis for band n96 20+80MHz CCs.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 80 | 801000 | 804332 | 6015.00 | 6064.98 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.86 | 120 | -20 |
| 20 | 80 | 806332 | 809668 | 6094.98 | 6145.02 | 3336 | 50.04 | 50 MHz + {-5, 0, 5} | 49.86 | 180 | 40 |
| 20 | 80 | 811668 | 815000 | 6175.02 | 6225.00 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.86 | 120 | -20 |
| 20 | 80 | 817000 | 820332 | 6255.00 | 6304.98 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.86 | 120 | -20 |
| 20 | 80 | 822332 | 825668 | 6334.98 | 6385.02 | 3336 | 50.04 | 50 MHz + {-5, 0, 5} | 49.86 | 180 | 40 |
| 80 | 20 | 799000 | 802332 | 5985.00 | 6034.98 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.86 | 120 | -20 |
| 80 | 20 | 804332 | 807668 | 6064.98 | 6115.02 | 3336 | 50.04 | 50 MHz + {-5, 0, 5} | 49.86 | 180 | 40 |
| 80 | 20 | 809668 | 813000 | 6145.02 | 6195.00 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.86 | 120 | -20 |
| 80 | 20 | 815000 | 818332 | 6225.00 | 6274.98 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.86 | 120 | -20 |
| 80 | 20 | 820332 | 823668 | 6304.98 | 6355.02 | 3336 | 50.04 | 50 MHz + {-5, 0, 5} | 49.86 | 180 | 40 |

Table 5: Analysis for band n96 20+100MHz CCs.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 100 | 799668 | 803668 | 5995.02 | 6055.02 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 59.94 | 60 | 0 |
| 20 | 100 | 806332 | 810332 | 6094.98 | 6154.98 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 59.94 | 60 | 0 |
| 20 | 100 | 810332 | 814332 | 6154.98 | 6214.98 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 59.94 | 60 | 0 |
| 100 | 20 | 799668 | 803668 | 5995.02 | 6055.02 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 59.94 | 60 | 0 |
| 100 | 20 | 803668 | 807668 | 6055.02 | 6115.02 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 59.94 | 60 | 0 |
| 100 | 20 | 871668 | 867668 | 7075.02 | 7015.02 | 4000 | -60 | 60 MHz + {-5, 0, 5} | 59.94 | 60 | 0 |
| 100 | 20 | 870332 | 866332 | 7054.98 | 6994.98 | 4000 | -60 | 60 MHz + {-5, 0, 5} | 59.94 | 60 | 0 |

Table 6: Analysis for band n96 40+40MHz CCs.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | 40 | 797668 | 800332 | 5965.02 | 6004.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.96 | 0 | -40 |
| 40 | 40 | 800332 | 803000 | 6004.98 | 6045.00 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.96 | 60 | 20 |
| 40 | 40 | 803000 | 805668 | 6045.00 | 6085.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.96 | 60 | 20 |
| 40 | 40 | 805668 | 808332 | 6085.02 | 6124.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.96 | 0 | -40 |
| 40 | 40 | 808332 | 811000 | 6124.98 | 6165.00 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.96 | 60 | 20 |
| 40 | 40 | 811000 | 813668 | 6165.00 | 6205.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.96 | 60 | 20 |
| 40 | 40 | 813668 | 816332 | 6205.02 | 6244.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.96 | 0 | -40 |
| 40 | 40 | 816332 | 819000 | 6244.98 | 6285.00 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.96 | 60 | 20 |
| 40 | 40 | 819000 | 821668 | 6285.00 | 6325.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.96 | 60 | 20 |
| 40 | 40 | 821668 | 824332 | 6325.02 | 6364.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.96 | 0 | -40 |
| 40 | 40 | 824332 | 827000 | 6364.98 | 6405.00 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.96 | 60 | 20 |
| 40 | 40 | 827000 | 829668 | 6405.00 | 6445.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.96 | 60 | 20 |
| 40 | 40 | 829668 | 832332 | 6445.02 | 6484.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.96 | 0 | -40 |
| 40 | 40 | 832332 | 835000 | 6484.98 | 6525.00 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.96 | 60 | 20 |
| 40 | 40 | 835000 | 837668 | 6525.00 | 6565.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.96 | 60 | 20 |
| 40 | 40 | 837668 | 840332 | 6565.02 | 6604.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.96 | 0 | -40 |
| 40 | 40 | 840332 | 843000 | 6604.98 | 6645.00 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.96 | 60 | 20 |
| 40 | 40 | 843000 | 845668 | 6645.00 | 6685.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.96 | 60 | 20 |
| 40 | 40 | 845668 | 848332 | 6685.02 | 6724.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.96 | 0 | -40 |
| 40 | 40 | 848332 | 851000 | 6724.98 | 6765.00 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.96 | 60 | 20 |
| 40 | 40 | 851000 | 853668 | 6765.00 | 6805.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.96 | 60 | 20 |
| 40 | 40 | 853668 | 856332 | 6805.02 | 6844.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.96 | 0 | -40 |
| 40 | 40 | 856332 | 859000 | 6844.98 | 6885.00 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.96 | 60 | 20 |
| 40 | 40 | 859000 | 861668 | 6885.00 | 6925.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.96 | 60 | 20 |
| 40 | 40 | 861668 | 864332 | 6925.02 | 6964.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.96 | 0 | -40 |
| 40 | 40 | 864332 | 867000 | 6964.98 | 7005.00 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.96 | 60 | 20 |
| 40 | 40 | 867000 | 869668 | 7005.00 | 7045.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.96 | 60 | 20 |
| 40 | 40 | 869668 | 872332 | 7045.02 | 7084.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.96 | 0 | -40 |

Table 7: Analysis for band n96 40+60MHz CCs.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 40 | 799668 | 803000 | 5995.02 | 6045.00 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.92 | 60 | -20 |
| 40 | 60 | 800332 | 803668 | 6004.98 | 6055.02 | 3336 | 50.04 | 50 MHz + {-5, 0, 5} | 49.92 | 120 | 40 |
| 60 | 40 | 805000 | 808332 | 6075.00 | 6124.98 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.92 | 60 | -20 |
| 40 | 60 | 805668 | 809000 | 6085.02 | 6135.00 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.92 | 60 | -20 |
| 60 | 40 | 869000 | 872332 | 7035.00 | 7084.98 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.92 | 60 | -20 |
| 40 | 60 | 869668 | 873000 | 7045.02 | 7095.00 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.92 | 60 | -20 |
| 60 | 40 | 863668 | 867000 | 6955.02 | 7005.00 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.92 | 60 | -20 |
| 40 | 60 | 864332 | 867668 | 6964.98 | 7015.02 | 3336 | 50.04 | 50 MHz + {-5, 0, 5} | 49.92 | 120 | 40 |
| 40 | 60 | 859000 | 862332 | 6885.00 | 6934.98 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.92 | 60 | -20 |
| 60 | 40 | 858332 | 861668 | 6874.98 | 6925.02 | 3336 | 50.04 | 50 MHz + {-5, 0, 5} | 49.92 | 120 | 40 |

Table 8: Analysis for band n96 40+80MHz CCs.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | 80 | 800332 | 804332 | 6004.98 | 6064.98 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 59.82 | 180 | 0 |
| 40 | 80 | 805668 | 809668 | 6085.02 | 6145.02 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 59.82 | 180 | 0 |
| 40 | 80 | 811000 | 815000 | 6165 | 6225 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 59.82 | 180 | 0 |
| 40 | 80 | 816332 | 820332 | 6244.98 | 6304.98 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 59.82 | 180 | 0 |
| 40 | 80 | 821668 | 825668 | 6325.02 | 6385.02 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 59.82 | 180 | 0 |
| 80 | 40 | 799000 | 803000 | 5985 | 6045 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 59.82 | 180 | 0 |
| 80 | 40 | 804332 | 808332 | 6064.98 | 6124.98 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 59.82 | 180 | 0 |
| 80 | 40 | 809668 | 813668 | 6145.02 | 6205.02 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 59.82 | 180 | 0 |
| 80 | 40 | 815000 | 819000 | 6225 | 6285 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 59.82 | 180 | 0 |
| 80 | 40 | 868332 | 872332 | 7024.98 | 7084.98 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 59.82 | 180 | 0 |

Table 9: Analysis for band n96 40+100MHz CCs.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | 100 | 805668 | 810332 | 6085.02 | 6154.98 | 4664 | 69.96 | 70 MHz + {-5, 0, 5} | 69.72 | 240 | -40 |
| 100 | 40 | 803668 | 808332 | 6055.02 | 6124.98 | 4664 | 69.96 | 70 MHz + {-5, 0, 5} | 69.72 | 240 | -40 |
| 100 | 40 | 814332 | 819000 | 6214.98 | 6285.00 | 4668 | 70.02 | 70 MHz + {-5, 0, 5} | 69.72 | 300 | 20 |
| 40 | 100 | 816332 | 821000 | 6244.98 | 6315.00 | 4668 | 70.02 | 70 MHz + {-5, 0, 5} | 69.72 | 300 | 20 |
| 100 | 40 | 825000 | 829668 | 6375.00 | 6445.02 | 4668 | 70.02 | 70 MHz + {-5, 0, 5} | 69.72 | 300 | 20 |
| 40 | 100 | 827000 | 831668 | 6405.00 | 6475.02 | 4668 | 70.02 | 70 MHz + {-5, 0, 5} | 69.72 | 300 | 20 |
| 40 | 100 | 859000 | 863668 | 6885.00 | 6955.02 | 4668 | 70.02 | 70 MHz + {-5, 0, 5} | 69.72 | 300 | 20 |

Table 10: Analysis for band n96 60+60MHz CCs.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 60 | 799668 | 803668 | 5995.02 | 6055.02 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 60 | 0 | 0 |
| 60 | 60 | 805000 | 809000 | 6075 | 6135 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 60 | 0 | 0 |
| 60 | 60 | 810332 | 814332 | 6154.98 | 6214.98 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 60 | 0 | 0 |
| 60 | 60 | 815668 | 819668 | 6235.02 | 6295.02 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 60 | 0 | 0 |
| 60 | 60 | 821000 | 825000 | 6315.00 | 6375.00 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 60 | 0 | 0 |
| 60 | 60 | 826332 | 830332 | 6394.98 | 6454.98 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 60 | 0 | 0 |
| 60 | 60 | 869000 | 873000 | 7035.00 | 7095.00 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 60 | 0 | 0 |

Table 11: Analysis for band n96 60+80MHz CCs.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 80 | 799668 | 804332 | 5995.02 | 6064.98 | 4664 | 69.96 | 70 MHz + {-5, 0, 5} | 69.9 | 60 | -40 |
| 80 | 60 | 799000 | 803668 | 5985 | 6055.02 | 4668 | 70.02 | 70 MHz + {-5, 0, 5} | 69.9 | 120 | 20 |
| 60 | 80 | 805000 | 809668 | 6075 | 6145.02 | 4668 | 70.02 | 70 MHz + {-5, 0, 5} | 69.9 | 120 | 20 |
| 80 | 60 | 804332 | 809000 | 6064.98 | 6135 | 4668 | 70.02 | 70 MHz + {-5, 0, 5} | 69.9 | 120 | 20 |
| 60 | 80 | 810332 | 815000 | 6154.98 | 6225 | 4668 | 70.02 | 70 MHz + {-5, 0, 5} | 69.9 | 120 | 20 |
| 80 | 60 | 809668 | 814332 | 6145.02 | 6214.98 | 4664 | 69.96 | 70 MHz + {-5, 0, 5} | 69.9 | 60 | -40 |
| 80 | 60 | 868332 | 873000 | 7024.98 | 7095 | 4668 | 70.02 | 70 MHz + {-5, 0, 5} | 69.9 | 120 | 20 |
| 60 | 80 | 863668 | 868332 | 6955.02 | 7024.98 | 4664 | 69.96 | 70 MHz + {-5, 0, 5} | 69.9 | 60 | -40 |
| 80 | 60 | 863000 | 867668 | 6945 | 7015.02 | 4668 | 70.02 | 70 MHz + {-5, 0, 5} | 69.9 | 120 | 20 |

Table 11: Analysis for band n96 60+100MHz CCs.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 100 | 805000 | 810332 | 6075 | 6154.98 | 5332 | 79.98 | 80 MHz + {-5, 0, 5} | 79.8 | 180 | -20 |
| 100 | 60 | 799668 | 805000 | 5995.02 | 6075 | 5332 | 79.98 | 80 MHz + {-5, 0, 5} | 79.8 | 180 | -20 |
| 100 | 60 | 810332 | 815668 | 6154.98 | 6235.02 | 5336 | 80.04 | 80 MHz + {-5, 0, 5} | 79.8 | 240 | 40 |
| 100 | 60 | 814332 | 819668 | 6214.98 | 6295.02 | 5336 | 80.04 | 80 MHz + {-5, 0, 5} | 79.8 | 240 | 40 |
| 60 | 100 | 815668 | 821000 | 6235.02 | 6315 | 5332 | 79.98 | 80 MHz + {-5, 0, 5} | 79.8 | 180 | -20 |
| 60 | 100 | 826332 | 831668 | 6394.98 | 6475.02 | 5336 | 80.04 | 80 MHz + {-5, 0, 5} | 79.8 | 240 | 40 |
| 100 | 60 | 825000 | 830332 | 6375 | 6454.98 | 5332 | 79.98 | 80 MHz + {-5, 0, 5} | 79.8 | 180 | -20 |
| 100 | 60 | 867668 | 873000 | 7015.02 | 7095 | 5332 | 79.98 | 80 MHz + {-5, 0, 5} | 79.8 | 180 | -20 |
| 100 | 60 | 863668 | 869000 | 6955.02 | 7035 | 5332 | 79.98 | 80 MHz + {-5, 0, 5} | 79.8 | 180 | -20 |
| 60 | 100 | 862332 | 867668 | 6934.98 | 7015.02 | 5336 | 80.04 | 80 MHz + {-5, 0, 5} | 79.8 | 240 | 40 |

Table 13: Analysis for band n96 80+80MHz CCs.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 80 | 80 | 799000 | 804332 | 5985 | 6064.98 | 5332 | 79.98 | 80 MHz + {-5, 0, 5} | 79.98 | 0 | -20 |
| 80 | 80 | 804332 | 809668 | 6064.98 | 6145.02 | 5336 | 80.04 | 80 MHz + {-5, 0, 5} | 79.98 | 60 | 40 |
| 80 | 80 | 809668 | 815000 | 6145.02 | 6225 | 5332 | 79.98 | 80 MHz + {-5, 0, 5} | 79.98 | 0 | -20 |
| 80 | 80 | 815000 | 820332 | 6225 | 6304.98 | 5332 | 79.98 | 80 MHz + {-5, 0, 5} | 79.98 | 0 | -20 |
| 80 | 80 | 820332 | 825668 | 6304.98 | 6385.02 | 5336 | 80.04 | 80 MHz + {-5, 0, 5} | 79.98 | 60 | 40 |
| 80 | 80 | 825668 | 831000 | 6385.02 | 6465 | 5332 | 79.98 | 80 MHz + {-5, 0, 5} | 79.98 | 0 | -20 |
| 80 | 80 | 831000 | 836332 | 6465 | 6544.98 | 5332 | 79.98 | 80 MHz + {-5, 0, 5} | 79.98 | 0 | -20 |
| 80 | 80 | 836332 | 841668 | 6544.98 | 6625.02 | 5336 | 80.04 | 80 MHz + {-5, 0, 5} | 79.98 | 60 | 40 |
| 80 | 80 | 841668 | 847000 | 6625.02 | 6705 | 5332 | 79.98 | 80 MHz + {-5, 0, 5} | 79.98 | 0 | -20 |
| 80 | 80 | 847000 | 852332 | 6705 | 6784.98 | 5332 | 79.98 | 80 MHz + {-5, 0, 5} | 79.98 | 0 | -20 |
| 80 | 80 | 852332 | 857668 | 6784.98 | 6865.02 | 5336 | 80.04 | 80 MHz + {-5, 0, 5} | 79.98 | 60 | 40 |
| 80 | 80 | 857668 | 863000 | 6865.02 | 6945 | 5332 | 79.98 | 80 MHz + {-5, 0, 5} | 79.98 | 0 | -20 |
| 80 | 80 | 863000 | 868332 | 6945 | 7024.98 | 5332 | 79.98 | 80 MHz + {-5, 0, 5} | 79.98 | 0 | -20 |

Table 14: Analysis for band n96 80+100MHz CCs.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 80 | 100 | 804332 | 810332 | 6064.98 | 6154.98 | 6000 | 90 | 90 MHz + {-5, 0, 5} | 89.88 | 120 | 0 |
| 100 | 80 | 803668 | 809668 | 6055.02 | 6145.02 | 6000 | 90 | 90 MHz + {-5, 0, 5} | 89.88 | 120 | 0 |
| 80 | 100 | 815000 | 821000 | 6225 | 6315 | 6000 | 90 | 90 MHz + {-5, 0, 5} | 89.88 | 120 | 0 |
| 100 | 80 | 814332 | 820332 | 6214.98 | 6304.98 | 6000 | 90 | 90 MHz + {-5, 0, 5} | 89.88 | 120 | 0 |
| 80 | 100 | 825668 | 831668 | 6385.02 | 6475.02 | 6000 | 90 | 90 MHz + {-5, 0, 5} | 89.88 | 120 | 0 |
| 100 | 80 | 825000 | 831000 | 6375 | 6465 | 6000 | 90 | 90 MHz + {-5, 0, 5} | 89.88 | 120 | 0 |
| 80 | 100 | 857668 | 863668 | 6865.02 | 6955.02 | 6000 | 90 | 90 MHz + {-5, 0, 5} | 89.88 | 120 | 0 |
| 100 | 80 | 857000 | 863000 | 6855 | 6945 | 6000 | 90 | 90 MHz + {-5, 0, 5} | 89.88 | 120 | 0 |

Table 15: Analysis for band n46 20+20MHz CCs.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 20 | 745332 | 746668 | 5179.98 | 5200.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 746668 | 748000 | 5200.02 | 5220 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 748000 | 749332 | 5220 | 5239.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 749332 | 750668 | 5239.98 | 5260.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 750668 | 752000 | 5260.02 | 5280 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 752000 | 753332 | 5280 | 5299.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 753332 | 754668 | 5299.98 | 5320.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 754668 | 756000 | 5320.02 | 5340 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 756000 | 765332 | 5340 | 5480 | 9332 | 139.98 | 20 MHz + {-5, 0, 5} | 19.98 | UNII-2B gap | UNII-2B gap |
| 20 | 20 | 765332 | 766668 | 5479.98 | 5500.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 766668 | 768000 | 5500.02 | 5520 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 768000 | 769332 | 5520 | 5539.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 769332 | 770668 | 5539.98 | 5560.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 770668 | 772000 | 5560.02 | 5580 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 772000 | 773332 | 5580 | 5599.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 773332 | 774668 | 5599.98 | 5620.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 774668 | 776000 | 5620.02 | 5640 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 776000 | 777332 | 5640 | 5659.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 777332 | 778668 | 5659.98 | 5680.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 778668 | 780000 | 5680.02 | 5700 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 780000 | 781332 | 5700 | 5719.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 781332 | 783000 | 5720 | 5745 | 1668 | 25.02 | 20 MHz + {-5, 0, 5} | 19.98 | UNII-2ext. gap | UNII-2ext. gap |
| 20 | 20 | 783000 | 784332 | 5745 | 5764.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 784332 | 785668 | 5764.98 | 5785.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 785668 | 787000 | 5785.02 | 5805 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 787000 | 788332 | 5805 | 5824.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 788332 | 789668 | 5824.98 | 5845.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |
| 20 | 20 | 789668 | 791000 | 5845.02 | 5865 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 791000 | 792332 | 5865 | 5884.98 | 1332 | 19.98 | 20 MHz + {-5, 0, 5} | 19.98 | 0 | -20 |
| 20 | 20 | 792332 | 793668 | 5884.98 | 5905.02 | 1336 | 20.04 | 20 MHz + {-5, 0, 5} | 19.98 | 60 | 40 |

Table 16: Analysis for band n46 20+40MHz CCs.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 40 | 744000 | 746000 | 5160 | 5190 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 20 | 40 | 746668 | 748668 | 5200.02 | 5230.02 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 20 | 40 | 749332 | 751332 | 5239.98 | 5269.98 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 20 | 40 | 752000 | 754000 | 5280 | 5310 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 20 | 40 | 753332 | 755332 | 5299.98 | 5329.98 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 20 | 40 | 765332 | 767332 | 5479.98 | 5509.98 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 20 | 40 | 768000 | 770000 | 5520 | 5550 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 20 | 40 | 770668 | 772668 | 5560.02 | 5590.02 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 20 | 40 | 776000 | 778000 | 5640 | 5670 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 20 | 40 | 778668 | 780668 | 5680.02 | 5710.02 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 20 | 40 | 784332 | 786332 | 5764.98 | 5794.98 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 20 | 40 | 785668 | 787668 | 5785.02 | 5815.02 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 20 | 40 | 788332 | 790332 | 5824.98 | 5854.98 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 20 | 40 | 791000 | 793000 | 5865 | 5895 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 40 | 20 | 744668 | 746668 | 5170.02 | 5200.02 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 40 | 20 | 746000 | 748000 | 5190 | 5220 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 40 | 20 | 748668 | 750668 | 5230.02 | 5260.02 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 40 | 20 | 751332 | 753332 | 5269.98 | 5299.98 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 40 | 20 | 754000 | 756000 | 5310 | 5340 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 40 | 20 | 766000 | 768000 | 5490 | 5520 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 40 | 20 | 767332 | 769332 | 5509.98 | 5539.98 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 40 | 20 | 770000 | 772000 | 5550 | 5580 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 40 | 20 | 772668 | 774668 | 5590.02 | 5620.02 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 40 | 20 | 775332 | 777332 | 5629.98 | 5659.98 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 40 | 20 | 778000 | 780000 | 5670 | 5700 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 40 | 20 | 783668 | 785668 | 5755.02 | 5785.02 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 40 | 20 | 786332 | 788332 | 5794.98 | 5824.98 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |
| 40 | 20 | 787668 | 789668 | 5815.02 | 5845.02 | 2000 | 30 | 30 MHz + {-5, 0, 5} | 29.7 | 300 | 0 |

Table 17: Analysis for band n46 20+60MHz CCs.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 60 | 744000 | 746668 | 5160 | 5200.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 20 | 60 | 745332 | 748000 | 5179.98 | 5220 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 20 | 60 | 749332 | 752000 | 5239.98 | 5280 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 20 | 60 | 750668 | 753332 | 5260.02 | 5299.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.78 | 180 | -40 |
| 20 | 60 | 752000 | 754668 | 5280 | 5320.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 20 | 60 | 765332 | 768000 | 5479.98 | 5520 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 20 | 60 | 766668 | 769332 | 5500.02 | 5539.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.78 | 180 | -40 |
| 20 | 60 | 770668 | 773332 | 5560.02 | 5599.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.78 | 180 | -40 |
| 20 | 60 | 772000 | 774668 | 5580 | 5620.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 20 | 60 | 776000 | 778668 | 5640 | 5680.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 20 | 60 | 777332 | 780000 | 5659.98 | 5700 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 20 | 60 | 783000 | 785668 | 5745 | 5785.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 20 | 60 | 788332 | 791000 | 5824.98 | 5865 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 20 | 60 | 789668 | 792332 | 5845.02 | 5884.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.78 | 180 | -40 |
| 60 | 20 | 745332 | 748000 | 5179.98 | 5220 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 60 | 20 | 746668 | 749332 | 5200.02 | 5239.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.78 | 180 | -40 |
| 60 | 20 | 748000 | 750668 | 5220 | 5260.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 60 | 20 | 752000 | 754668 | 5280 | 5320.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 60 | 20 | 753332 | 756000 | 5299.98 | 5340 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 60 | 20 | 766668 | 769332 | 5500.02 | 5539.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.78 | 180 | -40 |
| 60 | 20 | 768000 | 770668 | 5520 | 5560.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 60 | 20 | 769332 | 772000 | 5539.98 | 5580 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 60 | 20 | 773332 | 776000 | 5599.98 | 5640 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 60 | 20 | 774668 | 777332 | 5620.02 | 5659.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.78 | 180 | -40 |
| 60 | 20 | 778668 | 781332 | 5680.02 | 5719.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.78 | 180 | -40 |
| 60 | 20 | 784332 | 787000 | 5764.98 | 5805 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |
| 60 | 20 | 785668 | 788332 | 5785.02 | 5824.98 | 2664 | 39.96 | 40 MHz + {-5, 0, 5} | 39.78 | 180 | -40 |
| 60 | 20 | 791000 | 793668 | 5865 | 5905.02 | 2668 | 40.02 | 40 MHz + {-5, 0, 5} | 39.78 | 240 | 20 |

Table 18: Analysis for band n46 20+80MHz CCs.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 80 | 744000 | 747332 | 5160 | 5209.98 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.86 | 120 | -20 |
| 20 | 80 | 749332 | 752668 | 5239.98 | 5290.02 | 3336 | 50.04 | 50 MHz + {-5, 0, 5} | 49.86 | 180 | 40 |
| 20 | 80 | 765332 | 768668 | 5479.98 | 5530.02 | 3336 | 50.04 | 50 MHz + {-5, 0, 5} | 49.86 | 180 | 40 |
| 20 | 80 | 770668 | 774000 | 5560.02 | 5610 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.86 | 120 | -20 |
| 20 | 80 | 776000 | 779332 | 5640 | 5689.98 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.86 | 120 | -20 |
| 20 | 80 | 788332 | 791668 | 5824.98 | 5875.02 | 3336 | 50.04 | 50 MHz + {-5, 0, 5} | 49.86 | 180 | 40 |
| 80 | 20 | 746000 | 749332 | 5190 | 5239.98 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.86 | 120 | -20 |
| 80 | 20 | 747332 | 750668 | 5209.98 | 5260.02 | 3336 | 50.04 | 50 MHz + {-5, 0, 5} | 49.86 | 180 | 40 |
| 80 | 20 | 752668 | 756000 | 5290.02 | 5340 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.86 | 120 | -20 |
| 80 | 20 | 767332 | 770668 | 5509.98 | 5560.02 | 3336 | 50.04 | 50 MHz + {-5, 0, 5} | 49.86 | 180 | 40 |
| 80 | 20 | 768668 | 772000 | 5530.02 | 5580 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.86 | 120 | -20 |
| 80 | 20 | 774000 | 777332 | 5610 | 5659.98 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.86 | 120 | -20 |
| 80 | 20 | 785000 | 788332 | 5775 | 5824.98 | 3332 | 49.98 | 50 MHz + {-5, 0, 5} | 49.86 | 120 | -20 |

Table 19: Analysis for band n46 20+100MHz CCs.

| PCC CBW [MHz] | SCC CBW [MHz] | PCC NR-ARFCN | SCC NR-ARFCN | PCC Fc [MHz] | SCC Fc [MHz] | Delta NR-ARFCN | Configured Channel Spacing [MHz] | Adjacent Carrier (AC) NCS [kHz] 5.4.1.1 | IB-CA NCS [MHz] 5.4A.1 | Deviation from IB-CA NCS [kHz] | Deviation from "nominal" AC-NCS [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 100 | 749332 | 753332 | 5239.98 | 5299.98 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 59.94 | 60 | 0 |
| 20 | 100 | 787000 | 791000 | 5805 | 5865 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 59.94 | 60 | 0 |
| 100 | 20 | 746668 | 750668 | 5200.02 | 5260.02 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 59.94 | 60 | 0 |
| 100 | 20 | 768000 | 772000 | 5520 | 5580 | 4000 | 60 | 60 MHz + {-5, 0, 5} | 59.94 | 60 | 0 |