3GPP TSG-RAN WG4 Meeting #110 						                     R4-2400393

Athens, Greece, February 26 – March 1, 2024

Agenda item:	8.16.1.3

Source:	Apple

Title:	Discussion on improvement on FR2 SCell/SCG setup delay

Document for:	Discussion

# Introduction

Improvement on FR2 SCell/SCG setup delay have been discussed widely for several meetings. The latest agreement can be found in the approved WF [1]. Besides, it was also discussed in the previous RANP#102. In this contribution, we will continue discussing the remaining issues.

# Discussion

RAN4 core requirements for improvement on FR2 SCell/SCG setup delay were introduced in RAN4#109 [1]. However, RAN2 signaling support for this objective hasn’t been finalized. In RANP#102, it was agreed to extend the WI with only focus on solution based on existing measurement [2]:

Task(s) within work which are not complete:

WI objective#7, focus on solution based on existing measurement, as below:

- RAN2 to define time-based measurement result validation configuration based on RAN4 agreements.

- RAN2 signaling to enable reporting of cell reselection measurement or EMR for fast CA/DC setup.

- NOTE 1: RAN4 shall not work on any new requirements for this functionality in Rel-18. Only essential corrections are allowed.

- NOTE 2: If RAN2 is not able to complete the work, the functionality will be removed from Rel-18.

- NOTE 3: Existing measurement means that no additional measurement is performed during RRC Setup/Resume procedure.

In RAN4, the remaining issue is to discuss and agree on UE feature. Since solution based on enhanced measurement is not going to be supported, RAN4 only needs to discuss the new UE capability for solution based on existing measurement.

Proposal 1: UE feature list for improvement on SCell/SCG setup/resume delay:

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [39-X1] | Improvement on SCG/SCell setup/resume delay | Support of solution based on existing measurement to reduce SCG/SCell setup/resume delay. | N/A | Yes | No | Network doesn’t know if UE can support validity check upon RRC connection setup/resume procedure. | Per UE | No | No | N/A |  | Optional with capability signalling |

# Conclusion

In this contribution, we provide further discussion on improvement on FR2 SCell/SCG setup delay. After discussion, the following conclusions are provided:

Proposal 1: UE feature list for improvement on SCell/SCG setup/resume delay:

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [39-X1] | Improvement on SCG/SCell setup/resume delay | Support of solution based on existing measurement to reduce SCG/SCell setup/resume delay. | N/A | Yes | No | Network doesn’t know if UE can support validity check upon RRC connection setup/resume procedure. | Per UE | No | No | N/A |  | Optional with capability signalling |

# References

- R4-2321641, Big CR to TS 38.133 on Further NR mobility enhancement, MediaTek Inc., Apple

- RP-233969, Rel-18 Work Item Exception for Further NR Mobility Enhancements, Rapporteur (MediaTek Inc., Apple)

- R4-2317328, WF on R18 Further NR mobility enhancement – Improvement on SCell/SCG setup delay and Enhanced CHO, Apple

- R4-2314332, WF on NR Mobility Enhancements (Part 2), Apple

- R4-2310064, WF on NR Mobility Enhancements RRM requirements (part 2), Apple

- R4-2306395, WF on NR Mobility Enhancements RRM requirements (part 2), Apple

- R4-2220415, WF on improvement on FR2 SCell/SCG setup/resume, Apple, MediaTek Inc.