3GPP TSG-RAN WG4 Meeting #110                                                             R4-2400420

# Athens, Greece, February 26 – March 1, 2024

Agenda item:	8.26.4

Source:	Apple Inc.

Title:	Discussion on UE demodulation performance and CSI requirements for Network Energy Savings for NR

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

Network energy saving is of great importance for environmental sustainability, to reduce environmental impact (greenhouse gas emissions), and for operational cost savings. As 5G is becoming pervasive across industries and geographical areas, handling more advanced services and applications requiring very high data rates (e.g. XR), networks are being denser, use more antennas, larger bandwidths and more frequency bands. The environmental impact of 5G needs to stay under control, and novel solutions to improve network energy savings need to be developed. Energy consumption has become a key part of the operators’ OPEX, ~23% of the total operator cost, most of it coming from the RAN and in particular from the Active Antenna Unit (AAU).

Based on the agreed BS energy consumption model, and the evaluation methodology and assumptions, potential network energy saving techniques in various domains were evaluated with respect to the energy saving gains and the corresponding performance impact considering the above KPIs. The studied techniques are classified into time, frequency, spatial and power domains, such as:

- Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2]

- Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2]

- Note: Above objectives are only for UE specific channels/signals

- Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements

# 2. Discussion

The focus of this Demod WI is to analyze and then specify demodulation performance and CSI reporting requirements for Network Energy Savings, if any is needed. Based on the previous way forward [1], we base our discussion on the following open issues:

Issue 1-1: Whether to introduce SSB-less Scell requirements for inter-band CA

- Way forward

  - Option 1: RAN4 shall evaluate the performance of SSB-less SCells CA performance with Rel-15 compliant demodulator implementation (without TRS based TOE) and decide, if the observed performance constitutes a practical operating point of the feature, or if new requirements capturing improved demodulator implementations are needed, or if SSB-less SCell compliant UE shall be tested with normal CA requirements.

  - Option 2: Define the SSB less Scell requirements for inter-band CA by reusing the existing minimum PDSCH CA demodulation requirements with RTD=[0]us

    - Apply the same requirements as the minimum CA demodulation requirements, TS38.101-4 5.2A.2.1 for 2Rx UE and 5.2A.3.1 for 4Rx UE.

    - Add a new test parameter table for SCell where SSB is not configured:

Table 5.2A.x.y-1: Additional test parameters for SCell Parameter; Unit; Value. Common serving cell parameters; Physical Cell ID; Same as PSell. SSB position in burst; Not configured. SSB periodicity; ms; N/A.

    - Applicable only for inter-band CA in FR1

  - Option 3: Not to introduce the requirement.

During the RAN4#108b meeting, concerns have been brought on the demodulation performance in the SSB-less SCell [2]. More specifically, they are interested in 1) Demodulation performance in multipath channels with RTD/time offset close to CP, and in 2) TRS based time (& frequency) synchronization performance, when used initial time offset and frequency offset estimates from PCell are wrong on the order of CP.

From Apple’s point of view, we believe there is no reason for the UE to not keep the same performance relying on TRS for frequency and timing compensation with the coarse timing information obtained from the other CC. From the RRM point of view, the requirement that RTD is less than CP is the working assumption, and this hasn’t changed. Since for inter-band CA the UE will operate with two separate RF chains anyways, we don’t expect RTD to be a problem.

Observation 1: In inter-band SSB-less operation, the UE is expected to operate with two separate RF chains, hence separate tracking loops and FFT blocks will exist.

In addition, the main argument relies in the selection of a rather extreme channel condition such as TDLC300-100.  However, from Apple’s point of view and other companies, any considered “performance degradation” is simply the results of the selected channel and not of a problem with the implemented signal processing in the UE. From the UE point of view, the tracking loop algorithms will not change. In addition, in most of the reasonable propagation scenarios the RTD is small enough that the UE can lock onto TRS where performance will be optimal.

Observation 2: Current test proposal relies on a single scenario based on TDLC300-100 channel, which is not a common operating scenario but rather a corner case. In most scenarios it is expected that RTD will be minimal via BS handling this in collocated scenarios.

Observation 3: CA requirements are mostly functional tests set under TDLA30-10. More stringent channel conditions are usually reserved for single-carrier requirements.

Observation 4: Release 19 already considers the SSB on demand based on, at least, cell load as an input parameter, which will dilute any performance concern in Release 18.

Observation 5: There is an argument that there are better TRS estimation algorithms that can be implemented. However, this WI is not about proposing new algorithms for TRS-based estimation.

Proposal 1: Do not introduce new requirements for CA in SSB-less scenarios.

Issue 2-1: Whether to introduce CSI requirements for power/spatial domain adaptation

- Way forward

  - Option 1: Yes, RAN4 to introduce:

    - CQI requirements under AWGN channel for power domain adaption to check CQI performance with multiple CPUs and if UE can calculate CQI for each power offset correctly.

    - PMI requirements for type1 and type2 spatial domain adaption to check PMI performance with multiple CPUs and if UE can calculate PMI for each spatial pattern correctly.

  - Option 2: Not to introduce CSI requirements for power/domain adaption.

As mentioned in our previous contribution during RAN4#108bis, there is no actual need of additional requirements. During the SI phase, the evaluation results for both semi-static TXRU reduction and dynamic TXRU adaptation have been provided (TR38.864-6.3.1), settling on Case 3: Dynamic TXRU adaptation with multi-CSI. Moreover, during the WI phase, RAN1 has identified two types of spatial domain (SD) adaptations to be supported:

- Type 1 SD adaptation: all antenna elements associated to a logical antenna port is disabled/enabled

- Type 2 SD adaptation: part/subset of antenna elements associated to a logical antenna port is disabled/enabled

However, there is no new codebook and subsequently no CQI impact since this feature is mostly related to processing timeline instead of actual signal processing. Moreover, even if processing timeline is an issue, this is solved by creating a specific UE capability in this regard. Hence, we propose again that we should refrain from introducing a new requirement.

A similar argument can be said about power offsets. Since this is nothing new and there’s no impact on signal processing, we should also avoid defining new requirements in this regard.

Proposal 2: Do not introduce new CSI requirements for NES.

Proposal 3: Do not specify new tests intended to new power offset values an adaptation, since this does not meaningfully impact UE internal signal processing.

# 4. Conclusion

Our observations and proposals are summarized below:

Observation 1: In inter-band SSB-less operation, the UE is expected to operate with two separate RF chains, hence separate tracking loops and FFT blocks will exist.

Observation 2: Current test proposal relies on a single scenario based on TDLC300-100 channel, which is not a common operating scenario but rather a corner case. In most scenarios it is expected that RTD will be minimal via BS handling this in collocated scenarios.

Observation 3: CA requirements are mostly functional tests set under TDLA30-10. More stringent channel conditions are usually reserved for single-carrier requirements.

Observation 4: Release 19 already considers the SSB on demand based on, at least, cell load as an input parameter, which will dilute any performance concern in Release 18.

Observation 5: There is an argument that there are better TRS estimation algorithms that can be implemented. However, this WI is not about proposing new algorithms for TRS-based estimation.

Proposal 1: Do not introduce new requirements for CA in SSB-less scenarios.

Proposal 2: Do not introduce new CSI requirements for NES.

Proposal 3: Do not specify new tests intended to new power offset values adaptation, since this does not meaningfully impact UE internal signal processing.

# References

- R4-2321130	, “WF on [109][330] Netw_Energy_NR_demod”, Huawei.

- R4-2318353, “Discussion on Network energy saving for NR UE demodulation performance and CSI requirements”, Nokia, Nokia Shanghai Bell.