3GPP TSG-RAN WG4 Meeting #110		R4-2400429

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	8.3.2.1

Source:	Apple

Title:   	On general aspects for NR FR2 multi-Rx chain DL reception

Document for:	Discussion

# 1	Introduction

At RAN4 meeting#109, a WF on general aspects was agreed [1]. In this contribution, we continue to discuss the following issues:

- Indication of multi-Rx operation: associated UE capabilities

- UE capability

# 2	Discussion

## 2.1 Indication of multi-RX operation: associated UE capabilities

The following options are captured in [2]:

#### Issue 1-2-5: Indication of multi-Rx operation

- FFS

  - Option 1:

    - For the new UE indication on UE preference on not supporting simultaneous reception with different QCL-typeD, following related UE capabilities in FR2-1 are identified to be involved:

      - simultaneousReceptionDiffTypeD-r16

      - defaultQCL-PerCORESETPoolIndex-r16

      - defaultQCL-TwoTCI-r16

      - mTRP-PDCCH-TwoQCL-TypeD-r17

      - sfn-SchemeA-r17

      - sfn-SchemeA-DynamicSwitching-r17

      - sfn-SchemeA-PDCCH-only-r17

      - sfn-SchemeA-PDSCH-only-r17

      - sfn-SchemeB-r17

      - sfn-SchemeB-DynamicSwitching-r17

      - sfn-SchemeB-PDSCH-only-r17

      - sfn-SimulTwoTCI-AcrossMultiCC-r17

      - sfn-QCL-TypeD-Collision-twoTCI-r17

      - UE capabilities to be introduced in Rel-18 NR_FR2_multiRX_DL which will be finalized in RAN4 feature list.

    - RAN4 to send LS to RAN1/2 to inform the involved UE capabilities related to the new UE indication on UE preference on not supporting simultaneous reception with different QCL-typeD

With UE indication of multi-RX operation, it is necessary for the network to know which UE capabilities will not be supported by the UE when the UE indicates its fallback to single RX operation. As to how to share the set of those multi-RX dependent UE capabilities, we prefer the following handling:

- For RAN1-specified UE capabilities, RAN1 can share the list with RAN2.

- For RAN4-specified UE capabilities, RAN4 can share the list with RAN2.

In this way, we can avoid missing any RAN1-specified UE capabilities or wrongfully sending any RAN1-specified UE capabilities that do not depend on multi-RX operation at UE side.

Proposal 1: In the follow-up LS to RAN2, RAN4 can share RAN4-specified UE capabilities with RAN2, while asking RAN1 to share RAN1-specified UE capabilities that depend on multi-RX operation at UE side.

To this end, we prepare a draft LS in [3].

## 2.2 UE capability

In the WF [1], two UE capabilities were tentatively agreed upon:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_FR2_multiRX_DL | 30-1 | Supports scheduling restriction relaxation and measurement restriction relaxation | - Supports simultaneous reception of CSI-RS for layer 1 measurement and PDSCH with different QCL Type-D on overlapping OFDM symbols. - Supports Simultaneous layer 1 measurement of CSI-RS overlapping with another CSI-RS with different QCL Type-D on overlapping OFDM symbol(s). | 16-2c, 23-5-1, [at least one of 16-2a, 16-2b-1, 16-2b-2 and 16-2b-3] | Yes | N/A |  | [Per FSPC] | TDD only | FR2-1 only |  | [Note: It can be supported for all power classes excepted PC6. ] | Optional with capability signalling |
| 30. NR_FR2_multiRX_DL | [30-2] | Fast beam sweeping for layer 1 measurement | Supports beam sweeping factor reduction for SSB-based layer 1 measurement. |  | [No] | N/A |  | [Per band] | N/A | FR2-1 only |  | Candidate values for Component 2: {2,4,6} for FR2-1 [Note: It can be supported for all power classes excepted PC6.] | [Optional with capability signalling] |

Our views are as follows:

- On [30-2], Fast beam sweeping for layer 1 measurement, as discussed at the last meeting, we think some side condition needs to be added to clearly show that this feature depends on the fact that the UE is in the multi-RX operation. We understand that there are views that this capability should be designed to be independent of UE implementation or operation mode, but having such a capability is outside the scope of this WI and should not be considered in R18. Therefore, we propose to revise the description in the cell “Components” that this capability is activated only when the UE is in multi-RX operation.

- Feature Type: For both 30-1 and [30-2], as they may be extended for CA/DC in future releases, and involve BB processing, a better granularity of per CC gives UE the desired flexibility. Therefore, we propose to use Per FSPC. That said, it should be OK to use Per band for [30-2]. Therefore, we propose to remove the brackets.

Table 1: Multi-RX UE feature list

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_FR2_multiRX_DL | 30-1 | Supports scheduling restriction relaxation and measurement restriction relaxation | - Supports simultaneous reception of CSI-RS for layer 1 measurement and PDSCH with different QCL Type-D on overlapping OFDM symbols. - Supports Simultaneous layer 1 measurement of CSI-RS overlapping with another CSI-RS with different QCL Type-D on overlapping OFDM symbol(s). | 16-2c, 23-5-1, [at least one of 16-2a, 16-2b-1, 16-2b-2 and 16-2b-3] | Yes | N/A |  | Per FSPC | TDD only | FR2-1 only |  | [Note: It can be supported for all power classes excepted PC6. ] | Optional with capability signalling |
| 30. NR_FR2_multiRX_DL | [30-2] | Fast beam sweeping for layer 1 measurement | Supports beam sweeping factor reduction for SSB-based layer 1 measurement when the UE is in multi-RX operation. | 16-2c, 23-5-1 | [No] | N/A |  | Per band | N/A | FR2-1 only |  | Candidate values for Component 2: {2,4,6} for FR2-1 [Note: It can be supported for all power classes excepted PC6.] | [Optional with capability signalling] |

Proposal 2: It is proposed to specify two UE capabilities as in Table 1.

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: In the follow-up LS to RAN2, RAN4 can share RAN4-specified UE capabilities with RAN2, while asking RAN1 to share RAN1-specified UE capabilities that depend on multi-RX operation at UE side.

Proposal 2: It is proposed to specify two UE capabilities as in Table 1.

# 4	References

R4-2321602, " WF on core maintenance and performance part for multi-Rx WI," vivo, Ericsson

R4-2317332, "WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1)," vivo

R4-2400430, "Draft LS on associated UE capabilities for UE indication of FR2 multi-RX operation," Apple