3GPP TSG-RAN WG4 Meeting # 110	R4-2400462

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	8.21.2.1

Source:	Apple

Title:	On RRM requirements for TDCP measurement

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#109 RRM impacts for R18 MIMO evolution were discussed and way forward [1] was agreed.  In this contribution we present our views on RRM requirements for TDCP measurement.

# 2. Discussion

In [1] the following agreements were made for feasibility of introducing accuracy requirements for TDCP :

Issue 1-1-1: Is it feasible to define TDCP accuracy requirement for TDCP? Agreement: - No RRM core requirement to be defined, and further discuss the following options: - Option 1: Define accuracy and test cases as part of performance requirement - Option 2: Do not define accuracy requirement but define test cases for particular configurations as part of performance requirements - Option 3: Do not define accuracy requirement and test cases as part of performance requirements

In RAN4#108bis it was agreed to study feasibility and simulation assumptions for the study were agreed in [2]. In RAN4#109 we provided results for feasibility study for defining accuracy requirements in [3] and we provide them again below to corroborate our proposals.

Link level simulation assumptions for evaluating TRS based TDCP measurements in NR Parameter; Value. Delay (between TRS symbols); 1slot. Channel model; TDL-A, delay spread=30ns. Doppler Spread; 10, 30, 75, 100, 200, 300. SNR; 5:5:20. Number of averaging samples:; one shot as baseline, 4 samples. Channel BW; 10MHz. SCS; 30KHz as baseline, 15KHz 2nd priority. Reference Channel estimation; LS CE for TRS as baseline, MMSE CE as 2nd priority. Correlation matrix and antenna configuration; 1x2 Low. Simulation results to be considered: - CDF curves of (including the 90% and 50% and 10% CDF points) for estimated TDCP. TDCP calculation reference: ct,∆t=n=0N-1hnt+∆t∙hn*tnhnt+∆t2∙nhnt2

The simulation assumptions for the feasibility are provided in the table below:

| Parameter | Value |
| --- | --- |
| Delay (between TRS symbols) | 1slot |
| Channel model | TDLA, delay spread=30ns TDLB, delay spread=30ns |
| Doppler Spread | 10, 30, 75, 100, 200, 300 |
| SNR | 5:5:20 |
| Number of samples | One shot measurement |
| Channel BW | 10MHz |
| SCS | 30KHz |
| Reference Channel estimation | LS CE for TRS |
| Correlation matrix and antenna configuration | 1x2 Low |

TDLA-30

TDLB-100

Companies were suggested to provide the median, 10,90 percentile points of the TDCP measurement. In the tables below we provide the unquantized TDCP amplitude statistics.

Table 1: Statistics for TDLA-30 with 30KHz SCS

| Doppler | SNR | 10 Perc | 90 Perc | Median |
| --- | --- | --- | --- | --- |
| 10 | 5 | 0.5037 | 0.8272 | 0.7028 |
|  | 10 | 0.7367 | 0.9337 | 0.8715 |
|  | 15 | 0.8901 | 0.9754 | 0.9516 |
|  | 20 | 0.9572 | 0.9903 | 0.9809 |
| 75 | 5 | 0.4766 | 0.8193 | 0.6613 |
|  | 10 | 0.6874 | 0.9295 | 0.8465 |
|  | 15 | 0.8476 | 0.9727 | 0.9375 |
|  | 20 | 0.9279 | 0.9876 | 0.9712 |
| 100 | 5 | 0.4577 | 0.8085 | 0.654 |
|  | 10 | 0.6763 | 0.9251 | 0.8375 |
|  | 15 | 0.8306 | 0.9696 | 0.9293 |
|  | 20 | 0.9028 | 0.9846 | 0.964 |
| 200 | 5 | 0.4273 | 0.7923 | 0.6182 |
|  | 10 | 0.6138 | 0.9119 | 0.8002 |
|  | 15 | 0.7195 | 0.9596 | 0.895 |
|  | 20 | 0.7657 | 0.9769 | 0.9322 |
| 300 | 5 | 0.386 | 0.7702 | 0.5976 |
|  | 10 | 0.553 | 0.89 | 0.7648 |
|  | 15 | 0.65 | 0.9425 | 0.8555 |
|  | 20 | 0.6928 | 0.9617 | 0.8913 |

Table 2: Statistics for TDLB-100 with 30KHz SCS

| Doppler | SNR | 10 Perc | 90 Perc | Median |
| --- | --- | --- | --- | --- |
| 10 | 5 | 0.5939 | 0.8165 | 0.7253 |
|  | 10 | 0.8113 | 0.9303 | 0.8875 |
|  | 15 | 0.9250 | 0.9742 | 0.9575 |
|  | 20 | 0.9714 | 0.9902 | 0.9830 |
| 30 | 5 | 0.5882 | 0.8158 | 0.7148 |
|  | 10 | 0.8067 | 0.9293 | 0.8821 |
|  | 15 | 0.9214 | 0.9739 | 0.9545 |
|  | 20 | 0.9659 | 0.9907 | 0.9797 |
| 75 | 5 | 0.5655 | 0.8073 | 0.7057 |
|  | 10 | 0.7801 | 0.9225 | 0.8726 |
|  | 15 | 0.8970 | 0.9673 | 0.9449 |
|  | 20 | 0.9436 | 0.9853 | 0.9713 |
| 100 | 5 | 0.5628 | 0.8000 | 0.6984 |
|  | 10 | 0.7689 | 0.9175 | 0.8628 |
|  | 15 | 0.8833 | 0.9629 | 0.9366 |
|  | 20 | 0.9256 | 0.9802 | 0.9632 |
| 200 | 5 | 0.5078 | 0.7740 | 0.6609 |
|  | 10 | 0.6781 | 0.8897 | 0.8148 |
|  | 15 | 0.7672 | 0.9372 | 0.8848 |
|  | 20 | 0.8035 | 0.9556 | 0.9099 |

In the table below we capture the measurement error for different Doppler, SNR and channel models.

Table 3: TDCP Measurement Error Statistics

| Doppler | SNR (dB) | 15KHz SCS |  |  | 30KHz SCS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ideal | % Error |  | Ideal | % Error |  |
|  |  |  | TDLA-30 | TDLB-100 |  | TDLA-30 | TDLB-100 |
| 10 | 5 | 0.999 | 32% | 19% | 0.9998 | 30% | 27% |
|  | 10 |  | 14% | 7% |  | 13% | 11% |
|  | 15 |  | 5% | 3% |  | 5% | 4% |
|  | 20 |  | 2% | 1% |  | 2% | 2% |
| 30 | 5 | 0.9911 |  | 19% | 0.9978 |  | 28% |
|  | 10 |  |  | 7% |  |  | 12% |
|  | 15 |  |  | 2% |  |  | 4% |
|  | 20 |  |  | 0% |  |  | 2% |
| 75 | 5 | 0.9452 | 29% | 16% | 0.9862 | 33% | 28% |
|  | 10 |  | 11% | 4% |  | 14% | 12% |
|  | 15 |  | 3% | 1% |  | 5% | 4% |
|  | 20 |  | 1% | 2% |  | 2% | 2% |
| 100 | 5 | 0.9037 | 31% | 15% | 0.9755 | 33% | 28% |
|  | 10 |  | 11% | 2% |  | 14% | 12% |
|  | 15 |  | 1% | 4% |  | 5% | 4% |
|  | 20 |  | 3% | 6% |  | 1% | 1% |
| 200 | 5 | 0.6425 | 13% | 4% | 0.9037 | 32% | 27% |
|  | 10 |  | 12% | 23% |  | 11% | 10% |
|  | 15 |  | 24% | 31% |  | 1% | 2% |
|  | 20 |  | 29% | 34% |  | 3% | 1% |
| 300 | 5 | 0.2906 | 76% |  | 0.79 | 24% |  |
|  | 10 |  | 128% |  |  | 3% |  |
|  | 15 |  | 153% |  |  | 8% |  |
|  | 20 |  | 164% |  |  | 13% |  |

Based on the results, we observe a large dependence on SNR, channel model and Doppler on the TDCP measurement. Requirements if defined would need to be for a specific SNR, channel model and Doppler. Such accuracy requirements are not useful.

- Simulation results show that the TDCP measurement is very sensitive to SNR and depends on channel model and Doppler.

- It is not feasible to define accuracy requirements for TDCP measurement that would be applicable to a range of SNR, and all channel conditions.

Based on the agreements in previous meeting, the feasibility study and evaluation are limited to TDL channel model. Also, the ideal or genie value is only for a specific channel model. The accuracy requirements specified in RRM are typically agnostic to propagation channel condition and applicable to a range of SNR, although the test case might be defined for a certain channel model and SNR. If in case it is determined that it is feasible to define inaccuracy requirements for TDCP measurement based on certain channel model- TDL in this case, such accuracy requirements would only be for that channel model and not applicable in real deployment. Such a requirement is inadequate and not useful, as UE meeting the accuracy requirement in the test setup wouldn’t ensure accuracy in actual deployment.

- The accuracy requirements for RRM are agnostic to propagation channel condition and applicable to a range of SNR.

- The feasibility study of defining accuracy requirements for TDCP measurement is limited to TDL channel model.

- Defining an accuracy requirement for TDCP measurement in a certain channel model is inadequate and not useful in real deployment.

Another issue is with testability. We typically have performance tests for accuracy in static channel condition, since the measurement could change based on the fading channel and we cannot determine if the error is due to instantaneous channel condition or measurement error.

- Testability issue with defining such accuracy requirements as static channel conditions are used for accuracy requirements and TDCP measurement accuracy would need a TDL channel.

- Using a fading channel for accuracy requirement test might lead to false errors, as the measurement can change with channel conditions.

Based on the above observations, we don’t think it is feasible to define accuracy requirements for TDCP measurement and propose not to introduce any accuracy requirements for TDCP measurement.

- Do not define accuracy requirements for TDCP measurement.

Further considerations in RAN4

Based on the results there seem to be limited scenarios when the TDCP measurement is reliable. To make the feature useful in deployment, there might be a necessity to introduce criteria for TDCP reporting. One way is that the TDCP measurement is only triggered when the reporting L1-RSRP or L1-SINR for serving cell is larger than some threshold. Another way is that associated L1-RSRP or L1-SINR measurement is reported along with the TDCP measurement to provide additional indication on the reliability of the measurement.

- Introduce additional criteria for TDCP measurement reporting.

- Introduce additional L1-SINR or L1-RSRP reporting along with TDCP reporting

The network could either configure new measurement or use the existing reports of L1 measurements from the serving cell to trigger aperiodic CSI for TDCP measurement reporting. The network could trigger a report only when the serving cell L1 measurements are above a certain threshold.

- Network can trigger aperiodic CSI for TDCP measurement reporting based on serving cell L1 measurement report

- UE is expected to receive aperiodic CSI for TDCP measurement reporting only when reported L1-RSRP or L1-SINR from serving cell are larger than a threshold.

# 3. Conclusion

In this paper, we provide our views on open issues on RRM requirements for TDCP measurement. Our observations and proposals are captured below:

- Simulation results show that the TDCP measurement is very sensitive to SNR and depends on channel model and Doppler.

- It is not feasible to define accuracy requirements for TDCP measurement that would be applicable to a range of SNR, and all channel conditions.

- The accuracy requirements for RRM are agnostic to propagation channel condition and applicable to a range of SNR.

- The feasibility study of defining accuracy requirements for TDCP measurement is limited to TDL channel model.

- Defining an accuracy requirement for TDCP measurement in a certain channel model is inadequate and not useful in real deployment.

- Testability issue with defining such accuracy requirements as static channel conditions are used for accuracy requirements and TDCP measurement accuracy would need a TDL channel.

- Using a fading channel for accuracy requirement test might lead to false errors, as the measurement can change with channel conditions.

- Do not define accuracy requirements for TDCP measurement.

- Introduce additional criteria for TDCP measurement reporting.

- Introduce additional L1-SINR or L1-RSRP reporting along with TDCP reporting

- Network can trigger aperiodic CSI for TDCP measurement reporting based on serving cell L1 measurement report

- UE is expected to receive aperiodic CSI for TDCP measurement reporting only when reported L1-RSRP or L1-SINR from serving cell are larger than a threshold.

# Reference

- R4-2321615, “WF on NR_MIMO_evo_DL_UL WI”, Samsung

- R4-2317371, “WF on R18 NR MIMO RRM requirement”, Samsung

- R4-2318583, “On RRM requirements for TDCP measurement”, Apple