3GPP TSG-RAN WG4 Meeting # 110	R4-2400530

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	8.27.2

Source:	MediaTek Inc.

Title:	Simulation results on PDCCH requirements for DSS enhancement

Document for:	Information

- Introduction

In RAN4#109 meeting, a WF for DSS enhancement requirements was agreed [1]. RAN4 will define requirements with the assumption “PDCCH channel estimation is assumed to use only the clean PDCCH symbol”. In this contribution, we present simulation results based on simulation assumptions in Table 1 [1].

Table 1. Simulation assumptions for PDCCH requirements for DSS enhancement

| Parameters | Values |
| --- | --- |
| Duplex mode | FDD |
| SCS | 15 kHz |
| Bandwidth | 10 MHz |
| Channel model | TDLA30-10 and TDLC300-100 |
| Antenna configuration | 2x2, 2x4, 4x2, 4x4 |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 39 bits |
| Interleaving | Non-interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 |
| Shift index | 0 |
| CORSET duration | 2 (2nd symbol and 3rd symbol) |
| CORSET RB | 48 |
| AL | 4, 8 |
| - Note 1: The bandwidth of NR CORESET is within the bandwidth of LTE. - Note 2: gNB punctures the PDCCH data and DMRS REs overlapped with LTE CRS. Note 3: PDCCH channel estimation is assumed to use only the clean PDCCH symbol. |  |

- Simulation Results

In Table 2, we provide our initial simulation results for SNR values achieving 1% BLER for PDCCH under DSS scenario.

Table 2. SNR to achieve 1% BLER

| Test case | SCS(KHz)/CBW (MHz) | CORESET RB | Payload | AL | Propagation condition | Antenna configuration | SNR @ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 2x2 Low | 2.25 |
| 2 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 2x2 Low | -2.80 |
| 3 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 2x4 Low | -2.25 |
| 4 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 2x4 Low | -5.90 |
| 5 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x2 Low | 1.70 |
| 6 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x2 Low | -2.98 |
| 7 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x4 Low | -2.31 |
| 8 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x4 Low | -6.40 |

| Figure 1. SNR versus BLER for 2T2R, FDD | Figure 2. SNR versus BLER for 2T4R, FDD |
| --- | --- |
| Figure 3. SNR versus BLER for 4T2R, FDD | Figure 4. SNR versus BLER for 4T4R, FDD |

- Conclusion

In this contribution, we provide PDCCH simulation results for DSS enhancement WI.

- Reference

- R4-2321139, “WF for Enhancement of Dynamic Spectrum Sharing demodulation requirements”, Ericsson