3GPP TSG-RAN WG4 Meeting # 110	R4-2400559

# Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	8.24.2

Source:	Qualcomm Inc.

Title:	On NR SL relay

Document for:	Discussion

# Introduction

In this contribution, we present our view SL relay RRM requirements.

# Discussion

Proposal 1: The existing R17 SL relay test is applicable to the concerned SL UEs in R18, and no new test cases are needed.

We consider the accuracy requirement in clause 10.4.5 is applicable to R18 scenarios with proper condition. The proposal from the previous meeting WF:

Existing accuracies of SD-RSRP and SL-RSRP in clause 10.4.5 shall also apply for the remote UE in the multipath scenario provided that the remote UE:

- is synchronised to the sidelink relay UE that is measured and

- is in-coverage on the frequency used for sidelink if both the direct path and the SL on the indirect path are on the same frequency or

- is out of coverage on the frequency used for sidelink if the direct path and the SL on the indirect path are on different frequencies.

It’s not obvious to us whether the last bullet is still multipath scenario given that the remote UE is out of coverage on the frequency use for SL. In fact, if it is out of coverage for SL frequency, how can the UE measure SD-RSRP and SL-RSRP?

Proposal 2: Do not consider out of SL frequency coverage scenario for SD-RSRP and SL-RSRP measurement accuracy requirement.