3GPP TSG-RAN4 Meeting #110	R4-2400572

Athens, Greece, 26th Feb 2024 – 1st Mar 2024

Title:	Input for WP5D sharing studies

Source:	Qualcomm Incorporated

Agenda item:	12.1.2

Document for:	Discussion

# 1.	Introduction

In [1] WP 5D is seeking from RAN4 technical and operational characteristics of terrestrial IMT systems that would operate in frequency bands of 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz being considered under WRC-27 AI 1.7. In this paper we provide our high-level view on workplan and possible way forward on how to address WP 5D requests in RAN4 and 3GPP.

# Discussion

## 2.1	Work plan aspect

An initial input is requested by June 13th. However, technical and operational characteristics needed for sharing and compatibility studies should generally be available by 31 Dec 2024 (could be extended to 1st July 2025 if necessary).

In case RAN4 wants to provide initial feedback by June, the initial response may need to be sent from RAN4#111 May 2024 to allow some time for RAN4 to provide its technical response.

It is proposed to at least enable technical discussion in RAN4 by allocating an agenda in RAN4 for input. RAN should also discuss with separate submissions whether to agree study item to enable this work or enable WG level work under dedicated RAN task. The amount of work might suggest that proper TU allocation is required.

## 2.2	Technical discussion

The request is targeting studies for IMT-2030. The influence of some forward-looking items should be carefully considered when rafting the reply to WP5D.

Duplex method: 3GPP is working on enabling full duplex (SBFD) operation on unpaired spectrum. This may cause changes in assumptions in sharing studies for synchronised vs non-synchronised TDD networks.

Channel bandwidths: In general, wider channel bandwidths should be enabled with increasing center frequency following same approach as FR1 (3 to 100 MHz) to FR2 (50 to 400 MHz). Many bands have enabled intra-band CA that indicated need for wider bandwidths.

Maximum output power: The legacy FR1 default power class PC3 has become overrun by higher power in almost all bands. Many features such as ULFPTx and TX Diversity are built into NR standard for the purpose of enabling enhancing higher output power.

Larger MIMO and its impact to EIRP especially for base stations. For increasing frequency and shorter wavelengths, in equally sized physical housing and TRP power can accommodate more antenna elements and enable narrower beamwidths with higher EIRP/TRP ratio.

## 2.3	Discussion on specific frequency ranges

3GPP would need to provide an answer on core RF parameters and models for several bands, namely 4.4-4.8 GHz, 7.125-8.4 GHz and 14.8-15.35 GHz.

Some high-level observations on the bands of interest are provided below:

4 400 - 4 800 MHz: RAN4 has a band n79 defined for this range with defined parameters. Further discussion might be needed if and how to modify these existing normative parameters to address WP 5D request. This frequency range is likely easiest to come up with parameters. PC1.5 is defined for this frequency range and maximum channel BW is 100 MHz and intra-band contiguous CA (CA_n79D) is defined up to 300 MHz.

7.125 - 8.4 GHz: this frequency range is adjacent to existing band n104. It should be noted that PC2 is enabled for this band n104 already. No intra-band CA is defined for this band but this frequency range is rather new entry in 3GPP and it is possible it will b added.

14.8 - 15.35 GHz: this new frequency range may need most of the discussions since there are no immediate adjacent bands with normative requirements that can act as a reference. Potential architectures for gNB and UE are required to fill out the fields listed in Table 1. Advanced AAS on the gNB side are likely required with potential implications on transmit power and EIRP. UE architecture may be driven by the required EIRP to achieve a balanced uplink-downlink coverage. Alignment on the overall system requirements is required to produce the most appropriate parameter settings.

Several general considerations need also to be made in RAN4 about the potential impact of the different parameters selection and assumptions on the ITU-R sharing and compatibility studies outcome.

Based on the above, it is observed that an adequate technical discussion needs to be carried out in RAN4. As a consequence, we invite RAN to open a dedicated study item to address the questions.

# Conclusion

In this paper we have provided our general view on how to address WP 5D request in RAN4. RAN4 should be able to conclude the request in the LS [1] after proper technical discussion. Proper container for the discussion should be enabled and concluded in RAN#103.

# Reference

[1]	R4-2400333, “Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz”, WP5D, 3GPP TSG-RAN4 Meeting #110, Athens, Greece, 26th Feb 2024 – 1st Mar 2024

[2]	R4-2015681, “LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (6.425 to 10.5 GHz)”, TSG RAN WG4, 3GPP TSG-RAN WG4 Meeting # 97-e, Electronic Meeting, 2-13 Nov., 2020