3GPP TSG-WG4 Meeting #110 	R4-2400638

# Athens, Greece, Feb 26 - Mar 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.858 | CR | draftCR | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR 38.858 update to Technical Report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Charter Communications, CableLabs, Ericsson, Spark NZ ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_Duplex_Evo |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Finalized during RAN4#109 meeting Techical Report 38.858 with RAN4 part includes in some places different types of errors. In some places some additional clarifications are added, which are aimed not to change any conclusion, but to ensure better readability of TR. This draft CR includes proposal to correct these errors in TR that were not noticed during drafting phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 11.1: We suggest adding the column "Aggressor baseline" in Table 11.1-2 to help clarifying how the throughput degradation is calculated. This new column is present in many of the agreed WFs on coexistence. For instance: R4-2305921 and R4-2302888. Added to clarify that the tput degradation is obtained in form of % in the document "Summary of all simulation results". Clause 11.3: Section 11.2 describes the ACIR baseline and the different ACIR offsets applied for each coexistence case. In Section 11.3, however, it is unclear to the reader which of the ACIR configurations was used to derive the conclusions. Therefore, we think this sentence should be included. Clause 11.3.1: Changing 49 dBm to 46 dBm according to the SBFD BS Tx power options. The results in the excel spreadsheet show that with BS Tx power of 46 dBm, the degradation is higher than 10% for the cell-edge: Annex E.1: Table E.1-1: The priority was agreed to be removed as it was only for study item internal prioritization. This has already been removed in section 11.1. and should be followed here as well. Table 11.1-2: We suggest adding the column "Aggressor baseline" in Table 11.1-2 to help clarifying how the throughput degradation is calculated. This new column is present in many of the agreed WFs on coexistence. For instance: R4-2305921 and R4-2302888. If the column is added in 11.1-2, it should be added here as well. Annex E.3.1: We propose these changes to accurately detail the methodology followed in this RAN4 coexistence study. The new description is now aligned with the methodology used by companies to fill up the excel spreadsheet "Collection of simulation results from all companies" in R4-2321079. This screenshot of "Collection of simulation results from all companies.xlsx" indicates how the throughput degradation in the green area (use for deriving the coexistence conclusions) is calculated in the spreadsheet. The used formula matches the described in step 9 of the methodology: |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Technical Report 38.858 will include in some places errors, that may cause illegibility or ambiguity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1, 11.3, Annex E.1, Annex E.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

## 11.1	Introduction

The adjacent channel co-existence studies were performed to the deployment scenarios described in Table 11.1-1 below. The co-existence cases are described in the Table 11.1-2 below, and they were performed for each scenario listed in Table 11.1-1. The detailed assumptions associated with these scenarios and cases can be found in Annex E.

Table 11.1-1: Adjacent channel co-existence scenarios

| Scenario | FR | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro |
| 2 | FR1 | Urban Hotspot | Urban Hotspot |
| 3 | FR1 | Indoor | Indoor |
| 4 | FR1 | Urban Macro | Micro |
| 5 | FR1 | Micro | Micro |
| 6 | FR2-1 | Urban Macro | Urban Macro |
| 71 | FR2-1 | Urban Hotspot | Urban Hotspot |
| 8 | FR2-1 | Urban Dense | Urban Dense |
| 9 | FR2-1 | Indoor | Indoor |
| Note 1: This scenario has been down-selected. |  |  |  |

Table 11.1-2: Adjacent channel co-existence cases

| Case | Aggressor | Aggressor baseline | Victim | Slot allocation Aggressor Victim |
| --- | --- | --- | --- | --- |
| 1 | SBFD | TDD DL | TDD DL |  |
| 2 | SBFD | TDD UL | TDD UL |  |
| 3 | TDD DL | No aggressor network | SBFD |  |
| 4 | TDD UL | No aggressor network | SBFD |  |
| Note: Case 3 and Case 4 are down-selected for Scenario 4. |  |  |  |  |

The Urban Hotspot reuses most parameter assumptions as Urban Macro, except that Urban Macro adopts random dropping method for UE while Urban Hotspot adopts cluster-based dropping method for UE. Other differences are described in Annex E, in Table E.2.1-1, Table E.2.1-2, and Table E.2.1-3.

The co-existence evaluation captures cases where TDD and SBFD are both victim and aggressor networks. This to evaluate impact on legacy TDD networks if SBFD is introduced in an adjacent channel as well as to understand the impact of legacy TDD network on SBFD network, as described in Annex E, Table E.1-2. It is worth noting that RAN4 has only considered the case of {D, U} as an SBFD configuration as it is comparable to the {D, U, D} SBFD configuration in terms of performance (based on the agreed RAN4 models and parameters related to leakage).

Additionally, it should be noted that the RAN4 co-existence studies have special assumptions due to the adopted simulation methodologies such as:

-	Power control scheme is only used to compensate path loss. That’s the reason why final SINR for UL is less than assumed target SINR. But commercial UE UL SINR could meet target SINR value according to the power control scheme in TS 38.213.

-	It is assumed that all the slots configurations are the same with the time-invariant ACLR modelling assumption. Compared with the average throughput over all time slots with different configuration, this is the worst case with largest degradation value.

Moreover, in the following sections, all the throughput degradation data are given in percentage [%] and  in a range, where these data are defined as follows:

-	The {positive number} means the co-existence evaluation shows throughput loss;

-	The {negative number} means the co-existence evaluation shows throughput gain;

-	The {n/a} means the co-existence evaluation finds the performance baseline have no throughput, thus throughput degradation percentage cannot be mathematically calculated from such baseline.

# The throughput degradation is calculated following the methodology described in Annex E, Section E.3.1.

<End of change 1>

<Start of change 2>

## 11.3	Conclusion

This section will capture adjacent channel co-existence simulation results categorized by cases and scenarios. The conclusions below are derived from the coexistence results with baseline ACIR.

### 11.3.1	Case 1: aggressor SBFD DU victim NR TDD DL

Case 1 considers legacy TDD in DL slot as a victim while SBFD is operating in the adjacent channel for both FR1 and FR2-1. The conclusions are listed per scenario in Table 11.3.1-1.

Table 11.3.1-1: Case 1 co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 and Scenario 6 | FR1 and FR2-1 | No DL throughput degradation on the victim legacy TDD DL network for both average throughput and cell edge throughput is observed for different BS Tx powers (46dBm to 53 dBm for FR1 and 30 dBm for FR2-1), grid-shifts (5% to 100%), and different SBFD BS antenna configurations. |
| Urban Hotspot -> Urban Hotspot Scenario 2 | FR1 | DL throughput degradation is observed only at cell edge due to inter-UE CLI for different grid-shifts (5% to 100%) and BS Tx powers (46 dBm to 53 dBm). |
| Urban Macro -> Urban Micro Scenario 4 | FR1 | No DL throughput degradation for both average throughput and cell edge throughput is observed. |
| Indoor -> Indoor Scenario 3 and Scenario 9 | FR1 and FR2-1 |  |
| Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8 | FR1 and FR2-1 |  |

<End of change 2>

<Start of change 3>

# E.1	RAN4 co-existence simulation scenarios

The coexistence evaluation is conducted considering the different scenarios listed in Table E.1-1.

Table E.1-1: Adjacent channel co-existence scenarios

| Scenario | FR | Aggressor | Victim |  |
| --- | --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro |  |
| 2 | FR1 | Urban Hotspot | Urban Hotspot |  |
| 3 | FR1 | Indoor | Indoor |  |
| 4 | FR1 | Urban Macro | Micro |  |
| 5 | FR1 | Micro | Micro |  |
| 6 | FR2-1 | Urban Macro | Urban Macro |  |
| 71 | FR2-1 | Urban Hotspot | Urban Hotspot |  |
| 8 | FR2-1 | Urban Dense | Urban Dense |  |
| 9 | FR2-1 | Indoor | Indoor |  |
| Note 1: Scenario 7 has been down selected. |  |  |  |  |

The Urban Hotspot scenario reuses most parameter assumption as Urban Macro, except that Urban Macro assumes random deployment of UEs while Urban Hotspot assumes a clustered-based approach with different outdoor-to-indoor UE ratios and others as described in Annex E, in Table E.2.1-1 and Table E.2.1-2 and Table E.2.1-3.

The co-existence evaluation captures cases where TDD and SBFD are both victim and aggressor networks. This to evaluate impact on legacy TDD networks if SBFD is introduced in a neighbouring channel, also to understand impact of the legacy TDD network on SBFD network, as described in Annex E, Table E.1-2. It is worth noting that RAN4 has only considered the case of {D, U} as an SBFD configuration as it is comparable in terms of performance (based on RAN4 models and parameters) to the {D, U, D} SBFD configuration.

Table E.1-2: Adjacent channel co-existence cases

| Case | Aggressor | Aggressor baseline | Victim | Slot allocation Aggressor Victim |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | SBFD | TDD DL | TDD DL |  |  |  |
| 2 | SBFD | TDD UL | TDD UL |  |  |  |
| 3 | TDD DL | No aggressor network | SBFD |  |  |  |
| 4 | TDD UL | No aggressor network | SBFD |  |  |  |
| Note 1: Case 3 and Case 4 are down-selected for Scenario 4. |  |  |  |  |  |

<End of change 3>

<Start of change 4>

## E.3.1	Coexistence evaluation methodology

The coexistence evaluation methodology can be summarized as:

- Aggressor and victim network are generated. UEs are distributed as described by parameter assumptions.

- UEs are associated to BS based on coupling loss.

- Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS and UE. This is done for both victim and aggressor networks.

- Throughput is computed in the victim systems without considering Adjacent Channel Interference (ACI) as:

ThputNo_ACI[bit/s/Hz]=f(SINRICI)=f(S/(N+IICI)), where IICI is the intra-network co-channel inter-cell interference.

- Throughput is computed in the victim systems considering ACI from the baseline aggressor (see Table E.1-2) as:

ThputACI_baseline[bit/s/Hz]=f(SINRICI+ACI_baseline)=f(S/(N+IICI+IACI_baseline)), where IACI_baseline is the adjacent channel interference due to the baseline aggressor network.

- Throughput is computed in the victim systems considering ACI from the aggressor (see Table E.1-2) as:

ThputACI[bit/s/Hz]=f(SINRICI+ACI)=f(S/(N+IICI+IACI)), where IACI is the adjacent channel interference due to the aggressor network.

- The throughput degradation due to the presence of the baseline aggressor network is determined as:

LossACI_baseline=1-ThputACI_baseline/ThputNo_ACI

- The throughput degradation due to the presence of the aggressor network is determined as:

LossACI=1-ThputACI/ThputNo_ACI

- The ratio between the throughput degradation due to the presence of baseline aggressor and aggressor network is used to derive the coexistence conclusions and it is determined as:

Losscoexistence=1-((ThputACI/ ThputNo_ACI)/(ThputACI_baseline/ ThputNo_ACI))

The simulation results should be in the form of the throughput degradation with adjacent network baseline and with adjacent network and the ratio between the two can be compared at average and cell edge performance.

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput (Tp) in bps/Hz over a channel with a given SNIR, when using link adaptation:

Tp=0∝SSNIR∝SSNIRMAXfor SNIR≤SNIRMIN for SNIRMIN≤SNIR<SNIRMAXfor SNIR≥SNIRMAX

Where:

| S(SNIR) | Shannon bound, S(SNIR) =log2(1+SNIR) bps/Hz |
| --- | --- |
|  | Attenuation factor, representing implementation losses |
| SNIRMIN | Minimum SNIR of the code set, dB |
| SNIRMAX | Maximum SNIR of the code set, dB |

The parameters can be chosen to represent different modem implementations and link conditions. A parameter set relevant for eMBB is listed in Table E.3.1-1.

Table E.3.1-1: Parameters describing baseline Link Level performance for NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| , attenuation | 0.6 | 0.4 | Represent implementation losses |
| SNIRMIN | -10 | -10 | Based on QPSK, 1/8 rate (DL) and 1/5 rate (UL) |
| SNIRMAX | 30 | 22 | Based on 256QAM, 0.93 (DL) and 64QAM 0.93 (UL) |

<End of change 4>