3GPP TSG-RAN WG4#110	R4-2400644

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	7.1.1.2

Source: 	Qualcomm France

Title: 	NR-U Nominal Channel Spacing and Intra-band CA combinations

Document for:	Discussion

# Introduction

Considerations on how to resolve the existing discrepancy between specified NR-U Intra-band Contiguous CA combinations and nominal channel spacing are provided in this contribution.

- Discussion

It was pointed out in [1] that nominal channel spacing is not always met with some NR-U channel combinations. WF to study alternatives was agreed in [2], which basically suggest that either nominal channel spacing may be exceeded by certain amount for NR-U, or that combinations exceeding nominal channel spacing are non-contiguous.

The allowed NR-U channel locations are specified explicitly by NR-ARFCN numbers in Tables 5.4.2.3-2, 5.4.2.3-3, and 5.4.2.3-4 for NR-U bands n46, n96, and n102, respectively.

Figure 1 Allowed NR-ARFCN's for NR-U bands

Nominal NR channel spacing is specified in 5.4A.2:

Figure 2 Nominal Channel Spacing for CA

To re-cap nominal channel spacing rules, Intra-band CA combination with channel spacing with is larger than nominal channel spacing is considered Intra-band Non-Contiguous Combination. Notably, there is no limit how much larger the channel spacing needs to be to be called non-contiguous; any number larger than 0 kHz suffices.

We did calculate the Channel positions for all NR-U bands and looked which Contiguous Intra-band Channel combinations are possible using the current nominal channel spacing. Please note that “Applicable with current spec?” column compares respective NR CA configuration existing in current TS38.101-1. If all BW combinations included in current TS38.101-1 are possible, then that column shows “Yes”. If only some of the BW combinations are possible, ten this column shows “Partially”. Finally, if none of the BW combinations are possible, this column shows “No”.

Table 1 Applicable Channel Bandwidth combinations using Nominal Channel Spacing

For convenience we constructed a respective table for TS38.101-1

Table 2 Intra-band Contiguous CA combinations for NR-U bands

Comparing Table 1  and Table 2, it is obvious that several channel BW combinations are not feasible with nominal channel spacing rules. For instance, for 2CC combinations 20-40, 20-80, 40-60, 40-80 channel BW combinations are not feasible for n46, n96, and n102 as per current nominal channel spacing

To resolve this issue, we envision three alternatives:

- To modify nominal channel spacing to align with specified NR-U NR-ARFCN’s to allow using all currently specified Intra-band contiguous CA combinations

- To modify the allowed NR-U NR-ARFCN’s in a way that they would allow using all currently specified Intra-band contiguous CA combinations.

- To modify the specified Intra-band contiguous CA combinations by removing the unfeasible ones and by adding new Intra-band Non-contiguous CA combinations to allow using specified NR-U NR-ARFCN’s and to keep compliance with nominal channel spacing

None of the options is superior to others in our view.

Modifying nominal channel spacing might have unknown impacts to chipset operation, and in addition it would have potential backwards compatibility issue.

NR-ARFCN’s were specified with great attention at the time, so changing those should not be done with light grounds. Backwards compatibility issues could emerge in this alternative as well.

Modifying specified Intra-band contiguous CA combinations would resolve the nominal channel spacing issue, but it would create several new kind of combinations; just to give a few examples e.g CA_n46N with 20+20+20+20 could translate to CA_n46(B-B) or to CA_n46(A-M), CA_n46O with 20+20+20+20+20 could translate to CA_n46(B-M) and CA_n46O with 60+40+40+40+40 could translate to CA_n46(A-B-B) and so on.

This matter should be discussed in RAN4 among the interested companies. Initially, we expect final decisions would not be made during Athens meeting but during April/May instead.

- Conclusion

Considerations how to address the existing discrepancy between Nominal Channel Spacing and specified NR-U Intra-band contiguous CA band combinations were provided in this contribution.

To get interested companies’ comments, discussions should be carried out in RAN4 among interested companies with decisions expected during April/May 2024 timeframe.

- Reference

[1] R4-2321677, “On nominal channel spacing for NR-U”, Skyworks Solutions Inc.

[2] R4-2321928, “WF on nominal channel spacing for NR-U”, Skyworks Solutions inc.

- Appendix