3GPP TSG-RAN WG4 Meeting #110	R4-2400698

Athens, Greece, 26th February – 1st March 2024

Source:	AT&T

Title:	TP for TR 36.770 Conclusions

Agenda item:			9.3.1

Document for:	Approval

# 1. Introduction

A text proposal for TP for TR 36.770 to add the conclusions for the evaluations during the work item.

# 2. Text Proposal

---Start of changes---

# 9	Conclusions

In this work item, different aspects of enabling PC2 in NR FDD band 14 were addressed. The contents of the work included the applicable schemes to comply with SAR limits with 26dBm UE Tx power, the maximum output power limits, A-MPR requirements, the REFSENS degradation raised by FDD PC2, and UE implementations.

To accommodate the SAR limits of the E-UTRA PC2 FDD High Power UE, only the UE-based solution was considered. The UE-implementation based methods (i.e., P-MPR) are considered feasible to make sure SAR regulation is not violated.

In order to support 26dBm UE Tx power, only one RF architecture (i.e. 1Tx×26dBm) was considered. It was found that FDD HPUE with 1Tx architecture may need to use newly designed components. However, the design and performance of the components were determined to be feasible.

In conclusion, it has been shown in this work item that high power UE (power class 2) for NR FDD band 14 is feasible. The corresponding core requirements for maximum output power, A-MPR for NS_06 for PC2, and the reference sensitivity degradation were determined.

---End of changes---

# References