3GPP TSG-RAN WG4 Meeting#110					R4-2400730

26th Feb, 2024 – 1st Mar, 2024

Source: 	Qualcomm Incorporated

Title: 	NR MIMO: Views on UE demodulation performance and CSI requirements

Agenda item:	 8.21.4

Document for:	Discussion

# 1 Introduction

In this paper, our views on NR MIMO evolution following discussion in [1] are discussed.

# 2 Increased DMRS Orthogonal Ports

Issue 2-3-2: DMRS ports

Way forward:

- {1008} if Rank 1 test is selected

- {1008, 1009} if Rank 2 test is selected

- {1008-1010} if Rank 3 test is selected

- If Rank 4 is selected:

      - Option 1: {1008-1011}

      - Option 2: {1000, 1001, 1008, 1009}

2.1 PDSCH demodulation performance requirements

Option 2 enables all DMRS ports to be of the same CDM group which is the new feature being introduced. Moreover, since 4FD-OCC can reduce DMRS overhead and therefore increase throughput in a 4-layer (4L) case, we propose to use DMRS ports of Option2.

Issue 2-3-5: Cases need to be defined for FR1 Rel-18 DMRS

- Option 6: define one test for TDD rank 4 with 4Rx

      - For rank 4 with 4 Rx, Test 4-1 in clause 5.2.3.2.1

Proposal1: RAN4 to use Rel-18 DMRS ports {1000, 1001, 1008, 1009}  for Rank4 case.

Observation1: From earlier submissions of other companies, simulation results show no difference in performance for Rank1, and Rank2 between R15, and R18 DMRS ports. [2]

Therefore, new requirements for Rank1, Rank2 are likely to be redundant for performance specifications. For 4L case, with above proposal1, all 4 DMRS ports are in a single CDM group, R18 DMRS performance can be addressed with a single scenario, and an additional Rank3 case is not essential. Moreover, to keep the number of scenarios to be minimum, a single rank4 scenario for both FDD and TDD is our proposal.

Proposal2: RAN4 to reuse Test 4-1 of clause 5.2.3.2.1 and clause 5.2.3.1.1 of TS 38.101-4.

Issue 2-3-4: Number of Rx for tests need to be defined for Rel-18 DMRS

Way forward:

- Option 1: both 2Rx and 4Rx

- Option 2: Only 4Rx

- Option 3: Only 2Rx

Extending the approach, we support Option2 of Issue 2-3-4

Proposal3: RAN4 to define PDSCH demod. requirements for only 4Rx.

Issue 2-3-7: Minimum requirements for tests need to be defined for Rel-18 DMRS

Way forward:

- Option 1: reuse legacy value.

- Option 2: new value according simulation results

- Other options are not precluded.

Since a new scenario is going to be evaluated, we propose to revisit with  new simulation results, and support Option2.

Proposal4: RAN4 to use new value according to latest simulation results.

## 3 R18 TypeII Doppler

Issue 2-1-1: Propagation channel

Way forward:

- Start with TDLA30-30, TDLA30-50, and TDLA30-100 for TypeII Doppler feasibility study.

Since the feature is to address vehicles with medium speeds targeting FR1 (3GHz), and TDLA30-50 is used in RAN1 discussions, we propose TDLA30-50.

Proposal5: RAN4 to use TDLA30-50 for TypeII doppler PMI requirements.

Issue 2-2-4: paramCombination-CJT-L-r18

Way forward:

- Option 1: Set paramCombination-CJT-L-r18 as 7 ({4, 4})

- Option 2: Set paramCombination-CJT-L-r18 as 4 ({2, 2})

- Other options are not precluded.

Issue 2-2-5: paramCombination-CJT-r18

Way forward:

- Option 1: Set paramCombination-CJT-r18 as 4 (pν=14and18, β=12) or 7 (pν=12and12, β=12) for paramCombination-CJT-L-r18 = 7

- Option 2: Set paramCombination-CJT-r18 as 1 (pν=18, β=14) for paramCombination-CJT-L-r18 = 4

- Other options are not precluded.

4. R18 TypeII CJT

We support Option2 of issue 2-2-4 since L= 4 has lower UE complexity. With L=4, Option2 of Issue 2-2-5 is the only feasible option.

Proposal6: RAN4 to set paramCombination-CJT-L-r18 as 4 ({2, 2}).

## 5. Conclusions

Following is the summary of the proposals:

Proposal1: RAN4 to use Rel-18 DMRS ports {1000, 1001, 1008, 1009}  for Rank4 case.

Proposal2: RAN4 to reuse Test 4-1 of clause 5.2.3.2.1 and clause 5.2.3.1.1 of TS 38.101-4.

Proposal3: RAN4 to define PDSCH demod. requirements for only 4Rx.

Proposal4: RAN4 to use new value according to latest simulation results.

Proposal5: RAN4 to use TDLA30-50 for TypeII doppler PMI requirements.

Proposal6: RAN4 to set paramCombination-CJT-L-r18 as 4 ({2, 2}).

# References

- WF R4-2321141, “WF on  NR_MIMO_evo_DL_UL_demod”, 3GPP TSG RANWG4 meeting, #109, Nov 13 –  Nov 17, 2023, Xiamen, China.

- R4- 2320230, “Discussion on BS demodulation requirements for MIMO evolution”, 3GPP TSG RANWG4 meeting, #109, Nov 13 –  Nov 17, 2023, Xiamen, China.