# 3GPP TSG-RAN WG4 Meeting #110                                                              R4-2400823

# Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR | 0057 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.106 updating reference measurement channels |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Core |  |  |  |  |  | Date: |  |  | 2024-2-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For FR1, in #108 meeting, the agreement of FRC for FR1 NCR-MT is listed as below: - Agreement: - For LA NCR-MT: reusing legacy UE assumption on NF - Use the revised Table 2-1 for FRC parameters below for FR1 NCR-MT - Table 2-1: FRC parameters for FR1 reference sensitivity level for NCR-MT. In above agreements, there is no clear statement of whether the FRC is applicable only for FR1 WA NCR-MT or only for LA NCR-MT or both for WA and LA NCR-MT. From operators point of view, both WA and LA NCR-MT should support all operation bands including FDD and TDD. For FDD bands, 15kHz SCS is the common choice for operators. Therefore, 15kHz SCS is also necessary for both WA and LA NCR-MT FRC. However, current WA NCR-MT reuse the same FRC as IAB-MT which doesn’t consider the 15kHz SCS case. Therefore, the FRC for FR1 WA NCR-MT should be updated. Accordingly, 15kHz SCS conducted REFSENSE should also be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the FRC for conducted REFSENSE, ACS, in-band blocking, out-of-band blocking and receiver intermodulation requirements for FR1 WA NCR-MT, i.e. new annex C.1. - Add the conducted REFSENSE with 15kHz SCS for FR1 WA NCR-MT. i.e. 6.16. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Fixed reference channels of 15kHz SCS for WA NCR-MT are missing. Conducted REFSENSE of 15kHz SCS for WA NCR-MT is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.16, 6.18, 6.19, 6.21, and C.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... TS 38.115-1/2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This formal CR based on the draft CR R4-2311803 which was endorsed in RAN4 #108 meeting. |  |  |  |  |  |  |  |  |

<< Start of change >>

6.16.1	NCR-MT reference sensitivity level

6.16.1.1	General

The reference sensitivity power level PREFSENS is the minimum mean power received at the antenna connector for NCR-MT type 1-C or TAB connector for NCR-MT type 1-H at which a throughput requirement shall be met for a specified reference measurement channel.

6.16.1.2	Minimum requirements for NCR-MT type 1-C and 1-H

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex C.1.

The Wide Area NCR-MT reference sensitivity level is specified is table 6.16.1.2-1.

Table 6.16.1.2-1: Wide Area NCR-MT reference sensitivity levels

| NCR-MT channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 5,10,15 | 15 | G-FR1-A1-21 (Note 1) | -101.5 |
| 10, 15 | 30 | G-FR1-A1-22 (Note 1) | -102.0 |
| 10, 15 | 60 | G-FR1-A1-23 (Note 1) | -99.0 |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-24 (Note 1) | -95.2 |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-25 (Note 1) | -95.4 |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-26 (Note 1) | -95.6 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full NCR-MT channel bandwidth. |  |  |  |

The Local Area NCR-MT reference sensitivity level is specified the same as specified in TS 38.101-1 [13] clause 7.3.

Referenced requirements applying to NB IoT are not applicable to the NCR-MT.

<< Next of change >>

6.18.1	Adjacent Channel Selectivity (ACS)

6.18.1.1	General

Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency at the antenna connector for NCR-MT type 1-C or TAB connector for NCR-MT type 1-H in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

6.18.1.2	Minimum requirement for NCR-MT type 1-C

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex C.1. For Wide Area NCR-MT type 1-C, minimum requirement is the same as specified for BS type 1-C in TS 38.104 [2], clause 7.4.1.2.

For Local Area NCR-MT type 1-C, minimum requirement is the same as specified in TS 38.101-1 [13], clause 7.3.2.

6.18.1.3	Minimum requirement for NCR-MT type 1-H

For Wide Area NCR-MT type 1-H, minimum requirement is the same as specified for IAB-MT type 1-H in TS 38.174 [22], clause 7.4.1.3.

For Local Area NCR-MT type 1-H, minimum requirement is the same as specified in TS 38.101-1 [13], clause 7.3.2.

<< Next of change >>

6.19	Conducted blocking characteristics

6.19.1	General

The in-band blocking characteristics is a measure of the receiver's ability to receive a wanted signal at its assigned channel at the TAB connector for NCR-MT type 1-C and NCR-MT type 1-H in the presence of an unwanted interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

6.19.2	Minimum requirement for NCR-MT type 1-C

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex C.1. For Wide Area NCR-MT type 1-C, minimum requirement is the same as specified for BS type 1-C in TS 38.104 [2], clause 7.4.2.2.

For Local Area NCR-MT type 1-C, minimum requirement is the same as specified in TS 38.101-1 [13] clause 7.6.2.

6.19.3	Minimum requirement for NCR-MT type 1-H

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex C.1. For Wide Area NCR-MT type 1-H, minimum requirement at TAB connector is the same as specified for IAB-MT in TS 38.174 [22], clause 7.4.2.3.

For Local Area NCR-MT type 1-H, minimum requirement at TAB connector is the same as specified in TS 38.101-1 [13], clause 7.6.2.

<< Next of change >>

6.21	Conducted intermodulation characteristics

6.21.1	General

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency at the antenna connector for NCR-MT type 1-C or TAB connector for NCR-MT type 1-H in the presence of two interfering signals which have a specific frequency relationship to the wanted signal.

6.21.2	Minimum requirement for NCR-MT type 1-C

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex C.1. For Wide Area NCR-MT type 1-C, minimum requirement at antenna connector is the same as specified for BS type 1-C in TS 38.104 [2], clause 7.7.2.

For Local Area NCR-MT type 1-C, minimum requirement at antenna connector is the same as specified in TS 38.101-1 [13], clause 7.8.2.

6.21.3	Minimum requirement for NCR-MT type 1-H

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex C.1. For Wide Area NCR-MT type 1-H, minimum requirement at TAB connector is the same as specified for IAB-MT type 1-H in TS 38.174 [22], clause 7.7.3.

For Local Area NCR-MT type 1-H, minimum requirement at TAB connector is the same as specified in TS 38.101-1 [13], clause 7.8.2.

<< Next of change >>

C.1	Fixed Reference Channels for reference sensitivity level, ACS, in-band blocking, out-of-band blocking and receiver intermodulation (QPSK, R=1/3)

The parameters for the FR1 WA NCR-MT reference measurement channels are specified in tables C.1-1 for FR1 reference sensitivity level, ACS, in-band blocking, out-of-band blocking, receiver intermodulation.

Table C.1-1: FRC parameters for FR1 reference sensitivity level for FR1 WA NCR-MT.

| Reference channel | G-FR1-A1-21 | G-FR1-A1-22 | G-FR1-A1-23 | G-FR1-A1-24 | G-FR1-A1-25 | G-FR1-A1-26 |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 30 | 60 | 15 | 30 | 60 |
| Allocated resource blocks | 25 | 11 | 11 | 106 | 51 | 24 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 | 9 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| NOTE 1: DL-DMRS-config-type = 1 with DL-DMRS-max-len = 1, DL-DMRS-add-pos = pos2 with = 2, = 6 and 9 as per Table 7.4.1.1.2-3 of TS 38.211 [3]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size for receiver sensitivity |  |  |  |  |  |  |

<< End of change >>