3GPP TSG-RAN WG4 Meeting # 110 											R4-2400835

Athens, GR, Feb 26 - Mar 1, 2024

Agenda Item:	8.9.5

Source: 	CMCC

Title: 			 (NR_ATG-Core) Discussion on the remaining FFS UE features for NR ATG

Document for:	Discussion

- Introduction

The UE feature has been discussed in last meeting, the WF was approved in [1]. Three UE features are still FFS, which need to be further discussed in the maintenance phase.

| Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location-based measurement report trigger | Indicates whether the UE supports location-based triggered measurement reporting (i.e., event D1) |  | Yes | N/A | If UE does not support this feature, location-based triggered measurement reporting cannot be configured. | [Per Band] | No | FR1 only | N/A |  | Optional with capability siganling |
| SR triggered by a TA report | Indicates whether the UE supports triggering of SR when a TA report is triggered and there are no available UL-SCH resources. |  | Yes | N/A | If UE does not support this feature, SR cannot be triggered by a TA report. | [Per Band] | No | FR1 only | N/A |  | Optional with capability siganling |
| TA reporting during initial access | It is mandatory to support TA reporting during initial access for UEs supporting uplink-TA-Reporting-r17 as specified in TS 38.321 [10]. |  | Yes | N/A | If UE does not support this feature, TA reporting during initial access cannot be configured. | [Per Band] | No | FR1 only | N/A |  | Optional with capability siganling |

In this contribution, we provide our views on the introduction of above UE features.

- ATG UE feature

L2/L3 ATG UE feature: Location-based measurement report trigger

In NR NTN, due to severe near-far effect, the traditional RSRP based measurement report trigger may cause UE report the measurement result earlier or later. If UE report the measurement result earlier, then unnecessary power consumption will be caused. If UE report the measurement results later, then UE mobility may be affected. Therefore, the location-based measurement report trigger was introduced in NR NTN to compensate the near-far effect.

In ATG scenario, the power saving is not the bottleneck of ATG UE. The UE mobility effects can be eliminated by network configuration with higher or lower thresholds (depends on specific events), which means that the location-based measurement report trigger in ATG is not as essential as it in NTN. We are fine to not introduce such UE capability in ATG.

Proposal 1: No strong demand of introducing the UE feature ‘Location-based measurement report trigger’ .

L2/L3 ATG UE feature: SR triggered by a TA report

Regarding the UE feature ‘ SR triggered by a TA report’, RAN2 has agreed to introduce such feature, the related signaling and description has been introduced in TS38.306 and TS38.331 as shown below. We think we should follow RAN2’s agreement.

Proposal 2: For the UE feature of ‘SR triggered by a TA report’, follow the agreement from RAN2 that introducing such feature for ATG.

L2/L3 ATG UE feature: TA reporting during initial access

Regarding the UE feature ‘TA reporting during initial access’, since TA reporting in ATG is mainly for addressing the throughput loss issue due to long guard period in ATG TDD network, we think the mandatory feature as in NTN is not necessary for ATG.

Due to the data transmission may be not very urgent after the initial access, the limited TP loss is tolerable, we are ok not to introduce any UE feature about ‘TA reporting during initial access’.

Proposal 3: No strong demand of introducing the UE feature ‘TA reporting during initial access’.

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: No strong demand of introducing the UE feature ‘Location-based measurement report trigger’ .

Proposal 2: For the UE feature of ‘SR triggered by a TA report’, follow the agreement from RAN2 that introducing such feature for ATG.

Proposal 3: No strong demand of introducing the UE feature ‘TA reporting during initial access’.

- Reference

- R4-2321608, WF on NR ATG RRM requirements, CMCC