3GPP TSG-RAN WG4 Meeting # 110		R4-2400837

Athens, Greece, Feb 26 - Mar 1, 2024

Agenda Item:	8.9.6

Source: 	CMCC

Title: 			 Discussion on RRM performance requirements for ATG

Document for:	Discussion

- Introduction

In last meeting, the test case list and test configurations were discussed, the WF has been approved in [1]. In this contribution, we will provide our views on test method of ATG UE with antenna array and so on.

- Discussion

  - ATG RRM performance requirement

Issue 2-1: TDD pattern

Agreement:

- The legacy TDD pattern are used as baseline TDD pattern for all ATG test cases.

- In Section A.3.1.4, define a new TDD configuration with the pattern of  ‘30D4S6U’ for ATG UE.

- Introduce the new TDD pattern‘30D4S6U’in one or more ATG test cases. FFS which test cases.

- In the test, the new TDD pattern configuration only applies to UE supporting increasing the number of HARQ processes and K1 range extension.

- If UE pass the test cases with new TDD pattern, the same test cases with legacy TDD pattern can be skipped.

In last meeting, RAN4 agreed to choose one or more ATG test cases to verify the new TDD pattern ‘30D4S6U’. Considering most of RRM requirements are not sensitive to TDD pattern configuration, we propose to only choose one test case, timing advance adjustment delay and accuracy test case can be considered.

Proposal 1: Introduce the new TDD pattern‘30D4S6U’in ‘timing advance adjustment delay and accuracy’ test case.

Issue 2-5: UE mobility assumption

Agreement:

- GNSS changed during the test for location-based CHO.

- FFS on the UL transmit timing test

  - Option 1: Constant GNSS

  - Option 2: Changed GNSS

- FFS how to set the GNSS change,

  - Option 1: 1200km/h.

  - Option 2: modeled using the Doppler shift

  - Option 3: The GNSS change should be set with the consideration of two distance threshold istanceThreshFromReference1 and distanceThreshFromReference2, e.g. max{distanceThreshFromReference1,distanceThreshFromReference2}+50m.

When RAN4 defined the core requirements for UL transmit timing, RAN4 assumed ATG UE mobility as 1200km/h, and further relaxed the gradual timing adjustment requirements to Tp_ATG/Tq_ATG. Therefore, the related UE behavior that UE could perform the gradual timing adjustment with requirements Tp_ATG/Tq_ATG to compensate the additional timing drift introduced by ATG UE mobility 1200km/h should be verified in RRM tests, GNSS should be set as changed.

Proposal 2: Set changed GNSS in the UL transmit timing test cases.

Regarding how to set GNSS change, for UL transmit timing test case, the test system should emulate and send the GNSS signal to the test UE with mobility assumption of 1200km/h. The specific positioning of test UE is up to test system’s design.

Proposal 3: For UL transmit timing test case, The test system should emulate and send the GNSS signal to the test UE with mobility assumption of 1200km/h. The specific positioning of test UE is up to test system’s design.

For location-based CHO test case, we think Option 1 and Option 3 are not conflict options. In order to design the test procedure and guarantee the location condition can be triggered at certain time node, RAN4 is responsible to design the initial UE positioning, UE speed, reference location, and distance threshold.

There are two cases should be verified, which are:

- Case 1: condition D1-1 and D1-2 occurs earlier than TEvent_DU + Tidentify_intra

- Case 2: condition T1-1 occurs later than TEvent_DU + Tidentify_intra

To save the test efforts, we propose to verify the Case 1 in intra-frequency distance-based CHO, and to verify the Case 2 in inter-frequency distance-based CHO.

We give an example for test parameter configuration in Table 1, D1-1 and D1-2 Location condition can be fulfilled at T2 (earlier than TEvent_DU + Tidentify_intra).

Table 1: An example for location-based CHO specific parameters configuration

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| UE position (N,S,H) at T1 start | m | [(0, 0, 3000)] | Set by AT command |
| UE moving speed | km/h | [(1200, 0, 0)] | Set by AT command |
| referenceLocation1-r17.condEventD1-r17 | m | [(-4600, 0, 0)] | Reference location for serving cell |
| referenceLocation2-r17.condEventD1-r17 | m | [(14479, 0, 0)] | Reference location for target cell |
| distanceThreshFromReference1-r17.condEventD1-r17 | 50m | [200] | D1-1 Location condition is fulfilled at T2 |
| distanceThreshFromReference2-r17.condEventD1-r17 | 50m | [200] | D1-2 Location condition is fulfilled at T2 |
| hysteresisLocation-r17.condEventD1-r17 | 10m | 0 |  |
| timeToTrigger-r17.condEventD1-r17 | s | 0 |  |
| T1 | s | 15 |  |
| T2 | s | 6 |  |

Proposal 4: For location-based CHO test case, RAN4 is responsible to design the initial UE positioning, UE speed, reference location, and distance threshold.

Proposal 5:

- In the intra-frequency distance-based CHO test case, D1-1 and D1-2 location condition can be fulfilled at T2 (earlier than TEvent_DU + Tidentify_intra).

- In the inter-frequency distance-based CHO test case, D1-1 and D1-2 location condition can be fulfilled later than T2 (later than TEvent_DU + Tidentify_intra).

Besides, Option 2 also make sense in both UL transmit timing and location-based CHO test cases. The channel model should also consider the doppler caused by UE mobility, which should be AWGN + xx doppler, the xx doppler is the maximum value of doppler. The specific doppler shift trajectory could up to test system’s design considering of BS location and UE GNSS emulation. In the case of 15kHz SCS, we propose to use 2170MHz as center frequency, the maximum doppler is 2412Hz, in the case of 30kHz SCS, we propose to use 5GHz as center frequency, the maximum doppler is 5556Hz.

Proposal 5: In both UL transmit timing and location-based CHO test case, the channel model should be AWGN+2412Hz for 15kHz SCS and AWGN + 5556Hz for 30kHz SCS, which 2412Hz and 5556Hz are the maximum value of doppler. The specific doppler shift trajectory is up to test system’s design considering of BS location and UE GNSS emulation.

Issue 2-6: Test method for UE with antenna array

Proposals

- Option 1: The approach of only to introduce the scaling factor in the RRM core requirement and not to have the scaling factor in the tests is more simpler and cheaper. (CATT)

- Option 2: Conducted test should be reused for ATG UE with antenna array, the scaling factor need to be considered in the test requirements. (CMCC, LGE, HW)

- Option 3: RAN4 to consult RAN5 feedback on whether it is feasibility to define test cases for ATG UEs in FR1 with beam sweeping capability. (Ericsson)

- Option 4: Two alternatives for the test method with antenna array assumption: (ZTE)

  - Alternative 1: Define OTA test to verify beam sweeping for ATG UE with phase antenna array capability. (CATT open to discuss the feasibility)

  - Alternative 2: Not to distinguish the test between conductive test and OTA test, only focus on the requirements of delay. Leave the test details to actual implementation.

Currently, OTA is not feasible for FR1 UE test. Besides, ATG RF didn’t define the OTA requirements and test methods. Therefore, we prefer to use conducted test for ATG UE with antenna array. In conducted test, due to UE may doesn’t know that whether it is under test or in real network, UE will still follow the core requirements that generate the measurement results every identification/evaluation/measurement period, which is scaled for ATG UE with antenna array.

Proposal 6: Conducted test should be reused for ATG UE with antenna array, the scaling factor need to be considered in the test requirements.

Issue 2-7: Neighbour cell configuration

Proposals

- Option 1: For the TDD cell re-selection test and and intra/inter-frequency measurement test for UE with antenna array, choose some test cases to configure 2 neighbour cells.

Our intention of introducing 2 neighbour cells in the test is to verify the UE behavior of beam sweeping. However, if conducted test will be adopted, then such UE behavior could not be tested. We are fine to only introduce one neighbour cell in conducted test.

Besides, in the conducted test, the UE could measure the neighbour cell directly based on configured SMTC, such UE behavior is more like the UE behavior in practical network with the neighbour cell assistance location information. Therefore, in the test case, we think the neighbour cell assistance location information should be provided to UE, and the test requirement should use the related scaling factor.

Proposal 7: If conducted test is used for antenna array UE, we are fine to only design one neighbour cell in the test .

Proposal 8: In the test case with neighbour cell measurement, the network assistance on ATG cells reference location should be provided, and related scaling factor should be used in the test requirement.

BS location

In the 38.331, two parameters are related with BS location, which are:

atg-gNB-Location-r18                 ReferenceLocation-r17       OPTIONAL,    -- Need R

heightgNB-r18                         INTEGER (-16384...16383)    OPTIONAL,     -- Need R

Firstly, the height of gNB can be 0m for every test cases as a common parameter.

Secondly, the specific gNB location (Referencelocation) should be emulated by test system, which could consider the gNB location and UE location simultaneously.

Proposal 9: The BS location should be emulated by test system, with the common height of 0m.

cellSpecificKoffset

The cell specific koffset can be configured as a common parameter. The upper bound 3ms can be considered to cover all ATG deployments. The value could be FFS based on RAN2’s signalling design.

Proposal 10: The cell specific koffset can be configured as a common parameter. The upper bound 3ms can be considered when RAN4 define the specific value.

Issue 2-10: Test scope

Proposals

- Option 1: All the tests listed in last meeting’s agreement should be applied for ATG UE. (CMCC, ZTE)

- Option 2: Test case list presented in R4-2317341 is adopted for ATG performance requirements with following changes: (Ericsson)

  - Only non-DRX mode tests are defined in CONNECTED mode.

  - For HO: only inter-frequency handover with unknown target cell is defined.

  - For RRC re-establishment, following tests are defined:

    - Intra-frequency RRC Re-establishment with known target cell

    - Inter-frequency RRC Re-establishment with unknown target cell

- The tests listed in last meeting’s agreement:

| Test scope RRC_IDLE state mobility | Test number |
| --- | --- |
| Cell reselection to intra-frequency neighbour cell | ATG 1-1 |
| Location based cell reselection to intra-frequency neighbour cell | ATG 1-2 |
| Cell reselection to inter-frequency neighbour cell | ATG 1-3 |
| Cell reselection to inter-frequency NR cell for UE configured with [enhanced requirements] | ATG 1-4 |
| Location based cell reselection to inter-frequency neighbour cell | ATG 1-5 |

| Test scope RRC_CONNECTED state mobility | Test number |
| --- | --- |
| Intra-frequency HO with known target cell | ATG 2-1 |
| Intra-frequency HO with unknown target cell | ATG 2-2 |
| Inter-frequency HO with known target cell | ATG 2-3 |
| Inter-frequency HO with unknown target cell | ATG 2-4 |
| Intra-frequency location based CHO | ATG 2-5 |
| Inter-frequency location based CHO | ATG 2-6 |
| Intra-frequency RRC Re-establishment with known target cell | ATG 2-7 |
| Intra-frequency RRC Re-establishment with unknown target cell | ATG 2-8 |
| Inter-frequency RRC Re-establishment with known target cell | ATG 2-9 |
| Inter-frequency RRC Re-establishment with unknown target cell | ATG 2-10 |
| Inter-frequency RRC Re-establishment in FR1 without serving cell timing | ATG 2-11 |
| 4-step RA type contention based random access test | ATG 2-12 |
| 4-step RA type Non-Contention based random access test | ATG 2-13 |
| 2-step RA type contention based random access test | ATG 2-14 |
| 2-step RA type Non-Contention based random access test | ATG 2-15 |
| RRC Connection Release with Redirection | ATG 2-16 |

| Test scope Timing | Test number |
| --- | --- |
| UE transmit timing test | ATG 3-1 |
| Timing advance adjustment delay and accuracy | ATG 3-2 |

| Test scope Signalling characteristics | Test number |
| --- | --- |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode | ATG 4-1 |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode | ATG 4-2 |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode | ATG 4-3 |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode | ATG 4-4 |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode | ATG 4-5 |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode | ATG 4-6 |
| DCI-based and Timer-based Active BWP Switch | ATG 4-7 |
| RRC-based Active BWP Switch | ATG 4-8 |
| UE specific CBW change | ATG 4-9 |
| Pathloss reference signal switching delay | ATG 4-10 |

| Test scope Measurement procedure | Test number |
| --- | --- |
| Intra-frequency measurements event triggered reporting tests without gap under non-DRX | ATG 5-1 |
| Intra-frequency measurements event triggered reporting tests with per-UE gaps under non-DRX | ATG 5-2 |
| Intra-frequency measurements event triggered reporting tests without gap under non-DRX with SSB index reading | ATG 5-3 |
| Intra-frequency measurements SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | ATG 5-4 |
| Inter-frequency measurements event triggered reporting tests for FR1 without SSB time index detection when DRX is not used | ATG 5-5 |
| Inter-frequency measurements event triggered reporting tests for FR1 with SSB time index detection when DRX is not used | ATG 5-6 |
| Inter-frequency measurements event triggered reporting tests for FR1 without gap when DRX is not used | ATG 5-7 |
| SSB based L1-RSRP measurement when DRX is not used | ATG 5-8 |
| CSI-RS based L1-RSRP measurement when DRX is not used | ATG 5-9 |
| L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is not used | ATG 5-10 |
| L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used | ATG 5-11 |
| L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used | ATG 5-12 |
| SA intra-frequency CGI identification of NR neighbor cell in FR1 | ATG 5-13 |

| Test scope Measurement Performance requirements | Test number |
| --- | --- |
| SS-RSRP intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-1 |
| SS-RSRP inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-2 |
| SS-RSRQ intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-3 |
| SS-RSRQ inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-4 |
| SS-SINR intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-5 |
| SS-SINR inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-6 |
| L1-RSRP measurement SSB based L1-RSRP measurement | ATG 6-7 |
| L1-RSRP measurement CSI-RS based L1-RSRP measurement on resource set with repetition off | ATG 6-8 |
| L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off | ATG 6-9 |
| L1-SINR measurement with SSB based CMR and dedicated IMR | ATG 6-10 |
| L1-SINR measurement with CSI-RS based CMR and dedicated IMR | ATG 6-11 |
| CSI-RSRP intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-12 |
| CSI-RSRP inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-13 |
| CSI-RSRQ Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-14 |
| CSI-RSRQ Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-15 |
| CSI-SINR intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-16 |
| CSI-SINR Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-17 |

Some updates should be made based on the table above.

Firstly, since location-based cell re-selection is not introduced in the core requirements, the corresponding test cases (ATG 1-2 and ATG 1-5) should be removed.

Secondly, to verify the UE behavior with less test effort, we are fine to only consider unknown case for handover and RRC Re-establishment test case.

Last, consider the common network deployment scenario, we think there is no need to design the inter-frequency cell re-selection test which neighbour cell configured with high priority frequency.

Proposal 11: The location based cell re-selection test cases should be removed.

Proposal 12: Only consider unknown case for handover and RRC Re-establishment test case.

Proposal 13: No need to design the inter-frequency cell re-selection test which neighbour cell configured with high priority frequency.

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: Introduce the new TDD pattern‘30D4S6U’in ‘timing advance adjustment delay and accuracy’ test case.

Proposal 2: Set changed GNSS in the UL transmit timing test cases.

Proposal 3: For UL transmit timing test case, The test system should emulate and send the GNSS signal to the test UE with mobility assumption of 1200km/h. The specific positioning of test UE is up to test system’s design.

Proposal 4: For location-based CHO test case, RAN4 is responsible to design the initial UE positioning, UE speed, reference location, and distance threshold.

Proposal 5:

- In the intra-frequency distance-based CHO test case, D1-1 and D1-2 location condition can be fulfilled at T2 (earlier than TEvent_DU + Tidentify_intra).

- In the inter-frequency distance-based CHO test case, D1-1 and D1-2 location condition can be fulfilled later than T2 (later than TEvent_DU + Tidentify_intra).

Proposal 5: In both UL transmit timing and location-based CHO test case, the channel model should be AWGN+2412Hz for 15kHz SCS and AWGN + 5556Hz for 30kHz SCS, which 2412Hz and 5556Hz are the maximum value of doppler. The specific doppler shift trajectory is up to test system’s design considering of BS location and UE GNSS emulation.

Proposal 6: Conducted test should be reused for ATG UE with antenna array, the scaling factor need to be considered in the test requirements.

Proposal 7: If conducted test is used for antenna array UE, we are fine to only design one neighbour cell in the test .

Proposal 8: In the test case with neighbour cell measurement, the network assistance on ATG cells reference location should be provided, and related scaling factor should be used in the test requirement.

Proposal 9: The BS location should be emulated by test system, with the common height of 0m.

Proposal 10: The cell specific koffset can be configured as a common parameter. The upper bound 3ms can be considered when RAN4 define the specific value.

Proposal 11: The location based cell re-selection test cases should be removed.

Proposal 12: Only consider unknown case for handover and RRC Re-establishment test case.

Proposal 13: No need to design the inter-frequency cell re-selection test which neighbour cell configured with high priority frequency.

- Reference

- R4-2321608, WF on RRM requirements for NR ATG, CMCC

- R4-2317341, WF on RRM requirements for NR ATG, CMCC