3GPP TSG-RAN4 Meeting #110	R4-2400849

# Athens, Greece, Feb 26 - Mar 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | - | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (IoT_NTN_enh-Core) CR to TS 36.133: Correction of Cat-M1 conditional HO for IOT-NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-1-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The TEvent_DU doesn’t cover the cases of time and location only based CHO. - The measurement time for time and location-based CHO for eMTC over IOT-NTN doesn’t cover the following cases: - condition T1-1/D1-1,D1-2 occurs earlier/later than TEvent_DU + Tidentify inter/intra /TMeasurement_Period, Intra/Inter - Known cell and unknown cell are introduced in R18 IOT-NTN enh WI to determine the value of Tsearch. The concept of known cell and unknown cell before R18 IOT-NTN enh WI is not needed. - The introduction under 5.5A.3.3 should be CEModeB instead of CEModeA - Some editorial error and typo exists. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the TEvent_DU to cover the cases of time and location only based CHO. - Introduce following cases when defining the measurement time for IOT-NTN eMTC time and location-based CHO. - condition T1-1/D1-1,D1-2 occurs earlier/later than TEvent_DU + Tidentify inter/intra /TMeasurement_Period, Intra/Inter - In the interruption time, delete the concept of known cell and unknown cell which was introduced before R18 IOT-NTN enh WI. - Revise the “CEModeA” to “CEModeB” under 5.5A.3.3 - Refine some editorial error and typos. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For IOT-NTN Cat M1, the time based CHO and location based CHO requirements are not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.2.3, 5.5A.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change#1>

5.5A.2.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handovers for a Cat-M1 FDD UE in CEModeA.

5.5A.2.3.1	Handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 36.331 [2].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

Dhandover = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tinterrupt

Where:

-	TRRC is the maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [2].

-	If only condEventT1 is configured, then TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until the condition T1-1 is fulfilled. If only condEventD1 is configured, then TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until the condition D1-1 and condition D1-2 are fulfilled. Otherwise, TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until a condition exists at the measurement reference point which will trigger the conditional handover.

-	Tmeasure is the measurements time stated in clause 5.5A.2.3.2.

-	TCHO_execution is the conditional execution preparation time in clause 5.5A.2.3.3.

-	Tinterrupt is the interruption time stated in clause 5.5A.2.3.4.

5.5A.2.3.2	Measurement time

The measurement time delay Tmeasure is defined as the time period from the end of TEvent_DU until the UE begins the preparation time for handover execution. If only condEventD1 or condEventT1 is configured, then Tmeasure = 0.

The measurement event evaluation delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra defined in clause 8.13A.2.1 for intra-frequency conditional handover or Tidentify inter defined in clause 8.13A.2.2 for inter-frequency conditional handover.

For time-based conditional handover:

For intra-frequency conditional handover:

-	If condition T1-1 occurs earlier than TEvent_DU + Tidentify_intra defined in clause 8.13A.2.1, then the measurement time delay equal to Tidentify_intra defined in clause 8.13A.2.1.

-	If condition T1-1 occurs later than TEvent_DU + Tidentify_intra defined in clause 8.13A.2.1, then the measurement time delay equals to the time from the end of Tevent_DU until condition T1-1.

For inter-frequency conditional handover:

-	If condition T1-1 occurs earlier than TEvent_DU + Tidentify_inter defined in clause 8.13A.2.2, then the measurement time delay equal to Tidentify_inter defined in clause 8.13A.2.2.

-	If condition T1-1 occurs later than TEvent_DU + Tidentify_inter defined in clause 8.13A.2.2, then the measurement time delay equals to the time from the end of Tevent_DU until condition T1-1.

For location-based conditional handover:

For intra-frequency conditional handover:

-	If both condition D1-1 and condition D1-2 are fulfilled earlier than TEvent_DU + Tidentify_intra defined in clause 8.13A.2.1, then the measurement time delay equal to Tidentify_intra defined in clause 8.13A.2.1.

-	If both condition D1-1 and condition D1-2 are fulfilled is later than TEvent_DU + Tidentify_intra defined in clause 8.13A.2.1, then the measurement time delay equal to the time from the end of Tevent_DU until time when both condition D1-1 and condition D1-2 are fulfilled.

For inter-frequency conditional handover:

-	If both condition D1-1 and condition D1-2 are fulfilled earlier than TEvent_DU + Tidentify_inter defined in clause 8.13A.2.2, then the measurement time delay equal to Tidentify_inter defined in clause 8.13A.2.2.

-	If both condition D1-1 and condition D1-2 are fulfilled is later than TEvent_DU + Tidentify_inter defined in clause 8.13A.2.2, then the measurement time delay equal to the time from the end of Tevent_DU until time when both condition D1-1 and condition D1-2 are fulfilled.

When TTT or L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra defined in clause 8.13A.2.1 or Tidentify inter defined in clause 8.13A.2.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TMeasurement_Period, Intra or TMeasurement_Period, Inter provided the timing to that cell has not changed more than ± 50 Ts and the L3 filter has not been used.

For time-based conditional handover:

For intra-frequency conditional handover:

-	If condition T1-1 occurs earlier than TEvent_DU + TMeasurement_Period, Intra , then the measurement time delay equal to TMeasurement_Period, Intra.

-	If condition T1-1 occurs later than TEvent_DU + TMeasurement_Period, Intra, then the measurement time delay equals to the time from the end of Tevent_DU until condition T1-1.

For inter-frequency conditional handover:

-	If condition T1-1 occurs earlier than TEvent_DU + TMeasurement_Period, Inter, then the measurement time delay equal to TMeasurement_Period, Inter.

-	If condition T1-1 occurs later than TEvent_DU + TMeasurement_Period, Inter, then the measurement time delay equals to the time from the end of Tevent_DU until condition T1-1.

For location-based conditional handover:

For intra-frequency conditional handover:

-	If both condition D1-1 and condition D1-2 are fulfilled earlier than TEvent_DU + TMeasurement_Period, Intra, then the measurement time delay equal to TMeasurement_Period, Intra.

-	If both condition D1-1 and condition D1-2 are fulfilled is later than TEvent_DU + TMeasurement_Period, Intra, then the measurement time delay equal to the time from the end of Tevent_DU until time when both condition D1-1 and condition D1-2 are fulfilled.

For inter-frequency conditional handover:

-	If both condition D1-1 and condition D1-2 are fulfilled earlier than TEvent_DU + TMeasurement_Period, Inter, then the measurement time delay equal to TMeasurement_Period, Inter.

-	If both condition D1-1 and condition D1-2 are fulfilled is later than TEvent_DU + TMeasurement_Period, Inter, then the measurement time delay equal to the time from the end of Tevent_DU until time when both condition D1-1 and condition D1-2 are fulfilled.

When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

5.5A.2.3.3	Preparation time

Preparation time is the time when the UE prepares the target cell for conditional handover. It begins after measurement time.

-	TCHO_execution is the time needed for preparing the conditional handover to the target cell.

-	TCHO_execution can be up to 10 ms.

5.5A.2.3.4	Interruption time

The interruption time is the time between when the UE completes the preparation time TCHO_execution and the time when the UE starts the transmission of the PRACH to the target cell.

When intra-frequency conditional handover or inter-frequency conditional handover is commanded and the field sameSFN-Indication and mib-RepetitionStatus [2] are included in the handover command then the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + 20 ms

When intra-frequency contional handover or inter-frequency conditional handover is commanded and the field sameSFN-Indication or mib-RepetitionStatus [2] is not included in the handover command then the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TMIB + TIU + 20 ms

Where:

-	TIU is the time required to complete the transmission of PRACH in the target cell. The actual value of TIU shall depend upon the uncertainty in acquiring the first available PRACH occasion based on the PRACH configuration used in the target cell and the PRACH coverage enhancement level used by the UE for sending the random access preamble to the target cell.

-	Tsearch is the time required to search the target cell. If the target cell is known, then Tsearch = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = 80 ms. Otherwise, Tsearch shall be according to the non-DRX cell identification requirements specified in Clause 8.13A.2.1 with KSatellite_intra = 1 for intra-frequency handover for a UE configured with CEModeA or Tsearch shall be according to the non-DRX cell identification requirements specified in Clause 8.13A.2.2 with Ksatellite_inter_i =1 and Nfreq = 1 for inter-frequency handover for a UE configured with CEModeA. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

-	In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement for a time duration equal or longer than the time duration required for the cell identification. Otherwise, it is unknown. For intra-frequency handover the time duration required for the cell identification is specified in relevant intra-frequency cell identification requirements with KSatellite_intra = 1 as described in Clause 8.13A.2.1 for CEModeA. For inter-frequency handover the time duration required for the cell identification is specified in relevant inter-frequency cell identification requirements as described in Clause 8.13A.2.2  with Ksatellite_inter_i  =1 and Nfreq = 1 for CEModeA.

-	TMIB is the time required for acquiring the MIB and SIB information of the target cell.

<End of change#1>

<Start of change#2>

5.5A.3.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handovers for a Cat-M1 FDD UE in CEModeB.

5.5A.3.3.1	Handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 36.331 [2].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

Dhandover = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tinterrupt

Where:

-	TRRC is the maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [2].

-	If only condEventT1 is configured, then TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until the condition T1-1 is fulfilled. If only condEventD1 is configured, then TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until the condition D1-1 and condition D1-2 are fulfilled. Otherwise, TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until a condition exists at the measurement reference point which will trigger the conditional handover.

-	Tmeasure is the measurements time stated in clause 5.5A.3.3.2.

-	TCHO_execution is the conditional execution preparation time in clause 5.5A.3.3.3.

-	Tinterrupt is the interruption time stated in clause 5.5A.3.3.4.

5.5A.3.3.2	Measurement time

The measurement time delay Tmeasure is defined as the time period from the end of TEvent_DU until the UE begins the preparation time for handover execution. If only condEventD1 or condEventT1 is configured, then Tmeasure = 0.

The measurement event evaluation delay measured without Time To Trigger (TTT) and L3 filtering shall be less than T identify intra defined in clause 8.13A.3.1 for intra-frequency conditional handover or Tidentify inter defined in clause 8.13A.3.2 for inter-frequency conditional handover.

For time-based conditional handover:

For intra-frequency conditional handover:

-	If condition T1-1 occurs earlier than TEvent_DU + Tidentify_intra defined in clause 8.13A.3.1, then the measurement time delay equal to Tidentify_intra defined in clause 8.13A.3.1.

-	If condition T1-1 occurs later than TEvent_DU + Tidentify_intra defined in clause 8.13A.3.1, then the measurement time delay equals to the time from the end of Tevent_DU until condition T1-1.

For inter-frequency conditional handover:

-	If condition T1-1 occurs earlier than TEvent_DU + Tidentify_inter defined in clause 8.13A.3.2, then the measurement time delay equal to Tidentify_inter defined in clause 8.13A.3.2.

-	If condition T1-1 occurs later than TEvent_DU + Tidentify_inter defined in clause 8.13A.3.2, then the measurement time delay equals to the time from the end of Tevent_DU until condition T1-1.

For location-based conditional handover:

For intra-frequency conditional handover:

-	If both condition D1-1 and condition D1-2 are fulfilled earlier than TEvent_DU + Tidentify_intra defined in clause 8.13A.3.1, then the measurement time delay equal to Tidentify_intra defined in clause 8.13A.3.1.

-	If both condition D1-1 and condition D1-2 are fulfilled is later than TEvent_DU plus Tidentify_intra defined in clause 8.13A.3.1, then the measurement time delay equal to the time from the end of Tevent_DU until time when both condition D1-1 and condition D1-2 are fulfilled.

For inter-frequency conditional handover:

-	If both condition D1-1 and condition D1-2 are fulfilled earlier than TEvent_DU + Tidentify_inter defined in clause 8.13A.3.2, then the measurement time delay equal to Tidentify_inter defined in clause 8.13A.3.2.

-	If both condition D1-1 and condition D1-2 are fulfilled is later than TEvent_DU plus Tidentify_inter defined in clause 8.13A.3.2, then the measurement time delay equal to the time from the end of Tevent_DU until time when both condition D1-1 and condition D1-2 are fulfilled.

When TTT or L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra defined in clause 8.13A.3.1 or Tidentify inter defined in clause 8.13A.3.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TMeasurement_Period, Intra or TMeasurement_Period, Inter provided the timing to that cell has not changed more than ± 50 Ts and the L3 filter has not been used.

For time-based conditional handover:

For intra-frequency conditional handover:

-	If condition T1-1 occurs earlier than TEvent_DU + TMeasurement_Period, Intra , then the measurement time delay equal to TMeasurement_Period, Intra.

-	If condition T1-1 occurs later than TEvent_DU + TMeasurement_Period, Intra, then the measurement time delay equals to the time from the end of Tevent_DU until condition T1-1.

For inter-frequency conditional handover:

-	If condition T1-1 occurs earlier than TEvent_DU + TMeasurement_Period, Inter, then the measurement time delay equal to TMeasurement_Period, Inter.

-	If condition T1-1 occurs later than TEvent_DU + TMeasurement_Period, Inter, then the measurement time delay equals to the time from the end of Tevent_DU until condition T1-1.

For location-based conditional handover:

For intra-frequency conditional handover:

-	If both condition D1-1 and condition D1-2 are fulfilled earlier than TEvent_DU + TMeasurement_Period, Intra, then the measurement time delay equal to TMeasurement_Period, Intra.

-	If both condition D1-1 and condition D1-2 are fulfilled is later than TEvent_DU + TMeasurement_Period, Intra, then the measurement time delay equal to the time from the end of Tevent_DU until time when both condition D1-1 and condition D1-2 are fulfilled.

For inter-frequency conditional handover:

-	If both condition D1-1 and condition D1-2 are fulfilled earlier than TEvent_DU + TMeasurement_Period, Inter, then the measurement time delay equal to TMeasurement_Period, Inter.

-	If both condition D1-1 and condition D1-2 are fulfilled is later than TEvent_DU + TMeasurement_Period, Inter, then the measurement time delay equal to the time from the end of Tevent_DU until time when both condition D1-1 and condition D1-2 are fulfilled.

When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

5.5A.3.3.3	Preparation time

Preparation time is the time when the UE prepares the target cell for conditional handover. It begins after measurement time.

TCHO_execution is the time needed for preparing the conditional handover to the target cell.

TCHO_execution can be up to 10 ms.

5.5A.3.3.4	Interruption time

The interruption time is the time between when the UE completes the preparation time TCHO_execution and the time when the UE starts the transmission of the PRACH to the target cell.

When intra-frequency conditional handover or inter-frequency conditional handover is commanded and the field sameSFN-Indication and mib-RepetitionStatus [2] are included in the handover command then the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + 20 ms

When intra-frequency contional handover or inter-frequency conditional handover is commanded and the field sameSFN-Indication or mib-RepetitionStatus [2] is not included in the handover command then the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TMIB + TIU + 20 ms

Where:

-	TIU is the time required to complete the transmission of PRACH in the target cell. The actual value of TIU shall depend upon the uncertainty in acquiring the first available PRACH occasion based on the PRACH configuration used in the target cell and the PRACH coverage enhancement level used by the UE for sending the random access preamble to the target cell.

-	TMIB is the time required for acquiring the MIB and SIB information of the target cell.

-	Tsearch is the time required to search the target cell when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = 80 ms. Otherwise, Tsearch shall be according to the non-DRX cell identification requirements specified in Clause 8.13A.3.1 with KSatellite_intra = 1 for intra-frequency handover for a UE configured with CEModeB or Tsearch shall be according to the non-DRX cell identification requirements specified in Clause 8.13A.3.2  with Ksatellite_inter_i  =1 and Nfreq = 1 for inter-frequency handover for a UE configured with CEModeB. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

-	In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement for a time duration equal or longer than the time duration required for the cell identification. Otherwise, it is unknown. For intra-frequency handover the time duration required for the cell identification is specified in relevant intra-frequency cell identification requirements with KSatellite_intra = 1 as described in Clause 8.13A.3.1 for CEModeB. For inter-frequency handover the time duration required for the cell identification is specified in relevant inter-frequency cell identification requirements as described in Clause 8.13A.3.2  with Ksatellite_inter_i =1 and Nfreq = 1 for CEModeB.

<End of change#2>