3GPP TSG-RAN WG4 Meeting #110	R4-2400887

Athens, Greece, Feb 26 – Mar 1, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	On MultiRx Demodulation performance and CSI requirements - Simulation Results

Agenda item:	8.3.4.2

Document for:	Discussion

Introduction

During RAN4#109 [1] discussions the PDSCH demodulation configurations for the simulations and the test cases for most of the mDCI and sDCI related scenarios were agreed. Agreements related to the channel model and some of the parameter values were reached in the RAN4#107 [2], RAN4#108 [3] and RAN4#108bis [4] discussions.

This paper contains Nokia’s updated simulation results.

Simulation Configuration

Based on the agreements in RAN4#109 [1], the following applies for simulation setup used by Nokia:

Table 1: The simulation parameter setup used by Nokia.

|  |  | Notes |
| --- | --- | --- |
| sDCI Separate Processing | MIMO detector processing 2Rx and li layers per TRP |  |
| mDCI Separate Processing | MIMO detector processing 2Rx and li layers per TRP |  |
| Receiver (separate and joint) | MMSE-IRC |  |
| Channel model | TDLA30-75 + correlation/cross-talk model |  |
| Carrier frequency | FR2-1, 30 GHz |  |
| CBW/SCS | 100MHz/120kHz |  |
| Antenna configuration | 2x2, XPL per TRP |  |
| DMRS symbol allocation | (2,11) symbol indexes |  |
| Cross-correlation coefficient (Gamma) | 0.0625 |  |
| PDSCH resource allocation | 66 per TRP & 33 per TRP | 33 per TRP for non-overlapping 66 per TRP for fully-overlapping |
| MCS index (MCS table 1) | 64QAM, 0.43 (MCS17) & 16QAM, 0.48 (MCS13) | MCS13 for for non-overlapping MCS17 for for fully-overlapping |
| Time offset / Frequency offset | (-0.0625us, 600Hz) & (0.25us, 0Hz) | (0.25us, 0Hz) for non-overlapping (-0.0625us, 600Hz) for fully-overlapping |
| Error Vector Magnitude (EVM) | 0% |  |

Simulation Results

mDCI non-overlapping

Agreed simulation configuration (RAN4#109)

After RAN4#109 [1] and RAN4#108bis [4] discussions following simulation parameters were agreed for the mDCI non-overlapping configuration.

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | 2+2: {1000,1001} | 2+2: {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | non-overlapping |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Simulation results for rank 2+2 non-overlapping with MCS13 – Separate processing.

Simulation results for the agreed simulation configuration of multi TRP scheme with (2+2) layer combination and a crosstalk interference (rho) of -12dB using MCS13 is stated below.

Table 2: Results for mDCI non-overlapping rank 2+2 with MCS13 – Separate processing.

| Simulation Configuration | Bandwidth (MHz) / Subcarrier spacing (kHz) | TDD UL-DL pattern | MCS | Rank | Channel Model | Antenna Configuration (2Tx 2Rx) (α=β=0, γ=0.0625) | RB Allocation Size | Cross talk factor (rho) - dB | Fraction of maximum throughput (%) | SNRBB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mDCI non-overlapping Rank 2+2 (0.25us, 0Hz) | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 33/33 | -12 | 70 | 11.2 |

mDCI Fully-overlapping

Agreed simulation configuration (RAN4#109)

After RAN4#109 [1] and RAN4#108bis [4] discussions following simulation parameters were agreed for the mDCI fully-overlapping configuration.

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | 1+1: {1000} | 1+1: {1002} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Fully-overlapping |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Simulation results for rank 1+1 fully-overlapping with MCS17 – Separate processing.

Simulation results for the agreed simulation configuration of multi TRP scheme with (1+1) layer combination and a crosstalk interference (rho) of -12dB using MCS17.

Table 3: Results for mDCI fully-overlapping rank 1+1 with MCS17 – Separate processing.

| Simulation Configuration | Bandwidth (MHz) / Subcarrier spacing (kHz) | TDD UL-DL pattern | MCS | Rank | Channel Model | RB Allocation Size | Cross talk factor (rho) - dB | Fraction of maximum throughput (%) | SNRBB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mDCI fully-overlapping Rank 1+1 (-0.0625us, 600Hz) | 100/120 | FR2.120-1 | MCS17 | 1+1 | TDLA30-75 | 66 | -12 | 70 | 10.6 |

sDCI SDM

Agreed simulation configuration (RAN4#109)

After RAN4#109 [1] and RAN4#108 [3] discussions following simulation parameters were agreed for the sDCI SDM configuration.

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 |  |
|  |  | CORESETPoolIndex |  | 0 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | 1000 for 1+1 | 1002 for 1+1 |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full-overlapping |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) |  |  |  |  |  |

Simulation results for rank 1+1 with MCS17 – Separate processing.

Simulation results for the agreed simulation configuration of multi TRP scheme with 2Tx2Rx antenna configuration and (1+1) layer combination with a crosstalk interference (rho) of -12dB using MCS17.

Table 4: Results for sDCI SDM rank 1+1 with MCS17 – Separate processing.

| Simulation Configuration | Bandwidth (MHz) / Subcarrier spacing (kHz) | TDD UL-DL pattern | MCS | Rank | Channel Model | RB Allocation Size | Cross talk factor (rho) - dB | Fraction of maximum throughput (%) | SNRBB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sDCI SDM Rank 1+1 (-0.0625us, 600Hz) | 100/120 | FR2.120-1 | MCS17 | 1+1 | TDLA30-75 | 66 | -12 | 70 | 11.0 |

Conclusion

Within this contribution we have presented our simulation results for the performance alignment of PDSCH requirements of FR2 multi-RX downlink demodulation requirements.

- References

- R4-2321140 - WF on FR2_Multi-Rx_Demod (RAN4#109)

- R4-2309824 - WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception (RAN4#107)

- R4-2313977 - WF for Rel-18 Multi-Rx performance requirements (RAN4#108)

- R4-2316963 - WF on NR_FR2_multiRX_DL_Demod (RAN4#108bis)