3GPP TSG-RAN WG4 Meeting # 110 	R4-2400897

# Athens, Greece, Febuary 26 – March 01, 2024

Source:		Verizon, AT&T, Samsung, Ericsson

Title:			TP for TR 38.880: PC2 and PC1.5 3Tx inter-band CA_n66-n77

Agenda item:		7.25.2

Document for:		Discussion/Approval

# Introduction

This is a TP to TR 38.880 to introduce both PC2 and PC1.5 inter-band CA_n66_n77 with 3Tx requirements.

# Reference

[1]  RP-232941, WID revision for Rel-18 basket WI 3Tx NR inter-band UL Carrier Aggregation (CA) and EN-DC

[2] TS 38.101-1 V18.4.0

# Text proposal

====================== <Start of Text Proposal> ======================

### 5.x 	CA_n66A-n77A

### 5.x.1	Operating bands for CA

Table 5.x.1-1:  CA band combination of band n66+n77

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n66-n77 | n66, n77 |

### 5.x.2	Maximum output power for inter-band CA

Table 5.x.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n66A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. NOTE 4: The UE supports PC2 with UL MIMO in either one of the TDD bands and PC2 in the other TDD band. |  |  |  |  |  |  |

Table 5.x.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n66A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.x.3	MSD scenario studies

Table 5.x.3-1 and 5.x.3-2 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_n66-n77.

Table 5.x.3-1: MSD scenarios for CA_n66-n77 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | N/A | N/A | N/A |
| Cross band leakage | n77 UL | n66 DL | Yes |
| IMD | IMD2 (1*f_n77-1*f_n66) | n66 DL | Yes |
|  | IMD5 (2*f_n77-3*f_n66) | n66 DL | Yes |

Table 5.x.3-2: MSD scenarios for CA_n66-n77 with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | N/A | N/A | N/A |
| Cross band leakage | n77 UL | n66 DL | Yes |
| IMD | IMD2 (1*f_n77-1*f_n66) | n66 DL | No |
|  | IMD5 (2*f_n77-3*f_n66) | n66 DL | No |

### 5.x.4	REFSENS requirements

#### 5.x.4.1	REFSENS requirements for total power class 2

In previous discussion, RAN4 agreed that the following rules will be as the baseline to define MSD requirement for PC2 inter-band CA or EN-DC:

- Same MSD requirements can be applied for the band combination with either 1Tx or 2Tx in TDD aggressor band UL with same power class for harmonic mixing and cross band isolation, and

- the requirements specified based on 1Tx-1Tx UL configuration are applicable for 1Tx-2Tx UL configuration

Based on the agreement, the PC2 MSD results are referred in both Table 5.x.4.1-1 and Table 5.x.4.2-1. Mainly, they are the same as the PC2 2Tx configuration for this 3Tx requirement to the RAN4 specifications [2].

According to the RAN4 specifications [2], both IMD2 and IMD5 are the subjects to n66DL in CA_n66-n77. The value of PC2 IMD2 and IMD5 are the summarized in Table 5.x.4.1-1.

Table 5.x.4.1-1: Derived MSD values for PC2 UL configurations

| CA Combination | Power Class | NR Band | PUL (dBm) | IMD Source | IMD Order | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n66-n77 | 3 | n66 | 20 | IMD2 | 1 | 31 |
|  |  | n77 | 20 |  | 1 |  |
|  | 2 | n66 | 23 |  | 1 | 34.3 |
|  |  | n77 | 23 |  | 1 |  |
|  | 3 | n66 | 20 | IMD5 | 3 | 5.0 |
|  |  | n77 | 20 |  | 2 |  |
|  | 2 | n66 | 23 |  | 3 | 11.3 |
|  |  | n77 | 23 |  | 2 |  |

For the proposed MSD requirements, the required values are summarized in the Table 5.x.4.1-2, and the MSD value are based on discussed above.

Table 5.x.4.1-2: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 34.3 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 11.3 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |

For CA_n66-n77 band combination, RAN4 has concluded that the harmonic mixing is not a subject to this combo. Thus, the same conclusion is held in this TP.

For the cross-band isolation to this PC2 UL CA, the same requirement from Table 7.3A.6-1a of 38.101-1 is maintained in this proposal, and it is summarized in the Table 5.x.4.1-3

Table 5.x.4.1-3 (same as Table 7.3A.6-1a of 38.101-1): Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n77 | n66 | 3350 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | 1.0 | >ACLR2 |

#### 5.x.4.2	REFSENS requirements for total power class 1.5

According to the RAN4 specification [2], both IMD2 and IMD5 are the subjects to n66DL in CA_n66-n77. For IMD2, the IMD composition is 1st order in n66 UL and 1st order in n77 UL. And, the IMD composition is 3rd order in n66 UL and 2nd order in n77 UL for IMD5.

Table 5.x.4.2-1 summarizes the MSD analysis results where the MSD for PC3 and PC2 UL configurations are referenced to the RAN4 specification [2]. And, the MSD for PC1.5 UL configuration is derived based on linear extrapolation.

Table 5.x.4.2-1: Derived MSD values for PC1.5 UL configurations

| CA Combination | Power Class | NR Band | PUL (dBm) | IMD Source | IMD Order | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n66-n77 | 3 | n66 | 20 | IMD2 | 1 | 31 |
|  |  | n77 | 20 |  | 1 |  |
|  | 2 | n66 | 23 |  | 1 | 34.3 |
|  |  | n77 | 23 |  | 1 |  |
|  | 1.5 | n66 | 23 |  | 1 | 40.0 |
|  |  | n77 | 27.8 |  | 1 |  |
|  | 3 | n66 | 20 | IMD5 | 3 | 5.0 |
|  |  | n77 | 20 |  | 2 |  |
|  | 2 | n66 | 23 |  | 3 | 11.3 |
|  |  | n77 | 23 |  | 2 |  |
|  | 1.5 | n66 | 23 |  | 3 | 26.0 |
|  |  | n77 | 27.8 |  | 2 |  |

For the proposed MSD requirement, the required values are summarized in Table 5.x.4.2-2, and the MSD value are based on discussed above.

Table 5.x.4.2-2: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 40.0 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 26.0 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |

For CA_n66-n77 band combination, RAN4 has concluded that the harmonic mixing is not a subject to this combo. Thus, the same conclusion is held in this TP.

For the cross-band isolation to this PC1.5 UL CA, the same requirement from Table 7.3A.6-1b of 38.101-1 is maintained in this proposal, and it is summarized in Table 5.x.4.2-3.

Table 5.x.4.2-3 (same as Table 7.3A.6-1b of 38.101-1): Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR single UL band for DL NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n77 | n66 | 3350 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | 1.8 | >ACLR2 |

====================== <End of Text Proposal> ======================