# 3GPP TSG-RAN WG4 Meeting #110	R4-2400996

# Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	8.4.4

Source:	OPPO

Title:	On RRM test case design of PSCell (de)activation for FR1+FR1 NR-DC

Document for:	Discussion

- Introduction

As agreed in the WF [1], the TC list and CR slitting are approved as below:

| TC# | The RRM requirement for test case | Detailed Scope | Volunteer Company |
| --- | --- | --- | --- |
| TC1 | HO with PSCell from FR1-FR1 NR-DC to FR1-FR1 NR-DC | FFS: Only consider one of parallel processing and sequential processing for test cases of HO with PSCell to reduce the number of test cases. FFS: for TC2, considering the testability issue discussed in maintenance part, this test case may not be tested | Qualcomm |
| TC2 | HO with PSCell from FR1-FR1 NR-DC to FR1-FR2 NR-DC |  | Huawei |
| TC3 | HO with PSCell from FR1-FR2 NR-DC to FR1-FR1 NR-DC |  | OPPO |
| TC5 | PSCell addition and release delay of unknown FR1 PSCell |  | Nokia |
| TC6 | PSCell activation and deactivation for FR1+FR1 NR-DC | Define both RACH-based and RACH-less based PSCell activation/deactivation in one TC | Ericsson |
| [TC7] | Conditional PSCell addition and release delay of NR PSCell in FR1 | Conditional PSCell addition and release delay of NR PSCell in FR1(without SSB index measurement) | Vivo |

According to the Work Plan, we will discuss the test case design and prepare the draft CR[2] in this meeting.

3GPP RAN4 #110 meeting (Feb, 2024) - Discuss and agree on configurations/parameters for test cases [RAN4] - Initial discussion on draft test cases [RAN4] - Draft CR for test cases are expected

- Discussion on test case design

#### Issue 1-2: Tsearch requirement for RACH-less PSCell activation Way Forward: - For performance part in R18 - Option 1: Further discuss the test case to verify the procedure to RACH-based and RACH-less SCG activation in RRM performance part. - Option 1a: Instead of changing the core requirement, verify UE behavior in certain test environment to avoid UE to fallback to RACH based activation. - Option 1b: Update the current FR1-FR2 NR-DC SCG activation test case A.7.5.15 to guarantee the UE performance of RACH-less PSCell activation. - Others solutions are not precluded.

Companies proposed to discuss how to verify the procedure for RACH-based and RACH-less SCG activation in RRM performance part, e.g., both RACH-based and RACH-less based PSCell activation/deactivation in one TC, e.g.,

- 1st step: RACH-based unknown PScell activation

- 2nd step: deactivation of PScell,

- 3rd step: RACH-less based known PScell

We are open to design a test to cover both RACH-based and RACH-less based PSCell activation/deactivation in to verify the related core requirements.

Proposal 1: Agree to introduce one TC to cover both RACH-based and RACH-less based PSCell activation/deactivation.

- Conclusion

In this contribution, we discuss the work plan for test case design as well as the work splitting.

Proposal 1: Agree to introduce one TC to cover both RACH-based and RACH-less based PSCell activation/deactivation.

References

[1] R4-2321586, WF on NR FeRRM (part2), OPPO, RAN4 #109

[2] R4-24xxxx, DraftCR on TC for HO with PSCell from FR1-FR2 NR-DC to FR1-FR1 NR-DC, OPPO