# 3GPP TSG-RAN WG4 Meeting # 110	R4-2401016

# Athens, Greece, 26th Feb. – 1st March, 2024

Agenda item:			8.18.6.1

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for NTN above 10 GHz bands and other enhancement

Document for:	Discussion

# Introduction

In this paper, our views are provided for RRM requirement for NTN above 10GHz.

- UL timing accuracy

### Issue 1-6: Te_NTN for 60kHz and 120kHz

Agreement:

- UL 60kHz SCS:

  - 13 Ts for all cases with 120kHz/240kHz SSB

- UL 120kHz SCS:

  - Case 1 and case 2: 7.5 Ts

    - FFS for the applicable side condition on case 2

  - Case 3: Higher than 7.5 Ts, FFS for the exact value

The RAN4 framework for UL timing requirement are discussed [1] and the Te_NTN has been agreed as follows [2] for  Case 1: fixed VSAT in GEO, Case 2: fixed VSAT in LEO, and Case 3: mobile VSAT in GEO, respectively.

Based on the agreed Te_NTN, the timing budget break-down is provided as below:

  - X = UE position estimation error in meters

  - Y = serving-satellite position estimation error in meters

| Freq. Range | SCS of SSB signals [kHz] | SCS of uplink signals [kHz] | 0.5 CP length [Ts = 64*Tc] | Tr | Ta | Tf | Te_NTN |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Td | Tp+Tg | Max X+Y X: UE pos., Y: Sat. Pos. [m] | Te_NTN |
| 2-1 | 120 | 60 | 18 | 2 | 2 | 0.48 (16ns) | 0.54 | 12.46 | 60m | 13 |
|  |  | 120 | 9 | 1 | 0.5 | 0.48 (16ns) | 0.54 | 6.96 | 34m | 7.5 |
|  | 240 | 60 | 18 | 2 | 2 | 0.48 (16ns) | 0.27 | 12.73 | 62m | 13 |
|  |  | 120 | 9 | 1 | 0.5 | 0.48 (16ns) | 0.27 | 7.23 | 35m | 7.5 |

It has been noted that allowing a relaxation of 1*Ts in the Te_NTN results in an estimation error tolerance of 4.9 meters. In the scenario of case 3 with UL 120kHz SCS, which involves a mobile UE, with an expected X+Y positioning error range of 45 to 55 meters, the corresponding Te_NTN is between 9.5 and 11.5 Ts.

Observation 1: For UL 120kHz SCS, if X+Y = 45~55 m for mobile UE, Te_NTN = 9.5 ~ 11.5 Ts.

An alternative method would involve using the PRACH for establishing initial timing, followed by subsequent adjustments to the uplink (UL) timing. This method would mitigate the limitation caused by the CP of UL signals. With respect to the PRACH timing requirements in the Ka band, as the PRACH CP depends on the preamble format as below, the requirement can be defined based on X+Y = 80 m (as in NTN FR1) and Tg = 3 (as in TN FR2) as a starting point, and RAN4 can further check the applicability of preamble format. It will give Te_NTN = Td + Tp + Tg = 20Ts (for 120 kHz SSB SCS) and 19.7 Ts (for 240 kHz SSB SCS), respectively.

Proposal 1: RAN4 to further study the timing requirement of PRACH. X+Y = 80 m and Tg = 3 Ts can be a starting point.

# Summary

Observation 1: For UL 120kHz SCS, if X+Y = 45~55 m for mobile UE, Te_NTN = 9.5 ~ 11.5 Ts.

Proposal 1: RAN4 to further study the timing requirement of PRACH. X+Y = 80 m and Tg = 3 Ts can be a starting point.

# Reference

- R4-2317374, WF on RRM WF and CR work split for NR_NTN_enh, RAN4#108bis, Oct. 2023.

- R4-2321362, WF on NTN RRM requirements, RAN4#109, Nov. 2023.