3GPP TSG-RAN WG4 Meeting #110	R4-2401082

Athens, GR, Feb.-Mar. 2024

Agenda item:			8.3.5

Source:	Moderator (Qualcomm)

Title:	Topic summary for [110][123] FR2_multiRx_UERF_part1

Document for:	Information

# Introduction

Agenda items : 8.3.1, 8.3.1.x.

Proposals pertaining to the UE RF requirement are treated in topic #1. Proposals pertaining to TR38.751 are treated in topic #2.

# Topic #1: Edits to the UE RF requirement

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2400427 | Apple | CR on FR2 multi-RX |
| R4-2400962 | Huawei, HiSilicon | Draft CR for TS 38.101-2: MultiRx PC3 RF requirement applicable frequency range |
| R4-2401204 | Xiaomi | CR for Rel-18 38.101-2 to change the suffix K to M for simultaneous reception or transmission in multiple directions. |
| R4-2401509 | vivo | Observation 1: The current multi-Rx requirements is applicable for both 28GHz and 39GHz bands. Observation 2: The performance of n262 lacks sufficient evaluation so far. Proposal: It is suggested to preclude n262 from the current spec until sufficient evaluations are presented. |
| R4-2401510 | vivo | draft CR to 38.101-2 on initial UE orientation of FR2-1 multiRx |
| R4-2401511 | vivo | draft editorial correction on FR2-1 multiRx |
| R4-2402252 | Samsung | Observation 1: It was agreed to define complete RF requirements for 28GHz bands first, and 39GHz and 47GHz bands can be evaluated if time allows. Observation 2:there was already baseline calibration method before RAN4 agreed to evaluate 28GHz firstly Observation 3: simulation results comparison between 39GHz and 28GHz shows that the performance gap is not very large but still not negligible even there is calibration process. Observation 4: Multi-RX DL 2AoA spherical coverage requirements for PC3 were agreed to be derived based on the antenna pattern from electromagnetic simulation. Proposal 1: RAN4 to complete the PC3 Multi-RX DL 2AoA spherical coverage requirement for 28GHz & 39GHz in Rel-18 stage including maintenance phase, and postpone the requirement applicability at 47GHz in future release when EM simulation is available. |
| R4-2402253 | Samsung | Clarification on operating bands for simultaneous reception or transmission in multiple directions |

## Open issues summary

(continued…)

### Applicable bands for the UE RF requirement

Background: An early agreement was to focus on 28 GHz, but a later agreement adopted a calibration procedure that applied a band specific DL power for test. The earlier agreement was not explicitly revisited after adoption of the new calibration procedure.

Options:

- Remove n259, n260, n262

- Remove n262

- Remove n262, also re-evaluate requierments for n259, n260

- Remove no bands, capture note in TR38.751 to leave door open to revisit requirements for one or more high frequency bands

- No band applicablity change

Discussion:

### Void section 5.5 K  (R4-2400427)?

Clause 5.5K references sub-clause 5.5, which in turn, only contains CA configurations. On the other hand, this feature is only defined for single CC operation in Rel-18.

Options: Agree/disagree

Discussion:

### Suffix change to M?

Options: Agree/disagree

Discussion:

### Annex L, include ’In the test, n is set to 2’?

‘n’ is the maximum number of unique AoApairs that the TE can generate with one source at coordinates (1,1).

Options:

- Yes, n = 2 should be included in the Annex

- No, most points have n = 2, but the poles can have more than 2 AoA pairs associated with them

Discussion:

### Annex L, remove applicability to ’transmission’

Background: Wording was intended to be inclusive of the STxMP confiured power requirment

Options: Agree/disagree

Discussion:

### Annex L, add reference coordinate system illustration

Background: Wording was intended to be inclusive of the STxMP confiured power requirment

Options:

- Yes, add illustration

- No, illustration of positioner is not general enough

Discussion:

### Annex J, add explicit list of permitted UE alignments

Proposed new table of UE orientations is reproduced below:

Options:

- Yes, spec. needs this list documented in the Annex

- No, current format and wording are sufficient

Discussion:

### Remove redundant RMC information from PC6?

General section 7.3K introduced RMC info, so not necessary to retain for PC6. Proposal:

Options: Agree/disagree

Discussion:

# Topic #2: Edits to TR38.751

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2400428 | Apple | CR on reasoning of defining multi-RX RF requirements for all FR2-1 bands based on 28GHz simulation results |
| R4-2401512 | vivo | draft CR to 38.751 on update of UE orientation |

## Open issues summary

### Band applicability justification

Options: Agree/disagree

Discussion:

### Documentation of additional UE orientation to meet the requirement

Options: Agree/disagree

Discussion: